Use of Blue Lights

Background

The use of blue lights as an Aid to Navigation, under the current IALA Recommendations, is discouraged due to the poor quality colour definition and low transmissivity properties.  They are easily confused with green lights and are only suitable for recognition over short ranges.  However, the increase in the levels of background lighting, street lights and dock-side industries, etc., has served to make the recognition of navigation leading lights much more difficult to the user. The ability to identify the Aid to Navigation quickly among the clutter of other lights is very important, especially with high-speed craft.

Consideration

A sub group of Working Group One discussed the use of Blue lights as an Aid to Navigation  and reviewed the various input papers submitted by RNAV, AIS, EEP Committees and input papers submitted by the Maritime Safety Administration of China and the UK General Lighthouse Authorities Research and Development Department.

Conclusions

The following conclusions were reached:

- Blue Lights can have a role as Aids to Navigation.

- Blue Lights can be effective in built up areas in distinguishing Aids to Navigation from background lights.

- Blue Lights are presently only effective at short range (less than 2 nautical miles) as it is difficult to distinguish them from other colours at ranges greater than this.

- Blue Lights can be an additional light to supplement the current range of colours approved by IALA.

- Blue is more effective with another colour if used in alternative flash mode. Yellow is a particularly suitable complimentary colour.

- In some countries the use of flashing and fixed Blue Lights is strictly controlled by laws and conventions.

- There are no clear IALA guidelines on the marking of supplementary traffic lanes.  A variety of systems are in use including the SIGNI system within Holland and Germany. Blue Lights have proved effective in China in marking such lanes, as they are immediately distinguishable from the normal IALA approved lateral marks.

Recommendations

It is recommended that:

- Blue Lights are accepted as an Aid to Navigation by IALA but are used only where the existing IALA recommended colours would prove ineffective.

- EEP to agree a colour signal standard for Blue Lights.

- IALA  produce recommendations for the marking of supplementary traffic lanes. As a first step member countries to be invited to submit their own systems for analysis.

- Blue and Yellow alternate flashing lights may be considered only for emergency wreck marking.

.