Recommandation pour la signalisation des

AISM Association Internationale de Signalisation Maritime      IALA International Association of Marine Aids to Navigation and Lighthouse Authorities

champs d’éoliennes

Recommandation AISM O-117

Mai 2000

(traduction française en cours)

### DRAFT Revision

# Recommendation for the marking of

Offshore Wind Farms

## IALA Recommendation O-117

May 2000

Revised April 2004

20ter, rue Schnapper – 78100 Saint Germain en Laye – France

Telephone : +33 1 34 51 70 01  Telefax : +33 1 34 51 82 05

E-mail : aismiala@easynet.fr              Internet : http://www.iala-aism.org

DRAFT IALA Recommendation on the marking of offshore wind farms

THE COUNCIL

RECOGNISNG the increase in the number of areas with multiple wind generators (classed as wind farms) being established and the consequent increased danger posed to navigation,

RECOGNISNG ALSO that it is a matter for a National Authority to assess the navigational requirement and the risk involved and decide on how wind farms need to be marked,

RECOGNISNG FURTHER that marking of wind farms is intended to preserve the safety of navigation, the marine environment and to protect the wind generators themselves,

HAVING CONSIDERED the proposals by the IALA ANM Committee and taking into account the IALA recommendations on the marking of Offshore Structures

RECOMMENDS that

Offshore Wind Generators should be marked so as to be conspicuous by day and night, with consideration given to prevailing conditions of visibility and vessel traffic.

The Council recognises that there are already sufficient provisions contained within the IALA Maritime Buoyage System (MBS) for the marking of wind generators.

Wind generators & associated structures should be marked as a single unit or as a block as follows:

- The tower of every wind generator should be painted yellow all round from the level of Highest Astronomical Tide (HAT) to 15 metres or at least the height of the Aid to Navigation, if fitted, whichever is greater.

Consideration may be given to the use of additional retro reflective   material.

- Due to the increased danger posed by an isolated structure, it should be lighted as per the IALA Recommendation on the marking of Offshore Structures( 0-114) i.e. a white light flashing morse code "U".

- A Significant Peripheral Structure (SPS) is the “corner” or other significant point on the periphery of the wind farm.  Every individual SPS  should be marked by sufficient lights so as to be visible to the mariner from all relevant  directions in the horizontal plane. Such lights should  display the character of an IALA “special mark”, should be flashing yellow, with a range of not less than five nautical miles . As a minimum, lights on individual SPSs should be synchronised but Administrations may require all SPSs to be synchronised.

- In the case of a large or extended wind farm, the distance between SPSs should not normally exceed 3 nautical miles.

- Selected intermediate structures on the perifery of a wind farm other than the SPSs in c) and d) above, should be marked with flashing yellow lights which are visible to the mariner from all relevant directions in the horizontal plane. The flash character of  such lights should be distinctly different from those displayed on the SPSs with a range of not less than two (2) nautical miles.The lateral distance between such lit structures or the nearest SPS should not exceed two (2) nautical miles.

- Depending on the lateral separation of individual turbines, National Authorities may require each turbine within a wind farm to be lighted. Depending on the marking and lighting of the periferal structures, the additional marking of the individual structures within a wind farm should be considered as follows:

- Unlit.

- Unlit with retro-reflective areas.

- Downlighting floodlights on ladders and access platforms.

- Flashing yellow lights with a range of not less than 2 nautical miles.

- Lit/unlit identifying numbers or names for each unit.

- The Aids to Navigation on the structure of a wind generator should be mounted below the lowest point of the arc of the rotor blades. They should be exhibited at a height above the level of the Highest Astronomical Tide (HAT) of not less than 6 metres nor more than 15 metres.

- The Aids to Navigation described herein should have an availability of not less than 99.0 %. ( IALA Category 2)

- A wind farm may be additionally marked by Racon(s).

- Consideration should be given to the additional use of Radar Reflectors and Radar Target Enhancers.

- Consideration should  be given to the provision of AIS as an Aid to Navigation (IALA Recommendation A-126) on selected structures.

- Consideration should  be given to the provision of sound signals where appropriate,  given prevailing conditions of visibility, topography and vessel traffic. (Typical range not exceeding 2 nautical miles & Morse Code “U” every 30 sec.)

- An Electrical transformer station or a meteorological or wind measuring mast, if considered to be a composite part of the wind farm, should be included as part of the overall wind farm marking. If not considered to be within the wind farm block it should be marked as an offshore structure. (i.e. a white light flashing morse code "U").

- As far as practicable, Aeronautical obstruction lights fitted to the tops of wind turbines should not be visible below the horizontal plane of these lights. Aviation authorities should be consulted regarding the specification of such lights.

Advisory Notes for National Authorities & Developers:

      - IALA is aware that there is an increasing number of structures which may affect shipping. It is therefore recommended that Authorities should continuously monitor these Recommendations to ensure that any navigational problems caused by wind farms are solved in a satisfactory manner. Authorities facing problems in this field are invited to bring them to the attention of IALA to obtain advice on current practice. IALA foresees the need to re-examine these Recommendations from time to time, and if necessary to update them.

      - Consultation between Developers and Administrations [and Lighthouse Authorities / aids to navigation providers / Competent Authority…stakeholders] should take place at an early stage. In general, development of offshore energy structures or wind farms should not prejudice the safe use of Traffic Separation Schemes, Inshore Traffic Zones and the safe access to anchorages, harbours and places of refuge.

      - In order to avoid confusion from a proliferation of Aids to Navigation in a high density wind farm, full consideration should be given to the use of synchronised lighting, different light characters and varied light ranges.

      - During the construction of an offshore wind farm, working areas should be established and marked in accordance with the IALA Maritime Bouyage System (MBS). National Authorities should also consider the use of guardships in areas of high traffic density.

      - [Preliminary] Notices to Mariners and / or Radio Navigational Warnings [Notice to Airmen] must be promulgated in advance of and during any offshore wind farm construction.

      - Administrations should advise Hydrographic offices that mariners should be warned on the navigational chart by way of a legend or note that wind generators and seabed electric cables may cause propagation problems and electro-magnetic interference to onboard navigation equipment.

      - Power cables between wind generators, between wind generators and the transformer station, and between the transformer station and the shore should be sufficiently trenched.

      - On a case-by-case basis, National Authorities may consider establishing Exclusion or Safety Zones which would prohibit or restrict vessels from entering wind farms.  Such information should be shown on the navigation chart, as appropriate.

      - IALA trials so far indicate that wind turbines do not affect radar, radio navigation and radio communications. Administrations/developers may wish to carry out individual trials to verify this.

Refer to IALA Navguide, Chapter….

Refer to IALA Navguide Chapter….