AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

DRAFT

IALA Recommendation [X-###]

On

the marking of offshore wave and tidal energy sites

Edition #

[Date approved by Council]

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      		Internet -  http://iala-aism.org

Recommendation on XXX [ Title Here]

(Recommendation [leave number blank, to be filled by IALA Secretariat])

THE COUNCIL:

RECALLING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment;

RECOGNISING the significant hazard to shipping posed by the proliferation of surface and sub-surface energy extraction devices;

RECOGNISING ALSO that it is a matter for a National Authority to assess the navigational requirement and the risk involved and decide on how wave and tidal devices need to be marked;

RECOGNISING FURTHER that marking of wave and tidal energy devices is intended to promote the safety of navigation, preserve the marine environment and to protect the generating devices themselves;

NOTING the [as above];

NOTING ALSO that;

NOTING FURTHER that;

HAVING CONSIDERED the proposals by the IALA ANM Committee and taking into account the IALA recommendation on the marking of offshore wind farms,

ADOPTS the [name of document] in the annex of this recommendation; and,

RECOMMENDS that

Wave and Tidal Energy Extraction sites are marked so as to be conspicuous by day and night, with consideration given to prevailing conditions of visibility and vessel traffic.

The Council recognises that there are already sufficient provisions contained within the IALA Maritime Buoyage System (MBS) for the marking of wave and tidal energy extraction sites.

Wave and Tidal energy extraction sites should be marked as a single unit or as a block as follows:

a) Areas containing surface or sub-surface energy extraction devices (EED) should be marked by appropriate navigation buoys, fitted with such topmarks, lights, radar reflectors/racons or AIS transponders as the level of traffic requires.

- A Significant Peripheral Point (SPP) is the corner or other significant point on the periphery of the wave and tidal energy extraction site.  Every individual SPP should be marked by a lighted navigational buoy, so as to be visible to the Mariner from all directions in the horizontal plane. Lights should have a range of not less than 5  miles. The Northerly, Easterly, Southerly and Westerly SPPs should be marked with the appropriate IALA Cardinal lightbuoy.

- In the case of a large or extended energy extraction site the distance between SPPs should not normally exceed 2 nautical miles.

- Where Individual EEDs extend above the surface they should be painted yellow above the waterline. The device should be marked by sufficent flashing yellow lights so as to be visible to the mariner from all diractions in the horizontal plane.  The flash character of such lights should be sufficiently different from those displayed on the SPPs with a range of not less than 2 Miles.

- Consideration should be given to the additional use of Radar Reflectors and Radar Target Enhancers.

- Consideration should be given to the provision of AIS as an Aid to Navigation (IALA Recommendation A-126) on selected peripheral EEDs.

- Where individual EEDs are not visible above the surface but are believed to be a hazard to surface navigation, they should be marked by an IALA special mark, flashing yellow, with a range of not less than 2 miles.

- The Aids to Navigation described herein should have an availability of not less than 99.0%.

- The relevant Hydrographic Office should be informed of the establishment of an energy extraction site, to permit appropriate charting of the site.

- Notices to Mariners should be issued to publicise the establishment of an energy extraction site.

- Operators of energy extraction sites should develop contingency plans, which include the possibility of individual EEDs breaking loose and becoming floating hazards.

Advisory Notes

  - IALA is aware of the rapid progress being made internationally with regards to the development of wind and tidal energy extraction devices (EEDs).  Authorities should therefore continuously monitor these Recommendations to ensure navigational problems caused by wave and tidal energy sites are re assessed as necessary.

  - IALA is aware of the significant hazard to navigation posed by surface and subsurface devices.

  - Consultation between Developers and Administrations should take place at an early stage.  In general, development of wave and tidal energy sites should not prejudice the safe use of Traffic Separation Schemes, Inshore Traffic Zones and the safe access to anchorages, harbours and places of refuge.

  - During deployment and construction of offshore wave and tidal energy devices and associated seabed transmitter stations the working areas should be established and marked in accordance with the IALA Maritime Buoyage System (MBS).  National Authorities should also consider the use of guard ships in areas of high traffic density.

  - Notice to Mariners must be promulgated in advance and during any offshore wave or tidal energy site construction.

  - Power cables between wave and tidal energy extraction devices and the shore should be sufficiently trenched and marked on the navigational chart.

  - National Authorities may consider establishing Exclusion or safety Zones which would prohibit or restrict vessels from entering wave or tidal energy extraction sites  Such information should be shown on the navigation chart as appropriate.