| From: ANM (ANM5) | ANM5/output/03 |
| --- | --- |
| To: ANIS Ad Hoc Group | Oct. 15, 2004 |

Liaison Note

Request for Input by ANIS Ad Hoc Group

- Introduction

ANM5 reviewed the request from the ANIS Ad Hoc Group for information on specific sections of the Draft Recommendation on Aids to Navigation Information Service.

- Comments

The ANM Committee reviewed the draft guideline and provided some general comments as well as response to the questions identified by the ANIS Ad Hoc Group.

In general, there are four (4) main concerns:

1 – The number of AtoNs that are monitored is limited at present, cost factor to increase (members of the ANM Committee indicated between 1% and 40% (approximately) of their AtoN, including lighthouses, are presently monitored);

2 – Reliability of the monitoring system itself (experience has shown that this may be less than the reliability of the AtoN);

3 – Responsibility of the Authority – in many administrations, there are different responsible authorities (AtoN Authority / Hydrographic Authority / Maritime Safety Information );

4 – Monitoring may provide information on performance percentages that is not of relevance to the mariner, some means of filtering information for the mariner would be required.

The ANM Committee recognises that the work of the ANIS group is developing, and requests that they keep these points in mind as work progresses.

ANM5 provides the following comments on the document.

Aids to Navigation Information Service

Part 2

The WG recommends that only 3 levels of impact on safety are considered…High, Medium and Low.

In assessing High, Medium or Low, many subjective factors need to be taken into account.

The WG inquired as to why a definitive percentage of 90% of nominal range was utilised and questioned how this could be measured.  A subjective criteria might be 50% of the luminous intensity, equating to a “significant” reduction to an observer and resulting in a “high” category level of impact on Safety of Navigation.

With regard to racons, these are considered to be either on or off. Range reduction events are considered improbable.

Re DGNSS stations, when nominal range falls to 70% (half power output)  is a more realistic alarm benchmark.

Almost all the examples of malfunction/defect in Part 2 are considered “high” except top marks on buoys and fog signals.

Part 3

Level 1	The two examples categorised as Level 1 are “high”

Level 2	All the examples categorised as level 2 are “high” with the exception of total loss of monitoring signals. This item should perhaps not be included at all. It is information that the mariner probably does not require.

Level 3	In general, reduction to 50% of range of a lighted  AtoN is considered “high”.

All aspects of fog signal performance are considered “low”.

See comments re racons and DGNSS above. Failure of a DGNSS station where dual coverage exists is “low”, otherwise “medium” but this is subject to assessment in the particular case.

The ANM Committee recognises the efforts to provide “total information” on one display for the benefit of the mariner. However, when so many mariners complain of information overload, it is important that “extra” information is by way of “demand” ie a dialogue box on the display.

Comment: In Para 2, Description, it states that a “normal status” message will be broadcast every 30 minutes. The ANM Committee considers that in keeping with the theme of the preceding paragraph above, this is unnecessary information.

- Action Requested

The ANIS Ad Hoc Group is asked to take these comments into account in the development of the recommendation as appropriate.  The ANM Committee noted with interest the work of the ANIS Ad Hoc Group, and requests further updates with possible demonstration of the prototype display, at ANM6.