Use of Blue Lights

Background

The use of blue lights as an Aid to Navigation, under the current IALA Recommendations, has been discouraged due to the poor quality colour definition and the low transmissivity properties of filtered blue lights. They are easily confused with green lights and are only suitable for recognition over short ranges.

With the advent of LEDs,  filtering is not  a requirement and effective and efficient blue lights are now a reality. Colour confusion is reduced significantly.

The increase in levels of background lighting, street lights and dock-side industries, etc., has served to make the recognition of navigation lights much more difficult to the user. The ability to identify the Aid to Navigation quickly among the clutter of other lights is very important, especially with high-speed craft.

Consideration

The WG1 discussed the use of Blue lights as an Aid to Navigation  and reviewed the various input papers submitted by RNAV, AIS, EEP Committees and input papers submitted by the Maritime Safety Administration of China and the UK General Lighthouse Authorities Research and Development Department.

Conclusions

The following conclusions were reached:

- Blue Lights can have a role as Aids to Navigation.

- Filtered blue lights should not be considered for future applications.

- Blue Lights can be effective in built up areas in distinguishing Aids to Navigation from background lights.

Blue LEDS

- Blue LED Lights are now effective at ranges up to 5 miles and advances in technology  may well extend these ranges.

- Blue can be an additional light to supplement the current range of colours approved by IALA.

- A distinct set of characters should be considered for use with blue lights to reduce confusion. These characters should be confined to those with a flash duration not less than 1.0 sec. and/or fixed lights.

Blue/Yellow LEDs

- A Blue Light used in conjunction with its reciprocal colour (yellow) produces a conspicuous light and reduces the risk of colour confusion.

- An alternating flash character of 60 flashes per minute (fl b 1.0 sec, fl y 1.0 sec) and no quicker is considered most effective with a blue/yellow combination.

- Such a Blue/Yellow light should only be considered for Emergency Wreck Marking.

Restricted Use of Blue Lights

- In some countries the use of flashing and fixed Blue Lights is strictly controlled. Eg in the USA, use of blue lights is reserved solely for law enforcement agencies.

- There are no clear IALA guidelines on the marking of supplementary traffic lanes. Blue Lights have proved effective in China in marking such lanes, as they are immediately distinguishable from the normal IALA approved lateral marks.

Recommendations

ANM5 recommends that:

- Blue Lights be accepted as Aids to Navigation by IALA but use should be restricted to where the existing IALA recommended colours would prove ineffective.

- Only LED Blue lights should be considered for future applications.

- EEP should be tasked to specify a chromaticity region for Blue Lights.

- Blue/Yellow alternating light should be accepted solely for Emergency Wreck Marking.

- IALA should produce recommendations for the marking of supplementary traffic lanes. As a first step member countries to be invited to submit their own systems for analysis.

.