AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

DRAFT IALA Guideline XXXX

For the

Provision and Identification of aids to navigation in built-up areas

Edition 1

June 2005 (?)

20ter, rue Schnapper     -     78100     Saint Germain en Laye        -       France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05 Telex 695499 ialaism f

E-mail          :         iala-aism@wanadoo.fr      Internet      :      hjttp://iala-aism.org

ANM7 / output / 03

Approved in principle – electronic review process agreed

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

DRAFT IALA Guideline for the Provision and Identification of aids to navigation in built up areas.

# Introduction

The identification of Aids to Navigation (AtoN) is increasingly affected by the proliferation of urban developments in and around ports and harbours. The purpose of this guideline is to provide responsible authorities with techniques on how to deal with the identification of AtoN in built up areas.

# Scope

This information provides guidance for aids to navigation providers when assessing the level of AtoN effectiveness in built up areas and potential methods for improving the conspicuity of the AtoN.

# Definitions & Acronyms

Conspicuity- The descriptor used for how well something stands out from its surroundings.

Background Lighting- Encompasses ambient lighting either directly behind or adjacent to the AtoN.

Synchronised- A number of AtoN flashing together in character.

Sequenced- A number of AtoN flashing one after the other with the same character.

# Options for provisions of effective aids to navigation

Authorities should consider the actions described in the following flow chart and relate to the examples given in section 5. (For the more technical aspects of the options provided in the conspicuity section please refer to IALA Navguide)

Is identification of AtoN a problem?

General problems often associated with identification are:

- background lighting

- atmospheric pollution

- shadowing

- obstruction

- restricted visibility.

Can identification be improved?

Background lighting is a significant contributing problem to AtoN recognition. Reducing or eliminating background lighting should be the initial aim of the authority and every effort should be made to liaise with other authorities.

Similarly the approach mentioned above should be adopted to deal with atmospheric pollution, shadowing or obstruction.

Can the AtoN be relocated?

In addition to the mitigating measures identified above, authorities should not dismiss the option of relocating an AtoN to maintain its effectiveness.

(Any relocation should not in any way reduce the prime navigational objective of the AtoN)

Can the conspicuity of the AtoN be improved?

There are many ways to improve the conspicuity of an AtoN. The following measures should be considered:

- Alter the colour of the light and/or structure within the colour boundaries.

- Alter the rhythmic characteristics of the light.

- Alter the intensity of the light.

- Alter the light source of the AtoN.

- Alter the size/shape of the AtoN

- Provide an additional AtoN or AtoNs

- Sequence and/or synchronise AtoN lights

The above measures are not exhaustive and any one or combination can be used.

# Note - Care should be utilized with the introduction of blue lights.  Most people associate blue lights with law enforcement and emergency services.  Care should therefore be taken to ensure that the blue AtoN signal lights are not confused with these other lights.  Where flashing blue lights are to be used as aids to navigation, their rhythmic character should be restricted to ‘on’ periods of one second or greater.  A long presentation time would further reduce the risk of colour confusion.  [key point – character – blue light use fixed; only flashing blue for wreck marking buoy]

# Note – The number of blue lights should be kept to a minimum so as to keep them from becoming too common place.  If they become too common place the conspicuity of the emergency wreck marking will be significantly reduced.  Care should be utilized with the introduction of blue lights.  [key point – number of blue lights used]

# Examples

The problem:- AtoN light signal is at same height and colour as local street lighting.

Solution:- Liaise with local authorities and setup an exclusion of street lighting to seaward in a limited area.

Example 2

The problem:- Fixed Green filtered range lights were difficult to identify due to colour of background lighting.

Solution:- Change colour of range lights to fixed blue.

The problem:- Starboard channel light lost in background lighting.

Solution:- Additional AtoN and Sequence lights.

# Conclusions

Authorities can improve the effectiveness of their AtoN in built up areas by using the methods described in this guideline.