REPORT

on

the adoption of the LRIT System by IMO

Contents

1.	Introduction

2.	The LRIT System

3.	The new SOLAS Regulation V.19-1

4. 	Performance Standards and Functional Requirements for LRIT system

5.	Arrangements for the timely establishment of the LRIT system

Annexe

Amendment to the International Convention for the Safety of Life at Sea, 1974:

Chapter V, Safety of Navigation, Regulation 19-1 – Long-Range identification and tracking of ships

*  *  *

1.	Introduction

The Maritime Safety Committee, at its 81st session, 10 to 19 May 2006, adopted an amendment to SOLAS Chapter V introducing new obligations for ships regarding Long-Range Identification and Tracking (LRIT). The matter had been discussed since February 2002 as a part of the “Security Package” developed in the aftermath of the September 11 events. Undoubtedly, it was the most difficult and most sensitive element of the package to be adopted and it took a lot of effort to reach a consensus on the subject, mainly from the MSC and from the COMSAR Sub-Committee, including several sessions of dedicated working groups and correspondence groups.

IALA was present to most of the meetings and played an active and useful role in also organising two seminars in Victoria (Canada) in 2004 and in Kuala-Lumpur (Malaysia) in 2005.  Most of the ideas promoted during these seminars have been incorporated in the new amendment adopted by the Maritime Safety Committee. Inter alia, LRIT can be used not only for security purposes but also for safety and environmental reasons. It is closed to be a worldwide system since there is no limit for flag States and port States to track ships and the limit for coastal States has been fixed at 1000 nautical miles from coasts.

2.	The LRIT System

Whereas AIS is a broadcast system, LRIT data will be available only to the recipients who are entitled to receive such information and safeguards concerning the confidentiality of those data have been built into the regulatory provisions. Contracting Governments will bear all coasts of the system and they will not be authorized to impose any charges on ships in relation to the LRIT information they may seek to receive.

The Maritime Safety Committee considered that, at first, the purpose of the SOLAS amendment is to contribute to the enhancement of security and to aid search and rescue services, and agreed that the SOLAS Regulation shall enter into force as soon as possible and for this reason decided the 1st January 2008 as the date of the entry into force. This implies that new ships shall be fitted with a system to automatically transmit the information required – identity, position, date and time of the position provided – before the 31st December 2008, and for existing ships during the year 2009.

The Committee recognized that for the LRIT system to become operational it is still necessary to establish the International LRIT Data Centre and the International LRIT Data Exchange as well as to carry out tests and confirm the functioning of the system as described in the LRIT architecture. So the date of entry into force can appear a little optimistic but it is a way for the Committee to invite IMO Member States to act promptly.

3.	The new SOLAS Regulation V.19-1

The text of the new SOLAS amendment – Chapter V, new Regulation 19-1 – is annexed to the present report. It applies to passenger ships and to cargo ships of 300 gross tonnage and upwards on international voyages, and it includes high speed crafts and mobile offshore drilling units.

This new SOLAS regulation establishes a multilateral agreement for sharing LRIT information amongst SOLAS Contracting Governments, in order to meet the maritime security needs and other concerns of such Governments. It maintains the right of flag States to protect information about the ships entitled to fly their flag, where appropriate, while allowing coastal States access to information about ships navigating off their coasts. The SOLAS regulation on LRIT does not create or affirm any new rights of States over ships beyond those existing in international law, particularly, the United Nations Convention on the Law of the Sea (UNCLOS), nor does it alter or affect the rights, jurisdiction, duties and obligations of States in connection with UNCLOS.

The MSC also adopted MSC Resolution 210(81) on Performance Standards and Functional Requirements for LRIT and MSC Resolution 211(81) on Arrangements for the Timely Establishment of the LRIT System.

4. 	Performance Standards and Functional Requirements for LRIT system

The LRIT System is so described in Resolution 210(81):

“1 – The LRIT System provides for the global identification and tracking of ships.

2 – The LRIT System consists of the shipborne LRIT information transmitting equipment, the Communication Service Provider(s), the Application Service Provider(s), the LRIT Data Centre(s), including any related Vessel Monitoring System(s), the LRIT Data Distribution Plan and the International LRIT Data Exchange. Certain aspects of the performance of the LRIT system are reviewed or audited by an LRIT Co-ordinator acting on behalf of all Contracting Governments.

3 – LRIT information is provided to Contracting Governments and Search and Rescue services entitled to receive the information, upon request, through a system of National, Regional, Co-operative and International LRIT Data Centres, using, where necessary, the LRIT International Data Exchange.

4 – Each Administration should provide to the LRIT Data Centre it has selected, a list of the ships entitled to fly its flag, which are required to transmit LRIT information, together with other salient details and should update, without undue delay, such lists as and when changes occur. Ships should only transmit the LRIT information to the LRIT Data Centre selected by their Administration.

5 – The obligations of ships to transmit LRIT information and the rights and obligations of Contracting Governments and of Search and Rescue services to receive LRIT information are established in Regulation V/19-1 of the 1974 SOLAS Convention.”

The Shipborne LRIT Equipment should be capable of:

- automatically and without human intervention on board the ship transmitting the ship’s LRIT information at 6-hour intervals to an LRIT Data Centre,

- being configured remotely to transmit LRIT information at variable intervals,

- transmitting LRIT information following receipt of polling commands.

The Communication Service Providers (CSPs) provide services which link the various parts of the LRIT system using communications protocols in order to ensure the end-to-end secure transfer of the LRIT information

The Application Service Providers (ASPs) should:

- provide a communication protocol interface between the Communication Service Providers and the LRIT Data Centre to enable the remote monitoring of LRIT information transmission,

- ensure that LRIT information is collected, stored and routed in a reliable and secure manner.

ASP should be recognised by the Contracting Government(s) establishing the LRIT Data Centre or by the MSC. A CSP may also provide services as an ASP.

The International, National, Regional and Co-operative LRIT Data Centres should:

- establish and continuously maintain systems which ensure, at all times, that LRIT Data Users are only provided with the LRIT information they are entitled to receive,

- collect LRIT information from ships instructed by their Administrations to transmit the LRIT information to the centre,

- exchange LRIT information with other LRIT Data Centres through the International LRIT Data Exchange

- execute on request monitoring of LRIT information and archiving of this information.

National, Regional and Co-operative LRIT Data Centres are established by a Contracting Government or a group of Contracting Governments. Such Centres may also serve as Vessel Monitoring System (VMS) and may require, as VMS, the transmission from ships of additional information and at different intervals.

Ships, other than those which are required to transmit LRIT information to either a National, Regional or Co-operative LRIT Data Centre, should transmit the required LRIT information to the International LRIT Data Centre recognized by the MSC and established by Contracting Governments not participating in a National, Regional or Co-operative LRIT Data Centre.

The LRIT Data Distribution Plan should be established and maintained by IMO. It should include all information provided by Contracting Governments regarding their LRIT Data Centres, the list of their ports and port facilities as well as their wishes to receive LRIT information as flag, port and coastal States.

The International LRIT Data Exchange is established under the co-ordination of the MSC and is connected to all LRIT Data Centres. It should route LRIT information between LRIT Data Centres using information provided in the LRIT Data Distribution Plan.

The LRIT Co-ordinator should be appointed by the MSC to review the performance of the LRIT system and to audit its different components. The Co-ordinator should also assist the Committee in the establishment of the International LRIT Data Centre and the International LRIT Data Exchange.

5.	Arrangements for the timely establishment of the LRIT system

Resolution MSC 211(81) established a programme of decision that the MSC has to be taken at its two next sessions for the System to be able to work on time. Amongst other, the MSC has to establish the International LRIT Data Centre and the International LRIT Data Exchange and to designate the LRIT Co-ordinator, which could be the International Mobile Satellite Organization (IMSO). To that effect, the MSC requested Contracting Governments to submit their information necessary for the establishment of the LRIT Data Distribution Plan and the name of the LRIT Data Centres they have selected.

The Maritime Safety Committee also established an Ad Hoc Working Group on engineering aspects of LRIT, which should develop and submit to the 82nd session for approval:

- technical specifications for the International Data Exchange,

- technical specifications for the International LRIT Data Centre,

- technical specifications for communications within the LRIT System network (between the LRIT Data Centres and International LRIT Data Exchange and with the LRIT Data Distribution Plan),

- protocols for the development testing of the LRIT System and for testing the integration into the system of new LRIT Data Centres, and

- guidance which would assist the IMO in setting up and maintaining the LRIT Data Distribution Plan.

Several meetings of the Ad Hoc Working Group will be held before the next MSC meeting (29 November – 8 December 2006).

J.Ch. Leclair.

IALA Representative to IMO

20 June 2006.

Annexe

Amendment to the International Convention for the Safety of Life at Sea, 1974

CHAPTER V

SAFETY OF NAVIGATION

Regulation 19-1 – Long-Range identification and tracking of ships

1	Nothing in this regulation or the provisions performance standards and functional requirements adopted by the Organization in relation to the long-range identification and tracking of ships shall prejudice the rights, jurisdiction or obligations of States under international law, in particular, the legal regimes of the high seas, the exclusive economic zone, the contiguous zone, the territorial seas or the straits used for international navigation and archipelagic sea lanes.

2.1	Subject to the provisions of paragraph 4.1 and 4.2, this regulation shall apply to the following types of ships engaged on international voyages:

.1	passenger ships, including high-speed passenger craft;

.2	cargo ships, including high-speed craft, of 300 gross tonnage and upwards; and

.3	mobile offshore drilling units.

2.2	The term “ship”, when used in paragraph in paragraphs 3 to 11.1, includes the passenger and cargo ships, the high-speed craft and the mobile offshore drilling units which are subject to the provisions of this regulation.

3.	This Regulation establishes provisions to enable Contracting Governments to undertake the long-range identification and tracking of ships.

4.1	Ships shall be fitted with a system to automatically transmit the information specified in paragraph 5 as follows:

.1	ships constructed on or after 31 December 2008;

.2	ships constructed before 31 December 2008 and certified for operations:

.1	in sea areas A1 and A2, as defined in regulations IV/2.1.12 and IV/2.1.13; or

.2	in sea areas A1, A2 and A3, as defined in regulations IV/2.1.12, IV/2.1.13 and IV/2.1.14;

Not later than the first survey of the radio installation after 31 December 2008;

.3	ships constructed before 31 December 2008 and certified for operations

in sea areas A1, A2, A3 and A4, as defined in regulations IV/2.1.12, IV/2.1.13, IV/2.1.14 and IV/2.1.15, not later than the first survey of the radio installation after 1 July 2009. However, these ships shall comply with the provisions of subparagraph .2 above whilst they operate within sea areas A1, A2 and A3.

4.2	Ships, irrespective of the date of construction, fitted with an automatic identification system (AIS), as defined in regulation 19.2.4, and operated exclusively within sea area A1, as defined in regulation IV/2.1.12, shall not be required to comply with the provisions of this regulation.

5.	Subject to the provisions of paragraph 4.1, ships shall automatically transmit the following long-range identification and tracking information:

.1	the identity of the ship;

.2	the position of the ship (latitude and longitude); and

.3 	the date and time of the provision provided.

6	Systems and equipment used to meet the requirements of this regulation shall conform to performance standards and functional requirements not inferior to those adopted by the Organization. Any shipboard equipment shall be of a type approved by the Administration.

7	Systems and equipment used to meet the requirement of this regulation shall be capable of being switched off on board or capable of ceasing the distribution of long-range identification and tracking information:

.1	where international agreements, rules or standards provide for the protection of navigational information; or

.2	in exceptional circumstances and for the shortest duration possible where the operation is considered by the master to compromise the safety or security of the ship. In such aa case, the master shall inform the Administration without undue delay and make an entry in the record of navigational activities and incidents maintained in accordance with regulation 28 setting out the reasons for the decision and indicating the period during which the system or equipment was switched off.

8.1	Subject to the provisions of paragraph 8.1 to 11.1, Contracting Governments shall be able to receive long-range identification and tracking information about ships, for security and other purposes as agreed by the Organization, as follows:

.1	the Administration shall be entitled to receive such information about ships entitled to fly its flag irrespective of where such ships may be located;

.2	a Contracting Governments shall be entitled to receive such information about ships which have indicated their intention to enter a port facility, as defined in regulation XI-2/1.1.9, or a place under the jurisdiction of that Contracting Government, irrespective of where such ships may be located provided they are not located within the waters landward of the baselines, established in accordance with international law, of another Contracting Government; and

.3	a Contracting Government shall be entitled to receive such information about ships entitled to fly the flag of other Contracting Governments, not intending to enter a port facility or a place under the jurisdiction of that Contracting Government, navigating within a distance not exceeding 1000 nautical miles of its coast provided such ships are not located within the waters landward of the baselines, established in accordance with international law, of another Contracting Governments; and

.4	a Contracting Government shall not be entitled to receive, pursuant to sub-paragraph .3, such information about ship located within the territorial sea of the Contracting Government whose flag the ship is entitled to fly.

8.2	Contracting Governments shall specify and communicate to the Organization relevant details, taking into account the performance standards and functional requirements adopted by the Organization, to enable long-range identification and tracking information to be made available pursuant the provisions of paragraph 8.1. The Contracting Government concerned may, at any time thereafter, amend or withdraw such communication. The Organization shall inform all Contracting Governments upon receipt of such communication together with the particular thereof.

9.	Notwithstanding the provisions of paragraph 8.1.3, the Administration shall be entitled, in order to meet security or other concerns, at any time, to decide that long-range identification and tracking information about ships entitled to fly its flag shall not be provided pursuant to the provisions of paragraph 8.1.3 to Contracting Governments. The Administration concerned may, at any time thereafter, amend, suspend or annul such decisions.

9.1	The Administration concerned shall communicate, pursuant to paragraph 9, such decisions to the Organization. The Organization shall inform all Contracting Governments upon receipt of such communication together with the particulars thereof.

9.2	The rights, duties and obligations, under international law, of the ships whose Administration invoked the provisions of paragraph 9 shall not prejudiced as a result of such decisions.

10	Contracting Governments shall, at all times:

.1	recognize the importance of long-range identification and tracking information;

.2	recognize and respect the commercial confidentiality and sensitivity of any 		long-range and tracking information they may receive;

.3	protect the information they may receive from unauthorized access or disclosure; and

.4	use the information they may receive in a manner consistent with international law.

11	Contracting Governments shall bear all costs associated with any long-range identification and tracking information they request and receive. Notwithstanding the provisions of paragraph 11.1, Contracting Governments shall not impose any charges on ships in relation to the long-range identification and tracking information they may seek to receive.

11.1	Unless the national legislation of the Administration provides otherwise, ships entitled to fly its flag shall not incur any charges for transmitting long-range identification and tracking information in compliance with the provisions of this regulation.

12	Notwithstanding the provisions of paragraph 8.1, the search and rescue services of Contracting Governments shall be entitled to receive, free of charges, long-range identification and tracking information in relation to the search and rescue of persons in distress at sea.

13	Contracting Governments may report to the Organization any case where they consider that provisions of this regulation or of any other related requirements established by the Organization have not been or are not being observed or adhered to.

14.	The Maritime Safety Committee shall determine the criteria, procedures and arrangements for the establishment, review and audit of the provision of long-range identification and tracking information to Contracting Governments pursuant to the provisions of this regulation.