# Changes Made to Chromaticity Regions

## Colour Regions

The “general” colour regions recommended in 1977 have been replaced by “temporary” boundaries that will be valid for ten years after the publication of this recommendation. At that time, the boundaries will be further reviewed. Please note that each “temporary” boundary includes its appropriate recommended region.

## Changes Made to Colour Boundaries

### Red

The red boundary follows that of the 1977 “preferred” region for users with impaired colour vision. The boundaries towards purple and yellow follow those given in CIE S004/E-2001 but the boundary towards blue is closer to the spectral locus to ensure a more saturated colour reducing the risk of confusion with green for protan observers (approximately 2% of males). For protanopes lacking a red-sensitive retinal pigment, long-wavelength reds will not be seen. A red region cut-off at y = 0.29 is for such observers. A red “temporary” region has been provided to include a legacy of red filtered lights.

### Yellow

The yellow boundary follows that of the 1977 “preferred” region except that the boundary towards the red has been aligned with the D65 point at 6500K. The 1977 preferred region is small, close to the spectral locus and excludes the longer wavelengths of yellow known to improve low luminance recognition. Nevertheless, these boundaries were chosen in order to reduce potential confusion with white lights of low colour temperature, e.g. oil and gas, as well as filament lamps run below their design voltage to increase lamp-life. (N.B. In 1993, commenting on the draft of CIE Technical Report 107, the USCG was opposed to using yellow light, “The chromaticity of the white signal light is based on the use of relatively low-power incandescent lamps, and cannot be improved. This serves to stiffen the U.S. Coast Guard’s opposition to using yellow lights in all but the most restricted cases, and to insure that white and yellow signals do not share the same flash characteristics.”)

The proliferation of sodium street lighting has also had an impact on the conspicuity of yellow and yellowy-white lights, reducing their usefulness in some areas. Because it has a chromaticity close to the 1977 yellow general region, a background of high-pressure sodium lighting can effectively mask a foreground yellow signal. (e.g. A report on a viewing of an offshore wind farm stated, “When viewed from north of the wind farm whilst sailing past at a distance of 4M, the marine beacons were often indistinguishable against the background yellow streetlights on the coast.”)

A further reason for the tight boundaries of the 1977 preferred region was to help reduce the likelihood of yellow/green confusion for protan observers.

A yellow “temporary” region has been provided, extending the yellow boundary towards red, which should include many yellow light-emitting diode (LED) light sources.

### Green

The green boundary follows that of the 1977 “preferred” region except for the boundary towards the yellow, which has been aligned with that given in CIE S004/E-2001. “Yellow-greens” have been excluded in order to reduce the risk of confusion with red by persons with colour defective vision. The CIE green Class A region, as laid down in CIE S004/E-2001, shows the boundary extending further towards the blue than the IALA 1977 preferred green region. Although this has been identified as a useful area of signal green, there is the concern of increased confusion with blue and white in a five-colour system (CIE recommends a maximum of four colours in any one system).

A green “temporary” region has been provided, extending the green boundary towards the white, in order to include a legacy of green-filtered incandescent lights sources.

### White

The white boundary follows that of the 1977 “preferred” region. The white region extends to beyond 9,500ºK in order to include the majority of devices fitted with white phosphor-conversion LED’s (pcLED’s). Such devices are widely deployed by AtoN providers and many users prefer their colour, especially when viewed against a background of “yellowy” artificial lighting.

A white “temporary” region has been provided that extends the white boundary towards the yellow. The extent of this “temporary” boundary towards yellow has an x value corresponding to a colour correlated temperature (CCT) of 2,500ºK (X-value of .46) that will include the majority of incandescent filament lamps run at their designed voltage. (This temporary region is still under discussion. It transpires that there are still many AtoN providers using low wattage or underrun filament lamps that have a CCT of less than 2,500K.  If necessary, the IALA 1977 “general region could be used as a “temporary region but at the risk of confusion with yellow.)

### Blue

A blue region has been added. This region is a truncated version of that recommended in CIE S004/E-2001. The truncated region is designed to reduce the risk of colour confusion with white and possibly green. It will also exclude the chromaticity values of most filtered blue lights (filtered blue lights can appear as purple or with red halo when viewed at a distance under certain atmospheric conditions).

Another reason for truncation is to improve separation between blue lights and white lights of high colour temperature.

Please note that for reliable recognition of blue when illuminance at the eye is low (<1 μlux), CIE S004/E recommends truncation of the blue to below y ≤0.06. However, such a small region would be difficult to comply with.

# Colour Confusion

## Percentage of Colour Deficient Observers

| Type of Deficiency |  | % males | % females |
| --- | --- | --- | --- |
| Overall |  | ~8% | ~0.5% |
| Anomalous trichromasy | Protanomaly | 1% | 0.01% |
|  | Deutanomaly | 5% | 0.4% |
|  | Tritanomaly | rare | rare |
| Dichromasy | Protanopia | 1% | 0.01% |
|  | Deuteranopia | 1.5% | 0.01% |
|  | Tritanopia | 0.008% | 0.008% |