Annex 4

of

Input paper for IALA ANM 8, point 6 of the provisional agenda

Develop guidance on channel design and a minimum comprehensive mix of AtoN

| Navigation Channel width | Excellent A to N |  |  | Good A to N |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PIANC additional width | Navigation accuracy (IALA paper) 1) | a) Mix, b) buoys: 1: percent of stand. number 2: separation distance | PIANC additional width on each side 2) | Navigation accuracy (IALA paper) 1) | a) Mix, b) buoys: 1: percent of stand. number 2: separation distance |
| 100 | 0 | 10 | a) only buoys 1: 100 % 2: 300 m | 0,6 | 10,6 | a) only buoys 1: 94 % 2: 320 m |
| 200 | 0 | 20 | a) leading lights and buoys 1: 90 % 2: 660 m | 1,25 | 21,3 | a) leading lights and buoys 1: 86 % 2: 700 m |
| 400 | 0 | 40 | a) all included b) buoys 1: 90% 2: 1300 m | 2,5 | 42,5 | a) all included b) buoys 1: 84 % 2: 1430 m |

- based on the width of fairway – caution if this is much bigger than the width of the navigation channel !

- based on ships beam B, considered to be 1/8 of channel width