Our Reference:

IALA CONCERNS ON THE TRANSITION TO A COMMON CHART DATUM AND SURVEY STANDARDS

# References

IMO SN/Circ 213 Guidance on Chart Datums and the Accuracy of Positions on Charts.

IMO SN/Circ 255  Additional Guidance on Chart Datums and the Accuracy of Positions on Charts.

IALA  Aids to Navigation Guide (Navguide)

Dear Sir,

There are concerns amongst IALA members that there are significant problems and potential risks to mariners operating in areas where chart datum, other than WGS84, is used to reference navigational charts.

IALA recognises that the transition to the adoption of a single datum worldwide (WGS 84) is now well underway. However, there are still significant areas where this has yet to take place and as a consequence many different chart data are still in use.  It is also noted that whilst the vast majority of official electronic charts available worldwide are already referenced to the common WGS84 datum, a significant percentage of paper charts have yet to be converted to a common datum.  For example, there are countries where all available electronic navigation charts (ENCs) are referenced to WGS84 but only a few paper charts have been converted from local data.

Experience from IALA Members suggest that, with the advent of GNSS, ECDIS and Integrated Bridge Systems, mariners are deliberately navigating closer to hazards at night, in reduced visibility and under inclement weather and sea conditions.  As a result, members are finding that they often have to install new and additional Aids to Navigation to mitigate this risk.

Whilst it is accepted that in areas of heavy traffic density the seabed has been surveyed t the modern S57 standard, there is concern that in many parts of the world the quality of source data is poor and sometimes even relies on surveys undertaken during the 19th century.  As this data is then incorporated into modern ENCs it is not always easy to determine the quality of source data and indeed there may be a tendency for mariners to over rely on the presented electronic display without fully taking this into account.

IALA believes that significant investment is needed from IHO members on a worldwide basis to bring survey data up to modern standards in order to fully take advantage of the increased safety benefits realised with ECDIS and Integrated Bridge Systems.  Additionally, although there are now actions within IMO to mandate the use of ECDIS charts onboard SOLAS registered vessels there remain, and are likely to remain for the foreseeable future, a significant number of local and small craft users who will not invest in official electronic displays and instead continue to rely on paper charts and non-certificated electronic chart systems.

Anecdotal evidence suggests that some mariners do not always appreciate that data, other than WGS84, are still in use particularly as modern navigation systems including AIS invariably default or only operate using WGS84.  In addition in some places the conversion parameters to WGS84 have yet to be determined and IMO have recognised this problem by recently issuing SN.1/Circ 255 to bring mariners attention to this problem.  IALA echoes these concerns.

There are also concerns that the use of AIS when navigating in areas which have yet to make the transition to the WGS84 datum could cause significant problems and potential hazards as anti-collision and other safety related decisions could be made based on misinterpreted positions.  This would also have an affect on the planned roll out of AIS as an Aid to Navigation by IALA members.

Recognizing that IHO plays a leading role on the above mentioned issues, IALA remains committed to engaging in dialogue and assisting in anyway it can.  As always, IHO is invited to attend the next meeting of the IALA Aids to Navigation Management (ANM) Committee.  IALA, for its part, is willing to participate and assist at the IHO Conference in May 2007.

Sincerely