The design of aids to navigation systems

within the Canadian Coast Guard

Input Paper for ANM9, prepared by A. Châteauvert, Canadian Coast Guard

1. Aim

This input paper was prepared in relation to the task 1 of the ANM Committee looking to ‘develop a guidance (manual) on channel design and a minimum comprehensive mix of Aids to Navigation…’.

In submitting this paper, the Canadian Coast Guard (CCG) wishes to share its experience with the design and review of short-range aids to navigation systems and its guideline on channel design.

2. Analysis

In Canadian waters, every aid to navigation system provided by the Canadian Coast Guard is designed in accordance with national procedures. Launched in 1989, the Procedures manual “Design and Review of Marine Short-Range Aids to Navigation Systems” specifies the steps to follow in defining the extent (Provision of service), the quantity (System Design) and the quality (Reliability) of the aids to navigation service. This Procedures manual is annexed to this paper as:

- Annexe_A_CCG_1989_procedure_manual.pdf and,

- Annexe_B_CCG_1993_procedure_manual_supplement.pdf

The procedures for determining the requirements for short-range marine aids at any site are based upon four types of analysis:

- An analysis of users, traffic patterns, weather and possible navigational threats at the site (site analysis).

- An analysis of user needs for navigational markers and signals at the site (needs analysis) to help mariners cope with navigational threats.

- An analysis of outputs of various combinations of short-range aids that could be provided at the site (operational analysis) to identify feasible means of meeting user needs.

- A comparative analysis of cost of effective combinations of aids (cost-effectiveness analysis) to select between viable alternatives.

In brief, the steps to be followed in these procedures are:

- Specify the navigation site to be analyzed by identifying the user’s traffic patterns and plotting the track or tracks on the chart. Gather all user and site information such as size, type and number of vessels, type, size, output and characteristics of all established aids to navigation, weather and sea conditions, previous reviews, marine incidents, requests, etc.

- Identify and classify the size and number of users according to the three basic categories.

- Rate the threats at each site for each type of user, using guidelines and professional judgement.

- Identify any local means, other than short-range aids to navigation, that may meet any of the needs identified.

- For each of the threats in the waterway, identify the related navigation needs and the aid or aids which address these needs.

- Identify any threats that are not adequately reduced and the recommended aids or other means that could meet the navigational need.

- Identify any aids to navigation that are provided where the threats are not significant or service is redundant and recommend action to reduce or discontinue aids.

- Calculate the coverage and hazard marking requirements at any open water landfall situation, for each user group.

- Calculate the equipment output required to achieve the required visual, aural and radar coverage at the specified level of availability.  Identify where the requirements are not being met with established equipment and propose viable options.

- Calculate the size and output requirements for each aid to navigation established or proposed, in the confined section of the waterway, to meet the proposed availability for each user group. Identify where the proposed levels are not being met and propose viable options.

- Establish the comparative direct costs of each proposed alternative, to assist in determining the most cost-effective system of aids to navigation.

ANM Members should be informed that the application of the procedures manual is a very complex and precise process to perform. In fact, CCG Officers follow a course to learn the process and assess their knowledge. It should also be known that the procedures manual will be revised shortly following the new and changing requirements of today’s mariners based on the availability of new technology, both from a mariners and service provider point of view.

In order to facilitate the understanding of this complexe process, attached is an example of a design review (also known as a Level of Service Review) performed on St.Clair River. Please refer to Annexe_C_CCG_St_Clair_River_ Review.pdf .

Also attached to this input paper is a document called Annexe_D_CCG_Channel _Design_Guideline_Eng.pdf.  This document provides planners with a set of procedures to be used when determining waterway parameters required to provide efficient manoeuvrability with no less than minimum safety margins and allowances. Procedures are set forth for the determination of channel width, depth, side slope and curvature, as well as the alignment of channels.

The guidelines have been developed for waterways utilized primarily by large traffic, such as tankers, general cargo and bulk carriers, and are not meant to replace more extensive analyses for the final channel design. As with the application of any guidelines, good judgement, experience and common sense will be required in their application. The methods are based upon the operational requirements for ships, and the aim is to provide the conceptual requirements for safe and efficient navigation. The design procedure for each element of waterway geometry is provided in order to enable the planner to optimize the design.

3. Conclusion

I invite the ANM committee members to take the annexed documentation and the above information into account when drafting the guidelines related to task 1. Although, the submitted information may be large and cumbersome, I think that the Canadian Coast Guard had to share these with its international brotherhood for the good of navigation safety.

ANM9_input_paper_channel_design.doc