PROPOSAL FOR DEVELOP A GUIDELINE ON PERFORMANCE INDICATORS FOR SERVICE QUALITY

Juan F. Rebollo – Puertos del estado-Spain.

As document input for workshop on Quality in Madrid-2005 drafted a document that contained a certain number of quality indicators (characteristic). In the debate of the Workshop it was considered that he was better, at a first moment, to make a reference general to these indicators, but not to define his objective figures. This same has stayed in the new edition of the Guideline of year 2006.

There have already been commentaries in the sense that those indicators would have to be defined, at least with a basic character, facilitating objective figures,  that helped to the members and service providers to be able to have a reference of their level on quality.

My proposal is that the ANM Committee start a task to develop a basic definition of service quality indicators, based on the document input of workshop of Madrid and on Guideline 1052.

# INDICATORS

# BASIC INDICATORS

Among the key processes, special treatment shall be given to “Service Operation”, the most representative parameters of which are service AVAILABILITY and CONTINUITY, as the process whose output is directly received by the mariner.

AVAILABILITY of service refers to that percentage of time in which the service is rendered under the established conditions with respect to a time of reference. This always has to do with the availability of the service provider; i.e., the availability of the capacity to render the service required. The time of reference is established in accordance with the importance of the signal for certain marine operations. Objective availability as an indicator of service quality level assumes the values recommended by the IALA, but each country may establish more rigourous values.

The minimum admissible value for a navigation aid to be considered as such is 95% over a two-year period. If it cannot achieve the said level, the IALA recommendation is to abandon the aid and eliminate it.

Since the “objective availability” indicator is linked to the categorisation of the navigation aid, each Organization must draw up a table associating a category to each aid and assigning an objective availability indicator to each category. For example (merely by way of orientation):

- Lighthouse beacons, racoons and approach lights (Category I) = 99.8 %

- Lights on fixed buoys and channel markers (Category II) = 99.0 %

- Port lights and lights on other buoys (Category III) =  97.0 %

- Daytime markers and radar reflectors (Category III) = 97.0%

- DGP Systems = 99.8 % (measured over 30 days)

In order to achieve these service quality level values, aids with “unique” systems or redundant installations shall be deployed in such a way that the service is not interrupted in case one of the elements of the navigation aid’s equipment breaks down. Thus, it is important to make a prior estimate of availability using data on the reliability of the equipment or technical installation of the aid, through the expression:

A =

MTBF

Time of Reference

In which MTBF is the AtoN installation’s mean time between failures. This data is facilitated by the installation supplier (design value).

This method of estimation should be used to verify whether the installation is, at least, capable of complying with service level requirements. But measurement of real availability should be done continuously, recording incidents and the time required for service recovery to established levels.

The practical expression to be measured is:

A =

Time in Operation ( T on)

Time of Reference  (T tt)

A =

T on

T on  +   T off

In which the time of reference (total time) may be considered as the sum of time in correct operation (T on) and time not in correct operation ( T off). Although “T on” will depend exclusively on the quality of the equipment or installation (product quality), “T off” may be considered as composed of three addends that have to be recorded separately in order to draw up an analysis of causes and focus improvement actions in the correct direction:

## T off =  TD + TL + TR

TD is the time of detection of the incident. In order words, the interval that elapses between the onset of the incident and awareness of it on the part of the person responsible for the service.

Normally, unless systems of remote supervision are available, this time should be estimated as a mean value, constant for each specific installation. This implies that each entity responsible for the service must have a controlled and specific way of detecting incidents, either by itself or through communications by others.

TL is the time employed in the logistics to restore the service. It is the interval that elapses from the point in which the entity responsible for service continuity learns of the incident up to the point in which the technical and human means begin the work to restore the service, which may be in situ or through remote control.

The first action taken during this period is notifying the pertinent Agency so as to disseminate the necessary warnings to mariners. This time period includes all the actions necessary to restore the service, with emphasis on the importance of the conditions that depend on the availability of spare parts, manuals, diagnostic capacity for the causes of the incident, and the management of maintenance and the technical, human, and logistic resources for displacements.

TR is the time employed in the effective restoration of the service. It is the interval that the technicians/operators employ in restoring it. It is evident that training will be an element that will help improve this interval.

Knowing how the “stoppage” interval is broken down in each of these periods will make it possible to focus and design service management more effectively and efficiently in order to achieve and improve the service levels established for each aid.

As has already been said, these levels, independently of the reference values mentioned previously in general, must be established for each and every one of the aids or set of aids by each entity responsible for the service, in accordance with the risks and traffic existing in each area.

Another significant quality parameter, particularly in the new radioelectrical aids in which certain navigational operations depend on the availability of the pertinent signal for a certain time, is the so-called CONTINUITY of the service. It is defined as the probability that, when the signal involved is available, it will remain available for a certain time. The way to measure this is to assign probabilities to the intervals of service permanence: i.e., it is the histogram of the “T on” period statistics previously defined.

# OTHER MONITORING AND EVALUATION INDICATORS. QUALITY OF SERVICE CHARACTERISTICS.

All indicators must comply with the following requirements:

- Be representative of the process,

- Be objective,

- Be verifiable by third parties

- Be possible to control by the service provider itself

Only the rendering of those services having an objective measurement may be improved. On another hand, the indicator must be representative of the service and controllable on the part of the service provider, such that the service provider can act upon it, thereby improving the service.

Quality of Service Characteristics are those qualities that the Agency commits itself to fulfil. Fulfilment is also achieved through indicators, so characteristics have the same requirements as indicators. However, they must also imply an appreciable added value for the user.

Each characteristic may contain different aspects, which shall be called “details of the characteristic”.

QUALITY OF SERVICE CHARACTERISTICS

The different characteristics may be grouped under the following headings or areas:

A- Processing of beaconing applications and authorisations

B- Service start-up of marine navigation aids

C- Service operation

D- Personnel training

E- Inspection

F- Efficiency, continual improvement of the service and quality measurements

The characteristics or quality commitments established and the means that each service provider must use for control and management are defined. In all cases, the characteristics have been broken down into a series of attributes that flesh it out (details of each characteristic). As an initial premise, compliance with mandatory obligations applicable to the characteristics or commitments of service quality is a necessary, albeit insufficient, requirement in certification.

The basic characteristics of the navigation aids service must be considered as a basic, but not limiting, relationship.  The records, indicators or evidence necessary to demonstrate compliance have been identified for each characteristic. The records are reflected beside each characteristic.

## B.- SERVICE START-UP OF MARINE AIDS TO NAVIGATION

| CHARACTERISTIC COMMITMENT | DETAILS OF THE CHARACTERISTIC | RECORD, INDICATOR OR EVIDENCE |
| --- | --- | --- |
| B 1- Inventory | - The Organization shall have an updated inventory of all marine navigation aids under its direct management and in its area of inspection, their component elements, and their service status. | - Updated inventory and service status. |
| C 1- Maintenance Management | - The Organization has a Preventive Maintenance Plan, with actions for programmed revision that include periodic actions depending on the type of aid and technological characteristics, for: - Lighthouse and coastal beacons - Port beacons - Buoys - The Organization analyses and corrects breakdown causes and incidents in both infrastructure and equipment. | - Preventive Maintenance Plan. - Reports of maintenance actions. - Records of breakdowns, incidents and corrective actions. - Measurement of breakdown frequency. |
| C 2.- Interval for service restoration | - The Organization executes necessary actions with a service restoration interval from detection of breakdown not in excess of: - Lighthouse and land beacons: 72 hours - Radioelectrical aids: 72 hours - Lighthouse and island beacons, and beacons in buoys: One week, provided that climatic conditions allow. | - Interval for service restoration |
| C 3- Spare part logistics | - The Organization has defined logistics for the supply of spare parts based on the interval for service restoration and the frequency of breakdowns, permitting it to comply with its established intervals. | - Spare part acquisition plan. - Inventory of spare parts. |
| C 4- Availability and continuity of aids owned | - The Organization defines and achieves service availability and continuity, never falling below the following values (calculated over two-year periods): - Lighthouse beacons and lights deployed: 99.8% - Beacons: 99.0% - Bouys: between 97.0% and 99.9% - Radio aids: 99.6% - DGPS: 99.8% (over 30-day periods) As per the measurement system set forth in Annex 1 of this manual. - The Organization has the necessary means to report and substantiate the values achieved. - The Organization sends annual information to the IALA regarding the availability and continuity of the service rendered. | - Measurements of availability and continuity. - Annual Service Level Report. |
| C 5- Detection and communication of breakdowns and incidents in service status | - The Organization has a defined procedure for the detection of service breakdowns and incidents. - The Organization immediately notifies the proper agencies of service breakdowns and incidents arising before taking any corrective action. - The Organization notifies the proper agencies of service restoration. | - Procedure for the detection of breakdowns and incidents. - Record of notifications of breakdowns and incidents. - Record of notifications of service restoration. |
| D 1- Personnel training | - Every year, the Organization identifies its training needs and undertakes the action necessary to cover them. - The Organization evaluates training effectivity. - The IALA promotes and organizes technical training seminars. | - Yearly Identification of training needs. - No. of training actions per year. - No. of persons involved in the courses. - Results of training surveys - Evaluation of training effectivity. |
| E 1- Inspection | - The Organization prepares an annual plan that reflects the content and frequency of inspections. | - Annual Inspection Plan. - Inspection records. - Record of inspection communications. - Time spent in correcting anomalies. |
| F 1- Economic and financial efficiency | - The Organization prepares an economic table for the service, which includes the values of immobilised assets, their amortisations at source, investments in course, annual expenses assigned including personnel expenses, and income for any item related to the service or the use of its associated infrastructures. | - Updated economic profile. - Invoices and supporting documents for the economic values reflected. |
| F 2- Measurement of quality of service | - The Organization compiles and records measurements taken on all the quality indicators defined in its Quality Manual. | - Annual report on service status of each Port Authority and the Spanish system of marine navigation aids. |
| F 3- Promotion of quality | - The IALA promotes the adoption of quality management systems and the implementation of service quality systems for the marine navigation aids service in its member states. | - Number of Port Authorities with marine navigation aids service quality systems. |
| F 4- Client participation and quality measurement | - The Organization has a system of requests for information, complaints, claims and suggestions that includes the processing of such and a deadline for responses. - The Organization has a defined system for obtaining the needs, expectations and perceptions of marine navigation aids service users. | - Number of requests for information, complaints, claims and suggestions. - Response times. - Number of suggestions adopted. - Analysis of needs, expectations and perceptions obtained from service users. |
| ## F 5- Continual improvement | - The Organization analyses the service results achieved yearly and prepares an annual improvement plan. | - Record of annual improvement plans. |

# INTEGRATED AVAILABILITY INDICATOR

On many occasions it will be necessary to avail of a single indicator that represents level of Service Quality and makes it possible to compare different Organizations, as well as incorporate them into globally integrated Quality Management Systems, such as Integrated Balanced Scorecard techniques or EFQM, in which individual indicators for each aid, type of aid, or area, etc., cannot be assimilated.

Thus, the use of an integrated service availability indicator is proposed, defined as the weighted mean availability value of the different AtoNs grouped by categories (according to the IALA classification), to which the following weights are applied:

- Category I :  weight = 0.60

- Category II:  weight =  0.30

- Category III: weight =  0.10