6. FAIRWAY BUOYAGE

6.1 General

The purpose of buoyage is to indicate to the fairway user the fairway space, any rocks dangerous for shipping, and usually also the centre line of the fairway. The narrower the fairway, the more efficient must the buoyage be to enable sufficiently accurate navigation (compare with 4.2). From the navigational point of view, there is no significant importance associated with whether the buoyage system is lateral or cardinal – that is, whether the buoy shows from which side it must be passed or, rather, must be kept at a certain compass point. Essential is that the buoy can be identified and located on the chart. In narrow and winding passages where the navigator has no time to correlate the vessel's position with chart information, the buoyage must be so explicit and clear that navigational decisions can be made directly via visual and/or radar observation.

Accident statistics and analyses show that the narrowness or windingness of a fairway is not the primary reason for running aground; the usual cause is deviation from the fairway. Hence, enhancing the aids to navigation is the most efficient way to prevent grounding. Grounding and bottom contact occur mostly in the autumn when visibility conditions are poor and getting a fix on the vessel’s position thus is hard (see Annex 6.1). As the waters freeze over, the number of accidents takes a downturn since the vessels travel partly along channels opened through ice. Also, those vessels with lower-level equipment are not in traffic at that time of year. A slight increase in the number of accidents in the spring might be due in part to unexpectedly impaired visibility occasioned by spells of wet snow and the possibility of the floating aids to navigation being out of sight from time to time as the spring ice moves. In Finland, development led to positioning based on leading marks, by necessity, since fairways could not be marked with edge marks that can withstand the ice conditions. Leading marks constructed on shore were the only reliable aids to navigation in the earlier days. Edges of fairways were marked with temporary and unreliable wooden spars. Nowadays, both leading and edge marking can be used for fairway buoyage.

A fairway's aids to navigation must meet the following maritime requirements:

- Buoys and other marks may not move from their original location. This is why fixed aids to navigation located in water are constructed to withstand ice load and why buoys and spars are used only in areas where they keep their location reliably.

- The buoyage must be distinct and identifiable both visually and via radar from a sufficient distance in all navigation conditions.

Moreover, buoyage should be systematic, consistent, and clear for all fairways.

The following aids to navigation are used as fairway-specific buoyage:

- leading marks (boards and lights)

- fixed edge marks

- floating edge marks (buoys and spars)

- lighthouses and beacons

6.2 Leading marks

Leading marks normally are used to indicate the location of fairway line (see Figure 13). For sections that are difficult to navigate and for heavy-traffic fairways, the aim is to construct leading marks at both ends of the fairway. The utility of leading marks is impaired by the fact that, when located far from the observer, they often cannot be seen, due to fog, rain, or similar factors. Moreover, leading marks constructed on shore are not visible on radar. For the above reasons, navigation cannot be based on leading marks only.

Leading board size increases with observation distance (see Figure 13). Board visibility is lowest in observation against the light. In this case, one might use a board made of, for example, body-tinted plastic board that lets some light pass through. In poor visibility conditions, a board coated with daytime fluorescent sheeting or paint whose colour differs from the background stands out the best.

However, the use of daytime fluorescent sheeting and paints is limited by the short service life of fluorescent properties (five to 10 years) and relatively high cost.

In locating the leading marks, the following issues are among those that should be considered:

- The leading marks always should be constructed on shore for cost reasons, unless locating the mark on water can improve the visibility significantly.

- Investing in observation distances of more than 12 kilometres is not really worthwhile, since the visibility of these leading marks is very limited.

- If the line is marked from only one end, the marks should be at the north end of the line to minimise navigation against the light.

- Visibility of leading marks is impaired by smoke, lights, etc. in the background.

- In special cases, leading marks can be implemented with daytime lights with which board structures usually are not required, or with sector lights that show an extremely narrow and accurate sector (in cases where constructing a pair of leading marks is not possible).

Operational planning and determination of geometric values for leading marks is described in more detail in the Finnish Maritime Administration's instruction document 'Linjalaskennan perusteet', and corresponding calculation software for Windows environments has been compiled on the basis of these instructions. In addition, type drawings have been prepared for leading marks, upper structures of edge marks, buoys, and spars.

6.3 Edge marks

In the most difficult navigation conditions, a vessel is manoeuvred either partially or completely with the help of the fairway's edge marks. Hence, the edge marks must be visible both visually and with radar.

Edge mark spacing must be in relation to the width of the fairway, its curves, and the conditions in which the fairway is used. In the narrow sections of the fairway, the marks should be located in pairs on both sides of the fairway since a gate is the most efficient optical guide for a fairway. In areas where ice moves, the edge marks should be fixed since floating marks submerge, get damaged, and move as the ice moves.

Buoys have their main use in areas where ice does not move much, such as in archipelago and inland waterways. Buoys always should be anchored so firmly to the bottom that they do not move from their original location even during the seasons when ice builds up or breaks up. The most reliable method for securing a buoy is anchoring in bedrock (see Figure 14).

For inland waterways, a buoy with a frame made of plastic tubing may be used (see Figure 15). The buoy frame consists of polyethylene tubing in which batteries and a radar reflector are fitted. The buoy light device must withstand, for example, impacts by bundle rafts. Nowadays, the buoys are almost exclusively plastic tube buoys in which the radar reflector is fitted (see Figure 16). The plastic buoys can be seen clearly due to their bright and durable colours, and they also keep their position quite well. Wooden spars are used only where the spar is damaged almost every year. A fixed edge mark must be close to the fairway edge. A suitable distance from a fixed mark to the edge of the fairway is 0.5 to 1.0 times the width of the design vessel. The distance may not exceed 50 metres without a special reason. Floating edge marks always are located at the edge of the fairway. The so-called buoy-to-buoy rule is applied to edge marking. It means that no area above the confirmed depth may be on the fairway side of a line drawn between two consecutive buoys or spars that have identical identifications and belong to the same fairway line.

6.4 Lighthouses

Lighthouses are extremely expensive aids to navigation. This is why lighthouses are constructed only at the entrances of the most important fairways to ensure that a vessel coming from the high seas can find the entrance in all conditions. Hence, the lighthouse must be constructed right by the side of the fairway like the edge marks.

6.5 Buoyage examples

International buoyage practice for heavy-traffic fairways has become standardised as follows (see Annex 6.2):

- Marking of the fairway line(s) is done with boards and lights.

- Moreover, the fairway edges are buoyed with buoy pairs at 1–3-km spacing. The buoy gate spacing is determined primarily by the width of the fairway.

- If the fairway line is not marked, the buoy spacing for a straight fairway is usually one to 1.5 km.

On Finnish coastal fairways, where encounter situations are less frequent due to the small amount of traffic and conditions in the outer sections of the fairways make the use of floating aids to navigation difficult, edge marking often is more irregular, increasing the importance of marking dangerous rocks. Figure 17 shows a proposal for straight fairway buoyage. In case A, the amount to be dredged is so small that the fairway need not be narrowed. In case B, the fairway is narrowed from one side to decrease dredging costs (buoyage probably should be enhanced), and in case C the width of the fairway is narrowed to the minimum due to dredging costs, creating a need to enhance buoyage. The need for edge marks along curves depends on the turning angle, extent of the full-depth water area, fairway width, and shape of dredging (see Figure 18).

Case A shown in Figure 18 is applicable when there is plenty of full-depth water area around the curve area. In this case, the fairway has no precisely determined width. The edge mark indicates the turning point. In case B, there are shallow areas in both the inside and outside curve area but the small turning angle makes the curve area short. Case C shows a relatively tight curve in a dredged or narrow section. In addition to adding clarity, the relatively extensive buoyage is used for keeping the dredging area small. Case D is similar to case D, but, to decrease dredging costs further, the outer corner of the fairway is left outside the fairway area since the vessels cannot use it anyway. Annex 6.3 shows an example of narrow fairway buoyage. Vessels with a width of up to 0.5 x fairway width carrying partial or no cargo have used the fairway without tug assistance. Due to the narrowness of the fairway, the buoyage is more extensive and versatile than normally.