DRAFT IALA RECOMENDATION ON EXPECTED RANGE

(COMMITMENT TO THE USER POINT OF VIEW)

INPUT PAPER FOR THE IALA-ANM6

JUAN F. REBOLLO  (Puertos del Estado-Spain)

RECALLING the IALA definition of “Nominal Range” appearing in the NAVGUIDE (Ed. 4)

RECALLING ALSO that the “Nominal Range” is the data that appears published in nautical chart and other nautical documents used by mariners as a parameter  of the lighted visual aids ;

NOTING that the mariners expect to recognize, interpret and be able to use aids to navigation according to their characteristics

RECALLING that the “nominal range” it is not a value that users can directly make use of because it is affected by factors such as background light and atmospheric visibility;

NOTING that a quality service must provide useful information for users so that their needs and expectations are met;

NOTING ALSO that an aid to navigation must provide service with an availability of at least 95%;

RECOGNISING that the aids to navigation service can only be provided when the mariner can use the information and from the point of view for the “user”, a light that is not seen is a light not providing service;

ADOPTS that It is necessary to define “range” in order that the mariner can expect to use the aid at the noted distance with , at least, the minimum standard availability for an aid to navigation.

THEREFORE, the International Association of Marine Aids to Navigation and Lighthouse Authorities, IALA

RECOMMENDS

- The members adopting measures to set the “expected range” for each lighted visual aid use the attached Guideline.

- The dissemination of this value should be promoted, particularly in cases of highly important or relevant aids, together with the “nominal range” until a new term has been fully implanted, so as not to loose traceability of information.

- Members promote the implantation of this new term in international organisations such as the International Maritime Organisation (IMO) and the International Hydrograhpic Organisation (IHO), as applicable.

DRAFT IALA GUIDELINE ON EXPECTED RANGE

(COMMITMENT TO THE USER POINT OF VIEW)

The International Association of Marine Aids to Navigation and Lighthouse Authorities, (IALA) has defined the “Nominal Range” of a lighted visual aid as appears in the NAVGUIDE (ed. 4) as:

“the luminous range when the weather visibility is 10 nautical miles which is the equivalent of a transmission factor of T=0,74. Generally, this figure is used in official documents such as nautical charts, lighthouse books, etc.

The nominal range assumes that the light is observed against a dark background free from background light.”

Since luminous range is defined as “the maximum distance at which a light can be seen by the observer’s eye at any given time”

In other words, the figure indicating the nominal range of a lighted visual aid will fit the expectations of a mariner when the weather visibility is equal to the standard of 10 nautical miles and when there is no background light in the area of observation.

Although it can be said that there are conversion tables for nominal range as a function of atmospheric visibility in the area and at the time in question, these tables are rarely ever available to mariners and are difficult to use on board, particularly at night. And there are no tables, at least not sufficiently standardised, disseminated and used, that take into account background light and the reduction of the service distance of the AtoN.

The general result is that the user, the mariner, receives information through nautical charts and documents generating expectations of a much better service than what can actually be received. The mariner will not know whether a signal is off or whether environmental conditions are diminishing the range. In any event, the service will not be provided at the expected distance; it will not be available.

In this guideline the use of a new term is proposed, the “Expected Range” of a lighted visual aid that will take into account background light and the weather visibility with a 95% probability, so that the service of the light can be guaranteed with a the minimum availability of 95% which a navigation aid should have. This range should be related to the concept of availability perceived by the mariner or to the real availability of the service under the established conditions.

An example, for the Cape Peñas Lighthouse, in Spain, is as follows:

Its nominal range is 28 nautical miles (T= 0’74), however the expected range (for atmospheric transmissibility at a level of 95%) is only 14 nautical miles. This means that a mariner at a distance of 15 miles will rarely see the lighthouse and will not know whether not see it, or whether it is non-lighted. The general perception however is that the aid to navigation is off, even when the authority in charge considers that it is properly functioning.

Unlike nominal range, which lacks real meaning from a safety standpoint since the mariner is unaware of the distance at which the aid can be used, expected range provides information on the distance at which the lighted signal’s service  can be used.

Thus, the EXPECTED RANGE (D) of a lighted visual aid to navigation is defined as the range for the corresponding background light in the area of observation that corresponds, when applying the Allard formula, to a value of atmospheric visibility representing a 95% probability, in the area of study.

Because it is a service value for the mariner, the expected range must be independent from background light, and also from the colour and the rhythm of the light.

Insofar as information on the environment is concerned, a sub-index could be added representing a factor (1 – 5) related to the background light (Df ), but for each site the expected range would be related to the traffic and risks and would be a quality of aids to navigation service.

In order to establish the expected range for aids, a certain amount of statistics must be available for the area of study. The closer to reality, the better the quality of the service.

This value should appear on nautical charts and documents, since it represents a character of the service and should be geared towards the user’s perception. Actually, in many instances, the range at which the signal in question should be used is defined as the nominal range, and this is not useful. This often results in the signal actually only providing service at much shorter distances, although the nominal range value meets its definition, and this could lead to risky situations in navigation.

The calculation of the luminous intensity (candelas) of the source of light will be adjusted to the value for atmospheric visibility that, for the area of study, has a 95% probability, and will also take background light into account. In addition, in order to obtain the luminous intensity value to be installed, colour, rhythm, and the effects due to degradation for use should be taken into account according to the corresponding factors and formulae indicated in the NAVGUIDE.