ANNEX A

CHAPTER 10

INFORMATION SERVICES

  - INTRODUCTION

The e-Navigation initiative, led by the International Maritime Organization (IMO), aims to enhance safety and efficiency in the maritime sector by providing harmonized electronic information to navigational systems on board ships, as well as streamlining the exchange of information among maritime authorities, agencies, and stakeholders ashore. This initiative encompasses multiple disciplines and seeks to create a more integrated and standardized approach to the use of electronic navigational tools.

The development of e-Navigation began in 2006 when seven IMO Member States proposed the creation of a strategic vision for the utilization of navigational tools, particularly electronic ones, in a holistic and systematic manner. The proposal was driven by concerns that a lack of coordination in the introduction of new technology could lead to increased complexity and a lack of standardization on board vessels, potentially jeopardizing safety.

In response to this proposal, the IMO collaborated with other international organizations such as IALA and IHO to develop a strategy for e-Navigation in 2008. This strategy was further refined with the completion of the Strategy Implementation Plan (SIP) in 2014 and its subsequent review and update in 2018.

e-Navigation’s ultimate goal is to reduce navigation-related accidents and improve safety and incident prevention in the maritime sector. By providing seamless, customized, and efficient access to electronic information, e-Navigation enables better-designed navigational systems on board ships and streamlines the way maritime authorities and stakeholders gather and exchange information. The continued development and implementation of e-Navigation will play a crucial role in enhancing safety and efficiency in the maritime industry moving forward.

A plan to implement the strategy, termed the Strategy Implementation Plan (SIP), was completed in 2014 (NCSR1/28 Annex 7). This was reviewed and updated in 2018 (MSC.1/ Circ.1595).

  - IMO’S STRATEGY FOR THE DEVELOPMENT AND IMPLEMENTATION OF e-NAVIGATION

The IMO strategy for e-Navigation (MSC 85/26/Add.1, Annex 20) states that about 60% of collisions and groundings are caused by direct human error. Despite advances in bridge resource management training, it seems that the majority of watch-keeping officers make critical decisions for navigation and collision avoidance in isolation. This is partly due to a general reduction in manning, it states.

The IMO strategy also states that in human reliability analysis, the presence of someone checking the decision-making process improves reliability by a factor of 10. If e-Navigation can assist in improving this aspect, through well-designed on-board systems and closer cooperation with vessel traffic management (VTM) systems ashore, the risk of collisions and grounding (and their inherent liabilities and costs to administrations) can be dramatically reduced.

The IMO vision for e-Navigation includes the following general expectations for on- board, ashore and communications elements.

    - e-NAVIGATION DEFINITION

The IMO strategy defines e-Navigation as the “harmonised collection, integration, exchange, presentation and analysis of maritime information on-board and ashore by electronic means to enhance berth-to-berth navigation and related services, for safety and security at sea and protection of the marine environment.”

In other words, e-Navigation means:

      - The harmonised exchange and presentation of navigational information in electronic formats;

      - Harmonised data exchange and improved communications;

      - Creation of a “wide area navigation team”, which allows the Officer of the Watch (OOW) and the Vessel Traffic Services (VTS) Operator to share tactical and planning information; and

      - Improved design of navigational and communication equipment.

    - ON-BOARD AND ASHORE

Navigation systems that benefit from the integration of own ship sensors, supporting information, a standard user interface and a comprehensive system for managing guard zones and alerts. Core elements of such a system will include actively engaging the mariner in the process of navigation, to carry out their duties in the most efficient manner, while preventing distraction and overburdening.

The management of vessel traffic and related services from ashore, enhanced through better provision, coordination and exchange of comprehensive data in formats that will be more easily understood and utilized by shore-based operators in support of vessel safety and efficiency.

    - COMMUNICATIONS

An infrastructure providing authorized seamless information transfer on board ship, between ships, between ship and shore and between shore authorities and other parties with many related benefits.

    - WHAT DOES THE ‘e’ IN e-NAVIGATION STAND FOR?

It is generally accepted that the IMO concept of e-Navigation can be thought of as a brand, without the need for ’e’ to be specifically defined. The concept of e-Navigation was first proposed by seven IMO Member States in 2006 as a process for the harmonisation, collection, integration, exchange and presentation of maritime information. As such, the ’e’ could have stood for ’enhanced’ or ’electronic’ (just like the ’e’ in e-commerce), but this would limit what could be done within e-Navigation. It must be noted that the generic term electronic marine navigation already exists in many forms. It should not be confused with this particular IMO initiative.

    - KEY ELEMENTS

The key elements of the IMO strategy for e-Navigation, based on user needs include:

      - Architecture

      - Human element

      - Conventions and standards

      - Position fixing

      - Communication technology and information systems

      - Electronic Navigational Charts (ENC)

      - Equipment standardisation

      - Scalability

According to the strategy, the implementation of e-Navigation should be a phased, iterative process of continuous development, taking into account the evolution of user needs and the lessons learned from the previous phase(s).

As part of the basic requirements for the implementation of e-Navigation, it was agreed that e-Navigation should be based on user requirements and needs and not technology-driven.

    - e-NAVIGATION SOLUTIONS

The centrepiece of the current (2018) SIP is the following five e-Navigation solutions:

      - S1: improved, harmonised and user-friendly bridge design;

      - S2: means for standardized and automated reporting;

      - S3: improved reliability, resilience and integrity of bridge equipment and navigation information;

      - S4: integration and presentation of available information in graphical displays

received via communication equipment; and

      - S5: improved Communication of VTS Service Portfolio (not limited to VTS stations).

Solutions S1 and S3 promote the workable and practical use of the information and data on board. As regards S1, IMO finalised:

      - Guideline on Software Quality Assurance and Human-Centred Design for

e-navigation (MSC.1/Circ.1512)

      - Guidelines for the standardisation of user interface design for navigation equipment (MSC.1/Circ.1609)

      - Interim Guidelines for the Harmonised Display Of Navigation Information Received

Via Communication Equipment (MSC.1/Circ.1593)

      - Guidelines for the presentation of navigation-related symbols, terms and abbreviations, (SN.1/Circ.243/Rev.2)

      - Amendments to the Performance standards for the presentation of navigation-

related information on shipborne navigational displays (resolution MSC.191(79))

      - Guidance on the definition and harmonisation of the format and structure of maritime services in the context of e-navigation (IMO Resolution MSC.467 (101)).

Solutions S2, S4 and S9 focus on efficient transfer of marine information and data between all appropriate users (ship-ship, ship-shore, shore-ship and shore-shore).

The process at the IMO led to the specification of Maritime Services in the Context of e-Navigation.

  - IALA’S WORK ON DIGITAL COMMUNICATION TECHNOLOGIES, INFORMATION SERVICES AND e-NAVIGATION MATTERS

IALA has so far focused on three broad streams: Digital Information Systems

- Developing guidance on the description and implementation of “Maritime Services

in the Context of e-Navigation”.

- Facilitating harmonisation in the development of data models, technical services and platforms.

- Defining concepts, procedures, services and platforms for new initiatives such as

identity management.

Emerging Digital Technologies

- Evaluation of new technologies relevant to e-navigation, in particular digital maritime communications

- Maritime Autonomous Surface Ships (MASS)

- Single Window Data Exchange Digital Communication Systems

- IALA MARCOM manual

- The technical characteristics and operation of the VHF Data Exchange System (VDES)

- Autonomous Maritime Radio Devices (AMRD), Automatic Identification System (AIS) and other digital communication technologies

  - MARITIME SERVICES IN THE CONTEXT OF e-NAVIGATION

IALA Standard S1070 applies to Information Services. This Standard references normative and informative provisions, detailed in the listed IALA Recommendations, covering the following scope.

- Data models and data encoding

- Data exchange systems

- Terminology, symbology and portrayal

IALA Guideline G1155 The Development of a description of a maritime service in the context of eNavigation, which is aimed at assisting members and/or other international organizations in the development and implementation of a Maritime Service (MS) in the context of e-Navigation. This document will assist in the production of a Maritime Service description and is the interconnection between the high level description of the Maritime Services and the Technical Services to be developed.

    - BACKGROUND

The primary goal of e-Navigation is to provide harmonized information in electronic formats to better-designed navigational systems on board and streamline communication and data exchange among maritime authorities, agencies, and stakeholders. There are 16 digital maritime services recognized by the IMO at the moment, forming the core of navigation services in the coming years. These services include three VTS services (information, traffic organization, and navigational assistance), a chart service, and Maritime Safety Information (MSI). IALA is trying to update the VTS services and add the Marine Aids to Navigation service in the list of maritime services.

    - TERMINOLOGY AND DIRECTIVES

Maritime Services in the context of e-Navigation refer to the provision and exchange of maritime-related information and data in a harmonized, unified format. IMO Resolution MSC.467(101) provides guidance on the definition and harmonization of the format and structure of maritime services in the context of e-Navigation. IALA, in collaboration with IMO, is recognized as a coordinating body for the development of Maritime Service on VTS.

    - HARMONIZATION OF THE FORMAT AND STRUCTURE OF MARITIME SERVICES

In accordance with IMO Resolution MSC.467(101), all Maritime Services must adhere to the S-100 standard as a baseline. IALA offers recommendations and guidelines for the development and evaluation of harmonized platforms that provide Marine Aids to Navigation (AtoN) Services, including VTS, within the context of e-Navigation.

Recommendation R1019 acknowledges ongoing advancements in the field of Maritime Services, outlining general requirements for these services in the context of e-Navigation. Digitalization in the maritime domain is advancing rapidly. The shipping industry is experiencing increasing levels of digitization and automation on board and ashore, the growing electronic exchange of digital information and the advent of digital maritime services. It also puts forward higher requirements for the services under e-Navigation. This recommendation serves as a directive for creating new guidance documents on the topic.

Guideline G1161 offers information for evaluating harmonized and suitable platforms for delivering Marine Aids to Navigation (AtoN) Services, including VTS, within the context of e-Navigation, as suggested by R1019. As the e-Navigation domain continues to evolve with new digital platform solutions, a common understanding of requirements is essential. This document concentrates on the required criteria for these next generation platforms which support the provision of Maritime Services in the context of e-Navigation and presents an overview of relevant requirements that should be considered for the distribution of maritime services, which assists IALA members and the industry in identifying appropriate platforms and encourages platform providers to align their solutions with shared requirements.

Refer to IALA Publications:

- Recommendation R1019 - Provision of Maritime Services in the context of e-Nav- igation in the domain of IALA

- Guideline G1161 – Evaluation of Platforms for the Provision of Maritime Services in the context of e-Navigation.

Refer to IALA Publications:

- Recommendation R1019 - Provision of Maritime Services in the context of e-Nav- igation in the domain of IALA

- Guideline G1161 – Evaluation of Platforms for the Provision of Maritime Services in the context of e-Navigation.

    - TECHNICAL SERVICES

A Technical Service comprises a set of technical solutions, based on an agreed data model and communications means, to provide a Maritime Service as per IMO MSC.467 (101). Based on the concepts of service-oriented architectures, a Technical Service also refers to a set of related software functionalities that can be reused for different purposes, together with policies that govern and control their use. Therefore, harmonization and interoperability of data models and interfaces of Technical services play a key role in the realization of Technical Services.

Technical Services should also be designed modularly. In this way, digital services offered by one electronic device can directly be used by another electronic device without any modification. Often, Operational Services are implemented by electronic devices or software components that rely on one or more Technical Service.

IALA is working on developing the operational and technical aspects of maritime service on the Marine Aids to Navigation including VTS.

IALA Guideline G1128 on the specification of e-Navigation technical services enables service providers, consumers and regulatory authorities to have a common understanding of a technical service and its implementation. The Guideline differs between the actual service specification (functionality and interfaces), technical designs (technology specific considerations) and instance descriptions (specific setup of a single service instance). All sections of the guideline are characterized by a fixed scheme. This enables the standardized specification of services in a Service Oriented Architecture.

A Technical Service should be formally specified and documented, as described by IALA Guideline G1128. At the time of writing, this guideline aims at improving the visibility and accessibility of available Technical Services and information provided by them. The Guideline provides Service Specifications, Service Designs and Service Instance Descriptions for any kind of e-Navigation Technical Service in a standardised way. Any Technical Service Specification in the context of e-Navigation shall comply with this Guideline. Technical Service Specifications shall provide references to the appropriate S-100 Product Specifications if available or to the appropriate S-100 features and attributes in their data models.

    - ARCHITECTURES FOR e-NAVIGATION

The maritime domain is a complex ecosystem of multiple stakeholders and a mix of new and existing digital infrastructures. Secure, reliable, and efficient architectures are required to fulfil the requirements of a harmonized network of technical services. The Maritime Architecture Framework (MAF) is a framework for the design of maritime architectures. It can be used to describe the context of e-Navigation digital architectures and implement them.

IALA Guideline G1113 Design and implementation principle for harmonized system architectures of shore-based infrastructure describes the overarching foundation for e-Navigation Architecture. Most MS depend on the exchange of data and for that purpose it is essential that a common understanding of the data is described in a standardized data model and stored in an appropriate repository (for example the IHO or IALA Registries).

As stated in Recommendation R0140 (eNAV-140) The Architecture for Shore-based Infrastructure ‘fit for e-Navigation' and in G1113, the development of e-Navigation related services should be based on user (information) needs. This means that the objective of the MS, the user need, the information need, user requirements, etc. are described in a way that can later be translated into Technical Service Specifications and one or more technical designs, by engineers.

In recent years, platform solutions for connecting decentralized services and databases have been established in a large part of the transportation and other sectors. IALA recognizes platforms as an essential building block to implement the e-Navigation strategy, as they can facilitate harmonization, interoperability, and collaboration between different service providers and support Service Oriented Architectures (SOA). SOAs enable services to interact with other services and, furthermore, make it possible to build services based on existing services. Additionally, IP-based communication and web services open possibilities for efficient and more fine-grained communication. However, as this introduces a relatively new way of interacting with Maritime Services in a digital context, there are new requirements and risks that service architects and developers should be aware of. IALA Guideline G1161 on Platforms to support the provision of Maritime Services in the context of e-Navigation offers information for evaluating harmonized and suitable platforms for these issues.

IALA Guideline G1114 A Technical Specification for the Common Shore-based System Architecture（CSSA）is a best practice representation of a system layout which was designed, as a system engineering response to the prompt for a common technical shore-based system harmonized for e-Navigation (incl. its Human-Machine-Interfaces) as implied by IMO’s overarching architecture for e-Navigation and in accordance with the principles set out in the corresponding IALA Recommendation R0140.

As for the content of NAVGUIDE, certain sections remain pertinent and can be integrated into e-Navigation architecture discussions. These sections touch on crucial aspects such as technical infrastructure, data exchange, and communication protocols, all of which are vital for the successful implementation and operation of e-Navigation systems.

By combining the insights from the MAF, existing platform solutions, and relevant sections of the NAVGUIDE, a comprehensive understanding of the architectures for e-Navigation can be developed. This knowledge will enable the maritime community to design and implement effective digital infrastructures that enhance safety, efficiency, and collaboration among stakeholders in the maritime domain.

  - A LAY PERSON’S DESCRIPTION OF e-NAVIGATION

e-Navigation is based on the principles of data exchange as used in the apps that operate on smart phones.

For example, a weather app provides information to a user, this could be called a “Weather Service”. This weather service satisfies the user’s need for information about the weather. Similarly, a Maritime Service, in the context of e-Navigation, satisfies a user need for information concerning vessel navigation and other maritime considerations including safety, efficiency and the protection of the marine environment. Our weather service app has to communicate with a server that runs software that can provide weather information.

Figure 37. The Concept of a Client-Server based Maritime Service

The interaction between the app and the server is defined by a technical service specification, which describes the exchange of standardised messages and the language that is used in the message contents. The language is described by a data model.

The software running on the server hardware is described as being an instance of the technical service.

Figure 38. The relationship between specifications of Maritime Services, Technical Services and data models in e-Navigation

If somebody wants to develop an app that is able to communicate with the server or develop a server to be used by the app, then the developer must refer to the technical service specification and the referenced data models.

Similarly, an e-Navigation Maritime Service draws together information through a combination of one or more running instances of a technical services. A technical service facilitates the exchange of data, by receiving messages, processing the data, and sending a result. The data is formatted according to the Common Maritime Data Structure, which is based on the IHO S-100 series of data models, thus ensuring harmonisation and interoperability.

Figure 39. The dataflow between Maritime Services, Technical Services and data exchange

The relationship between Maritime Services, Technical Services, and Data Models can be further analyzed with reference to Figure 11, which elaborates on the example of a Weather Service App. In this scenario, the weather service app is assumed to be part of a set of e-Navigation Maritime Services, drawing parallels with Figure 10. The Maritime Weather Service App, developed by Stormy Corp Ltd., comprises several Technical Services. Each Technical Service communicates with servers containing various weather-related data, such as temperature, wind, and cloud cover. These datasets are collected from sensors dispersed across a geographical region and stored in databases. Technical Services are responsible for extracting relevant data based on the user input in the Weather Service app interface.

The Weather Service app connects to instances of Technical Service 1 for temperature, Technical Service 2 for wind, and Technical Service 3 for cloud cover (or sunlight). Upon examining the architecture presented in Figure 11, several characteristics can be observed:

- A single Maritime Service can utilize multiple Technical Services, exemplifying a

1:N relationship between Maritime Services and Technical Services.

- Instances of Technical Services can be deployed by either one organization or multiple organizations. For instance, BlowFelt Inc. hosts a Temperature Technical

Service on their servers and provides their temperature data. They also offer a Wind Technical Service with their wind database. However, StormyCorp Ltd.’s Weather Service App opts to use cloud cover data from GreySkies Inc. BlowFelt and GreySkies servers may be geographically distant.

  - Instances of Technical Services can be reused by other Maritime Services. Figure 11

depicts a Maritime Service that estimates the amount of harvestable solar energy using the Solar Energy Predictor App, developed by SunShine Inc. Users can input a geographical location in the app’s interface to determine the energy harvesting potential of a remote, solar-powered autonomous ship at its current location. SunShine’s App could utilize the same instance of GreySkies’ Cloud Cover Technical Service as employed by StormyCorp’s Weather App or choose the one hosted by BlueSkies. Decisions regarding which service provider’s technical service instance to use may depend on factors such as perceived data quality, subscription costs, or other considerations.

    - AN EXAMPLE OF AN IALA NATIONAL MEMBER’S VISION

In 2019, the Australian Maritime Safety Authority released Navigation Services in Australian Waters—outlook to 2030. This work provides an insight to the provision of navigation services (not just Aids to Navigation) in the coming years. It outlines the emerging trends and drivers in navigation technology and communications. It also describes the anticipated impacts these will have on the maritime industry. Importantly, it lists AMSA’s policy responses to these changes.

It can be found here:

https://www.amsa.gov.au/safety-navigation/navigation-systems/navigation-services- australian-waters-outlook-2030

    - PLANNING AND REPORTING OF TESTBEDS IN THE MARITIME DOMAIN

Several projects (completed and underway) have made noteworthy inroads in developing aspects of e-Navigation. The IALA website (https://www.iala-aism.org/technical/planning- reporting-testbeds-maritime-domain/) provides more details on known testbeds and their results.

IMO and IALA have guidelines to assist with the reporting of testbed results

      - Guidelines on Harmonisation of Testbed Reporting (MSC.1/Circ.1494)

      - IALA Guideline G1107 – Planning and reporting of testbeds in the maritime domain

  - MARITIME RESOURCE NAMES

The Maritime Resource Names (MRN) is a unique identifier system developed by IALA to support e-Navigation initiatives. MRNs are designed to facilitate the identification, management, and exchange of digital resources in the maritime industry. The MRN

structure is based on the Uniform Resource Name (URN) concept, which is part of the Internet Engineering Task Force (IETF) standards.

In the maritime sector, there are already numerous identification systems in place, such as the IMO number for ships, the Maritime Mobile Service Identity (MMSI) for vessel communication, and the Aid to Navigation (AtoN) number. While these identification systems serve their specific purposes, they are often limited in scope and do not provide a comprehensive solution for identifying all types of maritime resources.

The MRN system addresses this issue by offering a global, unified approach to identifying various maritime resources, regardless of their nature or purpose. By adopting the URN concept from IETF, the MRN system ensures that the identifiers are globally unique, persistent, and location-independent, which is critical for efficient communication and collaboration among various maritime stakeholders, such as ship operators, port authorities, coastal states, and service providers.

The MRN consists of several components that follow a hierarchical structure, similar to the URN:

- Namespace Prefix: This is a short code that represents the standardization

organization responsible for maintaining the MRN system. For IALA, the prefix is ‘urn’.

- Domain: This component indicates the specific organization or authority that is responsible for the resource. Examples include ‘mmsi’ for Maritime Mobile Service Identity, ‘imo’ for International Maritime Organization, or ‘ais’ for Automatic Identification System.

- Subdomain: This is an optional component that further specifies the type of

resource within the domain. For example, within the ‘ais’ domain, there could be subdomains like ‘message’ or ‘report’ to differentiate between different types of AIS resources.

- Resource Identifier: This is a unique alphanumeric code that distinguishes the

resource from others within the same domain and subdomain. The resource identifier is usually generated by the responsible organization or authority.

By implementing the MRN system, the maritime industry can streamline data exchange, avoid confusion or miscommunication, and improve the overall safety and efficiency of maritime operations. The MRN is a key component of the Maritime Connectivity Platform (MCP), an open and vendor-neutral communication framework that supports the seamless exchange of information between maritime actors in a secure and reliable manner.

For more information on MRNs, URNs, and their practical applications, you can consult the IALA website, explore IETF’s RFC documents, or review guidelines and best practices related to e-Navigation.1

  - MARITIME CONNECTIVE PLATFORM (MCP)

The Maritime Connectivity Platform (MCP) is a decentralised platform that facilitates secure and reliable information exchange within the maritime domain and beyond. The information exchanged can be almost of any nature, ranging from private confidential information between a vessel and the shore office of the shipowner, to public information provided by authorities, such as the provision of navigational warnings. As a decentralised platform, there is no single entity operating this. Several organisations are MCP service providers, and collectively they form “the Maritime Connectivity Platform”.

The MCP features - as one of its core components - an identity registry, where all entities that wish to exchange information are registered and have a digital certificate issued to them. Thus, a vessel registered with the MCP identity registry (having a digital certificate issued from it), can authenticate itself (cryptographically prove its identity) to the VTS centre, and thus provide data to the VTS centre which the VTS centre can trust the origin of. The principle of authentication is a cornerstone in contemporary digital solutions. The IALA guideline G1183, defines requirements for secure identities through the Maritime Identity Registry (MIR) of the MCP, targeting organizations planning to become MCP Identity Service Providers and those using MIR certificates. This aligns with IALA R1019’s recommendation on maritime services in e-navigation.

Maritime Service Registry, MSR for short, is a registry of the implementations of Technical Services and is a reference point for information on registered services and offers discoverability for the registered services. The functionality of the MSR is twofold: service discovery and service management. It enables service providers to register their services in the MCP and allows an end-user to discover those services. The MSR does not provide actual maritime information but a specification of various services, the information that they carry, and the technical means to obtain it. An MSR instance contains service specifications according to a Service Specification Standard (which is identical to IALA Guideline 1128) and provisioned service instances implemented according to these service specifications.

The MMS is a messaging service intended to offer transparent seamless information transfer across different communication links in a carrier agnostic and geolocation-context sensitive manner. The MMS primarily addresses ship-shore communication based on internet connectivity, yet any number of alternative communication services may be connected to and utilized by the MMS via dedicated gateways. The MMS have be standardised through RTCM, the official RTCM standard describing the overall architecture and the main MMS protocol (Maritime Message Transfer Protocol, MMTP).

For more information on MCP and their practical applications, you can consult the MCP and IALA website.

  - IHO’S S-100 UNIVERSAL HYDROGRAPHIC DATA MODEL

The IHO S-100 Standard is a comprehensive framework document designed for the development of digital products and services within the hydrographic, maritime, and GIS

1 Maritime Resource Name (MRN) see: http://mrnregistry.org

communities. It comprises multiple parts based on ISO geospatial standards. IHO has developed the S-100 Universal Hydrographic Data Model (UHDM) to meet future demands for digital products and services.

The S-100 Geospatial Information Registry hosts online databases of concepts, features, portrayal information, attributes, metadata, and other resources pertinent to communities developing S-100 based products and services. A centralized registry facilitates harmonization between product specifications in the maritime sector.

    - S-100 ROLE IN e-NAVIGATION

IMO oversees the e-Navigation concept, which aims to enhance safety and security in commercial shipping through improved organization of data on ships and onshore, as well as better data exchange and communication between them.

IHO had already established a system of standardized methods for codifying, encapsulating, transferring, and distributing hydrographic and charting data through its S-57 Electronic Navigational Chart standard. This concept can be integrated into the entire e-Navigation concept by ensuring that the foundational standards are in place from the outset through the S-100 family of product specifications and centralized information registry. The IHO maintains control only over its own standards. The principles are outlined in IHO publication S-99, Operational Procedures for the Organisation and Management of the S-100 GI Registry, available at www.iho.int. It is planned to introduce finalized S-100 products in the coming years, initially with the simultaneous usage of S-57 products. Later, a complete transition to S-101 and other S-100 products is planned.

    - S-100 DEPENDENT PRODUCT SPECIFICATIONS

To manage the development of S-100 based products and minimize duplication while encouraging conformity, the IHO Hydrographic Services and Standards Committee (HSSC) allocates S-XXX numbers for the development of S-100 dependent products. The HSSC has allocated the following Product Specification numbers:

      - International Hydrographic Organization (IHO) (S-101 to S-199)

Some examples are:

      - S-101 Electronic Navigational Chart (ENC)

      - S-102 Bathymetric Surface

      - S-104 Water Level Information for Surface Navigation

      - S-111 Surface Currents

      - S-121 Maritime Limits and Boundaries

      - S-122 Marine Protected Areas (MPAs)

      - S-123 Marine Radio Services

      - S-124 Navigational Warnings

      - S-125 Marine Navigational Services

      - S-126 Marine Physical Environment

      - S-127 Marine Traffic Management

      - S-128 Catalogue of Nautical Products

      - S-129 Under Keel Clearance Management (UKCM)

      - S-130 Polygonal Demarcations of Global Sea Areas

      - S-131 Marine Harbour Infrastructure

      - S-158 Validation Checks - Introduction and Structure

      - S-164 IHO Test Data Sets for S-100 ECDIS

IALA (S-201 to S-299)

Some examples are

      - S-201 Aids to Navigation Information

      - S-210 Inter-VTS Exchange Format

      - S-211 Port Call Message Format

      - S-212 VTS Digital Information Service

      - S-230 Application Specific Messages

      - S-240 DGNSS Station Almanac

      - S-245 eLoran ASF Data

      - S-246 eLoran Station Almanac

      - S-247 Differential eLoran Reference Station Almanac

Joint Technical Commission for Oceanography and Marine Meteorology (WMO/IOC JCOMM) (S-411 to S-414)

      - S-401 Inland ENC

      - S-402 Bathymetric Contour Overlay for Inland ENC

      - S-411 JCOMM Ice Information

      - S-412 JCOMM Weather Overlay

      - S-413 Weather and Wave Conditions

      - S-414 Weather and Wave Observations

International Electrotechnical Commission (IEC) (S-421 to S-430)

      - S-421 Route Plan

    - IALA AND S-100 DEVELOPMENT

IALA plays a significant role in the development of the IHO S-100 standard by contributing to its refinement, testing, and implementation. IALA collaborates closely with IHO to ensure that the standard meets the needs of the maritime community, particularly regarding aids to navigation and related services.

IALA focuses on creating guidelines and recommendations for S-100-based products and services to promote technical interoperability, harmonization, and the secure, reliable, and efficient exchange of maritime information. Consequently, IALA participates in the development and testing of new proposals for S-100, addressing any issues or challenges that may arise.

This collaborative effort between IALA and IHO ensures that the S-100 standard meets the evolving needs of the maritime community and supports the broader goals of e-Navigation.

The current edition of S-100 includes a section for online data exchange. IHO is actively working to refine S-100 to incorporate the concept of online communications. The initial challenges identified during the testing of the first proposal, contributed by IALA, have been addressed.

As the S-100 standard becomes increasingly important for exchanging maritime information, it is essential to ensure the secure, reliable, and efficient exchange of S-100 data. Standardizing a common Web Service interface (based on IP technology) for the exchange of S-100-based products will enable greater technical interoperability, allowing

the same service interface to be used for information exchange, regardless of operational use, and making it common for various Maritime Services. As a result, IALA has completed the formulation of a guideline for S-100-based Web Services, known as G1157 Web Service-Based S-100 Data Exchange. The guideline offers a general introduction to the topic and an overview of existing standardization and guidance documents.

IALA and IHO are actively formulating the S-125 product specification, which is the data product specification for maritime navigation assistance services. This data can be used as a Nautical Publication Information Overlay (NPIO) within an Electronic Chart Display and Information Systems (ECDIS).

    - IALA and S-200

IALA has collaborated closely with IHO and other organizations to establish product specifications in its respective areas of responsibility, particularly in AtoNs and VTS. To oversee this domain, IALA has created a supervisory structure, as outlined in IALA Guideline G1087, and has been granted the numbering series S-201 to S-299 through IHO approval. IALA focuses on creating guidelines and recommendations for S-200 and services to promote technical interoperability, harmonization, and the secure, reliable, and efficient exchange of maritime information. Consequently, IALA participates in the development and testing of new proposals for S-200, addressing any issues or challenges that may arise.

IALA has begun developing S-100 compliant product specification for the S-200 domain to meet the requirements of product specifications for developing and implementing maritime services (MS) in the context of e-Navigation. IALA has completed the specifications of the S-201 series products, which have been registered with IHO GI and are now in the usage stage. S-201 provides a common structure for describing the position, characteristics, operational status and maintenance information of AtoNs. It also supports metadata such as power supply, moorings, fixing methods, and equipment types, allowing for comprehensive management of marine aids to navigation assets.

  - CYBER SECURITY

As the IALA domains are undergoing rapid changes with regards to digitalisation, as well as the maritime domain as a whole, cyber security becomes an important aspect. Cybercrime and the associated risks have expanded from IT environments to Operational Technology like AtoN, VTS and vessels. Therefore, cyber security must be taken into consideration for all maritime domains to provide resilience for safety, continuity and for reliability of systems and data.

Cyber security is not only a technical issue. Human behaviour is equally important. Moreover, cyber security should not be treated as a feature or an add-on to a (digital) technology, but incorporated from the earliest stages of development throughout the technology’s entire lifetime. This is what is commonly known as “security by design”.

It is a good practice to secure everything unless this is explicitly unwanted for a valid reason. This will make a technology more resilient to evolving (and yet unknown) risks in the future. With “secure”, at least encryption and validation of data authenticity should be considered. Furthermore, it is recommended that a suitable normative cybersecurity standard be selected and applied throughout the development and operational use of the technology.

IALA has created Guideline 1182 Cyber Security Specifics from an IALA perspective, which offers guidance on possible measures to be taken to mitigate cyber security risks in the IALA domain, IALA also created the recommendation R1024 Cyber Security from the IALA domain, which provides valuable information for developers and users of digital technologies in the IALA domains. The document includes references to the most applicable normative standards that may be applied to technology for AtoN, VTS and maritime services.

NOTES FOR CHAPTER 10

- A registry is simply a bookkeeping device where definitions/specifications are kept in organised locations known as registers. The registry eases the tasks of development of new things, by providing a centralised source for finding definitions/ specifications

- GPS Performance Standards, 2008

- United Nations Office for Outer Space Affairs, “Current and Planned Global and Regional Navigation Satellite Systems and Satellite-based Augmentations Systems”, 2011

- At the time of writing, further information on BeiDou may be found on the internet hhttp://www.en.beidou.gov.cn/csnclist.html

- Further information on Galileo can be found at the following website: http:/ / ec. europa. eu/ growth/ sectors/ space/ galileo/

- A 1kW transmitter will generally allow position fixing to better than 10 metres over a radius of about 200 Nautical Miles

- United Nations Office of Outer Space Affair