# Use Cases for Multi-Domain MCP Scenarios

# Document Revision

| # ID: Use Cases for Multi-Domain MCP Scenarios |  |  |  |
| --- | --- | --- | --- |
| # Version | # Author(s) | # Nature of change | # Date of adoption |
| 1.0 | Nikolaos Vastardis | Initial version |  |

# Purpose

The way MCP instances interact with each other, as well as with their associated services, is crucial for the envisaged distributed MCP operation (as this is described in the IALA G1183 [1] and G1191 [2] guidelines). This document intends to support the discussion on the interoperability of multiple MCP instances, each facilitating its own authentication domain.

# Introduction

To facilitate the discussion on the interoperability of MCP instances, a list of use cases has been compiled in the following sections. To keep things simple, Figure 1 presents an overview of the generic multi-domain MCP environment, including the main actors involved.

Figure 1: Overview of a generic environment with multiple MCP domains.

In scenario of Figure 1 we consider two MCP domains. Each domain has one user and one service, both associated with the same MCP instance. The overview of the environment does not display the internal components of each MCP instance (i.e. Identity Registry - MIR, Service Registry – MSR, Messaging Service – MMS), for the sake of clarity. We may however refer to them as actors, while describing the operations of each use case. Apart from that however, the involved MCP instances are seen as black boxes, providing the necessary functionality.

In the current context, two MCP domains are considered to be interconnected, when the actors from one domain, can contact and are permitted to interact with the actors of the other. This requires the root Certificate Authority (CA) certificates of each domain to be known and accepted by all actors in the other domain(s).

Table 1 provides a brief description of six different use cases, utilising the environment and actors present in Figure 1. The detailed information on each use case can be found in Section Error: Reference source not found. This is by no means an exhaustive list, but it represents the most important use cases that have been identified to date.

It should be noted that some use cases may already be included (in part or in full), within the process of other more complex use cases. However, if a use case has merit on its own, it is still be captured and presented independently.

Table 1: Use Case Overview

| Use Case | Description |
| --- | --- |
| Use-Case A | A user from an MCP domain wants to discover and use a technical service registered in a different domain. |
| Use-Case B | A consumer from an MCP domain wants to access data from a technical service registered in a different domain. |
| Use-Case C | A technical service from an MCP domain wants to validate the identity of a consumer registered in a different domain. |
| Use-Case D | A consumer from an MCP domain wants to validate the identity of a technical service registered in a different domain. |
| Use-Case E | A consumer from an MCP domain wants to subscribe to data from a technical service in a different domain. |
| Use-Case F | A technical service from an MCP domain wants to send data to a consumer from a different MCP domain. |

All digital communications in the aforementioned use cases are assumed to take place over IP, using SSL/TLS encryption (e.g. over HTTPS). This is made possible by employing security certificates of public internet CAs. These are not to be confused with the MCP certificates and MCP CAs. MCP certificates, issued by MCP instances (acting as CAs), are used solely in the context of MCP interactions, in order to verify the validity of requests themselves (including the maritime data transmissions), as per IALA G1183 [1]. An example of this can be found in the IEC SECOM [3] standard.

Finally, an important clarification is that the interactions of a consumer or a service with an MCP instance are not considered to be limited only to actions towards a remote system. For the purposes of this document, any action involving components or functionality of the MCP, including the validation of certificates or signatures and the checks against certificate revocation lists (CRL/OSCP), is considered to be an interaction with an MCP instance.

# Use Cases

## Use Case A – MSR Global Search

- Who: A consumer registered with a legitimate MCP instance.

- Wants to: Discover and use a technical service registered a different MCP domain.

- So that: It will obtain the information required for its task (e.g. plan a route).

### Description

This use case is taken directly from the IALA MSR technical specification (IALA G1191 [2]). A consumer W is a registered user of a legitimate MCP instance in Domain 1. They intend to make a trip towards the destination and will need to obtain some information necessary for the trip (e.g. for planning a route, such as Navigational Warnings or AtoN information). To find the appropriate service(s) (service Z) that can provide that information, the consumer performs a search query to their respective MCP, submitting as query parameters:

- The geometry of interest (e.g. route path);

- the service design MRN (see G1128 [3]) and versions which are compatible with their onboard equipment; and

- the service status which should be Released.

A visual overview of this use case, making use of the generic environment established previously can be seen in Figure 2. The figure includes the numbered steps of the ordinary sequency of events.

### Actors

- User using the consumer information system.

- Consumer (ECDIS, Route-Planning System or other information system).

- Maritime Technical Service (such as for Navigational Warnings, AtoN information) in its own MCP domain.

- MSR service of the consumer's MCP domain.

- MSR service of the technical service’s MCP domain.

Figure 2: The outline of the ordinary sequence of events for Use Case A.

- Maritime Technical Service (such as for Navigational Warnings, AtoN information) in different MCP domain.Frequency of Use

Typically triggered when the consumer is performing the task (e.g. planning for a trip) for a destination which is far away from their base of operations.

### Pre-Conditions

- The consumer is registered with a legitimate MCP instance.

- The consumer's MCP instance is interconnected with a compatible instance associated with a technical service, which meets the consumer's requirements.

- The MCP instance of the technical service is also a legitimate MCP instance.

- All actors support the search service request functionality.

- The consumer of the search has the ability to poll an MSR for further results.

- Identity verification is a given in this use case and is not described.

### Ordinary Sequence

- The consumer sends a search request to the MSR of its own MCP domain, including its route path and other criteria.

- The consumer's MSR searches its internal database and finds any matching registered entry.

- The consumer's MSR will reply with the local search result and supply a transaction ID that can be used when returning results from the other MCPs.

- The consumer's MSR propagates the search request (along with the geometry provided description of the route) to interconnected MCP instances, which potentially have services meeting the requirements specified in the received request.

- The MSRs of the interconnected MCP instances will validate the request and ensure that it comes from a reliable source, already vetted and allowed to perform service search.

- The interconnected MSRs will respond with the search results from their own internal databases. If a search via the search platform does not produce any results, an MSR should not respond with an empty result.

- The consumer's MSR will collect all valid responses identified by the transaction ID and compile a single list of search response entries.

- The consumer will receive the service information list by polling the MSR to get the results as they arrive via global search. If no results are received, an empty list is returned.

- The consumer will select which of the services it will contact, based on its own set of criteria.

- The consumer will contact the selected technical service (Service Z in the example of Figure 2), sending a request to be processed.

- The technical service will respond to the consumer with the data matching the request made.

- The data is rendered and displayed to the user.

### Post-Conditions

The correct maritime information is received by the consumer.

## Use Case B – Technical Service Information Retrieval

- Who: A consumer registered with a legitimate MCP instance.

- Wants to: Access data from a technical service registered in a different MCP domain.

- So that: It will obtain the information required for performing a task (e.g. plan a route).

### Description

This is basically a subset of the previous Use Case A. It involves the operations that take place when a consumer W needs to access information from a technical service Z, which is associated with a different MCP domain. The initial consumer request to the service should therefore first get validated and if the consumer belongs to a recognised and interconnected MCP domain, then the service should be allowed to respond. Likewise, the consumer is required to ensure that the response, along with any cryptographic signatures, is authentic and still valid.

A visual overview of this use case, making use of the generic environment established previously can be seen in Figure 3. The figure includes the numbered steps of the ordinary sequency of events.

Figure 3: The outline of the ordinary sequence of events for Use Case B.

### Actors

- User using the consumer information system.

- Consumer (ECDIS, Route-Planning System or other information system).

- Maritime Technical Service (such as for Navigational Warnings, AtoN information) in its own MCP domain.

- The MIR service of the consumer's MCP domain.

- The MIR service of the technical service’s MCP domain.

### Frequency of Use

Typically triggered when the consumer is required to access information from services that belong to different MCP domains.

### Pre-Conditions

- The consumer is registered with a legitimate MCP instance.

- The technical service is registered with a legitimate MCP instance.

- The consumer's MCP instance is interconnected with a compatible instance associated with a technical service, which meets the consumer’s requirements.

- The consumer has already all the necessary knowledge on how to access the technical service.

- Both MCP instances provide the facilities to validate MCP certificates.

### Ordinary Sequence

- The consumer will send a data retrieval request to the technical service, based on the already known contact information (e.g. from a previous MCP request).

- The technical service receives the incoming requests and validates the identity of the consumer and the validity of the incoming request, through the consumer’s MCP instance (revocation list check using the MIR).

- If the identity of the consumer is indeed valid, the technical service will send back a response with the requested data.

- The consumer will receive the technical service response and will first have to first validate the identity of the sender, through the technical service’s MCP instance (revocation list check using the MIR).

- Once the identity of the sender is verified, the consumer should also check the validity of the incoming data, through the included cryptographic signatures.

### Post-Conditions

The correct maritime information is received by the consumer and presented to the user.

## Use Case C – Consumer Identity Validation by Technical Service

- Who: A technical service registered with a legitimate MCP instance.

- Wants to: Validate the identity of a consumer registered in a different MCP domain.

- So that: It can establish a secure communication and allocate the appropriate resources.

### Description

This particular use case can be considered as a subset of both previous A and B use cases. It involves the operations that take place when a technical service Z needs to verify the identity of a consumer W, which is registered in a different MCP domain. The reason why this interaction was initiated is irrelevant, as it forms part of multiple operations such as data retrievals, subscriptions, notifications etc. Therefore, the identity of the consumer needs to be first validated and if it belongs to a recognised and interconnected MCP domain, then the technical service should be allowed to accept the consumer as a legitimate user.

A visual overview of this use case, making use of the generic environment established previously can be seen in Figure 4. The figure includes the numbered steps of the ordinary sequency of events.

Figure 4: The outline of the ordinary sequence of events for Use Case C.

### Actors

- User using the consumer information system.

- Consumer (ECDIS, Route-Planning System or other information system).

- Maritime Technical Service (such as for Navigational Warnings, AtoN information) in differentits own MCP domain.

- The MIR service of the consumer's MCP domain.

- The MIR service of the technical service’s MCP domain.

- Frequency of Use

Typically triggered when the consumer is required to send messages to services that belong to different MCP domains.

### Pre-Conditions

- The consumer is registered with a legitimate MCP instance.

- The technical service is registered with a legitimate MCP instance.

- The consumer's MCP instance is interconnected with a compatible instance associated with a technical service, which meets the consumer’s requirements.

- The consumer has already all the necessary knowledge on how to access the technical service.

- The consumer’s MCP instance provides the facilities to evaluate MCP certificates.

### Ordinary Sequence

- The consumer will send a relevant request, regardless of whether it is an update/response/notification, to the technical service.

- The technical service will receive the consumer’s request and will first have to validate the identity of the sender, through the consumer’s MCP instance (revocation list check using the MIR).

- Once the identity of the consumer is verified, the technical service should also check the validity of the incoming data, through the included cryptographic signatures, before proceeding to process the incoming message.

### Post-Conditions

The consumer message is received and processed by the technical service.

## Use Case D – Technical Service Identity Validation by Consumer

- Who: A consumer registered with a legitimate MCP instance.

- Wants to: Validate the identity of a technical service registered in a different MCP domain.

- So that: It can establish a secure communication and trust the transmitted information.

### Description

This particular use case can also be considered as a subset of both previous A and B use cases. It involves the operations that take place when a consumer W needs to verify the identity of technical service Z, which is registered in a different MCP domain. The reason why this interaction was initiated is irrelevant, as it forms part of multiple operations such as data uploads, subscriptions, notifications etc. Therefore, the identity of the technical service needs to be first validated and if it belongs to a recognisable and acknowledged MCP domain, then the consumer should be allowed to accept the technical service as legitimate.

A visual overview of this use case, making use of the generic environment established previously can be seen in Figure 5. The figure includes the numbered steps of the ordinary sequency of events.

Figure 5: The outline of the ordinary sequence of events for Use Case D.

### Actors

- User using the consumer information system.

- Consumer (ECDIS, Route-Planning System or other information system).

- Maritime Technical Service (such as for Navigational Warnings, AtoN information) in its own MCP domain.

- The MIR service of the consumer's MCP domain.

- The MIR service of the technical service’s MCP domain.

### Frequency of Use

Typically triggered when technical services need to communicate with consumers that belong to different MCP domains.

### Pre-Conditions

- The consumer is registered with a legitimate MCP instance.

- The technical service is registered with a legitimate MCP instance.

- The consumer's MCP instance is interconnected with a compatible instance associated with a technical service, which meets the consumer’s requirements.

- The technical service has already all the necessary knowledge on how to access the consumer.

- The technical service’s MCP instance provides the facilities to evaluate MCP certificates.

### Ordinary Sequence

- The technical service will send a relevant request, regardless of whether it is an upload/response/notification, to the consumer.

- The consumer will receive the technical service message and will first have to validate the identity of the technical service, through the technical service’s MCP instance (revocation list check using the MIR).

- Once the identity of the technical service is verified, the consumer should also check the validity of the incoming data, through the included cryptographic signatures, before proceeding to process the incoming message.

### Post-Conditions

The technical service message is received and processed by the consumer.

## Use Case E – Consumer Subscription to Technical Service

This use case is fairly similar to Use Case C described in Section 2.3, and is adequately discussed there.

## Use Case F – Technical Service Data Upload to Consumer

This use case is fairly similar to Use Case D described in Section 2.4, and is adequately discussed there.

References

| [1] | IALA, “G1183 'The provision of Maritime Connectivity Platform (MCP) identities'”. |
| --- | --- |
| [2] | IALA, “G1191 'Maritime Service Registry (MSR) – Technical Specification'”. |
| [3] | IEC, “IEC 63173-2: Maritime navigation and radiocommunication equipment and systems - Data interfaces - Part 2: Secure communication between ship and shore (SECOM),” IEC, 2022. |
| [4] | IALA, “G1128 'The Specification of e-Navigation Technical Services'”. |