IALA Guideline

Guideline number

EXCHANGING GNSS INTERFERENCE DATA FOR NAVIGATIONAL SAFETY

Edition x.x

Date (of approval by Council)

Revokes Guideline [number]

urn:mrn:iala:pub:gnnnn:ed1.0

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

1.	INTRODUCTION	6

1.1.	SCOPE	6

1.2.	Related documents and relations to thesE	6

1.3.	BACKGROUND	6

2.	NORMATIVE COMPONETS	7

2.1.	OPERATIONAL CONTEXT	7

2.2.	Information Exchange Principles	8

2.2.1.	Structured and Machine-Readable Information	8

2.2.2.	Interoperability and Standardisation	8

2.2.3.	Timeliness and Update Mechanisms	9

2.2.4.	Differentiation of Information Status	9

2.2.5.	Severity and Confidence Indication	9

2.2.6.	Complementarity with Existing Safety Services	9

2.3.	ALIGNMENT WITH S-100 AND S-200 FRAMEWORK	9

2.3.1.	Use of S-100 Data Modelling Principles	10

2.3.2.	S-200 Product Specification Considerations	10

2.3.3.	Service-Based Exchange Mechanisms	10

2.3.4.	Interoperability with Shipborne Systems	10

2.3.5.	Lifecycle and Version Management	11

3.	IMPLEMENTATION OF GNSS INTERFERENCE INFORMATION SERVICES	11

3.1.	Service Architecture	11

3.1.1.	Architectural Principles	11

3.1.2.	Functional Roles	11

3.1.3.	Service Interfaces and Behaviour	12

3.1.4.	Communication Patterns	12

3.1.5.	Resilience and Continuity	12

3.2.	3.2 Data Model Considerations	13

3.2.1.	Information Object Structure	13

3.2.2.	Core Data Elements	13

3.2.3.	Compatibility with Service-Based Exchange	13

3.3.	Interference Event Reporting	13

3.3.1.	Reporting Mechanisms	13

3.3.2.	Reporting Content	14

3.3.3.	Validation and Aggregation	14

3.4.	Interference Zone Dissemination	14

3.4.1.	Generation of Interference Zones	14

3.4.2.	Dissemination Mechanisms	14

3.4.3.	Operational Use	15

3.5.	Authentication and Data Integrity	15

3.5.1.	Authentication	15

3.5.2.	Data Integrity	15

3.5.3.	Time Synchronisation	15

3.5.4.	Audit and Traceability	15

4.	DATA MANAGEMENT	15

4.1.	Functional Roles of Stakeholders	15

4.1.1.	Data Producer	15

4.1.2.	Data Provider	16

4.1.3.	Service Provider	16

4.1.4.	Competent Authority	16

4.1.5.	Allocation and Combination of Roles	17

4.1.6.	Accountability and Traceability	17

4.2.	Dataset Packaging	17

4.3.	Guidance on Query Processing	17

4.4.	Guidance on Service Subscription	17

5.	DEFINITIONS	18

6.	ACRONYMS	18

List of Tables [List of Figures]

No table of figures entries found.

List of Figures [List of Figures]

No table of figures entries found.

List of Equations [List of Figures]

- No table of figures entries found.

# INTRODUCTION

## SCOPE

This Guideline provides guidance on the structured and interoperable exchange of information related to Global Navigation Satellite System (GNSS) interference that may affect maritime navigation. It supports the development and implementation of digital maritime services for the reporting, aggregation and dissemination of GNSS interference information between shipborne and shore-based systems within the framework of IALA e-Navigation and the S-100/S-200 environment.

The Guideline addresses operational and information exchange aspects only. It does not define GNSS receiver performance requirements, spectrum management measures, or technical detection equipment, nor does it replace or modify established Maritime Safety Information (MSI) or Global Maritime Distress and Safety System (GMDSS) services.

## Related documents and relations to thesE

This Guideline shall be read in conjunction with:

• IALA Guideline G1128 – Specification of e-Navigation Technical Services
• IALA Guideline G1157 – Web Service Based S-100 Data Exchange

• IALA Guideline G1157 – Web Service Based S-100 Data Exchange
• IALA Guideline G1089 – Vessel Traffic Services Manual
• IHO S-100 Universal Hydrographic Data Model
• IEC 63173-2 (SECOM) – Secure Exchange of S-100 based Products

This Guideline provides operational and architectural guidance. Technical realization should refer to SECOM and relevant S-100 documentation.

## BACKGROUND

Global Navigation Satellite Systems (GNSS) provide the primary source of Positioning, Navigation and Timing (PNT) information for maritime navigation. Over recent decades, maritime operations have evolved toward increasing digitalisation and system integration, resulting in extensive reliance on GNSS-derived data across both shipborne and shore-based environments.

On board vessels, GNSS information supports core navigation and control functions, including position fixing in Electronic Chart Display and Information Systems (ECDIS), Automatic Identification System (AIS) position reporting, radar overlay stabilisation, route monitoring, track control, and dynamic positioning operations. GNSS-derived timing is further used for synchronisation of onboard sensors, communication systems, and voyage data recording.

Within shore-based systems, GNSS data contributes to Vessel Traffic Service (VTS) traffic image correlation, sensor fusion, surveillance processing, and the maintenance of a coherent operational picture. Accurate and reliable PNT information is therefore essential not only for individual vessel safety but also for coordinated traffic management and maritime domain awareness.

GNSS signals are inherently weak and susceptible to interference. Disruptions may arise from unintentional sources, such as malfunctioning equipment or atmospheric phenomena, or from deliberate actions including jamming and spoofing. Such interference may result in degraded positioning accuracy, inconsistent sensor alignment, loss of PNT capability, or the presentation of misleading navigational information. These effects may reduce situational awareness, compromise decision-making, and increase navigational risk.

In many maritime regions, instances of GNSS interference have been observed with increasing frequency. The operational impact may extend beyond a single vessel and affect multiple ships or shore systems within a defined geographic area. Consequently, awareness of interference conditions is of relevance not only to the affected vessel but also to other vessels and competent authorities operating in the same region.

In the aviation domain, structured reporting of GNSS degradation has been standardised through ADS-B operational status messages, enabling air traffic services to assess signal reliability and maintain oversight of traffic safety. This model demonstrates the value of harmonised, machine-readable reporting mechanisms that support coordinated situational awareness.

A comparable structured and interoperable framework is required in the maritime domain. The exchange of GNSS interference information between ships and shore authorities should enable timely reporting, aggregation of observations, assessment of affected areas, and dissemination of relevant information to mariners. Such a framework supports enhanced situational awareness and contributes to navigational safety while remaining consistent with established Maritime Safety Information (MSI) and regulatory structures.

# NORMATIVE COMPONETS

## OPERATIONAL CONTEXT

The operational context of GNSS interference information exchange is founded on the need to maintain navigational safety in an environment where maritime operations are increasingly dependent on reliable Positioning, Navigation and Timing (PNT) data.

GNSS interference events may affect individual vessels, groups of vessels, or shore-based systems within a defined geographical area. As such, the impact of interference is rarely isolated and may influence traffic management, route monitoring, surveillance correlation and navigational decision-making beyond the initially affected platform.

The exchange of GNSS interference information is therefore intended to support coordinated situational awareness among shipborne systems, Vessel Traffic Services (VTS), maritime authorities and, where appropriate, other competent entities.

Within this operational context, the following functional interactions should be supported:

- Reporting of detected or suspected GNSS interference from shipborne systems or other detection sources to a designated shore-based authority or service provider;

- Aggregation, correlation and assessment of multiple interference reports by competent authorities or authorised service providers;

- Generation of assessed interference information, including the identification of affected geographic areas, severity levels and time validity;

- Dissemination of structured interference information from shore to vessels to enhance navigational awareness;

- Exchange of relevant information between authorities at national or regional level where cross-border coordination is required.

The operational objective of this exchange is to enable timely awareness of degraded GNSS conditions, support informed navigational decision-making, and facilitate coordinated traffic management measures where necessary.

The exchange mechanism is intended to complement existing navigational services and digital maritime service architectures. It shall not interfere with mandatory safety communications and shall be implemented in a manner consistent with recognised e-Navigation service frameworks.

The structured exchange of GNSS interference information enhances operational awareness and supports risk assessment processes. However, such exchange does not in itself constitute a Maritime Safety Information (MSI) broadcast.

Where interference conditions are assessed to meet the criteria for promulgation as MSI, dissemination shall be conducted in accordance with established MSI procedures and applicable international regulations, including SOLAS and relevant IMO instruments. The GNSS interference information exchange service may provide supporting input to MSI decision-making but does not replace or modify existing MSI obligations.

## Information Exchange Principles

The exchange of GNSS interference information shall be governed by principles that ensure interoperability, reliability, traceability and operational relevance within the maritime domain. These principles are intended to provide a consistent foundation for the development and implementation of GNSS interference information services under the IALA e-Navigation framework.

### Structured and Machine-Readable Information

GNSS interference information shall be structured and machine-readable to enable automated processing, correlation and display within shipborne and shore-based systems.

Information shall be encoded in a manner compatible with S-100 based data models and service exchange mechanisms. The structure shall support clear identification of:

- the reporting entity or detection source;

- the time of detection or assessment;

- the geographic location or affected area;

- the classification of interference (where known);

- the assessed severity or impact level;

- the confidence level associated with the information.

Unstructured free text should be avoided unless required to provide supplementary explanatory information.

### Interoperability and Standardisation

GNSS interference information exchange shall support interoperability between systems from different manufacturers, service providers and national administrations.

To achieve this, services should:

- use harmonised data models;

- apply consistent terminology and classification schemes;

- conform to recognised S-100/S-200 based product specifications;

- implement service interfaces consistent with relevant IALA guidance.

Interoperability shall extend to cross-border information exchange where interference conditions affect more than one maritime administration.

### Timeliness and Update Mechanisms

The exchange mechanism should support timely reporting and dissemination appropriate to the operational context.

Where near real-time awareness is required, the service should enable prompt transmission of new or updated information. Update mechanisms shall clearly indicate:

- new interference events;

- modifications to previously issued assessments;

- withdrawal or cancellation of information.

Service implementations should minimise unnecessary data transmission and ensure efficient use of communication bandwidth.

### Differentiation of Information Status

GNSS interference information services shall clearly distinguish between:

- raw observations or unvalidated reports;

- validated detection events;

- aggregated or assessed interference zones;

- authoritative advisories issued by competent authorities.

This differentiation is necessary to prevent misinterpretation and to support appropriate operational decision-making. Systems displaying the information should allow users to recognise the status and origin of the data.

### Severity and Confidence Indication

Each interference report or assessed interference zone shall include an indication of severity and an associated confidence level, where practicable.

Severity may reflect the operational impact on navigation, while confidence may indicate the degree of certainty associated with the detection or assessment. These indicators support risk-based decision-making and prioritisation of operational responses.

### Complementarity with Existing Safety Services

The GNSS interference information exchange service shall complement, and not duplicate, established Maritime Safety Information (MSI) and GMDSS services.

Structured interference information may support internal assessment processes and situational awareness. Where promulgation as MSI is required, this shall be conducted through established MSI procedures in accordance with applicable international regulations.

The design of the service shall ensure clear separation between structured situational awareness data exchange and mandatory broadcast safety communications.

## ALIGNMENT WITH S-100 AND S-200 FRAMEWORK

The implementation of GNSS interference information exchange shall be aligned with the IHO S-100 Universal Hydrographic Data Model and associated service-based exchange mechanisms. Alignment with S-100 ensures interoperability with existing and emerging maritime digital products and supports integration within the broader e-Navigation architecture.

### Use of S-100 Data Modelling Principles

GNSS interference information shall be modelled using S-100 data modelling principles to ensure consistency of feature definitions, attributes, enumerations and metadata. The data model should clearly define object classes representing, at a minimum:

- interference event reports;

- assessed interference areas or zones;

- associated temporal validity;

- source attribution and confidence indicators.

The model shall support spatial geometry consistent with S-100 standards, enabling portrayal within compliant shipborne and shore-based systems. Where geometry is provided, coordinate reference systems and precision shall be explicitly defined in accordance with S-100 requirements.

### S-200 Product Specification Considerations

GNSS interference information services should be developed under the S-200 series framework for S-100 based product specifications. An S-2XX Product Specification should define:

- the feature catalogue;

- portrayal considerations;

- encoding rules;

- exchange set structure (if applicable);

- service metadata;

- update and cancellation mechanisms.

The Product Specification shall clearly describe the intended operational use and identify dependencies on other S-100 products, such as S-101 Electronic Navigational Charts or other S-100 based overlays.

### Service-Based Exchange Mechanisms

Where GNSS interference information is exchanged via online services, the implementation shall conform to S-100 Part 14 provisions for service-based data exchange. Service definitions should include:

- service identification;

- service interfaces and operations;

- supported communication patterns (e.g. request-response, subscription-based updates);

- metadata and discovery mechanisms;

- update and withdrawal procedures.

Where secure exchange is required, implementation should be consistent with IEC 63173-2 (SECOM) or equivalent secure exchange frameworks adopted within the maritime domain.

### Interoperability with Shipborne Systems

The alignment with S-100 shall enable GNSS interference information to be displayed, where appropriate, as an overlay within S-100 capable shipborne systems. Portrayal rules should ensure that interference information is clearly distinguishable from other navigational data and that the operational status (e.g. reported, assessed, withdrawn) is visible to the user.

The design of portrayal should support situational awareness without obscuring critical navigational information. Where multiple S-100 products are displayed concurrently, harmonised portrayal principles should be applied to avoid ambiguity.

### Lifecycle and Version Management

Product and service specifications developed under the S-200 framework shall include clear version control and lifecycle management procedures. Changes to feature definitions, classification schemes or encoding structures shall be documented and communicated in accordance with S-100 governance processes.

Backward compatibility and migration considerations should be addressed to ensure continuity of service and minimise disruption to operational systems.

# IMPLEMENTATION OF GNSS INTERFERENCE INFORMATION SERVICES

## Service Architecture

GNSS interference information services shall be implemented within a structured and clearly defined service architecture that supports interoperability, operational reliability and scalability across maritime stakeholders. The architecture shall enable the systematic detection, reporting, validation, assessment and dissemination of GNSS interference information in accordance with the IALA e-Navigation framework.

### Architectural Principles

The service architecture should:

- support end-to-end information flow from detection source to service consumer;

- ensure clear allocation of responsibilities;

- enable secure and resilient communication;

- support integration with existing maritime digital service infrastructures;

- accommodate national implementation differences while maintaining interoperability.

Deployment may be shore-based, cloud-based or hybrid, depending on operational and national requirements. Regardless of deployment model, the service shall ensure continuity appropriate to its intended safety function.

### Functional Roles

The architecture shall define and document functional roles, including but not limited to:

Data Producer
An entity or system that detects or observes GNSS interference. This may include shipborne GNSS receivers, dedicated monitoring stations, VTS sensor systems or other authorised detection platforms.

Data Provider
An entity responsible for receiving, validating, correlating and assessing interference reports. The Data Provider may aggregate multiple reports to determine spatial extent, severity and confidence levels.

Service Provider
An entity responsible for distributing validated or assessed GNSS interference information to authorised users. The Service Provider may manage access control, subscriptions, system monitoring and lifecycle management of information products.

These roles may be performed by separate organisations or combined within a single competent authority. Responsibilities for validation, approval, update and withdrawal of information shall be clearly defined to ensure traceability and accountability.

### Service Interfaces and Behaviour

GNSS interference information services should be specified in accordance with recognised IALA guidance for e-Navigation technical services. The service specification should define:

- service identification and purpose;

- operational context and constraints;

- supported operations (e.g. report submission, query, subscription, update, cancellation);

- input and output message structures;

- error handling and acknowledgement procedures;

- performance expectations.

The service should define clear behavioural rules governing the lifecycle of interference information, including creation, modification, update, suspension and withdrawal.

### Communication Patterns

The architecture should support multiple communication patterns as required by the operational context:

- Synchronous request-response mechanisms, enabling query-based retrieval of current interference information;

- Asynchronous or subscription-based mechanisms, enabling near real-time notification of new or updated information;

- Event-driven updates, triggered by significant changes in assessed interference conditions.

Communication mechanisms shall minimise unnecessary data transmission while ensuring timely awareness of relevant operational changes.

### Resilience and Continuity

The service architecture shall ensure resilience commensurate with its safety relevance. This includes:

- redundancy of critical components where appropriate;

- monitoring and logging of service performance;

- clearly defined fallback or degraded-mode procedures;

- controlled versioning and change management processes.

The design should prevent single points of failure and support continuity during partial system outages.

## 3.2 Data Model Considerations

The data model for GNSS interference information services shall ensure consistency, clarity and interoperability within the S-100 framework.

### Information Object Structure

At a minimum, the data model shall support representation of:

- individual interference event reports;

- aggregated or assessed interference zones;

- associated metadata describing validity, source and status.

Each information object shall be uniquely identifiable and include attributes sufficient to support automated processing and visualisation.

### Core Data Elements

Data elements should include:

- temporal attributes (e.g. time of detection, time of assessment, validity period);

- spatial attributes (e.g. position, area geometry);

- identification of affected GNSS constellation(s), where known;

- classification of interference type (e.g. jamming, spoofing, suspected, unknown);

- severity or operational impact indicator;

- confidence level;

- information status (e.g. reported, validated, assessed, withdrawn).

The model shall clearly differentiate between raw observations, validated detection reports and assessed operational information to avoid misinterpretation.

### Compatibility with Service-Based Exchange

Where a service-based implementation is adopted, the data model shall support efficient encoding, transmission and incremental updating consistent with S-100 service metadata provisions. The model should support:

- version control;

- update notifications;

- withdrawal or cancellation indicators;

- clear indication of data lifecycle status.

## Interference Event Reporting

GNSS interference event reporting shall enable the structured transmission of detected or suspected interference from shipborne systems or other authorised detection sources to a designated shore-based authority or service provider.

### Reporting Mechanisms

Reporting may be:

- voluntary;

- automated based on predefined detection thresholds;

- manually initiated by an operator.

The reporting interface shall clearly define required and optional data elements and provide acknowledgement of receipt.

### Reporting Content

Reports shall include sufficient information to enable validation and assessment, including:

- time of occurrence;

- geographic position of detection;

- nature of observed signal degradation;

- affected constellations (if identifiable);

- relevant signal quality indicators;

- reporting entity identification.

The reporting mechanism shall not interfere with mandatory distress or safety communications and shall operate proportionately to operational requirements.

### Validation and Aggregation

Upon receipt, the Data Provider should validate and, where appropriate, correlate reports from multiple sources to identify patterns, spatial extent and likely severity of interference.

## Interference Zone Dissemination

Following validation and assessment, competent authorities may generate structured information describing assessed interference zones or conditions.

### Generation of Interference Zones

An interference zone may represent:

- a defined geographic area affected by confirmed or probable interference;

- an estimated severity or risk level;

- a defined time validity;

- associated confidence level.

The methodology used for aggregation and zone creation should be documented by the responsible authority.

### Dissemination Mechanisms

Dissemination shall support timely situational awareness and may include:

- query-based retrieval;

- subscription-based updates;

- event-driven notifications.

The dissemination mechanism shall support efficient updating and withdrawal of information. When an interference condition ceases or is reassessed, the service shall clearly indicate cancellation or modification to prevent persistence of outdated information.

### Operational Use

The dissemination of interference zones is intended to support navigational awareness and risk-based decision-making. It does not, by itself, constitute a Maritime Safety Information broadcast unless formally promulgated under MSI procedures.

## Authentication and Data Integrity

GNSS interference information services shall incorporate appropriate technical and procedural measures to ensure authentication, integrity and protection against misuse.

### Authentication

Mechanisms shall be implemented to verify the identity of reporting entities and service consumers. Access control policies shall define authorised participants and permissible operations.

### Data Integrity

Transmitted data shall be protected against unauthorised modification, replay or falsification. Secure communication protocols shall be used for data exchange. Where digital signatures are applied, the signature and associated timestamp shall be cryptographically bound to the transmitted dataset.

### Time Synchronisation

Time synchronisation between communicating entities shall be maintained at a level appropriate to the operational requirements of the service to ensure correct interpretation of temporal attributes and prevent replay vulnerabilities.

### Audit and Traceability

Service providers should maintain appropriate logging and audit capabilities to support incident investigation, performance monitoring and regulatory compliance.

# DATA MANAGEMENT

## Functional Roles of Stakeholders

The effective management of GNSS interference information requires clearly defined functional roles and associated responsibilities. The allocation of these roles shall support accountability, data integrity, traceability and continuity of service throughout the information lifecycle.

The implementation of GNSS interference information services may involve several functional stakeholders. While organisational structures may vary between administrations, the following roles should be defined within the service architecture.

### Data Producer

The Data Producer is the entity or system responsible for the initial detection or observation of GNSS interference. Data Producers may include:

- shipborne GNSS receivers or integrated navigation systems;

- dedicated monitoring stations;

- shore-based sensor networks;

- authorised detection platforms operated by competent authorities.

The Data Producer is responsible for generating structured or semi-structured reports in accordance with the defined reporting interface. Where automated reporting mechanisms are implemented, detection thresholds and reporting criteria should be clearly defined and documented.

### Data Provider

The Data Provider is responsible for receiving, validating, correlating and assessing incoming interference reports. This role includes:

- verification of report completeness and format compliance;

- assessment of plausibility and consistency;

- correlation of multiple reports to identify spatial patterns;

- determination of severity and confidence levels;

- creation of structured interference objects or zones;

- lifecycle management of validated information.

The Data Provider may apply predefined validation rules and operational procedures. Where appropriate, manual oversight by qualified personnel may complement automated validation processes.

### Service Provider

The Service Provider is responsible for the controlled distribution of validated or assessed GNSS interference information to authorised service consumers. Responsibilities may include:

- managing service access and authentication;

- maintaining subscription and notification mechanisms;

- ensuring timely dissemination of updates and withdrawals;

- maintaining service availability and performance;

- ensuring compliance with cybersecurity requirements.

The Service Provider shall ensure that only validated or authorised information is disseminated and that updates or cancellations are clearly communicated to service consumers.

### Competent Authority

A designated Competent Authority may retain overall responsibility for governance of the service. This may include:

- defining operational procedures;

- approving classification schemes;

- determining when interference conditions warrant promulgation as MSI;

- establishing data retention and archival policies;

- ensuring compliance with national and international regulatory frameworks.

### Allocation and Combination of Roles

Depending on national arrangements, the roles of Data Producer, Data Provider and Service Provider may be performed by separate organisations or consolidated within a single authority. Regardless of organisational structure, the allocation of responsibilities shall be formally defined and documented.

Clear procedures shall be established for:

- validation and approval of information prior to dissemination;

- update, modification or withdrawal of previously issued information;

- audit and traceability of decisions and data changes;

- management of erroneous or malicious reports.

### Accountability and Traceability

The data management framework shall ensure that each interference report and derived information object can be traced to its source and processing history. Unique identifiers, timestamps and version control mechanisms shall be applied to support transparency and post-incident analysis.

Logging and audit mechanisms should be implemented to record:

- receipt of reports;

- validation and assessment actions;

- dissemination events;

- updates and withdrawals.

## Dataset Packaging

## Guidance on Query Processing

## Guidance on Service Subscription

# DEFINITIONS

[TBC]

# ACRONYMS

[Acronym]	[Acronym]

[Acronym]	[Acronym]

…	…