Input paper:  DTEC6-6.2.3.1

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☐ ARM	☐  ENG		☐ PAP			☒ Input

☒ DTEC	☐ VTS					☐ Information

Agenda item 			n.n

Technical domain/ Task number 	…………………………………

Author(s)/Submitter(s)			China MSA

# Proposal to Supplement the Guideline for Shore VDES Infrastructure with aN appendix on Data Service Interface Specification

# Summary

At the IALA DTEC1 meeting, it was proposed to develop a new Guideline on VHF Data Exchange System (VDES) Shore-based Infrastructure to integrate the contents of Recommendations R0124 and R1007. ALLFORLAND further refined the draft during DTEC4 and requested member assistance for its completion. At the IALA DTEC5 meeting, China MSA proposed adding a specification regarding the VDES shore-based system data service interface to the Guideline. Following review, the Committee decided that China MSA would be responsible for drafting an independent annex for this Guideline. This proposal outlines the framework for the "VDES Shore-based System Service Interface Specification" to serve as a reference for the subsequent development of the complete specification.

## Purpose of the document

The purpose of this document is to provide the "VDES Shore-based Service Interface Specification" as an annex to the VDES Shore-based Infrastructure Guidelines. This aims to incorporate the VDES data application protocol, based on the S-100 Universal Hydrographic Data Model, into the VDES infrastructure standards. The Committee is requested to consider the attached draft framework and include it as part of the Guideline for continuous improvement in future sessions.

## Related documents

[1]ITU-R M.2092-1, Technical characteristics for a VHF data exchange system in the VHF maritime mobile band, February 2022

[2]IALA G1117, VHF Data Exchange System (VDES) Overview, December 2022

[3]IALA R1007, The VHF Data Exchange System (VDES) for Shore Infrastructure, June 2017

[4]IALA R0124 Ed2.2, The AIS Service, December 2012

[5]IHO S-100 Universal Hydrographic Data Model, Edition 5.2.0, 2024

[6]IALA S-201 Aids to Navigation Information Product Specification, Ed.2.0.0, 2025

# Background

VDES shore-based infrastructure facilitates data exchange between shipborne terminals and other shore-based users. Following traditional AIS shore-based practices, user interfaces typically rely on NMEA sentences to transmit AIS messages, which is development-intensive and error-prone. As VDES capabilities far exceed traditional AIS, continued reliance on low-level sentence-based interfaces would increase complexity and restrict the interaction of new data types.

Data modeled under S-100 provides standardized structures and semantic identifiers. Competent authorities and users can integrate such data directly into S-100-compliant systems, ensuring consistency and interoperability with other maritime services. Adopting a VDES data application protocol based on the S-100 model abstracts the underlying communication details, providing upper-layer users with a concise and consistent data view, thereby reducing integration difficulty and enhancing security.

# Discussion

This proposal provides the VDES Shore-based Service Interface Specification as an annex to the Guideline. This document primarily defines the data exchange standards between the VDES shore-based system and external business systems (e.g., VTS, Port Authorities). The main technical content includes:

General Architecture

Data Model and S-100 Encapsulation

Interface Definitions

Security Mechanisms

Service Discovery

Specific chapter arrangements and technical requirements are detailed in the Appendix 1.

# Action requested of the Committee

The Committee is requested to:

Consider the draft framework of the VDES Shore-based Service Interface Specification, agree to include it as an independent annex to the Guideline, and continue its refinement in subsequent work.

# VDES Shore-based Service Interface Specification

# Scope

## Introduction

To ensure VDES can efficiently serve the growing demand for maritime digitalization and to reduce the complexity for users accessing the VDES network, establishing a standardized and easily integrated service-layer data interface is crucial. This annex defines the data exchange interface specification between the VDES system and external users (e.g., VTS, Ports) or other external systems. This specification aims to shield users from low-level physical link details, providing a high-level service interface based on geospatial and business logic.

## Scope of the Specification

This annex specifically specifies the following contents:

Data Model

- Defines S-100 Feature Classes for describing VDES transmission tasks, including content payloads, target objects (specific vessels or geographic areas), and business attributes.

- Defines data structures for reporting Transmission Status and Reception Reports.

- Defines complex attributes for controlling transmission behavior, such as scheduling strategies (time windows, periodicity) and channel preferences (ASM, VDE-TER, VDE-SAT).

Service Interaction

- Defines interaction logic based on "Publish/Subscribe" and "Request/Response" patterns.

- Specifies how to use Geography Markup Language (GML) to encode the aforementioned data models.

Underlying Mapping

Describes how the high-level objects defined in this specification map to the underlying Presentation Interface (PI) sentences.

Note: This specification does not redefine the VDES air interface or physical layer parameters; these contents are covered by ITU-R M.2092 and IEC test standards. This specification focuses on the exchange format of application-layer data before it enters the physical transmission queue.

## Applicable Entities

VDES Service

The system responsible for implementing the the server-side interface as defined in this specification. Its functions include receiving requests, parsing geospatial and scheduling constraints, and dynamically allocating physical base station (PSS) resources. It further converts these requests into Presentation Interface (PI) binary message structures for downlink to the physical equipment.

Service Consumers

External business systems that need to communicate using VDES links. Typical users include but are not limited to:

- Vessel Traffic Services (VTS): Used for transmitting navigational advice, traffic organization instructions, and related information.

- Maritime Safety Information (MSI) Distribution Systems: Used for broadcasting weather warnings and navigational warnings in specific sea areas.

- Port and Logistics Enterprises: Used for sending berth plans and logistics updates to inbound vessels.

- Government Regulatory Platforms: Used for acquiring ship reports and other regulatory data.

## Service Limitations

This specification assumes that the VDES service possesses topology awareness and unified scheduling capabilities for the underlying physical base station network.

Due to the stochastic nature of the radio channel, successful reception by the terminal cannot be confirmed unless an acknowledgment (ACK) mechanism is enabled.

# Normative References

- IEC 63514 - VDES Technical requirements

- ITU-R M.2092 - VDES Technical characteristics

- IALA G-1117 - VDES Overview

- IHO S-100 Ed. 5.2.0 - Universal Hydrographic Data Model

# Terms, Definitions and Abbreviations

TBD

# General Architecture

Overview

The interface defined in this document is positioned between the user system and the VDES service, primarily facilitating the interaction between users and the VDES Logical Shore Station (VDES-LSS).

This architecture aims to achieve the following objectives:

- Interoperability: Utilizing IEC 63173-2 (SECOM) as a unified interface to encapsulate S-100 based data formats, ensuring that user systems from various vendors can seamlessly access the VDES network.

- End-to-End Security: Implementing dual signature and multi-layer encryption mechanisms to ensure the confidentiality and integrity of data throughout the entire path from the shore-based user to the shipborne terminal.

- Compatibility: Supporting both real-time interaction over IP networks and store-and-forward interaction via the Maritime Messaging Service (MMS).

## System Components

The system architecture consists of the following core components:

User

Definition: External systems (e.g., VTS, shipping companies, meteorological service providers) that need to transmit or receive data via VDES.

Role: Acts as a Consumer or a Data Client.

Relationship Mapping: Corresponds to the Clients connected to the VDES-LSS via the External Interface.

VDES Logical Shore Station (VDES-LSS)

Definition: The core data routing component of the VDES shore-based system, responsible for filtering, converting, and routing data.

Role: Acts as a Provider or a Data Server, exposing RESTful APIs to external entities.

Function: Serves as the access point for users, abstracting the complexity of the underlying Physical Shore Station (PSS) and the VHF Data Link (VDL).

## Direct IP Interaction based on SECOM

This is the preferred mode, suitable for scenarios involving a direct connection between the user and the VDES-LSS.

Architecture Description

Shore-based users directly invoke the SECOM interfaces (HTTPS) exposed by the VDES-LSS. Upon receiving the data, the VDES-LSS converts it into the VDES Presentation Interface (PI) binary message structure according to internal logic. It then schedules the Physical Shore Station (PSS) for transmission to the vessel via the VDES-PCU (Physical Control Unit).

Protocol Stack and Data Encapsulation

- Transport Layer: HTTP/1.1 over TLS 1.2/1.3. Mutual TLS (mTLS) must be implemented to ensure the legitimacy of the user identity.

- Application Layer:

  - Interface Protocol: IEC 63173-2 SECOM RESTful API.

  - Payload Format: S-100 Dataset.

  - Encryption Protection: S-100 payloads are encrypted using AES-256 (optional, depending on data sensitivity).

  - Signature Protection (Dual Signature):

Data Signature: The data generator (user) signs the original S-100 content to ensure data traceability and non-repudiation.

Envelope Signature: The SECOM transport layer signs both the metadata and the payload as a whole to ensure transmission integrity.

## Interaction based on MMS Gateway

This path is applicable to scenarios involving store-and-forward communication between the user and the VDES-LSS via the Maritime Messaging Service (MMS).

In this mode, the VDES shore-based system acts as an MMS VDE-TER Gateway within the MMS network. Users access the system through the MMS gateway.

The specific architectural description is as follows:

- The user sends a request to the MMS SECOM Gateway.

- The MMS SECOM Gateway verifies the envelope signature and extracts the S-100 payload.

- The payload is encapsulated into SMMP (Security Layer) and MMTP (Transport Layer) protocol packets.

- The data packets are transmitted to the VDES-LSS via the MMS router network.

- The VDES-LSS parses the MMTP packets, converts them into the VDES Presentation Interface (PI) binary message structure, and schedules the physical base station for transmission to the vessel via the VDES-PCU.

Data Model and S-100 Encapsulation Definition

VDES Message S-100 Product Specification

This section defines a dedicated S-100 Product Specification designed to encapsulate VDES data and signaling.

ASM Transmission

ASM Transmission Data Structure

| Field | Type | Description |
| --- | --- | --- |
| Message Sequence ID | String | A unique identifier used for deduplication and sequencing. |
| Source ID (MRN) | String | The Maritime Resource Name (MRN) of the sender. |
| Destination ID (MMSI) | String | The MMSI or MRN of the receiver (specific values are used for broadcasting). |
| Destination ID (MRN) | String | The Maritime Resource Name (MRN) of the receiver. |
| ASM Channel | Boolean | Specifies the physical channel used for transmission (e.g., ASM1, ASM2, etc.). |
| Format | Boolean | Indicates the encoding method and whether Forward Error Correction (FEC) is included. |
| Data Length (Bytes) | Integer | The length of the payload in bytes. |
| Data Payload Content | String | The actual transmitted binary data or text (requires Base64 encoding). |

ASM Reception

...

Data Protection

Payloads shall be encrypted using the Advanced Encryption Standard (AES).

Interface Definitions

Base Capability Interfaces

- Capability Query: Allows users to query the protocol versions and communication capabilities supported by the VDES system.

- Heartbeat: Used to monitor the online status of the VDES management system.

- Upload

- Acknowledgement (ACK)

Data Interaction Interfaces

- Upload

- Acknowledgement (ACK)

...

Security and Key Interfaces

Key Exchange: Facilitates the exchange of temporary AES symmetric keys required for decrypting S-100 payloads at the receiving end.

Security Mechanisms

Transport Layer Security

Dual Signature Authentication

Service Discovery