Input paper:  DTEC6-6.2.3.10

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					X  Input

X  DTEC	□  VTS							□ Information

Agenda item 			6.1

Technical Domain / Task Number 2	6.3.5

Author(s) / Submitter(s)			Atsushi KATO (Mr.)			Koichi YOSHIDA (Mr.)

Director of Policy Research Dept.	Research Fellow

The Ocean Policy Research Institute (OPRI)

The Sasakawa Peace Foundation (SPF)

1-15-16 Toranomon, Minato-ku, Tokyo 105-8524, Japan

# Discussion on the draft Guidelines on VDES resource sharing and coordination/cooperation

# 1	Summary

This document provides the outcome of the discussion made at DTEC WG3 Intersessional meeting held in February 2026, and also provide discussion points on the development of the Guidelines for VDES resource sharing and coordination/cooperation.

## 2	RELATED DOCUMENTS

IALA G1060, G1117,

ITU-R M2092-1

ENAV30-5.1.3.3, DTEC1- 5.1.3.5. DTEC2-5.2.3.4, DTEC3-5.2.3.2, DTEC4-6.2.0.8, DTEC5-5.2.3.8

# 3	BACKGROUNDS

Ocean Policy Research Institute (OPRI) of Sasakawa Peace Foundation (SPF) Japan proposed a new work item of development of guidelines for VDES resource sharing and coordination/cooperation at IALA ENAV 30 by the document EVAV30-5.1.3.3. ENAC Committee agreed with the proposal and put the proposal to IALA Council.

IALA Council in 2022 approved the proposed work item of “Develop Guidelines on VDES resource sharing and coordination/cooperation” has been approved by the Council in 2022 for a work of DTEC, as shown in the list of IALA Committee work programme 20223-2027 as DTEC-6.3.5.

OPRI submitted the documents ENAV30-5.1.3.3, DTEC1- 5.1.3.5. DTEC2-5.2.3.4, DTEC3-5.2.3.2, DTEC4-6.2.0.8 and DTEC.5-6.2.3.8 for the development of the draft guidelines., and submitted a discussion paper to DTEC WG3 Intersessional meeting held in February 2026. The WG3 intersessional meeting discussed on this issue and agreed way forward to further develop the draft guidelines.

# 4	Terms of refernce of the work item

The terms of reference approved by the Council are as follows:

Committee work item No. DTEC-6.3.5

Title: Develop Guidelines on VDES resource sharing and coordination/cooperation

Committee: DTEC

Description: Develop a guideline that provides framework of VDES resource sharing and coordination / cooperation for VDES satellites providers, VDES land-stations and VDES users to realize smooth and effective VDES communications on both official and private communications.

Period of work: 2023-2027

# 5	DISCUSSIONS made at DTEC Intersessional meeting in February 2026

5.1 Documentation

After the DTEC5, the project leader uploaded the draft guidelines to the WG3 document holder of “Guidelines resource sharing”, and asked DTEC WG3 members to input their comments to the holder, or send comments to the project leader through e-mail.

After receiving some comments, the project leader, taking into the comments, developed the second draft and uploaded it to WG3 document holder.

Mr. Nishimura sent his commenting document at the beginning of the DTEC WG3 intersessional meeting.

5.2 Terminology of “resource sharing”

"resource sharing" is used in M2092-1 to indicate frequency (channel) and time slot resource sharing on TMDA. However, term “resource” used in the draft means facility for VDE communications. There were concerns raised that such usage of term “resource sharing” creates confusion among the document users. There was a proposal of use of “facility” instead of “resource”.

The intersessional meeting agreed to indicate the two options “resource sharing” and “facility sharing” in square brackets for further consideration.

5.3 Comments from Mr. Nishimura

Having noted the important points for consideration described in the commenting document submitted by Mr. Nishimura, the intersessional meeting agreed to place markings of “place holder” in the draft where further consideration and development of text are required based on the proposals. The project leader suggested that some IMO documents regarding legal procedures of participation of satellite communication system into IMO can be used for the development od draft texts of the IALA guidelines for resource sharing.

5.4 work plan

Noting the completion date of this project is 2027, the project leader proposed to work on the draft during DTEC5 at WG3 meeting and after the DTEC5 until DTEC6 by correspondence using the IALA DTEC WG3 document holder.

5.5 Resource sharing and coordination by shore-based infrastructures

DTEC WG3 already agreed that resource sharing and coordination by shore-based infrastructures shall be prescribed in the guidelines for shore-based infrastructures.

# 6	draft text of the committee

ANNEX of this document contains the draft guidelines on resource sharing and coordination/cooperation as of February 20, 2026. The text boxes are added for the purpose of further discussion and drafting.

# 7	ACTION requested of the committee

The Committee is invited to:

- note the discussion made at DTEC WG3 intersessional meeting (paragraph 5);

- instruct WG3 to further develop the draft guidelines for VDES resource sharing and coordination (paragraph 5.4); and

- instruct the project leader of the guidelines to continue drafting through correspondence with WG3 members for submission to DTEC7.

* * *

ANNEX

IALA Guideline

Gnnnn

Guideline title FOR VDES [RESOURCE] FACILITY] SHARING, COORDINATION AND COOPERATION

Draft: 2026-02-20

Edition X.X

Date of Approval of Council:

Urn:mrn:iara:pub:gnnnn

DOCUMENT REVISION

Revisions to this document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |

CONTENTS

(to be developed after drafting)

List of Tables

(to be developed after drafting)

List of Figures

(to be developed after drafting)

# INTRODUCTION

## PURPOSE OF TEH DOCUMENT

This document provides guidelines on methodology of VDES [resource] [facility] sharing, coordination and cooperation to those who wish to join such international activities as voluntary basis. This document does not provide regulatory or obligatory framework on operation of VDES. VDES shore-based resource sharing is specified in another document (Guidelines for VDES shore infrastructures).

## BACKGROUND

VDES communication shall follow ITU-R M.2092-1 [1]. IALA Guidelines G.1117 [2] provides many possible usages of VDES. Under these circumstances. It is beneficial to share and coordinate VDES resources (ship stations, shore-based stations, satellite stations) in order to implement the usages of VDES.

ITU-R M2092-1 Annex 5 VDE-SAT 3.1.9 mentions "Multiple satellite networks can be combined to represent a roaming network". However, ITU-R M.2092-1 and in particular its Annex 5 do not give detailed specification of such "roaming". The term “roaming”, as used in mobile telecommunication, may not be appropriate in the context of VDES satellite operations. Instead, the term 'resource sharing and coordination' better reflects the technical and operational arrangements needed among multiple satellite operators.

ITU-R M.2092-1 Annex 3 ASM specifies the Scheduled broadcast message (7.4), Broadcast message (7.5), Scheduled Addressed message (7.6), Addressed message (7.7), Acknowledgement message (7.8) and Geographical multicast message (7.9), noting that scheduled addressed message, addressed message and acknowledge message contain destination ID to which VDES systems shall transfer the message. This means the VDES message transmitters, VDES satellites, DES shore-based stations and message recipients need to coordinate in order to realize and complete the VDES communications.

It is beneficial for users of VDES to provide guideline for VDES [resource] [facility] sharing, coordination and cooperation which allows VDES messages originated by a VDES station can be transferred through any VDES satellites.

## DEFINITIONS

For the purpose of this document, following definitions apply:

NOTE: "roaming" is not used in this document.

### [resource] [facility]

### [“resource”] [“facility”] means facilities which provide capability of VDES communications, such as ship stations, shore-based stations, satellite stations.

### NOTE: In ITU-R M.2092-1, “radio resource” means radio frequencies and channels, time-slots for communications.

### [resource sharing] [facility sharing]

[“resource sharing”] [“facility sharing”] means the coordinated use of resources (see 1,3,1) by multiple stations or operators. The goal is to avoid interference and make efficient use of the available communication capacity, especially in areas where coverage overlaps or where several parties operate VDES communications.

# PARTICIPATION TO [RESOURCE] [facility] SHARING

## PRINCIPLE

VDES communication can be used for official communications for safety on sea and marine environment protection as well as for private communications, among ship stations, VDES satellites and shore-based stations. Therefore, it is important to protect the freedom and confidentiality of the VDES communication in the resource sharing and coordination and cooperation.

The coordination and cooperation on VDES resource sharing can be implemented by various types of bodies (governmental organizations, NGOs, private sectors) which operate VDES communications. Such bodies that wish to coordinate and cooperate VDES communication should provide specifications of their system (e.g., terrestrial stations, satellites) and coordinating methodologies.

Messages to be handled by the VDES resource sharing and coordination and cooperation include ship to shore messages though VDES satellites, shore to ship messages through VDES satellites and ship to ship messages through VDES satellites.

## PARTICIPATION

The resource sharing, coordination and cooperation of VDES communication can be established among those who wish to participate to the coordination and cooperation based on the principle in 2.1.

The participants of VDES resource sharing, coordination and cooperation shall be capable of handling of VDES messages in accordance with ITU-R M.2092-1 and IALA G.1117.

It is necessary to establish agreements among/between the participants on what architecture of messages is used. Architecture of messages should follow IALA G.1117 [2] (2.2.1 and 2.2.2) and IALA Guidelines on VDES Authentication [4].

## NOTIFICATION

The organizer/operator of VDES satellites, VDES shore-based stations and VDES ships stations should notify the intention to join the VDES resource sharing, coordination and cooperation to IALA.

IALA should issue notices of a list of the organizers/operators joining VDES resource sharing.

## COORDINATION PANEL

A coordination panel should be established among the participants of VDES resource sharing, coordination and cooperation. The panel should define the VDES messages protocol that the participants handle, and should notify the message protocol for use by ship VDES stations (See 3). The panel should also define the detail of methods of VDES satellite coordination (see 4.1) including method of radio resource sharing.

The coordination panel should determine the following issues, and notify them to the participants of the VDES [facility sharing].

The coordination panel should specify:

1.	Logical Channel and Frequency Coordination Procedure

- How operators declare intended channel usage.

- The process for resolving channel conflicts in overlapping coverage.

- The timeline for coordination responses.

- Interim mitigation procedures in case of detected interference.

- Documentation and notification obligations.

2.	Satellite Handover and Routing Method

- The principles governing satellite handover.

- Deterministic routing priority rules when multiple satellites can serve the same message.

- Responsibilities for ensuring continuity of addressed messages.

- Documentation requirements for handover procedures.

3	Inter-Operator Communication Method

- The requirement to designate operational contact points.

- The establishment of secure coordination channels.

- Advance notification periods for planned system modifications.

- Emergency communication protocols.

4	Interference Detection and Mitigation Procedure

- The reporting process when interference is detected.

- Maximum response time.

- Temporary mitigation steps.

- Escalation to the Coordination Panel.

5	Conflict Resolution Mechanism

- The technical review procedure within the Coordination Panel.

- Timeframes for resolution attempts.

- Escalation steps.

- Documentation requirements.

# PROTOCOLS OF VDES COMMUNICATION

Messages to be handled within the VDES resource sharing, coordination and cooperation includes, as defined in ITU-R M.2092-1, which includes ASM as defined in Annex 3 of ITU-R M.2092-1.

Other message protocols may be handled within the VDES resource sharing, coordination and cooperation, if such message protocols are accepted among the participants to the VDES resource sharing.

(more to come, if necessary)

# VDES RESOURCE SHARING

VDES resource sharing ensures efficient and fair use of communication resources among satellites, ships, and shore-based stations. Uncoordinated operations may cause interference and reduce the performance of VDES communication. Therefore, cooperation is essential. This chapter introduces the main approaches to resource sharing: satellite coordination (4.1), ship VDES station requirements (4.2), and shore-based VDES station participation (4.3).

4.1	SATELLITE VDES RESOURCE SHARING

VDES satellite communications, particularly with LEO satellites, require technically complex coordination mechanisms, including dynamic satellite handover between satellites and operators, due to limited visibility, communication periods and shared spectrum environments.

In order to realize the VDES resource shared communications, VDES satellite operators are encouraged to coordinate their use of VDES frequencies, logical channels, codes, etc. to avoid interference and improve overall system performance.

Given the technical complexity and operational implications of satellite coordination including frequency assignments, logical channel allocation, spreading code definitions and spatial coverage, these matters are best handled through direct coordination among the involved VDES satellite operators.

The procedure by which satellite operators register:

- Satellite Identifier (Satellite ID)

- Frequency usage

- Logical channel allocation

- Coverage parameters

- Operational contact points

- The format of registration submission.

- The authority responsible for maintaining the register.

- The update procedure when operational parameters change.

- Public or participant-level accessibility of the register.

However, rather than detailing such elements in the IALA guidelines, it is more appropriate to leave the establishment of a guide of for VDES satellite coordination to a relevant external industry group or body/panel to be established among VDES satellite operators, which are better positioned to address these technical aspects of VDES satellite communication operations, taking into account the possible future developments of VDES satellite communications.

4.2 SHIP VDES STATIONS

Ship stations participating in VDES resource sharing should be able to send and receive messages in accordance with ITU-R M.2092-1. The protocol of VDES messages transmitted from ship VDES station should be those acceptable to the participants of the VDES resource sharing, coordination and cooperation in order to ensure compatibility within the shared environment (see 2.4).

The procedure by which ship operators register:

- Contact point

- ID of ships

- (more to come)

4.3 SHORE-BASED VDES STATIONS

Shore-based VDES stations play a key role in VDES resource sharing, and their specifications are to be defined in the Guidelines for VDES Shore Infrastructures [5]. Shore-based VDES stations/operators is recommended to join the Coordination Panel (See 2.4) in order to properly handle the VDES messages communicated within the VDES resource sharing, coordination and cooperation.

The procedure by which shore-based VDES station register:

- Contact point

- Location(s) of shore station

- (more to come)

# handLing of messages

The confidentiality of the VDES messages handled in the VDES resource sharing and coordination and cooperation shall be kept. The messages shall not be opened or altered during the process of the VDES resource sharing and coordination and cooperation.

6	INTER=OPERATOR COMMUNICATION METHOD

Inter-Operator Communication Method should be described

- The requirement to designate operational contact points.

- The establishment of secure coordination channels.

- Advance notification periods for planned system modifications.

- Emergency communication protocols.

7	Governance–Industry Interface

To preserve cooperation with industry while ensuring institutional clarity:

- The baseline coordination methods defined by IALA.

- The role of external industry groups in developing technical specifications.

- The requirement that external specifications remain consistent with IALA baseline coordination methods.

This ensures institutional coherence without over-specifying technical details.

# RELATED DOCUMENTS

- ITU-R M.2092-1, Technical characteristics for a VHF data exchange system in the VHF maritime mobile band, February 2022

- IALA G1117, VHF Data Exchange System (VDES) Overview, December 2022

- IALA G1181, VDES VHF Data Link (VDL) Integrity Monitoring, December 2023

- IALA G……..  VDES Authentication Guidelines

- IALA G…….. Guidelines for VDES shore Infrastructure

Leave open if uncertain