Input paper: DTEC6-6.2.3.11

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☐ ARM	☐  ENG		☐ PAP			☐ Input

☒ DTEC	☐ VTS					☐ Information

Agenda item 			N/A

Technical domain/ Task number 2	6.3.10

Author(s)/Submitter(s)			Stefan Bober

# intersessional Report on the development of

# Guidelines for

# Shore-based VDES Infrastructure

# Summary

An IALA DTEC intersessional meeting was held, as announced in the DTEC 5 report and IALA DTEC calendar:

Date: 16 to 20 February 2026,

Location: IALA HQ

Topic: DTEC Task 6.3.10 “New IALA Guideline on Shore-based VDES infrastructure”

Working hours to accommodate remote participants,

Online:   9:00 to 13:00h – work on the document

Offline: 14:00 to 17:00h – drafting work

## Purpose of the Meeting

To continue working on the draft IALA Guideline on shore-based VDES infrastructure and the draft IALA Guidelines VDES Resource Sharing in the light of finalisation of a first edition before the IALA Conference in Q2 2027. The coordinator of the meeting was Stefan Bober.

# Agenda of the meeting:

1) DTEC Task 6.3.10: Guideline on Shore-based VDES infrastructure

2) DTEC Task 6.3.5 Guideline on VDES resource sharing and coordination/cooperation

Approach of the work: Review and align the documents as appropriate considering the developments at international organisations such as IMO, ITU, IEC in the field of VDES as well as the input provided by the members to the meeting.

# Input documents to the Meeting

Input papers to the meeting can be found in the IALA share folder: 20260216 Intersessional Saint Germain

- 20260215 Master_Draft Guideline for Shore VDES Infrastructure (20251121), Lukas Kim

- 20260211 Gxxxx draft2 IALA Guidelines VDES Resource Sharing Ed_1_0 Feb2026V2, Koichi Yoshida

- 20260215 Position Paper on DTEC Task 6.3.5 Guideline on VDES resource, Koichi Nishimura

- Contribution: Section § 5.3 R-MODE OPERATION, Ronald Raulefs

- Contribution: Section § 7 VDE-TER COORDINATION METHODS, Johan Lindborg

- Several input documents were developed from the participants during the meeting

## Issues addressed during the meeting:

3.2       IALA Guideline on Shore-based VDES infrastructure

3.2.1	Review of the Guideline on shore-bases VDES infrastructure

3.2.2	Architecture for service interface

3.2.3	VDES coordination methods

3.2.4	Authentication of AIS messages by VDES mobile station

3.2.5	Verification of received authenticated VDES messages

3.2.6.	ASM implemented in the VDES mobile station according IEC 63541

3.2.7.	Assess the impact on the VDL of the reporting behaviour of VDES mobile station
             “at anchor or moored” and on the “VDE-TER status report”

3.3	IALA Guideline on VDES resource sharing and coordination/cooperation

## IALA Guideline on Shore-based VDES infrastructure

### Review of the guideline on shore-based VDES infrastructure

The group reviewed the draft VDES shore infrastructure guideline during the meeting.

Editorial correction including harmonisation of technical terms. were introduced, like “AIS” instead of “legacy AIS”, “VDES shore station” instead of “VDES fix station”, etc.

The draft document was transferred to the IALA template. This version should be used as the baseline document for further development.

Following technical issues were discussed during the meeting:

- Limited VDES shore station, for special application, e.g. mobile control station, (ongoing),

- synchronisation source for VDE channel: UTC direct, UTC indirect, precision common time reference,  external,    -> require interface for external precise synchronisation (PTP), (ongoing)

- § 5.3: R-Mode station as an option (shore station with and without R-Mode capability)
interface for external precise synchronisation, synchronisation between R-Mode stations via glass fibre, future: enhanced R-Mode by synchronised base station transmission (orthogonal),

- no DSC receiver in VDES

- AIS SART and EPIRB AIS only, no other VDES functions for such devices (section 4)

- Figure 6 ASM and Figure 7 VDE TER, remove VDL box (ongoing China MSA)

- § 4.1.3: Nominal coverage PLAN (ongoing)
add content to assist in detecting excepted coverage in various VDES link IDs, i.e. link quality, multipath, noise floor, antenna path loss,

  - § 4.2.2: Interaction model, § 4.2.3 Data flow model, § 4.2.4 Data model (ongoing China MSA),

  - § 4.2.5 Interface model (ongoing China MSA)

  - Minimum requirements for the HMIs, direct, local, field deployed, remote, (ongoing)

  - § 5.2 Functional Definitions of MODE (ongoing Ronald)

    - Simplex mode is same physical channel for tx and rx

    - Duplex mode two different physical channels for tx and rx

    - R-Mode: In this mode VDES signals support radiolocation alongside data communication.

  - § 6 VHF DATA LINK INTEGRITY MONITORING (ongoing MSA China)

  - §7 Resource sharing management and coordination (ongoing Johan)

    - Centralized network coordination,

    - Time domain sharing,

    - Frequency domain sharing, a) 25 kHz, b) upper and lower band 100 kHz

    - Over the Air coordination (OTA), use for monitoring purpose,

  - Amend “VDE TER status report” for Priority and TBB version number (ongoing IEC)

  - § 8 INSTALLATION, IMPLEMENTATION AND MAINTENANCE (ongoing) 
Not addressed during the meeting

Further work on the Guideline shore-based infrastructure for the VDES service is required.

The IALA template was applied to this version of the document, which should serve as a basis for further development.

Output: “Master_Draft Guideline for Shore VDES Infrastructure (20260220)”

The following sections of the report reflects the discussion of issues of special focus during the meeting.

### § 4.2.5: Architecture for service interface

Shuaiheng Huai and Gaole Yao from China MSA presented two proposed VDES interface architectural diagrams: one depicting the VDES service interface in a SECOMM environment, and the other illustrating the VDES service interface with a compatible MMS connection. The group agreed that both architectures must be included in the Guideline. China MSA has been invited to submit a detailed version of the proposed architectures to DTEC6 for further discussion."^

### § 7: VDES Coordination Methods

The group revised § 7 Resource sharing management and coordination based on the input received during the meeting. §7 includes descriptions on functional definitions of VDES resource sharing, coordination of the function of AIS, ASM and VDE inside the own VDES shore station and with other VDES shore stations. This section describes common coordination mechanism depending on topology, connectivity and cross-border context: Centralized network coordination, Over-the-air (OTA) coordination, Frequency-domain sharing and Static Time-domain sharing.

The section provides examples of VDE-TER resource sharing management and coordination scheme.

Example for a combined approach: Centralized time domain sharing in “own” network, time domain coordination in “overlaps”

Conflict resolution between overlapping “service areas” using additional high priority TBB

### Authentication of AIS messages by VDES mobile station

The group discussed the authentication of AIS messages by the VDES mobile station as implemented in draft IEC 63614 Edition.

An authenticated AIS message consists of content of AIS message 1 and AIS message 5 and the signature.

The data is autonomously sent by the mobile VDES station on the VDE channel as a one slot short message using link ID 17 in RCA logical channel. Every 30 minutes the private key certificate is sent in addition in as a one slot short message.

The group discussed alternative transmission of the message on ASM but concluded that the same data would need 4 slots (without FEC) or 5 slots (with FEC) on ASM channels.

Same situation would appear when using VDE 25 kHz channel link ID 11, which would result in a data session.

The group drafted a liaison note to IEC proposing a longer reporting interval of 30 minutes for authenticated AIS message when sip is at anchor or moored.

The same liaison note contains a proposal on the VDE-TER Status Report. The group proposes to add information about the priority of the bulletin board and the bulletin board version to the VDE-TER Status Report.

The receiving VDES station will validate the signature in the authenticated AIS message according the received public key certificate and its stored private key certificate and provide the data of the message including the status of authentication.

Note: Technical detail on VDE resources needed for authenticated AIS messages

Assumption: Default bullion board in use on the ship 
All 5 data channels for RATDMA = 70 RAC per TDMA frame-> 10 500 slots per 6 min, 
One data channels for RATDMA = 14 RAC per TDMA frame -> 2 100 slots per 6 min

Figure: VDE-TER capacity simulated using RAC

### Verification of received authenticated VDES messages

The group discussed the process needed to verify a received authenticated VDES messages as follows:

  - VDL Payload received with signed AIS data

    - ASM channels

      - DAC identifying VPFI data with signature and generic AIS message

        - Defined in VDES Mobile standard

          - Will not be sent autonomously

          - On interrogation?

    - VDE-TER channels

      - VPFI with signature with Signed AIS data and generic AIS message

        - Defined in VDES Mobile standard

          - Message 1 and 5 will be sent together every 6 minutes, (or every 30 minutes if moored/at anchor)

  - Verification of received Signature

    - Public key/certificate of remote station

      - Delivery mechanism for public certificate (and intermediate certificates)

        - VDL delivered Certificate

          - VDE-Ter Channels

            - Payload and transmission schedule defined in VDES Mobile standard

            - Will be sent autonomously every 30 minutes from a mobile

              - On interrogation? To be verified.

        - Shore service?

          - Not defined. Need to standardize? MCP defined?

      - Verification of certificate (or certificate chain) against common source of trust

        - Need to have the same root certificate issuer

          - What organization will be able to issue root trust?

        - Mechanism defined in VDES Mobile station

          - Includes format for revocation lists

          - Includes possibility to filter on MRN “domain”

    - Use the public certificate of remote station to verify authenticity of signature

        - Mechanism defined in VDES Mobile station

  - Output on the PI the result of the Authentication

    - VDE Mobile PI Binary Message structure include flag identifying authentication status

      - Not signed

      - Authenticated signature

      - Authenticated signature, but message validity period is expired

      - Failed authentication

### ASM implemented in the VDES mobile station according IEC 63541

The group discussed the Application Specific Messages (ASM) which are available on the HMI of a VDES mobile station according IEC 63541.

ASM defined in IMO SN1/Circ 289, which can be send from the VDES mobile station on board a vessel using the own HMI of the station:

- POB addressed message, DAC FI 16, response to interrogation only

- Berthing FI20 + associated text FI30, addressed, initiated from ship as berthing request,

- Ship extended data FI24 + associated text FI 29, broadcast initiated from ship,

- Dangerous cargo FI25, addressed, response to interrogation only

- Rout info broadcast FI 27, broadcast initiated by ship

- Rout info addressed FI28, addressed, response to interrogation only

- Weather observation report FI21, broadcast, initiated by ship

The VDES mobile station provides a means for the visualisation of AIS message 21 and ASM received via the ASM channel on displays according IEC 62288. (Further explanation of legacy PI sentences required).

### Assess the impact on the VDL of the reporting behaviour of VDES mobile station “at anchor or moored” and on the “VDE-TER status report”

The group discussed a longer reporting interval of 30 minutes for authenticated AIS message when ship is at anchor or moored.

Amendments to the VDE-TER Status Report were discussed. The group proposes to add information about the priority of the bulletin board and the bulletin board version to the VDE-TER Status Report.

## IALA GUIDELINE ON VDES RESOURCE SHARING AND COORDINATION/COOPERATION

The group discussed the draft IALA guideline for resource sharing including an input document from Japan providing comments on the guidelines.

The current draft guidelines mainly outline governance arrangements and general principles for coordination of resource sharing but does not yet specify concrete methods for implementing resource coordination.

The guideline should include concrete implementation procedures for resource coordination like registration procedures, logical channel coordination mechanisms, interference mitigation steps, satellite handover procedures, information exchange obligations, conflict resolution mechanisms, compliance consequences, etc.

An initiative is underway to organize a Satellite VDES Workshop in Japan in November 2026. This workshop would provide an opportunity to gather broader technical and operational expertise from satellite operators, administrations, and industry stakeholders.

Following steps for the development of the guideline were disused:

- Review the document to include “placeholder” to address proposals from the received input,

- Submit the revised draft to DTEC 6 for further discussion

- Submit a further developed version of the document to DTEC 7

- The first edition of the guideline is targeted for completion in the first half of 2027.

# Participants

Barry Baker

Derek Love

Dmitry Rostopshin

Gaole Yao

Glen Coady

Jinrui Li

Johan Lindborg

Johnny Schutz (phy)

Kaisu Keikonen

Kayamori Natsuki

Kenji Kogo

Koichi Nishimura

Koichi Yoshida (phy)

Krzysztof Bronk (phy)

Luca Fiorentino

Lukas Kim

Moon A

Patrik Gallager

Ronald Raulefs

Ross Norsworthy (phy)

Sean Forster

Stefan Bober (phy)

Shuaiheng Huai

Yang Li

Yoshio Miyadera (phy)

Leave open if uncertain