Input paper: DTEC6-6.2.3.2

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☐ ARM	☐  ENG		☐ PAP			☒ Input

☒ DTEC	☐ VTS					☐ Information

Agenda item			n.n

Technical domain/ Task number 2	…………………………………

Author(s)/Submitter(s)			China MSA

# Draft Guideline on ASM Application Protocol

# Summary

At the DTEC5 meeting, the Committee considered and approved the proposal on improving the ASM data application protocol (DTEC5- 6.2.3.4), agreeing to develop a guideline for the data application protocol on VDES ASM dedicated channels (ASM 1 and ASM 2). This document aims to submit a draft technical guideline on the use of ASM messages on VDES ASM dedicated channels in accordance with the aforementioned resolution. Based on the technical characteristics defined in Recommendation ITU-R M.2092-1, this draft guideline specifies the structure, encoding, and application methods of ASM messages to support efficient data exchange.

## Purpose of the document

This proposal aims to submit a new IALA Guideline draft, which specifies the application layer protocol for data exchange on VDES ASM dedicated channels.

## Related documents

[1]	ITU-R M.2092-1, Technical characteristics for a VHF data exchange system in the VHF maritime mobile band, February 2022

[2]	ITU-R M.1371-5, Technical characteristics for an automatic identification system using time division multiple access in the VHF maritime mobile frequency band, February 2014

[3]	IMO SN.1/Circ.289, GUIDANCE ON THE USE OF AIS APPLICATION-SPECIFIC MESSAGES, June 2010

[4]	IALA G1117, VHF Data Exchange System(VDES) Overview, December 2022

# Background

At the DTEC5 meeting, the Chinese delegation submitted a proposal on improving the ASM data application protocol. The proposal pointed out that with the maturity of VDES technology, Recommendation ITU-R M.2092-1 has defined ASM dedicated channels and new message structures (ASM 0-6), but there is a gap in the application layer protocol. Consequently, advanced features like MITDMA (Multiple Incremental Time Division Multiple Access) cannot be fully utilized for high-capacity data transmission. The Committee adopted the proposal and agreed to establish a work item to draft a guideline specifically for defining the data application protocol on VDES specific ASM channels.

VDES introduces two specific channels, ASM 1 and ASM 2, aimed at alleviating congestion on the AIS VDL and providing higher data throughput. Unlike traditional AIS ASM messages (Messages 6, 8, 25, 26), VDES ASM messages (Nos. 0-6) have different slot structures and channel access mechanisms. To ensure interoperability between equipment from different manufacturers and to effectively utilize the MITDMA mechanism for transmitting large data volume products such as meteorological and hydrographic grid information, a unified application format standard must be established.

# Discussion

VDES introduces two specific channels, ASM 1 and ASM 2, aimed at alleviating congestion on the AIS VDL and providing higher data throughput. Unlike traditional AIS ASM messages (Messages 6, 8, 25, 26), VDES ASM messages (Nos. 0-6) have different slot structures and channel access mechanisms. To ensure interoperability between equipment from different manufacturers and to effectively utilize the MITDMA mechanism for transmitting large data volume products such as meteorological and hydrographic grid information, a unified application format standard must be established.

## Core Content of the Draft Guideline

The draft guideline is written based on Recommendation ITU-R M.2092-1 and relevant IALA guidance documents. The draft intends to focus on regulating the following contents.

ASM Message Classification and Structure

It clarifies the definitions of messages 0 to 6 used by VDES-ASM. This includes ASM Message 0 as an encapsulation message, and ASM Messages 1-6 used to carry specific application data (such as reservation broadcasts, non-communication status broadcasts, addressed messages, and area multicast messages, etc.).

Message Encoding Requirements

It details the bit structure, parameter rules, and filling requirements for various types of ASM messages. For example, it clarifies that the Application Identifier (AI) consists of a Designated Area Code (DAC) and a Function Identifier (FI), and specifies its position within the ASM message.

Channel Access and Slot Management

Combining MITDMA characteristics, the draft guideline provides calculation and allocation rules for the number of slots occupied by ASM messages under different data volumes to maximize channel utilization.

Application Examples

To facilitate developer understanding, the draft provides typical application data encoding examples based on the aforementioned structures.

## Compatibility with Existing Systems

This draft guideline aims to fill the gap in the VDES ASM application layer, parallel to and complementing the existing AIS ASM system (based on ITU-R M.1371-5 and IMO SN.1/Circ.289). It clarifies how to utilize advanced VDES features (such as Forward Error Correction FEC, longer message sequences) to transmit data content that is difficult for traditional AIS links to carry.

# Action requested of the Committee

The Committee is requested to consider the proposals in this document and take appropriate action.

IALA Guideline

Gnnnn

ASM Application Protocol

Edition 1.0

March 2026

Revisions to this document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |

1	Introduction	3

1.1.	Purpose	3

1.2.	Scope	3

1.3.	Text structure	3

2.	BACKGROUND	3

3.	CLASSIFICATION AND OVERVIEW	4

3.1.	ASM Message Classification	4

3.2.	VDES ASM message types	4

3.3.	CHANNEL AND ACCESS SCHEMES	5

4.	VDES ASM Message Encapsulation Structure	5

4.1.	General Message Structure	5

4.2.	Application Identifier	5

5.	ASM APPLICATION DATA STRUCTURE DEFINITION	5

5.1.	General Encoding Rules	5

5.2.	SHORE-TO-SHIP BROADCASTS	5

5.3.	SHIP-TO-SHORE INTERACTIONS	5

5.4.	SHIP-TO-SHIP INTERACTIONS	5

5.5.	Text and Functional Messages	5

6.	Data Interface Requirements	5

7.	Terms and Abbreviations	5

8.	References	6

Appendix A: Definitions of Code Tables	7

Appendix B: Typical Application Scenarios (USE CASES)	7

- Introduction

  - Purpose

The purpose of this guide is to address the existing standardization gap by specifying application-layer protocols for data exchange over VDES ASM dedicated channels. It defines the encapsulation structure of ASM messages 0–6, channel access policies, and data encoding formats for specific application identifiers, comprising the Designated Area Code (DAC) and function identifier (FI).

This guide aims to:

Ensure interoperability by establishing a unified data structure standard for VDES equipment from different manufacturers, enabling seamless exchange of ASM data between shore-based systems and onboard terminals.

Enhance transmission efficiency by standardizing the use of the MITDMA mechanism to support efficient dissemination of high-capacity data products, such as meteorological and hydrological information, over VHF links.

Facilitate application development by providing standardized data dictionaries and encoding rules for diverse application scenarios, including navigation assistance, weather warnings, and ship–shore interaction, thereby promoting the broader adoption of VDES in enhancing maritime safety and operational efficiency.

  - Scope

This guide specifies the application layer protocols and technical requirements for data exchange on the dedicated channels (ASM 1 and ASM 2) of the VHF Data Exchange System (VDES) for Application Specific Messages (ASM).

  - Text structure

Chapter 2 provides the technical background of VDES ASM.

Chapter 3 specifies the classification and access schemes of VDES ASM.

Chapter 4 specifies the encapsulation structure of VDES ASM messages.

Chapter 5 specifies the application data structure definitions of VDES ASM services.

Chapter 6 specifies the data interface requirements of VDES ASM messages.

Chapter 7 provides the terms and abbreviations.

Chapter 8 provides the references.

- BACKGROUND

The Very High Frequency Data Exchange System (VDES), as the next-generation maritime digital communication infrastructure, has its technical specifications defined in the ITU-R Recommendation M.2092-1. VDES introduces two dedicated Application Message (ASM) channels (ASM 1 and ASM 2) to alleviate congestion in the existing AIS Data Link (VDL) while delivering higher data throughput and more flexible transmission capabilities.

While ITU-R Recommendation M.2092-1 specifies the physical and link layer characteristics of VDES ASM—including new packet structures (ASM messages 0–6) and the Multiple Incremental Time Division Multiple Access (MITDMA) access mechanism—no unified application-layer protocol standard has yet been established for data exchange over these dedicated channels.

Current ASM applications primarily rely on IMO SN.1/Circ.289, which was originally designed for traditional AIS messages (message 6 and 8). As a result, it cannot support the long packet sequences and high-bandwidth requirements of VDES ASM, thereby limiting the practical realization of VDES’s technical advantages.

- CLASSIFICATION AND access schemes

  - ASM Message Classification

Special Application Messages (ASM) are categorized into two types—AIS-ASM and VDE-ASM—based on their transmission channels and applicable technical standards.

AIS-ASM: As defined in ITU-R Recommendation M.1371, this term refers to specific message types, including Message 6 (addressed binary message), Message 8 (broadcast binary message), Message 25 (single-slot binary message), and Message 26 (multi-slot binary message with communication status). These messages primarily operate on AIS 1 and AIS 2 frequency bands.

VDE-ASM: As defined in ITU-R M.2092 Recommendation, these messages primarily include ASM messages 0 to 6. Designed for VDES's dedicated ASM channels (ASM 1 and ASM 2), they support more efficient channel access mechanisms (e.g., MITDMA) and flexible data transmission.

  - VDES ASM message types

The VDE-ASM framework defines seven distinct message types (IDs 0-6) to support diverse service requirements including broadcast, addressing, acknowledgment, and area multicast. Table 1 details the functional specifications, access mechanisms, and communication state requirements for each message type.

VDE-ASM Message Types

| Message ID | Name | Description | Access Scheme | Communication Status |
| --- | --- | --- | --- | --- |
| 0 | Broadcast AIS ASM Message | Used to encapsulate traditional AIS ASM packets for transmission through the ASM channel. | RATDMA | None |
| 1 | Scheduled Broadcast Message | Broadcast data with communication status, supporting large data volume transmission | FATDMA, RATDMA, MITDMA | MITDMA |
| 2 | Broadcast Message | Broadcast data without a communication state, suitable for non-scheduled transmissions or short messages | FATDMA, RATDMA | None |
| 3 | Scheduled Addressed Message | Single-target addressing data with communication status requires receiver confirmation | FATDMA, RATDMA, MITDMA | MITDMA |
| 4 | Addressed Message | Single-target addressing data without communication status requires receiver confirmation | FATDMA, RATDMA | None |
| 5 | Acknowledgement Message | Used to provide link layer acknowledgments for one or more Addressing Messages (ASM 3 or 4) | FATDMA, RATDMA, MITDMA | None |
| 6 | Geographic Multicast Message | Send to a group of stations in a specific area defined by geographic coordinates without confirmation | FATDMA, RATDMA | None |

  - CHANNEL AND ACCESS SCHEMES

Channel Usage: VDE-ASM packets must be transmitted via the dedicated ASM channels ASM 1 (161.950 MHz) and ASM 2 (162.000 MHz) assigned by VDES, to alleviate AIS VDL congestion and achieve higher data throughput.

To enable efficient transmission of high-volume data (e.g., meteorological and hydrological information, navigation warnings, etc.), the ASM 1 and 3 messages incorporate the Multiple Incremental Time Division Multiple Access (MITDMA) mechanism. This mechanism allows the transmitter to combine up to 15 consecutive time slots on the ASM dedicated channel into a single large data block for transmission, significantly improving channel utilization and transmission efficiency.

- VDES ASM Message Encapsulation Structure

  - General Message Structure

All VDE-ASM messages (ASM 0~6) consist of a header and ASM information.

  - Application Identifier

The ASM information section contains an Application Identifier (AI) and Application Data. The Application Identifier comprises a Designated Area Code (DAC) and a Function Identifier (FI), which uniquely identify the data's service type.

- ASM APPLICATION DATA STRUCTURE DEFINITION

  - General Encoding Rules

  - SHORE-TO-SHIP BROADCASTS

### 5.2.1 Weather Forecast (Grid)

### 5.2.2 Weather Forecast (Regional)

### 5.2.3 marine environment forecast (grid)

### 5.2.4 marine environment forecast (regional)

### 5.2.5 nautical aid service

...

  - SHIP-TO-SHORE INTERACTIONS

  - SHIP-TO-SHIP INTERACTIONS

  - Text and Functional Messages

- Data Interface Requirements

- Terms and Abbreviations

| ADM | ASM VHF Data-Link Message |
| --- | --- |
| ADO | ASM VHF Data-Link Own-Vessel Report |
| AIS | Automatic identification system |
| ASM | Application Specific Messages |
| BAS | Basic AIS Service |
| CRC | Cyclic Redundancy Check |
| CSTDMA | carrier sense time division multiple access |
| DGNSS | Differential Global Navigation Satellite Systems |
| DOS | Denial of Service |
| ECDIS | Electronic Charts Display Information System |
| EDM | VDE Broadcast Message |
| EDO | VDE Data Message Sentence Own Report |
| ETA | Estimated Time of Arrival |
| FSR | AIS received frame summary |
| GNSS | Global Navigation Satellite Systems |
| IALA | International Association of Marine Aids to Navigation and Lighthouse Authorities |
| IMO | International Maritime Organization |
| MITDMA | Multiple Incremental Time Division Multiple Access |
| MMSI | Maritime mobile service identity |
| PSS | Physical Shore Station |
| ROT | Turn rate |
| SOG | speed over ground |
| SOTDMA | self organized time division multiple access |
| VDE | VHF Data Exchange |
| VDES | VHF Data Exchange System |
| VDE-SAT | VDE satellite (VHF Data Exchange Satellite) |
| VDE-TER | VDE-terrestrial (VHF Data Exchange-Terrestrial) |
| VDL | VHF Data Link |
| VDM | VHF Data Link Message |
| VDO | VHF Data Link message (Own) |
| VHF | Very High Frequency |
| VSI | VDL Signal Information |

- References

IMO Resolution MSC.140 (76), Recommendation for the protection of the AIS VHF data link, December 2002

IALA R0124 on The AIS Service

ITU-R Recommendation M.1371-5, Technical characteristics for an automatic identification system using time division multiple access in the VHF maritime mobile band, February 2014

ITU-R M.2092-1, Technical characteristics for a VHF data exchange system in the VHF maritime mobile band,February 2022

IALA G1117 on VHF Data Exchange System (VDES) Overview

NMEA 0183, Standard for Interfacing Marine Electronic Devices, November 2018

Note: The references are the latest documents as of the date of publication of the above guidelines. Readers should be aware that relevant information may be modified or revoked, and should follow the latest updates.

# Appendix A: Definitions of Code Tables

# Appendix B: Typical Application Scenarios (USE CASES)