Input paper:  DTEC6-6.2.3.3

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☐ ARM	☐  ENG		☐ PAP			☒ Input

☒ DTEC	☐ VTS					☐ Information

Agenda item 			n.n

Technical domain/ Task number 	…………………………………

Author(s)/Submitter(s)			China MSA

# Proposed Revision to Recommendation ITU-R M.2092

# Summary

This document is based on the outcomes of the 5th meeting of the IALA DTEC Working Group. It further refines the proposed revisions to Recommendation ITU-R M.2092 regarding the issues deferred from the meeting. Specifically, it proposes using Message 7 to suppress the transmission of ASM-SAT messages within shore-based coverage areas and provide mechanism protection for the downlink burst transmission of VDE-SAT. The document recommends that IALA continues the refinement of this Recommendation.

  - Purpose of the document

The purpose of this document is to propose revisions and enhancements to Recommendation ITU-R M.2092-1, update its technical details, and provide a reference for the Committee.

  - Related documents

- IALA Committee Work Program (2025-2027)

- ITU-R M.2092-1 Technical characteristics for a VHF data exchange system in the VHF

- Background

The 110th session of the IMO Maritime Safety Committee (MSC 110) has agreed in principle to incorporate VDES into the International Convention for the Safety of Life at Sea (SOLAS), and the implementation and deployment of VDES applications are poised for widespread promotion. The latest version of the VDES technical standard, ITU-R M.2092, is expected to be released in 2026. However, a full consensus has yet to be reached within IALA on certain technical issues in Recommendation ITU-R M.2092, and some contents require further discussion.

- Discussion

We propose:

Add a new field in Message 7 to enable the suppression of ASM-SAT message transmissions within shore-based coverage areas and its corresponding description;

To ensure the successful reception of VDE-SAT downlink long burst transmissions, the ship station may temporarily suppress own AIS transmissions for a maximum duration of 2.4 seconds, provided that no other ships are detected within a range of 1 nautical mile.

The revision of ITU-R M.2092-1 in detail is shown in the annex.

- References

ITU-R M.2092-1 Technical characteristics for a VHF data exchange system in the VHF

- Action requested of the Committee

The Committee is requested to:

Consider the suggestions in section 3.

Take appropriate action.

| Comment Number: Name-# | Change Log ID | Annex / Section | Section, Table, Figure | Type of change | Reason for the change, or what you want to accomplish | Proposed change to ITU-R M.2092-1, short editorial changes can be include here (large changes should be documented below) |
| --- | --- | --- | --- | --- | --- | --- |
| China MSA-1 | NA | Annex 3 | 7.10, Table 33 | Tech. Improvement | According to the consensus reached at the 4th meeting of the DTEC Committee, the decision to inhibit ASM-SAT transmissions from shipborne stations within coastal areas shall be controlled by shore stations. This issue has been deliberated during DTEC 5. In order to support this functionality, a control field has been added to Table 33, and corresponding instructions for the use of this field have been supplemented in Section 7.10. | Modify as shown below. |
| China MSA-2 | NA | Annex 6 | 2.1 | Tech. Improvement | The downlink duration of VDE-SAT is up to 2.4 seconds at most, while the firing interval of AIS for high dynamic ships is only 2 seconds. If the absolute priority of AIS is strictly implemented, the reception of long messages will be physically interrupted by 100% determinism, resulting in the unavailability of the link and invalid retransmission. Within a limited time and in the absence of AIS targets around, suppressing AIS emissions for a short period of time will not cause security risks and can significantly improve the downlink efficiency of VDE-SAT. | Modify as shown below. |

Comments:

The proposed revisions are shown in track changes and highlighted blue in the Attachment.

# China MSA-1

## 7.10	Message 7: ASM data link management message

In addition, this message is also used to control the enabling and disabling of the ASM-SAT function on shipborne stations within the coverage area of shore stations. For shipborne stations, ASM-SAT is enabled by default. If the shipborne station receives message 7 containing the value 1 in the "transmission control for ASM-SAT" field for at least 4 minutes within a 5-minute window, it shall disable ASM-SAT. Conversely, if no messages containing the value 1 in the "transmission control for ASM-SAT" field are received for 3 consecutive minutes within a 5-minute window, the shipborne station shall enable (or return to) the ASM-SAT enabled state.

Table 33

ASM data link management message

| Reservation Block | Parameter | Number of bits | Description |
| --- | --- | --- | --- |
|  | Message ID | 4 | 7 – ASM Data link management. |
|  | Retransmit flag | 1 | 0 (reserved for future use). |
|  | Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0 – 3; 0 = default; 3 = do not repeat any more. |
|  | Session ID | 6 | The Session ID associates the VDL transmission with a specific PI transaction. |
|  | Source ID | 32 | The Unique Identifier of the transmitting station as described in § 2.4, Annex 1. |
| Reservation Block 1 | Channel selection | 2 | 0 = ASM1; 1 = ASM2; 2 = ASM1 and ASM2; 3 = no reservation. |
|  | Offset number | 12 | Reserved offset number. |
|  | Number of slots | 4 | Number of reserved consecutive slots: 1-15. |
|  | Time-out | 4 | Time-out value in minutes. |
|  | Increment | 4 | Increment to repeat reservation block 1. 0 = one reservation block per frame. 1 = 30, 2 = 45, 3 = 50, 4 = 75, 5 = 90, 6 = 125, 7 = 150, 8 = 225, 9 = 250, 10 = 375, 11 = 450, 12 = 750, 13 = 1125, 14-15 not used. |
| Reservation Block 2 | Channel selection | 2 | 0 = ASM1; 1 = ASM2; 2 = ASM1 and ASM2; 3 = no reservation. |
|  | Offset number | 12 | Reserved offset number. |
|  | Number of slots | 4 | Number of reserved consecutive slots: 1-15. |
|  | Time-out | 4 | Time-out value in minutes. |
|  | Increment | 4 | Increment to repeat reservation block 2. 0 = one reservation block per frame. 1 = 30, 2 = 45, 3 = 50, 4 = 75, 5 = 90, 6 = 125, 7 = 150, 8 = 225, 9 = 250, 10 = 375, 11 = 450, 12 = 750, 13 = 1125, 14-15 not used. |
| Reservation Block 3 | Channel selection | 2 | 0 = ASM1; 1 = ASM2; 2 = ASM1 and ASM2; 3 = no reservation. |
|  | Offset number | 12 | Reserved offset number. |
|  | Number of slots | 4 | Number of reserved consecutive slots: 1-15. |
|  | Time-out | 4 | Time-out value in minutes. |
|  | Increment | 4 | Increment to repeat reservation block 3. 0 = one reservation block per frame. 1 = 30, 2 = 45, 3 = 50, 4 = 75, 5 = 90, 6 = 125, 7 = 150, 8 = 225, 9 = 250, 10 = 375, 11 = 450, 12 = 750, 13 = 1125, 14-15 not used. |
| Reservation Block 4 | Channel selection | 2 | 0 = ASM1; 1 = ASM2; 2 = ASM1 and ASM2; 3 = no reservation. |
|  | Offset number | 12 | Reserved offset number. |
|  | Number of slots | 4 | Number of reserved consecutive slots: 1-15. |
|  | Time-out | 4 | Time-out value in minutes. |
|  | Increment | 4 | Increment to repeat reservation block 4. 0 = one reservation block per frame. 1 = 30, 2 = 45, 3 = 50, 4 = 75, 5 = 90, 6 = 125, 7 = 150, 8 = 225, 9 = 250, 10 = 375, 11 = 450, 12 = 750, 13 = 1125, 14-15 not used. |
| Reservation Block 5 | Channel selection | 2 | 0 = ASM1; 1 = ASM2; 2 = ASM1 and ASM2; 3 = no reservation. |
|  | Offset number | 12 | Reserved offset number. |
|  | Number of slots | 4 | Number of reserved consecutive slots: 1-15. |
|  | Time-out | 4 | Time-out value in minutes. |
|  | Increment | 4 | Increment to repeat reservation block 5. 0 = one reservation block per frame. 1 = 30, 2 = 45, 3 = 50, 4 = 75, 5 = 90, 6 = 125, 7 = 150, 8 = 225, 9 = 250, 10 = 375, 11 = 450, 12 = 750, 13 = 1125, 14-15 not used. |
| Reservation Block 6 | Channel selection | 2 | 0 = ASM1; 1 = ASM2; 2 = ASM1 and ASM2; 3 = no reservation. |
|  | Offset number | 12 | Reserved offset number. |
|  | Number of slots | 4 | Number of reserved consecutive slots: 1-15. |
|  | Time-out | 4 | Time-out value in minutes. |
|  | Increment | 4 | Increment to repeat reservation block 6. 0 = one reservation block per frame. 1 = 30, 2 = 45, 3 = 50, 4 = 75, 5 = 90, 6 = 125, 7 = 150, 8 = 225, 9 = 250, 10 = 375, 11 = 450, 12 = 750, 13 = 1125, 14-15 not used. |
| Reservation Block 7 | Channel selection | 2 | 0 = ASM1; 1 = ASM2; 2 = ASM1 and ASM2; 3 = no reservation. |
|  | Offset number | 12 | Reserved offset number. |
|  | Number of slots | 4 | Number of reserved consecutive slots: 1-15. |
|  | Time-out | 4 | Time-out value in minutes. |
|  | Increment | 4 | Increment to repeat reservation block 5. 0 = one reservation block per frame. 1 = 30, 2 = 45, 3 = 50, 4 = 75, 5 = 90, 6 = 125, 7 = 150, 8 = 225, 9 = 250, 10 = 375, 11 = 450, 12 = 750, 13 = 1125, 14-15 not used. |
| Reservation Block 8 | Channel selection | 2 | 0 = ASM1; 1 = ASM2; 2 = ASM1 and ASM2; 3 = no reservation. |
|  | Offset number | 12 | Reserved offset number. |
|  | Number of slots | 4 | Number of reserved consecutive slots: 1-15. |
|  | Time-out | 4 | Time-out value in minutes. |
|  | Increment | 4 | Increment to repeat reservation block 6. 0 = one reservation block per frame. 1 = 30, 2 = 45, 3 = 50, 4 = 75, 5 = 90, 6 = 125, 7 = 150, 8 = 225, 9 = 250, 10 = 375, 11 = 450, 12 = 750, 13 = 1125, 14-15 not used. |
|  | Transmission control for ASM-SAT | 1 | 0 = default –Allow Mobile station to transmit messages via ASM-SAT within an ASM shore station coverage area. 1 = Mobile station stops transmission via ASM-SAT of within an ASM shore station coverage area. |
|  | Spare | 32 | Spare bits – reserved for the future. |

# China MSA-2

### 2	VHF data exchange system resource sharing principles

## 2.1	Automatic identification system priority

Care should be taken to respect the AIS transmission and reception as the highest priority.

Care should be taken to respect the AIS transmission and reception as the highest priority. However, to ensure the successful reception of VDE-SAT downlink long burst transmissions, if no other ships are detected within a range of 1 nautical mile, the ship station may advance or delay the AIS transmission that conflict with the VDE-SAT downlink. If the slot shifted (advance or delay) is less than 0.5 seconds from the adjacent AIS transmission slot, the transmission should be cancelled. Within the same DC, AIS transmissions should be cancelled no more than once.