Input paper:  DTEC6-6.2.3.6

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					□  Input

X  DTEC	□  VTS								X  Information

Agenda item			n.n

Technical Domain / Task Number	…………………………………

Author(s) / Submitter(s)		Takahiko Konishi / Japan Coast Guard

# Result of a Field Trial on VHF Digital Voice Communication

# Summary

Icom Incorporated (ICOM), Japan Aids to Navigation Association (JANA), Tokyo University of Marine Science and Technology (TUMSAT) and Japan Coast Guard (JCG) conducted a field trial on VHF digital voice communication on 27 November 2025 in Tokyo Bay.

In digital transmission, communication remains reliable when the RSSI difference between the desired and interfering radio waves is about 15 to 18dB, but performance degrades when the difference falls to 10dB or less, resulting in intermittent reception. When the interfering radio wave is stronger than the desired radio wave, analogue transmission produces noisy audio, whereas digital transmission often fails to demodulate the audio, though any successfully demodulated audio is clear.

## Purpose of the document

This document provides the result of a field trial on VHF digital voice communication in order to facilitate the discussion in the IALA committees.

## Related documents

- NCSR12/20 Report to the Maritime Safety Committee

- DTEC2-5.2.2.2 Demonstration paper for VHF digital communications

- VTS55-5.2.1 Inf paper VHF digital demo Japan

- DTEC5-6.2.3.2 Result of a Field Trial on VHF Digital Voice Communication

# Background of the digitalisation of VHF voice communications

VHF voice communication is an essential and widely used means of maritime communication to ensure the safety of navigation, and the efficient use of VHF frequency spectrum is being considered due to growing need for introduction of VHF Data Exchange System (VDES).

The VHF digital voice communication will contribute to the efficient use of spectrum, as it uses only half of the bandwidth of existing VHF analogue communication. In addition, the clear voice provided by the digital modulation could improve the Vessel Traffic Service (VTS) operations. On the other hand, however, there are concerns that VHF digital communication may introduce another risk due to the reduced background noise, although it is beneficial for the VTS operators.

The World Radiocommunication Conference 2023 (WRC-23) resolved that the consideration of improving the utilization of VHF marine radiocommunication should be included in the preliminary agenda of WRC-31(Preliminary Agenda Item 2.7). The digitalization of VHF voice communications could be considered in its scope.

In addition, to guide regulatory developments of the International Telecommunication Union (ITU), the International Maritime Organization (IMO) resolved that the development of a transition scheme for the introduction of digital technology for VHF voice communication be included in the biennial agenda of the Sub-Committee on Navigation, Communication and Search and Rescue (NCSR) for the 2026-2027 biennium as an urgent matter. And the provisional agenda for NCSR 13 an output on "Development of a transition scheme for the introduction of digital technology for very high frequency (VHF) voice communications", with a target completion year of 2027.

Similarly, at IALA, discussions on VHF digital voice communication are accelerated, as demonstrated by Japan in DTEC 2 and VTS 55.

Furthermore, JANA, Japan Radio Co., Ltd. (JRC), TUMSAT and JCG conducted a field trial for comparing the analogue and digital voice communication from December 2024 to January 2025. The result of a field trial was reported to DTEC5 in the [DTEC5-6.2.3.2]. It indicates that following points.

In general, digital voice communication was evaluated as clearer than analogue voice communication.

Analogue voice communication maintains constant quality under various conditions, as the quality of sound varies step by step according to propagation condition of radio waves, whereas digital voice communication degrades significantly in quality under certain conditions.

Transmission distance of digital voice communication was longer than analogue one, generally up to about 30km without noise.

Most evaluators assessed that the head loss in the digital voice communication was not a practical problem.

Although head loss in digital voice communication occurred, head loss also occurred in analogue voice communication, it is not due to connection delays, and it cannot be said that the lack of beginning of the transmitted content is a disadvantage specific to digital voice.

Many evaluators preferred digital voice due to its lower noise and clearer sound quality, but the other preferred analogue one.

In order to facilitate the discussion on the digitalisation of VHF voice communication in the IALA, it would be useful for the members to have a better understanding of the technology and of how digital voice is listened, for considering future usage. ICOM, JANA, TUMSAT and JCG conducted a field trial for which the information is provided in this document in November 2025.

# Discussion

## EXECUTIVE SUMMARY

This field trial focused on influence of interfering radio waves. The purpose of this trial is to [determine the maximum level of interfering radio waves under which the vessel could still receive the desired waves without causing any practical degradation to voice communication. In this trial, Japanese and English test phrases transmitted analogue and digital radio waves from two shore radio stations were received at the shipboard radio station, and the reception conditions were observed

## GENERAL INFORMATION

### Equipment

In this trial, three radio station were installed. The one was shipboard radio station and the others were shore radio stations. All radio station used same radio equipment, IC-F5400DP.

IC-F5400DP (radio equipment used in field trial)

Shipboard radio station was installed on Yayoi (survey and research ship of TUMSAT). The following tables show the particulars of radio equipment and the ship used in this trial.

Shipboard radio station (on board Yayoi)

| Place | Yayoi (Survey and research ship of TUMSAT) |
| --- | --- |
| Height of Antenna | 4.8m (sea level hight) |
| Radio Equipment | IC-F5400DP |
| Frequency | 157.150MHz (analogue) 157.1357MHz (digital) |
| Radio Wave Format | 16K0 F3E (analogue) 5K80 F1E (digital) |
| Antenna Power | 25W |
| Antenna | 7ABJD0004 (half-wavelength vertical dipole antenna) |
| Coaxial Cable | 8D-2V, 12m |
| Gain | 2.15dBi |

Particulars of Yayoi

| Length | 17.80m |
| --- | --- |
| Breadth | 4.28m |
| Draft | 1.49m |
| Gross Tonnage | 19 tons |
| Maximum Number of Persons Allowed on Board | 57 persons (55 persons as passenger and two persons as crews) |
| Material of Ship | Aluminium alloy |

Shore radio stations were installed on Urayasu and Futtsu. Each location is shown in the following figure 2. Point A, B and C in Figure 2 indicate the observation point where the Yayoi drifted and measurements were conducted during the trial.

Places where radio stations were [set]

The following tables show the particulars of radio equipment of each shore radio station.

Shore radio stations

| Shore radio station | Urayasu | Futtsu |
| --- | --- | --- |
| Height of Antenna | 2.6m (from ground) 6.2m (sea level hight) | 3.0m (from ground) 4.4m (sea level hight) |
| Radio Equipment | IC-F5400DP |  |
| Frequency | 157.150MHz (analogue) 157.1357MHz (digital) |  |
| Radio Wave Format | 16K0 F3E (analogue) 5K80 F1E (digital) |  |
| Antenna Power | 25W | 10W (at point A) 6W (at point B and C) |
| Antenna | 7ABJD0004 (half-wavelength vertical dipole antenna) |  |
| Coaxial Cable | 8D-2V, 12m |  |
| Gain | 2.15dBi |  |

### Conditions

The trial was conducted according to the timeline shown in Table 4.

Timeline of the trial

| Time | Event | Place |
| --- | --- | --- |
| 09:30 | Departed from port | TUMSAT |
| 10:30 | Arrived at point A and started measurement | Point A |
| 11:10 | Moved from point A to point B |  |
| 11:40 | Arrived at point B and started measurement | Point B |
| 12:20 | Moved from point B to point C |  |
| 13:00 | Arrived at point C and started measurement | Point C |
| 13:40 | Finished measurement at point C and started to go back to TUMSAT |  |
| 15:30 | Arrived at port | TUMSAT |

This trial was conducted under following conditions in table 5.

meteorological and sea conditions

| Time | Weather | Temperature (Celsius Degree) | Precipitation (mm) | Wind Direction | Wind Speed (m/s) | Wave Direction | Wave Hight (m) | Tide level(cm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 09:00 | cloudy | 13.2 | 0 | N | 3.0 | NW | 0.13 | 152 |
| 10:00 |  | 13.4 | 0 | NNW | 3.2 | WSW | 0.17 | 160 |
| 11:00 | clear | 14.2 | 0 | N | 2.6 | SE | 0.20 | 161 |
| 12:00 |  | 14.1 | 0 | NNE | 2.2 | S | 0.19 | 152 |
| 13:00 |  | 14.3 | 0 | NNW | 1.9 | WSW | 0.12 | 141 |
| 14:00 | cloudy | 14.7 | 0 | NNW | 2.0 | ENE | 0.15 | 132 |
| 15:00 |  | 15.1 | 0 | N | 1.1 | SSW | 0.16 | 130 |

The observation points for meteorological and sea conditions are following points and figure 3 indicates each point.

Tide level: 35°38.033'N, 139°45.150'E

Wave: 35°34.733'N, 139°48.817'E

Weather, temperature, precipitation and wind: 35°33.2'N, 139°46.8'E

Observation points for meteorological and sea conditions

## METHODS OF TRIAL

### Trial procedure

In this trial, three measurements were conducted at each point using the same methods as follows:.

Transmit a high-pitched audio from Urayasu to Yayoi and evaluators evaluated sensitivity and clarity.

Transmit a low-pitched audio from Futtsu to Yayoi and evaluators evaluated sensitivity and clarity.

Measure the two RSSI values (one is from Urayasu to Yayoi, and another is from Futtsu to Yayoi). Then compare the difference in two RSSI values.

Transmit radio wave from Yayoi and measure the RSSI value at each shore radio station.

Transmit following audio from each shore radio station at same time.

As desired radio wave, high-pitched audio was transmitted from Urayasu:

As interference radio wave, low-pitched audio was transmitted from Futtsu:

At each point, the above procedure was carried out four times: using analogue and digital modulation, and Japanese and English audio.

The above measurements were conducted in the same procedure at point A, B and C, in that order.

point A: 35°32.967'N, 139°52.540'E (app. 4.2NM from Urayasu, app. 14.8NM from Futtsu).

point B: 35°27.462'N, 139°51.963'E (app. 9.7NM from Urayasu, app. 9.5NM from Futtsu).

point C: 35°24.362'N, 139°49.116'E (app. 13.2NM from Urayasu, app. 5.8NM from Futtsu).

Since four measurements were conducted at each of the three points, a total of 12 measurements were conducted in the entire trial.

In addition, each shore radio station was prepared at each point as below.

Urayasu: 35°32.967'N, 139°52.540'E

Futtsu: 35°32.967'N, 139°52.540'E

For the RSSI value, the average of values that were automatically recorded every second during the periods when the radio wave was being received was used.

The Vector Error Rate (VER) was measured as displayed on the measurement instrument. The VER means an index similar in concept to Bit Error Rate (BER), it represents the proportion of errors that occur in the received voice data compared to the transmitted voice data.

### Evaluation methods

In this trial, sensitivity and clarity of the audios that shore radio station transmitted were evaluated in five scores as below.

Evaluation Criteria of the Sensitivity and Clarity

| Evaluation Scores |  | Evaluation Criteria |
| --- | --- | --- |
| 5 | Extremely good | Stable and good reception without noise |
| 4 | Good | Little noise, but it does not interfere with the call |
| 3 | Middle | Weak signal, lots of noise, but content can all be understood at once |
| 2 | Poor | Choppy but understandable if you listen again |
| 1 | Bad | Signal transmission is recognized, but contents cannot be understood. |
| - | No signal | No signal (in case of digital, it is not demodulated at all) |

For each measurement, the scores for sensitivity and clarity were calculated as the average of the evaluations provided by 10 evaluators aboard the Yayoi.

## RESULTS OF TRIAL

### Point A

Following table 7 shows the result of measurement at point A.

Result of the Measurement at Point A

| Modulation | Language | Transmitted | Score | RSSI (dBm) | VER (%) |
| --- | --- | --- | --- | --- | --- |
| analogue | Japanese | Urayasu | 4.6 | -89.8 | - |
|  |  | Futtsu | 2.6 | -111.6 | - |
|  |  | Both | 4.2 | -95.5 | - |
|  | English | Urayasu | 4.6 | -91.1 | - |
|  |  | Futtsu | 2.3 | -114.0 | - |
|  |  | Both | 4.4 | -99.9 | - |
| digital | Japanese | Urayasu | 5.0 | -96.7 | 0.00 |
|  |  | Futtsu | 4.8 | -111.7 | 0.55 |
|  |  | Both | 4.5 | -92.3 | 0.00 |
|  | English | Urayasu | 4.9 | -92.9 | 0.00 |
|  |  | Futtsu | 4.7 | -111.3 | 0.10 |
|  |  | Both | 4.4 | -95.5 | 0.00 |

The differences of two RSSI in each measurement is shown in following table 8.

Differences of RSSI at Point A

| Modulation | Language | Difference (dB) |
| --- | --- | --- |
| analogue | Japanese | 21.8 |
|  | English | 22.8 |
| digital | Japanese | 15.0 |
|  | English | 18.4 |

In addition, some evaluators commented in each measurement as below.

The audio transmitted in digital was clearer

In both analogue and digital, when transmitting at the same time, the audio from Futtsu was heard intermittently or not at all, and it could be heard clearly only after the transmission from Urayasu had ended.

From the result of measurement at point A, digital transmission can be inferred that when the RSSI difference between the desired radio wave and interfering radio wave is approximately 15 to 18dB, the affect of the interference remains at a level where the desired radio wave can be received without causing communication problems.

### Point B

Following table 9 shows the result of measurement at point B.

Result of the Measurement at Point B

| Modulation | Language | Transmitted | Score | RSSI (dBm) | VER (%) |
| --- | --- | --- | --- | --- | --- |
| analogue | Japanese | Urayasu | 3.8 | -106.1 | - |
|  |  | Futtsu | 2.9 | -110.5 | - |
|  |  | Both | 3.5 | -104.3 | - |
|  | English | Urayasu | 4.0 | -103.2 | - |
|  |  | Futtsu | 2.9 | -111.8 | - |
|  |  | Both | 3.6 | -104.5 | - |
| digital | Japanese | Urayasu | 5.0 | -104.8 | 1.80 |
|  |  | Futtsu | 4.5 | -114.0 | 2.80 |
|  |  | Both | 2.7 | -106.1 | 8.00 |
|  | English | Urayasu | 4.9 | -106.9 | 0.00 |
|  |  | Futtsu | 4.6 | -112.4 | 0.50 |
|  |  | Both | 3.4 | -105.2 | 4.00 |

The differences of two RSSI in each measurement is shown in following table 10.

Differences of RSSI at Point B

| Modulation | Language | Difference (dB) |
| --- | --- | --- |
| analogue | Japanese | 4.4 |
|  | English | 8.6 |
| digital | Japanese | 9.2 |
|  | English | 5.6 |

In addition, evaluators commented in each measurement as below.

In analogue transmission, there was a large amount of noise, and the audio from Futtsu could be heard either intermittently or only after the transmission from Urayasu had ended.

In digital transmission, it was felt that clarity decreased when there was little variation in pitch.

About Japanese audio transmitted in digital, the sound was sometimes interrupted, and only about half of it could be heard.

According to the result of measurement at point B, in digital transmission, when the RSSI difference between the desired radio wave and interfering radio wave is approximately 10dB or less, the desired radio wave was received intermittently due to interference, and in some cases only about half of the audio could be heard.

### Point C

Following table 11 shows the result of measurement at point C.

Result of the Measurement at Point C

| Modulation | Language | Transmitted | Score | RSSI (dBm) | VER (%) |
| --- | --- | --- | --- | --- | --- |
| analogue | Japanese | Urayasu | 4.1 | -104.9 | - |
|  |  | Futtsu | 4.6 | -98.9 | - |
|  |  | Both | 3.0 | -101.6 | - |
|  | English | Urayasu | 3.4 | -110.3 | - |
|  |  | Futtsu | 4.4 | -98.5 | - |
|  |  | Both | 1.9 | -102.0 | - |
| digital | Japanese | Urayasu | 5.0 | -107.1 | 2.20 |
|  |  | Futtsu | 4.9 | -100.1 | 1.20 |
|  |  | Both | 0.8 | -103.2 | 8.60 |
|  | English | Urayasu | 4.7 | -109.2 | 1.70 |
|  |  | Futtsu | 4.8 | -100.6 | 1.10 |
|  |  | Both | 1.7 | -101.1 | 9.00 |

The differences of two RSSI in each measurement is shown in following table 12.

Differences of RSSI at Point C

| Modulation | Language | Difference (dB) |
| --- | --- | --- |
| analogue | Japanese | -6.0 |
|  | English | -11.8 |
| digital | Japanese | -7.0 |
|  | English | -8.6 |

In addition, evaluators commented in each measurement as below.

In both modulation systems, the signal from Futtsu was stronger.

In analogue transmission, the audio from Futtsu could be heard, but there was a large amount of noise.

In digital transmission, when the inputs overlapped, the audio became silent and the content could not be understood.

In digital transmission, there were long periods of silence; however, when the audio was audible, it was clear.

From the measurement at point C, when the interfering radio wave was stronger that the desired radio wave, the interfering radio wave was demodulated with noise in analogue transmission. In digital transmission, however, audio was not demodulated and the content could not be understood; when demodulation did occur, the audio was recovered clearly.

### Modulation

Focusing on the modulation systems, a notable difference was observed: when the interfering radio wave was stronger than the desired radio wave, the desired radio wave was received as noise in analogue, whereas in digital neither radio wave could be demodulated.

### Language

Focusing on language differences, evaluation based on VER showed no significant difference in terms of data loss. However, the decrease in the evaluators’ score was greater for Japanese audio, and it is noteworthy that the score fell below one (signal transmission is recognized, but contents cannot be understood), particularly during two audios were transmitted at the same time when the interfering radio wave was dominant. Probable causes include compatibility between the codec and the language, but further research is required.

## CONCLUSIONS

This trial shows following points.

Digital transmission can be inferred that when the RSSI difference between the desired radio wave and interfering radio wave is approximately 15 to 18dB, the affect of the interference remains at a level where the desired radio wave can be received without causing communication problems.

In digital transmission, when the RSSI difference between the desired radio wave and interfering radio wave is approximately 10dB or less, the desired radio wave was received intermittently due to interference, and in some cases only about half of the audio could be heard.

When the interfering radio wave was stronger that the desired radio wave, the interfering radio wave was demodulated with noise in analogue transmission. In digital transmission, however, it was not demodulated and the content could not be understood; when demodulation did occur, the audio was recovered clearly.

When the interfering radio wave was stronger than the desired radio wave, the desired radio wave was received as noise in analogue, whereas in digital neither radio wave could be demodulated.

- The decrease in the evaluators’ score was greater for Japanese audio, and it is noteworthy that the score fell below one, particularly during two audios were transmitted at same time when the interfering radio wave was dominant.

# References

N/A

# Action requested of the Committees

The Committees are requested to note the information in this paper.