Input paper: DTEC6-6.2.3.7

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☐ ARM	☒  ENG		☐ PAP			☐ Input

☒ DTEC	☐ VTS					☒ Information

Agenda item 			n.n

Technical domain/ Task number 2	…………………………………

Author(s)/Submitter(s)			Kongsberg Discovery

# Maritime Alternative PNT Solution (MAPS)

# Summary

## Purpose of the document

This paper provides information to the committees on the VDES/MF Ranging Mode (R-Mode) activity  Maritime Alternative PNT Solution (MAPS) being conducted by a consortium consisting of Kongsberg Discovery (prime), Space Norway, German Aerospace Center and Telespazio UK.

# Background

Maritime shipping has widely adopted GNSS to enhance safety and operational efficiency; however, the availability and reliability of these systems have diminished in recent years. The MAPS initiative, undertaken within the framework of the ESA NAVISP Element 2 programme, seeks to demonstrate an Alternative PNT (APNT) solution that combines the maritime VHF Data Exchange System (VDES) with the Medium Frequency (MF) R-Mode service.

In alignment with IALA’s strategy for resilient PNT and e-Navigation, the MAPS concept integrates three interoperable R-Mode components—Terrestrial VDES R-Mode, MF R-Mode, and Space-based VDE-SAT R-Mode—forming a cohesive hybrid system designed to ensure safe navigation, continuity of timing services, and robust situational awareness.

Figure 2-1 MAPS project logo

# Discussion

Building upon previous European activities such as R-Mode Baltic, Independent Critical Navigation (ICING), and VDES-R Advanced User Technologies for Alternative PNT (VAUTAP), MAPS leverages existing AIS/VDES infrastructure and MF beacon networks to accelerate deployment and reduce cost. The system architecture is designed to promote:

- Complementary, diversity-based PNT sources

- Harmonised international standardization

- Support for VTS, e-Navigation and future autonomous vessel operations

- Regional interoperability across national administrations

VDES R-Mode provides VHF-based ranging and data channels capable of supporting both navigation and communication. MF R-Mode extends service reach with long-range, low-frequency signals proven to deliver stable positioning performance. The addition of VDES-SAT R-Mode enables wide-area coverage through low-earth orbit (LEO) satellites, ensuring service continuity beyond terrestrial limits. Together, these elements form a unified APNT capability that enhances maritime safety, mitigates GNSS outage risks, and supports future IMO/IALA regulatory evolution.

The ambition in phase 1 of MAPS is to develop a solution using R-Mode capable terrestrial VDES base stations and MF beacons and one VDE-SAT R-Mode satellite. The solution will be used to perform demonstrations and measure performance of current techniques, whilst experimenting with newer techniques and how these could be embedded in future evolutions. In the second phase, a scale up activity is envisaged with satellites that deliver an APNT maritime service towards a larger area.

The MAPS hybrid architecture emphasises interoperability and scalability, ensuring compatibility with both legacy and emerging maritime systems. Its design addresses critical operational factors such as signal propagation variability, time synchronisation without GNSS, multipath mitigation, and cross-border coordination of coastal infrastructure.

The MAPS concept strengthens the maritime navigation framework while providing practical, deployable APNT services aligned with IALA’s vision for safe, secure, and efficient maritime operations. Ongoing validation and test activities aim to contribute to the international standardisation pathways required for operational adoption.

# References

ESA's NAVISP Programmes - MAPS

# Action requested of the Committee

- The committee is requested to note the information presented.  It is envisaged that the project will provide further updates to the committee as the project progresses.

- Further information can be requested through Anders Bjørnevik at Kongsberg Discovery.

Leave open if uncertain