Input paper: DTEC6-6.2.3.8

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					X  Input

X  DTEC	□  VTS							□  Information

Agenda item 			n.n

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)	Guillermo Fernández (ESSP), José Luis Martín (ESSP), José Manuel Álvarez (ESSP), Elisabet Lacarra (ESSP).

# Proposal of VDES shore station upgrade to enable SBAS data retransmission

# Summary

IALA G1117 [1] provides an overview of VDES (VHF Data Exchange System) giving additional information about the related technology, concepts, and the use cases supported by VDES. VDES is the next-generation maritime VHF data system that extends and protects AIS (Automatic Identification System) functionality while adding new VHF and satellite data links, wider channel options and new message types allowing vessels and shore systems to exchange more information than AIS was designed to carry. Furthermore, this G1117 [1] guideline considers the option for VDES to retransmit SBAS (Satellite Based Augmentation System) corrections through this technology with the advantage to use directly the original RTCA format [12][13].

This functionality for VDES to retransmit SBAS data have been thoroughly analysed in the input paper “Analysis on the options for retransmission of SBAS data through VDES“ [7] and was presented in DTEC4 (in WG3) and ENG20 (in WG2) committees in 2025. Thus, this paper detailed the mechanisms proposed to enable the retransmission of SBAS data through VDES and specifically the definition of the recommended ASM, VDE-TER and VDE-SAT messages (as per ITU-R M.2092-1 [8]) as well as the required parameters within these messages to identify the type of augmentation data/corrections (e.g. SBAS, RTK, PPP…) and the content and format of these data. This first technical material (extended with the suitable VDES Link IDs for the transmission of these messages), which was proposed as an update to the G1117 [1] in DTEC5 WG3, was reviewed and accepted by DTEC members and incorporated in the G1117 working document to be submitted to next IALA Council for its approval in 2026.

In parallel, it was presented in ENG21 WG2 the input paper “Proposal of VDES ground and mobile stations upgrade to enable SBAS data retransmission. Update of G1129” which proposed the tasks to be accomplished to facilitate the upgrade of VDES shore and mobile stations in order to enable the seamless retransmission of SBAS corrections since VDES shore station receives the data from the original SBAS system/service source (SiS and/or terrestrial means through the corresponding data streams over internet) until VDES mobile station processes and forwards these data to the GNSS receiver on board, in order to improve the accuracy of vessel positioning and navigation while benefit as well from the integrity information within the SBAS data received.

## Purpose of the document

As a continuation of the work introduced in previous DTEC and ENG committees (as listed in previous paragraphs), the current input paper intends to describe the additional elements to deploy on a VDES shore station to enable the SBAS data retransmission, identifying the functionalities and requirements associated to these elements to succeed on this purpose. In relation to the communication between the components which participates in the process of SBAS data retransmission by the VDES shore station, the input paper refers to the different standards published or pending on approval and publication (currently in draft versions and under review and acceptance at IEC level) that shall be considered for the implementation.

## Related documents

- IALA Guideline G1129 – The Retransmission of SBAS Corrections Using MF‐Radio Beacon and AIS. Edition 2.0 December 2017.

- IALA Guideline G1152 – SBAS Maritime Service. Edition 1.0 December 2019.

- IALA Guideline G1117 VHF Data Exchange System (VDES) Overview. Update proposed in DTEC5 to Edition 3.0 (December 2022).

- IALA Recommendation (Informative) R0135 (R-135) The Future of DGNSS. Edition 2.1 September 2020

- ENG17-3.1.2.8 “Retransmission of SBAS data through VDES” input paper.

- DTEC4 -6.2.3.6 (or ENG20-3.1.2.4) Analysis on the options for retransmission of SBAS data through VDES

- DTEC5-6.2.3.10 Draft G1117 on VHF Data Exchange System (VDES)

- ENG21-3.1.2.6 Proposal of G1129 update to detail the retransmission of SBAS data through VDES

# Background

Global Navigation Satellite Systems (GNSS) is considered by IMO in its Resolution A.1046(27) [5] as the primary means of obtaining Position, Navigation and Timing (PNT) information at sea. GNSS alone does not meet the most stringent requirements for navigation (in coastal waters, harbour approaches and harbour entrances) requiring in this case the use of GNSS augmentation data. In such a way, the traditional GNSS augmentation is based on the marine radio beacon DGPS (Differential GPS) augmentation service, provided by maritime administrations, which improve accuracy while providing integrity for GPS (IALA G1112 [4]). As a complementary solution to DGPS (IALA G1129 [2] and G1152 [3]), SBAS (Satellite Based Augmentation System) provides increased accuracy and integrity to the PNT solution computed by a compatible receiver (as per IEC 61108-7 [9]), complementing the national Differential GNSS (DGNSS) services while providing additional capabilities such as the extended regional coverage area (for instance, EGNOS in Maritime as per [11]). Additionally, SBAS could be used as well as a source of GNSS corrections and integrity information to be retransmitted over the following Maritime Servicer Provider’s AtoN:

- DGNSS stations (maritime radio beacons)

- AIS (Automatic Identification System) stations using the AIS Message#17

- VDES (VHF Data Exchange) data channels

About the first two options, DGNSS and AIS stations, currently IALA G1129 [2] describes in detail the concept to enable the provision of SBAS L1 corrections and integrity information through them as well as it proposes the technical architectures to do so.

For VDES, being the evolution of AIS to overcome the main limitations this later presents, is seen as another option to continue providing SBAS corrections and integrity (IALA G1117 [1] and G1129 [2]), specially due to the increase of data bandwidth which characterizes VDES in comparison to AIS.

For VDES, the G1117 update [1] proposed in DTEC5 details the messages and channels (ASM, VDE-TER and VDE-SAT) to facilitate the SBAS data retransmission. For these new messages, it is proposed that VDES will retransmit current SBAS L1 messages as well as the forthcoming SBAS L5 messages (DFMC – Dual Frequency Multi-Constellation). Beyond the standardisation of the messages to provide this data, it is needed to perform the required upgrade on VDES shore and mobile stations to enable the SBAS data retransmission from the original SBAS service provider source to the final maritime user.

# Discussion

As it was described in the paper “ENG21-3.1.2.6 Proposal of G1129 update to detail the retransmission of SBAS data through VDES” [15], it is proposed in this input paper the required upgrade on VDES shore station architecture to enable the retransmission of SBAS data through the corresponding VDES messages assigned to do so (see the G1117 update proposal described in “DTEC5-6.2.3.10 Draft G1117 on VHF Data Exchange System (VDES)” [14] ). Thus, the architecture upgrade shall:

- Facilitate the reception of SBAS data from SBAS Signal-in-Space (SiS) broadcast by SBAS GEO satellites and/or received from alternative terrestrial means via internet (e.g. EDAS Services offered by EGNOS [18])

- Enable the required communications to retrieve and forward the SBAS data to the VDES functional components (VDES presentation interface, controller and transceiver) responsible to encapsulate and format the data as well as to generate the VDES messages and transmission packets for its subsequent transmission through VDES channels (ASM, VDE-TER, VDE-SAT).

  - The communication between those components (within VDES shore station) participating in the retransmission of SBAS data shall be defined according to the IEC 61162-450 [16] and IEC 63514 [17] standards.

## Assumptions

Following assumptions have been made in regards the VDES shore station upgrade to receive the SBAS data from the SBAS system/service provider:

- It is mounted as part of the VDES shore station a GNSS antenna(s) compatible with L1 and L5 to receive the SBAS data through SBAS SiS directly from SBAS GEO satellites.

- VDES shore stations integrates a GNSS and SBAS DFMC receiver (L1 and L5 at least) which is properly connected to the GNSS antenna mounted.

- VDES shore station belongs to a network deployed on the ground being connected (and monitored by the VDES service provider) through internet.

  - This is particularly relevant to enable the SBAS data reception through terrestrial means via internet (for instance, by adopting the NTRIP protocol). It is assumed a SW client could be deployed on the VDES shore station to enable the reception of the data through this interface.

- It is referred for this analysis to the model architecture already included in G1117 VDES Overview guidelines [1]. It is assumed this model architecture should not differ to the standard architecture that could be defined by IEC in the new Proposal Work Item 80-867 VDES shore station.

## Way forward proposed

- Review and consolidation of the analysis/conclusions detailed in Annex A.

- Identification of potential updates to be covered in further analysis. Indeed, the next proposal will cover the VDES mobile station architecture upgrade as well.

- Encourage the involvement of IALA experts to support further analysis. It is assumed intersessional work could be required between committees (plan to be defined and agreed among collaborators).

- Liaison among DTEC and ENG committees would be needed to work on this topic in both committees. It is understood the technical content proposed could be integrated in G1117 or G1129 (to be decided).

- Mature the technical description of the VDES shore station architecture upgrade (s) to retransmit SBAS data through VDES and its inclusion as part of IALA guideline material (for instance, G1129, G1117…) at the end of 2023-2027 quadrennial plan.

- Discuss on the need of additional workstreams to work on and follow-up, mainly with standardization bodies (IEC, ITU, RTMC…), to complement the technical description of the retransmission means and the shore and mobiles stations architecture definition. The required liaison process could be put in place if needed.

# References

IALA Guideline G1117 VHF Data Exchange System (VDES) Overview. Update proposed in DTEC5 to Edition 3.0 (December 2022).

IALA Guideline G1129 The retransmission of SBAS corrections using MF-radio beacon and AIS. Edition 2.0. June 2022

IALA Guideline G1152 SBAS Maritime Service. Edition 1.0 December 2019.

IALA G1112 Performance and Monitoring of DGNSS services in the frequency band 283.5 – 325 KHz

IMO Resolution A.1046(27) World Wide Radio Navigation System

IMO Resolution A.915(22) Revised Maritime Policy and Requirements for a future GNSS

DTEC4 -6.2.3.6/ ENG20-3.1.2.4 Analysis on the options for retransmission of SBAS data through VDES

ITU. Recommendation ITU-R M.2092-1 Technical characteristics for a VHF data exchange system in the VHF maritime mobile band. Ed Sept 2024 as a draft.

IEC 61108-7 SBAS L1 -Receiver equipment – Performance standards, methods of testing and required test results

EGNOS Maritime service status and status (ppt), Silvia Porfili, EUSPA. EGNOS Workshop 2024 (Dublin, 13/03/2024).

EGNOS Safety of Life assisted service for Maritime users (ESMAS). Service Definition Document (SDD). Issue 1.0. March 2024

- DO-229 Minimum Operational Performance Standard (MOPS) for GPS/WAAS airborne equipment

- ED-259A/RTCA DO-401 Minimum Operational Performance Standard (MOPS) for DFMC SBAS airborne equipment

- DTEC5-6.2.3.10 Draft G1117 on VHF Data Exchange System (VDES)

- ENG21-3.1.2.6 Proposal of G1129 update to detail the retransmission of SBAS data through VDES

- IEC 61162-450 Maritime navigation and radiocommunication equipment and systems - Digital interfaces - Part 450: Multiple talkers and multiple listeners - Ethernet interconnection

- IEC 63514 - Maritime navigation and radiocommunication equipment and systems - VHF data Exchange system - Requirements and test methods for shipborne mobile Station (draft)

- EGNOS Data Access Service. Service Definition Document (SDD). Issue 3.0. December 2024

# Action requested of the Committee

The Committee is requested to:

Review the input paper and the technical content written down in Annex A.

To agree on the way-forward proposed in section 3.2.

Shore Station Architecture Upgrade

Architecture Upgrade. Options proposed.

At the time this paper has been written down, the only available VDES shore station architecture standard (if any) is the one already included within G1117 VDES Overview in its Annex A section A.7, taking this as a reference for the two architecture upgrade options presented below.

Figure 1: VDES shore station reference architecture (as per IALA G1117)

- Option A: SBAS data received on VDES shore station only via SiS from SBAS GEOs

Figure 2: VDES shore station architecture upgrade (Option A)

- Option B: SBAS data received on VDES shore station via SiS from SBAS GEOs plus via terrestrial means from SBAS service provider

Figure 3: VDES shore station architecture upgrade (Option B)

In comparison with option A, this second option B integrates an additional module “SBAS data quality check” responsible to compare the SBAS data received from both sources (SBAS SiS and SBAS service provider terrestrial means) and to select the most suitable SBAS data to be retransmitted by the VDES shore station. An internal “decision logic” shall be defined in this module to perform this comparison and data selection.

Latency analysis

The retransmission of SBAS data from VDES means an alternative approach to provide SBAS data. This alternative approach has as a potential drawback, the latency introduced by the VDES shore station (and the mobile station) in the reception of the SBAS data by the user when it is compared with the data reception directly from GEOs satellites (via SiS). In this regard, it is performed an analysis of the maximum acceptable latency introduced through the retransmission process (considering the impact from both VDES shore and mobile stations) to comply with integrity requirement defined in IMO Res. A1046(27) to ensure a safe navigation in coastal waters, harbour approaches and harbour entrances, which specifically requests “An integrity warning of system malfunction, non-availability or discontinuity should be provided to users within 10 s”. This maximum time established to provide the integrity warning is known in GNSS domain as the Time-To-Alert (TTA) parameter.

To perform this analysis is worth of detailing the complete retransmission process, which comprises the following gradual steps:

- The reception of the SBAS data at the VDES shore station.

- The required process at VDES shore station to format and generate the related ASM, VDE-TER and VDE-SAT which contain the SBAS data and their broadcast through the corresponding VDES transmission channels.

- The reception of those ASM, VDE-TER and VDE-SAT SBAS data related messages at the VDES mobile station.

- The required process at VDES mobile station to retrieve the SBAS data from those messages and their subsequent provision to the GNSS primary receiver on board.

Assumptions for this latency analysis

Following assumptions have been made in regards the maximum admissible latency:

- On the SBAS system side:

  - For the SBAS legacy service (single-frequency and single-constellation augmentation), the maximum Time-To-Alert (TTA) commitment at SBAS system is 5.2 seconds. This TTA value is defined between the onset of the alarm condition and the time that the last bit of the alert message from SBAS SiS reaches the antenna of the user receiver.

  - For the SBAS terrestrial data provision (usually via internet adopting a NTRIP protocol), there is an additional latency associated to this terrestrial transmission mean compared to the broadcast of SBAS data through GEO satellites. This latency is defined as the time elapsed since the transmission of the last bit of the navigation message from the SBAS space segment until the same data leave the SBAS terrestrial (data provision service) system. This latency depends on the SBAS system and service provider. For this latency, it is assumed a maximum 1 second latency.

- On the VDES shore station side:

  - It should be accounted the time it takes to the VDES signal (containing the SBAS corrections and integrity alerts retransmitted) to reach the VDES mobile station from the VDES shore station. This time may be dependent on the VDES channel selected (ASM, VDE-TER or VDE-SAT) as well as the corresponding transmission means assigned to broadcast the message. For the current analysis, a maximum time of 1 second will be considered.

- On the VDES mobile station side:

  - It is assumed a maximum time of 0.5 seconds (to be discussed) for the VDES mobile station to retrieve the SBAS data from VDES messages and its provision to the GNSS primary receiver on board.

  - Additionally, it should be accounted the time required by the GNSS primary receiver on board to process and display the alert. This time is set to 0.8 seconds in the aviation domain as per MOPS DO-229 [12]. For the current analysis, considering there is no such a reference value for this receiver processing time in Maritime, it will be adopted the same value hereafter.

Latency quantification and apportionment.

The maximum latency values apportioned to the different components and processes within the VDES shore station is quantified considering the values already identified in the assumptions and the integrity warning requirement which specifies a maximum 10 seconds to alert the user. Then:

- For the architecture proposed in Option A, the total maximum latency allowed for the VDES shore station to process, encapsulate and retransmit the SBAS data is 2.5 seconds. These 2.5 seconds shall be split between the following functionalities identified:

  - Maximum latency for the encapsulation of the SBAS message received from SiS at VDES shore station (through the GNSS antenna and receiver) and its delivery to the VDES Presentation Interface as per the corresponding format defined in  IEC 61162-450 [16] and IEC 63514 [17] standards.

  - Maximum latency for the VDES Transceiver to retransmit the SBAS data over ASM, VDE-TER or VDE-SAT.

- For the architecture proposed in Option B, the total maximum latency allowed for the VDES shore station to process, encapsulate and retransmit the SBAS data is reduced to 1.5 seconds due to the additional latency introduced by the SBAS data terrestrial mean. Thus, these 1.5 seconds shall be apportioned as it follows:

  - Maximum latency for the SBAS Quality Check Module to compare and select the SBAS data from SiS or from the terrestrial mean (if any) sources to be retransmitted.

    - It is assumed the encapsulation of the SBAS message received from SiS and its delivery to the SBAS Quality Check Module is negligible or less than the 1 second delay associated to the SBAS data reception through the terrestrial mean.

  - Maximum latency for the delivery to VDES Presentation Interface of the SBAS data in the corresponding format.

  - Maximum latency for the VDES transceiver to start to retransmit the SBAS data over ASM, VDE-TER or VDE-SAT.

The following figure indicates a high-level view of all the elements and process that are followed when an alert occurs and needs to be displayed to the user in less than 10 seconds (TTA). The green lines are the new elements introduced due to the retransmission of SBAS data through VDES.

Figure 4: Latency apportionment for the SBAS data retransmission through VDES

SBAS Quality Check Module description (only for Option B). Minimum functionality and requirements.

The SBAS Quality Check Module is responsible to retrieve the SBAS data received from the SBAS GEO satellites via SiS (by means of the VDES shore station GNSS receiver) and from the SBAS data provision terrestrial mean. Once the retrieval process is completed, the main function of the module is to perform a consistency check on the data gathered from both sources, to ensure the appropriateness of the SBAS data to be retransmitted by VDES shore station to the user. Thus, the module will parse the timestamp of the SBAS messages and the SBAS PRNs to correctly compare the corresponding data received from both sources. If the SBAS data received from the two sources are not equivalent, the SBAS Quality Check Module shall register the inconsistency for potential notification to the VDES service provider.

The SBAS Quality Check module shall be configurable in accordance with the following options (inter alia):

- Terrestrial means connection parameters.

- Number of SBAS messages to be retransmitted per frequency (one or two different GEOs).

- VDES transmission means to be used for SBAS data retransmission (ASM, VDE-TER, VDE-SAT).

- Authentication features

Once the consistency check is successfully accomplished, the SBAS Quality Check Module shall retransmit to the VDES Presentation Interface the SBAS data using the format specified in IEC 61162-450 standard & IEC 63514 - VHF Data Exchange System (VDES) Shipborne mobile station draft standard (this draft standard is proposed until the corresponding IEC standard for the VDES shore station is developed).

Leave open if uncertain

The antennas already on the market are capable to receive several signals in L-band (e.g. L1 and L5) as well as other signals in that L-band (e.g. L2, E6…).

The current GNSS receivers on the market are capable to process messages from several constellations as well from SBAS (SBAS is not an additional feature and so this capability does not mean an additional cost to the receiver)

For ASM and VDE-TER it is understood 1 second latency is overestimated due to the VDES shore station coverage. For the VDE-SAT this latency value should be confirmed.

A “decision logic” will be defined for the module to perform the consistency check

This terrestrial means connection parameters configuration should be coherent with the configuration applied in the VDES shore station GNSS receiver. Thus, the same SBAS Service Provider should be selected for both SBAS data sources.