Input paper: DTEC6-6.2.2.8

Input paper for the following Committee(s): 	     		              Purpose of paper:

□ARM	□ENG		□PAP				☑ Input

XDTEC	□VTS						□ Information

Agenda item			8.7

Technical domain/ Task number	1.9.4………………………

Author(s)/Submitter(s)			China Maritime Safety Administration

# proposals for the Work Plan of the VTS Committee Based on the Advanced Decision Support System of Yangshan VTS

# Summary

The advanced decision support system of Yangshan VTS is a creative upgrade based on traditional VTS decision support tools (DST). It is centered on the core concepts of data-driven, risk-oriented and human-machine collaboration. By introducing the dual-engine architecture of "data-driven + mechanism model", it realizes the transformation of VTS decision support from "passive alarm" to "active planning", and comprehensively surpasses traditional decision support tools in terms of technical concepts, application methods and system performance. It is an intelligent decision-making core that conforms to the future VTS work vision of IALA.

## Purpose of the document

Introduce the basic situation of the advanced decision support system of Shanghai Yangshan VTS;

Recommend that the IALA VTS Committee establish a new task for the 2028-2030 work plan to revise Guideline G1110, incorporating the functional and performance standards of Advanced Decision Support Tools into the guideline to promote the coordinated development of advanced decision support tools worldwide.

## Related documents

DISCUSSION PAPER FUTURE VTS A LIVING DOCUMENT (Edition 2.2).

Resolution A.1158(32) GUIDELINES FOR VESSEL TRAFFIC SERVICES.

G1110 Use of Decision Support Tools for VTS Personnel.

# Background

The purpose of VTS is to contribute to the safety of life at sea, improve the safety and efficiency of navigation and support the protection of the environment within the VTS area by providing timely and relevant information on factors that may influence ship movements and assist onboard decision-making, monitoring and managing ship traffic to ensure the safety and efficiency of ship movements, and responding to developing unsafe situations.

Under existing VTS operational models, although decision support tools have improved the situational awareness of VTS operators to a certain extent and provided support for their decision-making processes, the achievement of VTS objectives still relies heavily on the professional experience of individual operators. Under the combined effects of high-density traffic flows and multiple types of incidents, physical fatigue and mental exhaustion among VTS operators persist.

With the in-depth application of new technologies and new practices such as MASS, just in time, route exchange, and MCP, the limitations of the traditional VTS decision support model have become increasingly obvious, and the operation mode of VTS will face a profound transformation. The mature application of technologies such as AI, large language model, and deep learning has provided the possibility for the optimization and upgrading of VTS decision support tools.

To adapt to the development of new forms of shipping industry and to promote the effective implementation of vessel traffic services under new shipping scenarios, Yangshan VTS has creatively optimized its decision support tools by deep learning, digital twin technology, and artificial intelligence. The resulting VTS Advanced Decision Support System partially replaces many repetitive, data-driven tasks traditionally performed by operators, effectively alleviating their workload. This enables operators to focus on more important decision-making tasks and more targeted traffic planning and management, while also laying the foundation for data flow and situational awareness models to support future developments in VTS.

# Discussion

The advanced decision support system of Yangshan VTS is based on the core logic of "data flow-driven and hierarchical progressive", and builds a four-stage full-chain technical architecture covering "situation awareness - risk identification - decision support - key services".

## Core elements

### Data standardization

The objective of data standardization is to establish a unified set of standardized data, ensuring consistent interpretation and usability. The key component is the development of an AI-based VTS data application dictionary, which defines clear semantics, formats, and value ranges for each data item. The primary task involves integrating multi-source, distributed, and multimodal data, including AIS, radar, electronic navigational charts, video surveillance, meteorological and hydrological information, navigational elements, and traffic management information, through temporal and spatial alignment and harmonization. In combination with capability features derived from environmental conditions, including time to encounter, maneuvering margin, and safe distance, these data are consolidated into a time-updated, standardized comprehensive situational data flow centered on vessels.

Data standardization logic diagram

Schematic diagram of AI analysis logic for standardized data implementation

### Risk estimation

The core task of risk estimation is to calculate the "safe activity space" of a single vessel based on the comprehensive situation data flow, predict the short-term movement trend of the vessel, and then upgrade from single-vessel situation perception to multi-vessel correlation analysis. A structured "risk event flow" containing elements such as time window, participating vessels, spatial location, and risk level is generated, and risk scenarios with management significance are extracted from it. The advanced decision support system of the Yangshan VTS, in combination with management requirements, can conduct risk estimation for traffic scenarios according to event type, vessel type, and areas.

Schematic diagram of a multi-vessel correlation analysis model based on single-vessel safety operating space

Risk estimation and awareness based on integrated situation data streams

### Incident handling

The core task of incident handling is based on the risk event flow and comprehensive situation data flow. On one hand, it visualizes the risk events and converts them into decision support content that the VTS operator can understand and execute, and collects the feedback from the VTS operator. On the other hand, it correlates the speed and course adjustment instructions, based on the logical loop of "risk incident-involved object-disposal plan-execution tracking", with authorization, many data-driven tasks that were previously executed by VTS operators are shifted to be autonomously handled by the system.

Case of incident handling

### Intelligent navigation

The core task of intelligent navigation is to extract the comprehensive situational data streams of ships that require special attention, such as LNG carriers and large oil tankers, based on the comprehensive situational data flow. It scans forward along their routes and continuously updates the potential navigation risks that may exist in the future. By combining safety requirements and efficiency demands, it generates dynamic navigation suggestions that change over time and presents them in an intuitive manner for prompt notification. The navigation suggestions and risk estimation results can be sent to the pilots and VTS personnel according to the risk level and personnel authority.

Intelligent navigation user interface

## Comparison OF CHARACTERISTICS

A comparison of the characteristics of the Yangshan VTS Advanced Decision Support system and the conventional decision support tool is presented in the table below.

Comparison of characteristics

| Dimension | Conventional DST | Yangshan VTS Advanced Decision Support system |
| --- | --- | --- |
| Technical Idea | Toolset Thinking Independent alarm module Limited rule expansion | Platform thinking Unified Decision Engine Continuous learning and model iteration |
| Data Utilization | It mainly relies on structured data such as radar and AIS, and has limited cross-source fusion capabilities. | Make full use of multi-dimensional data such as radar, AIS, CCTV, meteorology, hydrology and historical trajectories to construct a high-dimensional situation model. |
| Risk Assessment | Static rules + human experience Difficult to quantify dynamic risks | Introduce the quantitative concepts consistent with the risk management toolkit Combine with AI modeling to form a real-time risk field. |
| Adaptive Capability | The rules have limited universality and need to be adjusted manually according to the ship type and weather conditions. | Through cross-ship type and adaptive modeling, autonomous learning is conducted to understand the control characteristics and risk boundaries of different ship types and environments. |
| Response Mode | Primarily reliant on simple warning mechanisms and manual assessment, with responses being largely reactive. | Focus on prediction and prevention, and provide proactive intervention suggestions. |

# References

Resolution A.1158(32) GUIDELINES FOR VESSEL TRAFFIC SERVICES.

DISCUSSION PAPER FUTURE VTS A LIVING DOCUMENT (Edition 2.2).

G1110 Use of Decision Support Tools for VTS Personnel.

VTS49-8.2.8 The Application of a VTS Decision Support Tool based on Artificial Intelligence.

Committee-Input-paper-template.

# Action requested of the Committee

The Committee is requested to:

Pay attention to the content of this proposal;

Establish a new task to the 2028 - 2030 work plan to revise Guideline G1110, incorporating the functional and performance standards of advanced decision support tools into the guideline to promote the coordinated development of advanced decision support tools worldwide.

Footer will automatically populate