## Synchronising of flashing lights

Draft text for input paper to EEP10 on EEP9 action item 54.

#### Flash synchronisation

When two or more lights are to be synchronised, consideration must be given to the individual flash characters. It will be necessary to ensure that the flash rhythms are identical, or that the various periods are equal or multiples of a given period.

Examples

| Example | Light A | Light B | Light C |
| --- | --- | --- | --- |
| 1 | Flash 5 sec | Flash 5 sec | Flash 5 sec |
| 2 | Flash 5 sec | Flash (2) 5 sec | Flash (3) 5sec |
| 3 | Flash 5 sec | Flash 10 sec | Flash 15 sec |

In practice, the less complicated the relationship between characters, the better the system is likely to be.

#### Methods of implementation

Synchronisation requires having identical time information available at all the lights which are to be synchronised. This can be accomplished in at least two ways.

- A sufficiently accurate clock at each light location

- Reception at each light location of the same timing data from a single source

In practice the latter is usually employed, and implementation is done by either a radio broadcast system set up for the purpose, or by reception of an existing timing signal. Early synchronisation systems often employed a special local radio transmitter, emitting a timing signal received at each light.

Nowadays various local or global time signals are available and dedicated radio transmitters are not required. The most commonly used timing signal is that contained within the GPS GNSS system, but GLONAS can also be used.

In some countries, radio timing signals are broadcast at VLF and these have been used for synchronisation of lights. In Japan, the JJY signals at 40kHz and 60kHz have been used, with a small H-field antenna built into the lantern, but such products are now largely phased out in favour of GPS.

Note that synchronisation of the timing of the lights in the group is all that is needed. Synchronisation to GPS time or other standard is not needed.

The synchronisation accuracy need not be high. An accuracy of 100 parts per million is acceptable in most cases.

#### Field experience

There can be a risk of a mirror effect when a synchronised leading line uses the same characters in both lights.  If the rear light fails, the user can in some circumstances see the mirror of the front light in the water and then believe that he is in the line.

Work carried out in Finland, see below, suggests that having small groups of lights synchronised, such as 2 or three pairs, is easier for the mariner to use than synchronising all the lights in a channel.

However experience from Sweden is that when synchronising the lights on buoys in a channel, it is preferable that all the lights in the whole channel are synchronised. This gives the user a clear and distinct view over the whole channel and is reported as being appreciated by the user. Use of the same character for the port and starboard lights of a “gate” in a channel also is reported as assisting the user.

Trials in Sweden have shown that using “running lights” [or “ripple effect”] is not to be recommended.

Obviously GPS synchronisation is preferable to using a cable between the lights.

END

Drafted by

Christian Lagerwall

Ian Tutt

Mike Card