German Federal Waterways Administration

Traffic Technologies Centre (TTC)

Lights and Colour

Frank Hermann

IALA EEP 'surface colours'

Letter to the Group

Preface

This is an input to the group. It is intented to report the work the TTC did on surface colours. It is based on an internal report ‘surface colour and daytime visibility of buoys’ (‘Farbgebung und Tagessichtbarkeit von Tonnen’) and contains:

1. General remarks about colours for buoys

2. Three steps of perception

3. About threshold of colours

4. Colour versus shape

5. Desaturation caused by the atmosphere

In sequence to the report we worked on

6. New regions

7. Transformation to L*a*b*

8. Colour atlases

9. Supplement

with help from Sabina Schollmeier

Sabina Schollmeier

(49)261/9819-2412

sschollmeier@fvt.wsv.de

Frank Hermann

(49)261/9819-2413

fhermann@fvt.wsv.de

1. General remarks about colours for buoys

There are a lot of different discussions about buoys in Germany. The discussion is mainly influenced by the aim to reduce the cost for the buoys and their maintenance.

To set back the focus on the main task of a buoy, it became inevitable to make a list of the features of a buoy and to make a valuation between the features.

As a result we got the distinction between the so called primary and secondary features:

Primary features give reasons for a buoy (or a light) itself. They explain why we use buoys for a waterway and are linked to the nautical requirements.

Secondary features are features to ensure that a buoy (or a light) will work properly. They are necessary to support the primary features.

Example:

Primary features of a buoy are:

- colour and shape

- radar echo

- position

- availability

Secondary features are:

- mooring

- maintenance

- floating in water

- lifetime

- costs

In general the secondary features should not limit the quality of the primary features. However the primary features should not cause secondary features that cannot be fulfilled (e.g. because they are too expensive).

In detail the primary features of a (lighted) buoy are:

- colour (daytime: surface, nighttime: lights)

- shape (topmarks and special arrangement of coloured stripes or areas)

- flash characters of the light (nighttime)

- radar reflector

- position

- availability

The surface colour belongs to the primary features of a buoy and there have to be very hard 'secondary' arguments to justify the limitation the quality of that colour.

2. Three steps of visual perception

When approaching an object from a far distance we move through three different steps of visual perception (I directly translate the German words, might be you have different definitions):

(1) Object is visible: You see something but you don't know what it is.

For example: The object is still too small to be recognized as a certain type of buoy.

(2) Object is recognizable (identified): You see the object and know what it is, because you identify the colour or shape.

For example: The object can be recognized to be a certain type of buoy of the IALA buoyage system.

(3) Object is conspicuous: You cannot put the eyes away from the object.

Although we try to get distant buoys to become conspicuous, an object is conspicuous for your eye, when you just stand in front of it.

For the visual features (shape and colour) it becomes evident to distinguish between a visible (1) and an identified (2) object.

3. About Threshold of colours

To quantify the visual performance it is helpful to define the Maximum Distance of Recognition MDR.

MDR is the maximum distance the shape or the colour of the object can be identified.

In Germany we have the convention that the shape of a simple object (cone, cylinder or ball) can be identified when the angle is at least 3' (0.05°). This leads to an equation for MDR.

Fig. 1

In addition it is necessary that the object has enough contrast to the background.

For large objects, a distance MDR can be defined by the luminance contrast

C ≥ 0.05 (that is the international accepted value).

From IALA dictionary:

C=L2−L1L1

L1	luminance of the background

L2	luminance of the object

There are formulae for the threshold of the luminance contrast for small objects. However the concept of contrast can only be applied in a grey world.

When colour comes into consideration the contrast is not the right quantity to describe the visual performance.

Example: A red area with luminance 1 against a background with luminance 1 has a contrast of zero, but can appear conspicuous.

The identification of a colour does not depend on the luminance contrast.

The identification of colour depends on the luminosity and the saturation of the colour (The background at sea is assumed to be grey.).

Example: Assume a coloured area against a grey background. When the coloured area is very dark, you cannot identify the colour. Because of the luminance contrast you will see the area very well. However when you do not have a specific shape of the area, it will not show any information to the observer (perception step 1) except the information 'there is something'. When the colour gets brighter you will get the colour information (perception step 2). For large objects you get the information even if contrast is negative, for small objects it might be necessary to have positive contrast to identify the colour.

The luminosity of the colour can be described by the ratio L2/L1.

The saturation can be described by DIN 6164 [1]. There are alternative ways.

Unfortunately we have no theory for the identification of colour, which combines the influence of the size, luminosity and saturation of the coloured object, and that can be used to calculate MDR.

In general the distance MDR increases with the luminosity and the saturation of the colour [2].

In consequence fluorescent colours have a greater MDR.

4. Colour versus shape (German buoys)

In the following figures the shape of two typical German buoys are shown. The shape of the German buoys do not represent any information in accordance with IALA buoyage system (cylinder, cone , ball). For some buoys there are top marks.

Fig. 2

Fig. 3

The top mark may be omitted for lateral buoys, so colour is the only visible (primary) feature at daytime.

From the geometrical convention we use in Germany we can derive three distances for this certain type of buoy (LT 81).

(a) buoy becomes visible	3 300 m

(b) identification of the stripes 'cardinal west'	1 000 m

(c) identification of the shape of the top marks	 	690 m

For all three aspects it is necessary to have a contrast or colour difference against the background. (a) and (b) require a suitable colour to ensure that the buoy is identified according to the IALA buoyage system.

When a very saturated and bright colour is applied (for lateral green and red only) the buoy can be identified in the same moment as it becomes visible. So the MDR is 3 300 m for good weather conditions for lateral buoys.

Conclusions (for German buoys only):

Considering the primary feature 'colour' of a buoy, the lateral marks with bright and saturated colours (fluorescent) have the longest MDR and are therefore the most effective way to mark a waterway.

The distances to identify a buoy with two colours or with top mark (single danger, cardinal) are always much shorter than for the lateral buoys. This is a disadvantage caused by the current IALA buoyage system.

5. Desaturation caused by the atmosphere

Although there is no theory about the maximum distance of recognition for coloured objects, which includes luminosity, saturation and size of the object, it is possible to compare the colours.

One way to evluate the performance of colours is done with the law of Koschmieder [3], which can be transformed into spectral equations (Hoffmann, [4]).

Fig. 4

Figure 4 shows the geometry to explain the influence of the atmosphere on the light that reaches the observer.

Light from the object is scattered away from the direction to the observer (1).

Light from the background is scattered into the direction to the observer (2).

Remark:

In detail the amount of light scattered into one direction depends on the position of the sun. When the sun is behind the object, the amount of light scattered into the direction of the observer is higher than predicted. When the sun is behind the observer, it is less. So Koschmieder's law gives the mean value of the scattered light.

In general Koschmieder's law is used to explain the decrease of the contrast between the object and the background.

Hoffmann extended the law to spectral formulae. This can be used to calculate the change of colour when the light moves through the atmosphere (see appendix A1).

To simplify the calculation we did not yet take into account the spectral dependence of the atmospheric transmission. We will do this in future.

The scattering of light in the atmosphere causes two effects on the light the observer sees.

The first effect is that the visible luminance of the object approaches the luminance value of the background when the distance between object and observer increases. This is shown for two green colours in figure 5 and a meteorological visibility of 10 M (Ordinary green is RAL 6024).

Fig. 5

The second effect is the desaturation. This is shown in figure 6 for fluorescent colours.

Fig. 6

Figure 6 is based on outdoor measurements of coloured samples and application of the theory 'Koschmieder / Hoffmann' as explained in appendix A1.

We still have no threshold to evaluate between the different colours. We will not get a threshold, which is based on size, luminosity and saturation of the coloured object, since there is no theory. However looking at the saturation alone, we can define a threshold (see figure 7).

We chose the saturation S = 6 as the threshold. This decision is arbitrary of course, but 'S = 6 ' corresponds to the border of the region of red and green towards white (for green S = 5..6, and for red S = 6..7). So it represents an existing convention.

Fig. 7

In figure 7 we compared a fluorescent green and an ordinary green RAL 6024. We get an intersection at S = 6 for:

fluorescent green: 	3 700 m

green RAL6024:	 400 m

At the distance of 3 700 m fluorescent green produces nearly the same colour stimulus as RAL 6024 at 400 m.

So we can say that - in this model - the fluorescent colour is about 9 times more effective than the ordinary green.

We use this procedure to compare different green and red colours only. This is because the green and red colour is the only visible feature for most lateral buoys at daytime.

We have investigated six different colours and the results are presented in table 1.

| Colour | Distance for S = 6 | L2/L1 |
| --- | --- | --- |
| Fluorescent Green | 3700 m | 1,07 |
| Preferred Ordinary Green | 2200 m | 0,89 |
| Green RAL 6024 | 400 m | 0,78 |
| Fluorescent Red | 3900 m | 1,36 |
| Preferred Ordinary Red | 1800 m | 0,91 |
| Red RAL 3020 | 1500 m | 0,74 |

Table 1

Figure 8 shows the colorimetric values of the six colours.

Fig. 8

It can be read from table 1 that the RAL colours, which are used for road signs, are not suitable for buoys. There are ordinary colours (preferred) that ensure a greater distance of recognition than the RAL colours.

The preferred colours benefit from their higher degree of saturation and their luminosity.

6. New regions

There are a lot of aspects about the application of the theory of 'Koschmieder and Hoffmann' to describe the performance of colours. We did not investigated all aspects neither we got the universal theory.

But for green and red colour the consequence is easy and well known:

'Use bright and saturated colour to get the highest MDR.'

The results of our investigations only demonstrate the strength of the effect of saturation and luminosity on the distance of recognition.

In consequence we created new regions for the surface colours.

We now recommend the regions shown in figure 9. If required we can present explanations for every border.

Remarks:

In Germany we have two different types of regions (derived from German standard, DIN 6171) and we want to use these types in our new recommendation for German waterways:

The first type is for 'new condition' (start box): chromaticity limits when the colour on the buoy is new. Applied in Germany for measurements to give an approval or a denial.

The second type is for 'used condition': For the whole lifetime of the buoy the colour must stay within this chromaticity limits. When it leaves the region, a new painting is required.

The IALA regions can be compared to 'used condition'.

The differences between IALA and our new specifications include different values for luminance factor. This is not visible in x-y-diagrams.

As a consequence of our new regions the colours green RAL 6024 and red RAL 3020 are rejected now.

Fig. 9

‘New condition':

|  |  | Corners of the Chromaticity Regions |  |  |  | luminance factor |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | Ordinary | Fluorescent |
| Red | x | 0.649 | 0.602 | 0.701 | - | ≥ 0.12 | ≥ 0.25 |
|  | y | 0.351 | 0.348 | 0.249 | - |  |  |
| Yellow | x | 0.479 | 0.456 | 0.484 | 0.512 | ≥ 0.50 | - |
|  | y | 0.520 | 0.494 | 0.466 | 0.487 |  |  |
| Green | x | 0.100 | 0.222 | 0.300 | 0.300 | ≥ 0.12 | ≥ 0.25 |
|  | y | 0.831 | 0.543 | 0.605 | 0.693 |  |  |
| White | x | 0.295 | 0.305 | 0.335 | 0.325 | ≥ 0.75 | - |
|  | y | 0.325 | 0.315 | 0.345 | 0.355 |  |  |
| Black | x | 0.260 | 0.300 | 0.385 | 0.345 | ≤ 0.02 | - |
|  | y | 0.310 | 0.270 | 0.355 | 0.395 |  |  |

'Used condition':

|  |  | Corners of the Chromaticity Regions |  |  |  | luminance factor |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | Ordinary | Fluorescent |
| Red | x | 0.649 | 0.565 | 0.674 | - | ≥ 0.12 | ≥ 0.25 |
|  | y | 0.351 | 0.345 | 0.236 | - |  |  |
| Yellow | x | 0.465 | 0.427 | 0.470 | 0.522 | ≥ 0.50 | - |
|  | y | 0.534 | 0.483 | 0.440 | 0.477 |  |  |
| Green | x | 0.003 | 0.187 | 0.313 | 0.313 | ≥ 0.12 | ≥ 0.25 |
|  | y | 0.632 | 0.452 | 0.566 | 0.682 |  |  |
| White | x | 0.290 | 0.300 | 0.350 | 0.340 | ≥ 0.75 | - |
|  | y | 0.320 | 0.310 | 0.360 | 0.370 |  |  |
| Black | x | 0.260 | 0.300 | 0.385 | 0.345 | ≤ 0.02 | - |
|  | y | 0.310 | 0.270 | 0.355 | 0.395 |  |  |

7. Transformation to L*a*b*

My colleague Sabina Schollmeier has a lot of contact to paint suppliers. That is why she is keen on a transformation to other coordinates and standards.

The first choice is L*a*b*. However the transformation is not easy. L*a*b* might be better, when the physiological aspects are regarded.

I tried to discover the secrets behind L*a*b* and got many reasons to stay at Yxy (Sabina does not actually agree with that statement).

The most important advantage of Yxy is: The regions defined are independent of Y. This is not true for L*a*b*. L* represent something like the luminosity of the colour and it is called ‘lightness’. But when you have a fixed region in the x-y plane, the transformation into a*b*-plane looks different for different Y. For details see appendix A2.

Example:

We have the same chromaticity region for fluorescent and ordinary colour (in x-y-plane). In a*b* we get different regions for fluorescent and ordinary colours.

As a compromise we could define preferred colours in L*a*b* (as a point not a region) and give intervals for L*,a* and b* so that regions defined in L*a*b* are smaller than in Yxy.

So the paint suppliers can use L*a*b* as an alternative and get into the true Yxy-regions for sure.

8. Colour atlases

Sabina looked for colour atlases too. We did not find colours in the RAL atlas, that fit to our new regions. She contacted RAL institute, which is located near Cologne and asked, whether we can create a new RAL colour. It is possible and costs approx. 1000 € per colour.

As an alternative Sabina found a colour atlas from the Scandinavian Colour Institute (www.ncscolour.com). The atlas includes more colours than RAL and might be useful without creating new colours.

For both transformation to L*a*b* and the colour atlases it seems to be necessary to define a single point for each colour at first. Of course I recommend bright and saturated colours.

9. Supplement (from Sabina)

Terms and definitions to be discussed:

a) Hue/ Colour

Beside the term “colour” the term “hue” can be found e.g in the CIELab system.

When should the term “hue” or “colour” be used?

b) Luminosity

The International Dictionary of Aids to Marine Navigation defines luminositiy:

„ The attribute of visual sensation according to which an area appears to emit more or less light.“ The term „brightness“ instead of “luminosity” is used in the USA.

Furthermore there is the expression “lightness”, which can be found e.g. in the CIELab system.

Which one might be the most appropriate term for further discussions?

c) Saturation / Chroma

Is “chroma” used only within the CIELab system or could it be used as a synonym to saturation?

d) Colour atlas

This expression sounds strange to a German ear, but it might be correct.

Is there any other term to describe a collection of real colour samples for measurement and visual assessment?

Appendix 1:

Theory of Koschmieder and Hoffmann

1. 	Koschmieder's formula

The luminance of the object seen from a distant observer changes with distance r.

L(r)=L2∗e−z∗r+L1∗(1−e−z∗r)	(1)

L(r)	luminance seen from a distance r

LO	luminance of the object measured nearby the object

LU	luminance of the background

z	atmospheric extinction factor

r	distance between object and observer

2.	Hoffmann's formula

Koschmieder's formula is true for every small spectral interval (unless there is no fluorescence in the atmosphere):

Lλ(r)=L2λ∗e−zλ∗r+L1λ(1−e−zλ∗r)	(2)

L(r)	spectral radiance

L2	spectral radiance measured nearby the object

L1	spectral radiance of the background

z	spectral atmospheric extinction factor

r	distance between object and observer

3.	Transformation to tristimulus values

X=c∗Lλ(r)∗x(λ) dλ	(3a)

Y=c∗Lλ(r)∗y(λ) dλ	(3b)

Z=c∗Lλ(r)∗z(λ) dλ	(3c)

X, Y, Z	tristimulus

c	a constant

L(r)	spectral radiance

x(λ), y(λ), z(λ)	colour matching functions

4.	Transformation to chromaticity coordinates

x=XX+Y+Z	(4a)	y=YX+Y+Z	(4b)

5.	Extinction coefficient

Koschmieder's formula includes a lot of simplifications as Allard's law (*).

Although we looked for a spectral formula we now make an additional simplification and assume that the extinction coefficient is independent from wavelength. So we can calculate z from visibilty V0.05.

z=−ln σ1852∗m−1		(5)

z=−ln 0,05V0,05	V0,05 in metre, z in 1/metre	(6)

(*) Remark: In Koschmieder's theory there is no extinction of light. All light is scattered. We have the name 'coefficient of scattering' for z in Germany.

6.	Change of colour

Calculation of the tristimuls values of the object

X2=c∗L2λ(r)∗x(λ) dλ	(7a)

Y2=c∗L2λ(r)∗y(λ) dλ	(7b)

Z2=c∗L2λ(r)∗z(λ) dλ	(7c)

Calculation of the tristimulus values of the background:

X1=c∗L1λ(r)∗x(λ) dλ	(8a)

Y1=c∗L1λ(r)∗y(λ) dλ	(8b)

Z1=c∗L1λ(r)∗z(λ) dλ	(8c)

As the background colour we choose D65.

From formulae (2) and (3) we get the change of colour as a function of distance r:

X(r)=XO∗e−z∗r+XU∗(1−e−z∗r)	(9a)

Y(r)=YO∗e−z∗r+YU∗(1−e−z∗r)	(9b)

Z(r)=ZO∗e−z∗r+ZU∗(1−e−z∗r)	(9c)

7.	Transformation to luminance and chromaticity coordinates

This transformation is necessary to link the theory above to practical measurements.

The chromaticity coordinates of the object can be measured in the lab.

The chromaticity coordinates of the background are x=0.3127/y=0.3290.

The luminance can be taken from outdoor measurements. Therefore the Traffic Technologies Centre and the local office at Stralsund (baltic sea) took measurements of background and coloured specimen for one year.

Contrast:

C=L2−L1L1=Y2−Y1Y1	(10)

L2	luminance of the object

L1	luminance of the background

From formulae (4) we get:

colour of the object:

X2=x2y2∗Y2	Y2=(1+C)∗Y2	Z2=1−x2−y2y2∗Y2

(11a,b,c)

colour of the background:

X1=x1y1∗Y1	Y1	Z1=1−x1−y1y1∗Y1

(12a,b,c)

With these formulae we can calculate the tristimulus values the observer sees from the distance r:

X(r)Y1=(1+C)∗x2y2∗e−z∗r+x1y1(1−e−z∗r)	(13a)

Y(r)YU=(1+C)∗e−z∗r+(1−e−z∗r)	(13b)

Z(r)YU=(1+C)∗1−x2−y2y2∗e−z∗r+1−x1−y1y1∗(1−e−z∗r)	(13c)

Chromaticity coordinates:

x(r)=X(r)X(r)+Y(r)+Z(r)	y(r)=Y(r)X(r)+Y(r)+Z(r)	(14a,b)

As the atmosphere is assumed to be wavelength independent, the functions x(r) and y(r) are situated on a line between the colour of the object and the colour of the background D65.

8. 	Values from outdoor measurement

| No | Colour | type | Y/100 | x | y | L2/L1 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Background | - | 1 * | 0.3127 * | 0.3290 * | - |
| 1 | Red | F | 0.367 | 0.663 | 0.327 | 1.68 |
| 2 | Green | F | 0.273 | 0.211 | 0.672 | 1.12 |
| 3 | Yellow | O | 0.588 | 0.478 | 0.481 | 2.62 |
| 4 | Green | O | 0.171 | 0.253 | 0.602 | 0.84 |
| 5 | Red | O | 0.155 | 0.641 | 0.334 | 0.87 |
| 6 | White | O | 0.808 | 0.309 | 0.328 | 3.39 |
| 7 | Grey 1 | O | 0.287 | 0.308 | 0.329 | 1.34 |
| 8 | Grey 2 | O | 0.067 | 0.307 | 0.332 | 0.46 |
| 9 | Green RAL 6024 | O | 0.156 | 0.251 | 0.470 | 0.77 |
| 10 | Red RAL 3020 | O | 0.110 | 0.648 | 0.336 | 0.67 |

Explanations:

* 	by definition

F	fluorescent colour

O	ordinary colour

L2/L1	Here we took L1 to be the luminance of the water surface nearby the

horizon. For high bridges the background is the water. We have values

for the sky (nearby horizon) as background.

Appendix 2:

Transformation to L*a*b*

Given values:

Y	100 * luminance factor

x, y	chromaticity coordinates

x=XX+Y+Z		y=YX+Y+Z

Calculation of the tristimulus values:

X=xy∗Y		Y		Z=1−x−yy∗Y

Fixed values:

Xn, Yn, Zn 		Tristimulus values for nominally white object-colour

stimulus (here D65)

Xn = 95.05, 	Yn = 100.00, 	Zn = 108.90 (D65)

L*a*b* - system:

L∗116∗FYYn−16

a∗500FXXn−FYYn

b∗200FYYn−FZZn

where 	F(r) = r1/3			for r > 0.008856

F(r) = 7.787*r+16/116	for r ≤ 0.008856

r is either (X / Xn) or (Y / Yn) or (Z / Zn)

Examples:

The regions for green and red were transformed into L*a*b*. The a*-b*-plane is limited by the optimal colour which depends on Y or L*.

The regions are influenced by the luminance factor, so we get different regions for fluorescent and ordinary colour.

References:

[1] 	Color Science, Wyszeki & Stiles, 1982, page 509

[2] 	Das Wahrnehmen von Lichtzeichen an der achromatischen und

chromatischen Schwelle, Dissertation TH Darmstadt 1979, Dr. Gerdes

[3]	Theorie der horizontalen Sichtweite, Meteorologische Zeitschrift Heft 11,

1926, Koschmieder

[4]	Farbe und Sichtbarkeit, DFVLR Forschungsbericht 1977, Hoffmann