French recommendation about Daymark visibility

Physic laws

For each phenomena of transmission, we have

Something which creates signal

Something which conducts signal

Something which receipts signal

To study way of identification of a daymark is to define whole parameters which characterized these 3 things.

Daymark is defined by size, luminance, colours an shape. But have to take into account background just behind daymark. Background luminance et colour depend on geographical and meteorological conditions. These parameters are hazardous.

Atmosphere is characterized by meteorological visibility which is hazardous.

At the end, observer behaviour depend on physiologic and complex mind phenomena.

We are able to characterize why an object is sawn, but don’t know very well how that object is distinguished and identified. It’s why there isn’t here some accurate vision law.

# Definitions

## Contrast at short distance

If we have O an object, F uniform background with a very important size where object is projected; Lo and Lf luminance of object and background.

Contrast at short distance C(o) is :

C(0)=Lo−LfLf

Contrast characterized the possibility of the eye to distinguish an object with a background where the object is projected.

## Contrast with the sky

With Cc(o) contrast with the sky and Lc luminance of the sky.

Cc(0)=Lo−LcLc

## Contrast relative to the sky

When the object is projected on a different background than the sky, we have to keep sky luminance for reference because eye of the observer is adapted to the sky.

With Ls the luminance of the ground, contrast  relative to the sky Cs(o) is :

Cs(0)=Lo−LsLc

Order of size of contrast Cc(o) and Cs(o)

We consider two different cases : a uniform cloudy sky and a clear sky. In most of case, the reality is just between that 2 cases.

We consider that the daymark have a single colour and is in a vertical plane.

### ->Uniform cloudy sky

Ls and Lo are defined by :

Ls=2.34β'Lc

Lo=3βLc(0.308+0.389β')

With a ground in a horizontal plane.

Lc is luminance just near horizon. In case of uniform cloudy sky, horizon luminance is 1/3 zenith luminance.

 is illuminance factor of object and b’ illuminance factor of ground (0.6 for white sand, 1 for snow, ? for water).

### ->Clear sky

Luminance Lc just near horizon is very dependant on azimuth in relation to the sun, and height of the sun (zenith distance of sun). Lc is between 180Cd/m2 (Az 100° when sun is near horizon the evening) and 19000Cd/m2 (Az 0°, zenith distance 20°). Medium value, in France for Lc is 5000Cd/m2.

For all contrast calculi we have Lo(0) object luminance due to the sky, Ls(1) luminance of a white ground and ground luminance is Ls = ’ Ls(1).

For an object projected on sky

Lo=Lo(0)+12Ls

If object is not white we have to multiply Lo by 

For an object projected on background different of the sky

Lo−Ls=Lo(0)−12Ls

If object is not white we have

Lo−Ls=βLo(0)+β−22Ls

In France :

Ls is between 300Cd/m2 (dark ground) and 10000Cd/m2 (white ground with sun at zenith).

Lo is between 1Cd/m2 and 32000Cd/m2

So

Contrast with the sky

Cc(o)=1−βLo(0)Lc+12LsLc

Contrast relative to the sky

Cc(o)=βLo(0)Lc+1−2βLs2Lc

With CIE model of the sky we have Lc and Lo(0) for a white object. Like that we can calculate Lo(0) for coloured object and Ls (CIE110 Spatial distribution of daylight - Luminance distribution of various reference skies).

Contrast at distance

Atmospheric absorption, defined by meteorological visibility, has an action about conspicuity.

We admit that can be characterized by a modification of contrast of the object projected on background.

Contrast with the sky at distance X

Cc(0)=0.05XVCc(0)

Contrast relative to the sky at distant X

Cs(0)=0.05XVCs(0)

#### Vibility law of Daymark

Visibility conditions

We define d like the characteristic length of daymark (size of square with same area of daymark).

d2X2 is approximation of solid angle under daymark is seen.

Experience says that for a daymark visible at distance X we have to satisfy 2 conditions :

- Contrast at distance X have to be greater than 0.05

So C(0)=0.05XVC(0)>0.05

- Solid angle under daymark is seen have to be more important than a value which depend on X

So d2X2>0.038C(X) (with d in meter and X in kilometer)

##### Consequences

-> Because C(X) contrast have to be superior to 0.05

So d2X2≥0.038C(X)=0.76

It’s case solid angle which daymark is seen is superior or equal to 1/1000 St

-> When C(0) is equal to 1

C(0)=0.05XVC(0)>0.05 => X<V

It’s why you can’t see a daymark a distance of observation more important of meteorological visibility distance.

Visibility distance

If daymark is seen under a solide angle less than 1/1000 St (d/X<1) so

C(0)(0.05)XVd2X2=0.038 (with d in meter and X in kilometer)

If daymark is seen under a solide angle more than 1/1000 St (d/X>1) so

C(0)(0.05)XV=0.05

Distance of visibility depends on contrast and not size of daymark.

#### Readability and identification of daymark

Colour influence about visibility and identification

About colour, visibility depends on contrast and identification of colour depends on amount of light issued of daymark (with eye adaptation of sky).

Colour particularity at distance of observation d/X<1 (no fluorescent)

| Colour | Visibility | Colour identification |
| --- | --- | --- |
| Black | Good (contrast about 1) if relative to sky. Bad if relative to dark background | Good, no modification of rendering with illumination |
| White | Good for clear sky with face or lateral illumination and all background. Bad for cloudy sky for sky or sea background. | Bad if you are in front of the sun (seems dark) so modification of rendering |
| Red | Very good. Good contrast with sky and coast | Bad if you are in front of the sun (seems to be black) |
| Green | No really because colour degradation is important | Difficult identification of colour. Problem with vegetation on the coast. Important risk of confusion with blue at point source |
| Yellow | Not very good especially for clear background | Difficult identification of colour. Yellow seems to white. Good identification if just near a black colour |
| Blue | Not very good because of atmospheric absoption | Risk of confusion with sea and sky if the sunshine |

Fluorescent colour

Luminance of fluorescent colour is greater than ordinary colour. A part of energy of light in limited spectral radiation range is converted into the colour. Transfer energy is toward big wavelength (blue toward red).

Result is very good in red (3 or 4 time more light), good in yellow ( 2 or 3 time more light) and not so good in green (less than 2 time light) but interesting in some cases.

More important problem is lifetime of fluorescent colour in comparison of ordinary colour.

Shape

If we want that mark is seen in all direction in horizontal plane, we have to control it has same aspect in all direction of observation.

We have to adopt difference shapes if there is a lot mark in the same place. Shape can be defined to increase contrast, or increase illumination.

There is a lot of different shape. Determination of identification distance was done only for simple shape. The result shows for square and circle that shape is identified with probability of 90% at 1000 time size of square or diameter of circle.

For letters and numbers, distance of identification is 500 time the height.