IALA Guideline  XXXX

DRAFT

On

Lantern Beam Vertical Divergence, and Focal Height, Combined with the Effects of the Motion of a Floating Platform on the Ability of a Mariner to Recognize the AtoN

Edition 2

Wednesday, April 09, 2008

AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      		Internet -  www.iala-aism.org

Lantern Beam Vertical Divergence, and Focal Height, Combined with the Effects of the Motion of a Floating Platform on the Ability of a Mariner to Recognize the AtoN

# Purpose

# The purpose of this document is to assist the user with the selection of a light source and suitable floating platform such as a buoy. Vertical divergence and focal height are considered, as are the effects of the motion of the floating platform, on the ability of the mariner to recognise the light signal of the AtoN.

# Scope

# This guideline is limited to the light source vertical divergence, range and focal height. It does not advise on the selection of a particular floating platform size or shape to suit a geographical location.

# OVERVIEW

The Technical Note following the IALA Recommendations on the Determination of the Luminous Intensity of a Marine Aids to Navigation Light (1977) states that “the usefulness of a marine aid-to-navigation light depends greatly on the proportion of time for which it may be seen at any required distance”.

Moreover, the same text points out that “in enclosed waters the maximum distance at which it (the light) is required to be seen may be limited, but it is of interest to guarantee this range in as poor a visibility as it is economical to provide for”.

This Guideline will discuss the different factors affecting floating AtoN light signal recognition, and the consequent selection of lanterns, and of the platform on which it is to mounted, assuming that the range of the light is previously defined by the competent authority.

# factors affecting RECOGNITION OF A signal LIGHT  ON A FLOATING aTon light

## Light signal recognition

For identification of an AtoN, the mariner can observe two properties of the signal from the lantern during nighttime:

- color of the light

- character of the light

If the mariner observes a red or green light, he will be aware that if it is a floating aid, then it is a lateral mark. This means that he already has information useful for navigation. On cardinal and danger marks is it essential  for the mariner to recognize the character of light also, in order to have any navigation information.

### Definition of character recognition

The process of recognizing the character of the light can be divided into three steps :

- Detection : Mariner observes the signal

- Identification : Mariner identifies the character of the signal

- Confirmation : Mariner identifies the character of the signal once again, to verify

### Time required for recognition of the character of the light

There are two important time factors which define the time of recognition of the character . These are:

- Character duration (Tc)

- Observation duration (To)

The mariner therefore needs at least two character duration periods ( 2 x  Tc) to recognize the character of the light.

## Factors affecting recognition

A light intended for use on a floating AtoN is generally a lantern for which there is sufficient photometric data available. For this kind of lantern, the published luminous intensity and the spatial distribution of the emitted light are the useful parameters for establishing its range. Generally, it is not necessary to de-rate its luminous intensity by factors related to the installation itself, such as lantern housing, and, so, the main factors affecting its range are external to the installation and may be grouped as follows:

- Geographical range

- Light transmission and human perception

  - Meteorological visibility

  - Background lighting

- Dynamic effects

- Motion of the floating platform, and the lantern light distribution

- Variation in geographical range due to waves and swell, and obscuration by wave crests

### Geographical range

According to the NAVGUIDE, the geographical range is “the greatest distance at which an object or a light source could be seen under conditions of perfect visibility, as limited only by the curvature of the earth, by refraction of the atmosphere, and by the elevation of the observer and the object or light”.

This maximum distance is determined by the equation:

Rg=2.03×(ho+ Hm)

where:

| Rg = | geographical range (nautical miles) |
| --- | --- |
| ho = | elevation of observer’s eye (meters) |
| Hm = | elevation of the mark (meters) |

The factor 2.03 accounts for refraction in the atmosphere, which causes the light path from the source to the observer to be slightly curved, and also for the conversion of units between the heights in meters and the range in nautical miles. Climatic variations around the world may lead to different factors being recommended. Typical range of factors is 2.03 to 2.12.

The height of the light above the earth’s surface is usually referred to as “focal height” or focal plane height”, and is measured from the sea surface to the centre of the light source or to the centre of the lens of the lantern. (Note that for lights on fixed structures, the local land surface affects the focal height.) Revise to indicate focal height to bottom of beam. Delete phrase and possibly replace with height of light.

On floating AtoN, the focal height will vary due to the movement of the water surface, due to waves and swell.

The practical focal height for a light on a floating platform varies according to the design criteria adopted. Generally these criteria consider service conditions and allow floating platforms to be grouped in general categories. An example of a classification from one authority is as follows.

- Open sea buoys – focal heights varying from 4 to 6 meters.

- Moderate sea buoys – focal heights varying from 2.5 to 3.5 meters

- Coastal and Channel buoys – focal heights varying from 1.5 to 2.5 meters

- Restricted water buoys – focal heights at or below 1.5 meters.

Obviously, the focal heights indicated are not exclusive of each category and one can find different heights. These are only heights found in common models available at the market. Authorities also have their own designs, with a great variety of focal heights.

The formula above was used to calculate the geographical ranges in the table below. It shows that all categories of floating aids present geographical ranges at or above the available limits for luminous ranges, except for the lowest possible combination of heights. Therefore the focal height will not be the primary limiting factor of the range of a floating aid.

The following table is reproduced from the IALA Navguide, but with the 6 metre elevation of light added.

| Geographical Range in Nautical Miles |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Observer eye height (meters) | Elevation of light (meters) |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | 2.0 | 4.1 | 4.9 | 5.5 | 6.1 | 6.6 | 7.0 |
| 2 | 2.9 | 4.9 | 5.7 | 6.4 | 6.9 | 7.4 | 7.8 |
| 5 | 4.5 | 6.6 | 7.4 | 8.1 | 8.6 | 9.1 | 9.5 |
| 10 | 6.4 | 8.5 | 9.3 | 9.9 | 10.5 | 11.0 | 11.4 |
| 20 | 9.1 | 11.1 | 12.0 | 12.6 | 13.1 | 13.6 | 14.1 |
| 30 | 11.1 | 13.2 | 14.0 | 14.6 | 15.2 | 15.7 | 16.1 |

Another concern is the ability of the mariner to see the light at close range, when viewing angles are well above the focal plane. The problem can be solved to some extent by choosing a light source with adequate vertical divergence.

### Light transmission and human perception

The transmission of the light signal to the observer is affected by factors including

- Meteorological visibility of the atmosphere

- Background lighting

- Human factors

These factors are covered in other IALA documentation, and so are not discussed in detail here.

# Motion of A FLOATING ATON

## Describing the motion of a floating AtoN

Movement of a floating AtoN is described in different ways by various authors. In this document the following terms are used.

- Pitch – the platform tilts towards and away from the observer

- Roll – the platform tilts from side to side, relative to the observer

- Yaw – the platform angular deflection about its vertical axis.

- Heave – the platform moves up and down vertically.

- List – the average, permanent angle of roll of the platform.

- Trim – the average permanent deflection of the platform from the vertical. Refine but keep consistent with parent definitions

- Angle of Heel – a general term used to refer to a deflection from the vertical and is often used as a general term for pitch, roll, list or trim.

- Squat – the increased submergence of the platform due to dynamic forces arising from  water movement.

These movements, isolated or combined, contribute to affect the desired range, by lowering the focal height and/or by tilting the focal plane of the lantern away from the observer’s eye.

The nature of the movement of a floating aid presents a challenge for AtoN system designers. The solution may come from two complimentary approaches:

- Floating AtoN selection – choice of the most appropriate floating platform

- Lantern design and selection – which will be discussed in the rest of this Guideline.

## Effects on Light Recognition of Vertical Motion of the Floating AtoN

A floating aid and its light will undergo vertical motion as a result of

- Wave action

- Swell

- Heave, which is induced by the above two

This vertical motion will have the effect of altering the geographic range and may even lead to the light being obscured from the observer when the floating platform is in a wave trough.

It is necessary to consider expected sea conditions and the resultant vertical motion of the floating platform when selecting a floating AtoN. This will not be discussed further in this document.

## Effects on Light Recognition of Other Motion of the Floating AtoN – Two Studies

The illustration below is of a typical buoy lantern and the points where the intensity pattern crosses the focal plane represent the luminous intensity reduction towards the observer’s eye.

Note that the vertical divergence of a marine lantern is defined by IALA Recommendation E-122 as equal to Full Width Half Maximum, (FWHM), or 2θ in the above diagram where θ is the angle at which the intensity is 50% of the intensity at zero degrees.

Two studies presented during IALA Conferences dealt with the problem. Their outcomes will be summarized below to help understand the magnitude of the effects of floating platform motion in light perception. They will also be helpful in discussing the possible solutions.

### Detecting Buoy Lights: Effects of Motion and Lantern Divergence – XIIth IALA Conference – Holland – 1990.

This paper was presented by the U.S. Coast Guard and describes a practical method of measuring floating platform movement by digitalizing video images and measuring angles. The aim was to determine the range at which a mariner had an 80% probability of detecting the signal (by observation of its light). In this document the use of the term “list” should be read as “trim” in accordance with the above definitions.

The 80% probability of detection (POD) is part of the USCG design criteria for floating aids. The other assumptions were a 10-mile standard atmospheric visibility and no background lighting. The researchers recorded twelve data sets gathered from different buoys, all of them located in open sea environments, except one river buoy. The samples were considered representative of the different buoy models in use by USCG at that time.

The graph to the right shows how the combined effect of flashing light and buoy roll sum up to reduce the POD of a light on a floating aid. It is an actual 30-second roll recording of a buoy equipped with a red 155 mm lantern operating with 12V/1.15 Amp lamp. Steady light (the middle graph) would be seen flashing or flickering and the observer would detect only the first two flashes of the actual flashing light (the bottom graph). The remaining three flashes become invisible by the buoy motion.

Other important result was the unexpected list varying between 1º and 7.4º. The researchers could not find a direct correlation between those lasting tilts and tidal currents or other likely effects and concluded that the list could not be attributed to any single dominant force (e.g. wind/buoy sail area, moorings, balancing, etc.). The causes are yet to be determined.

The overall effect on POD of lights is that a permanent list angle is to be added to the roll, decreasing further the POD. The graph below illustrates this effect.

The study demonstrates that increasing the vertical divergence of the lights has a significant impact on improving the POD, but at the expense of some loss in lantern peak intensity, resulting in an overall reduction in the maximum range.

The paper presents six graphs similar to the one below, depicting the relationship between range and vertical divergence. There are three curves. The solid line represents de loss in peak intensity versus divergence, for a fixed aid in line with the observer’s eye. The composite dot/dash line is the effect of divergence versus 5.1º list and the dotted line represents the 2.6º list. It is clear that the three curves tend to align around 4 nautical miles nominal range for vertical divergence angles above 10 degrees. The study concluded that for the majority of samples, this alignment occurred for divergences of 7º to 10º.

In some graphs, the improvement of range in the presence of list continues well above 10 degrees, but the paper pointed out that it would not be practical to have lanterns with divergences tailored to each application.

The paper presented the following conclusions:

The designed buoy nominal ranges are never achieved with the slightest amount of buoy movement or list. Buoy movement and list are present in all but the calmest atmospheric and sea conditions.

The following general observations and trends are supported by the results:

* - Nominal range is not a realistic measure of the detection range of a lighted buoy.

* - Buoy list, more than buoy motion, is a severe problem, which merits further study.

* -Increasing the lens divergence can significantly increase the 80% POD range of a buoy.

### Well Balanced Ratio Between Horizontal Luminous Intensity and Vertical Divergence of Marine Lanterns For Light Buoys / Criteria for the Assessment of Luminous Intensity – XV IALA Conference – Australia – 2002.

The second study, presented by the German Federal Waterways Administration – Traffic Technologies Center (TTC) is summarised below.

This document compares vertical divergence of different lanterns for general aids to navigation use and also presents results for buoy movement measurements in the area of the North Sea.

The conclusions on buoy motion were that the reliable luminous intensity for 90% of the time is the value at 11.3° of the vertical intensity distribution. This is very close to the 10º divergence described in the previous USCG study. Furthermore, the researchers discovered that there is a slight probability (7%) that the buoy tilts to an angle of 30°. The Buoy Roll Angles X time graph of the USCG study (see page 8) records nearly 40º overall tilts.

The findings on vertical divergence can be viewed in the following sets of graphs extracted from the original document:

A – Different Light Sources	B – Different LED Lanterns

On column A, above, one can see the comparison between LED, Gas and standard 155 mm electric lantern samples. Below, there is the corresponding variation in luminous range for a fixed light under nominal conditions (V = 10 nm). The column B shows the same results for three different LED lanterns.

## Buoy Motion Effects on Character Recognition

Further studies have been carried out by Trinity House concentrating particularly on the interruption of a light characteristic by the dynamic motion of the floating platform.  In a simplified analysis of buoy motion, and in agreement with the other studies, the major effects are caused by pitch and trim.  The motion of a 3m diameter steel buoy was recorded by video and subsequently analysed to give the angle of heel shown as the lower curve in the graph below.

The apparent range of the lantern is plotted as the upper curve in the graph below.  This was obtained by equating the angle of heel of the buoy with the intensity as measured in the indoor light range, and then a calculation applied to convert intensity to nominal luminous range.

The results of the analysis above were then used as an input into a study to determine the apparent character of a lantern, as observed from a static distance of four miles, in the worst possible location with respect to the buoy motion.  A south cardinal character is represented as a series of dots and dashes at the top of the graph shown below.  The apparent character, as modified by the buoy behaviour, is shown as a second series of dots and dashes.  In this example, it is apparent that much of the character is lost.  It is also apparent that, over the observation period of one minute, the character of a south cardinal could be detected, and possibly identified, but not confirmed.

It is important to note some assumptions that have been made in order to simplify the study.

  - The observer is stationary, not closing in on the buoy

  - No allowance is made for the intelligence or experience of the observer.

Insert a second example, using wider divergence [Peter will arrange, or theoretical input from Omar.]

The final part of this study is a comparison of the modified characters of north and south cardinal lanterns, set in the same location.  It is clear that the two flash patterns are very different, but it is also clear that neither character could be confirmed in the methodology described in section 3 of this document.  It is also indicative of the problems that may be associated with identification and confirmation of east and west cardinal characters in that they are less salient.

# CONSIDERATIONS IN THE SELECTION OF THE light AND FLoaTING PLATFORM COMBINATION

Experience has shown that while the character of a light can be seriously affected by the movement of the floating platform, the colour of a light is unaffected. The use of cardinal marks (with their white lights) should therefore be minimised to take advantage of the unambiguous lateral colour.

This section sums up the various topics discussed earlier, from the standpoint of the Authority responsible for the establishment of floating aids.

The following steps are a general conceptual approach for a good floating AtoN design, considering the various factors affecting luminous range:

- Decide the visible range required of the light to meet the navigational needs of the AtoN station

- Consider the Meteorological Visibility profile in the region of interest.

- Measure or estimate the worst case background luminance in the intended directions of observation, .

- Calculate the luminous intensity needed to reach the desired range above all the limiting conditions mentioned in steps 2 and 3 above.

- Decide a suitable light character and calculate its effective intensity.

- Choose the appropriate floating platform for the location, taking into consideration the typical observers’ heights and expected wave amplitude when selecting the focal height.

- Consider the expected motion of the chosen floating platform and local conditions, and choose a lantern with a suitable vertical divergence curve.

- Now check whether, with the chosen lantern, and its character and intensity, together with the expected motion of the platform selected, the light signal will be recognized by the mariner and does not conflict with other nearby aids. .

- If the equipment selected does not correspond to your expectations of performance, retrace the steps above by considering:

  - other equipment, lantern and buoy or other floating platform;

  - a new light character with a better effective intensity and/or colour,; or

  - means for lowering the background lighting.

Sometimes, practical limitations can leave the designer with only the option to reduce the desired range and, consequently, to increase the number of aids with adverse consequences in overall cost.

# LANTERN DESIGN

## Study by Trinity House

This final section considers the recommendations of the TTC study mentioned above and another paper presented to IALA by Trinity House of London in 1982. The purpose of both documents was to establish an ‘optimum’ vertical profile for AtoN lanterns for buoys.

Lantern manufacturers can benefit from the following discussion for the conception and manufacture of suitable lanterns for buoys. It is also strongly recommended that they consult the original works for a complete discussion of the topics below.

The 1982 document author declared, based on his expertise that “the beam for a buoy principally in calm weather with tidal flow should be as the curve 2B and that for rough water should be as 3B”. The total spread for the two beams are 35º and 55º respectively.

The curve 1B is the result of a practical test and the author recommends the corresponding lantern for use on buoys “in still water giving a tilt not more than  10º”.

The aim of the document is “to prepare appropriate optical designs of lenses… to yield the 2B and 3B distributions of intensity…”

This proposal is amazingly similar to the conclusions taken from the other two studies and point out a kind of solution that cannot be ignored.

## Study by German Waterways Traffic Technology Centre

To complement this conclusion on the need for a wide divergence or “flat” pattern for buoy lanterns, the TTC document, noted above, contains an interesting proposal that is worth considering as a practical pattern to be adopted for lantern design.

The graph below summarizes the proposed pattern, where it can be seen that the required luminous intensity for the nominal range (I nominal) should be available through an angle of ± 10 degrees and the curve is further defined by 40% I nominal and 10% I nominal steps. Curves should be smooth rather than stepped.

END

Brown, D. M. “Probabilities of Detection and Recognition of Flashing Lights on Rolling Buoys”, U.S. Coast Guard Research and Development Center, Report no. CG-D-10-88, August 1987.

Please refer to IALA Recommendation E. 122 on the Photometry of Marine Aids-to-Navigation Signal Lights.