- Navguide Revision.

## For review

### Aids to Navigation Lights

#### Brief History

Until the first application of electricity for lights late in the nineteenth century, all artificial light was produced by fire.  Illuminants progressed from pyres of wood (used up until the 1800’s), to oil wick lamps, vaporised oil and gas burners, then electric arc and tungsten filament lamps.  Optical devices matched these developments, first with reflector systems and later with lenses.

It is interesting to note that the efforts to understand human perception of light.  To improve the efficiency and effectiveness of aids to navigation illuminants and optical apparatus were at the forefront of scientific endeavours for many years.

The glass lens design pioneered by Fresnel around 1820 remains a principal element of the modern aid to navigation light, although present day lenses are often made of plastic rather than glass.

A few countries still have gas lights that burn acetylene or propane. However, the majority of aids to navigation lights use electric lamps of various types.  Increasingly, these lamps draw their power from renewable energy sources such as solar, wind or wave power.

Electric lamps have been specifically designed for aids to navigation applications.  However, lamps selected from the enormous range of commercial products have also been used or adapted for aids to navigation.

Light emitting diode (LED) technology has emerged as an alternative to filament lamps.

#### Categories of Light Sources

Figure 3.2 shows the categories of the various light sources that are commonly used in AtoN.   The light sources that are described in this Guideline are identified by a reference to the sub-sections in which the information is contained.

Lamps using

Electric Discharge

Lamps using filament

Incandescence

Light Emitting Diode

Low Luminance

Tungsten Filament

High Luminance

Low pressure Sodium

Fluorescent

Compact Fluorescent

Tungsten Halogen

High Pressure Sodium

Mercury Vapour

Metal Halide

Coated Envelope Mercury Vapour

Light sources Using Gas

Acetylene

Propane

Xenon

Laser

Figure 1 Categories of light sources

#### Gas Lights

Acetylene

The acetylene light has a special place in the history of aids to navigation, primarily for being the first reliable means of automating lighthouses, buoys and beacons during the earlier part of the 20th century.  Acetylene lighting systems originated from the inventions of Gustaf Dalen and were made by a number of suppliers. Acetylene gas has the unusual property of burning with a white flame when correctly mixed with air. This enabled the development of exceptionally reliable open flame lanterns.

Acetylene lighting technology was further enhanced by the development of the Dalen “mixer” that allowed gas and air to be drawn into a chamber and then consumed in an incandescent mantle to produce a brighter light source than the open flame type.  The incandescent mantle can be operated as a flashing source inside a fixed lens or as a continuous source inside a rotating lens.  Related developments included a gas-operated mechanism for rotating a lens and a clockwork powered automatic mantle changing device.

Refer to IALA publication:

Practical Note for the Safe Handling of Gases, October 1993.

Propane and Butane

Propane and butane gases have been used as fuels for gas lighting systems.  The lighting equipment has to use an incandescent mantle burner as both gasses burn with yellow/orange flames when an open flame burner is used.

Gas lighting installations continue to be used in some countries, where they are chosen for their robustness and simplicity of operation. They have been replaced by solar/electric installations in many countries generally resulting in considerable savings in operating costs.

#### Electric Light Sources

##### Incandescent Lamps

Incandescent lamps are thermal radiators and generate light by heating a solid body to a high temperature. In electric incandescent lamps, this solid body is usually a filament made of tungsten, which has a high melting point and a low rate of vaporization.

Figure 2  A selection of lamps manufactured for AtoN applications (Courtesy of Tideland Signal Corporation, USA).

Tungsten filament lamps

This is typically a coiled tungsten filament in a glass envelope. The envelope is usually filled with an inert gas such as argon. Tungsten filament lamps are available in a variety of operating voltages and wattages.

Typical use

Applicable to all AtoN (from buoys to large optics) but use is diminishing due to high operation cost.

Tungsten halogen lamps

Tungsten halogen lamps feature a tungsten coil filament mounted in a quartz glass envelope that has been filled with an inert gas (usually krypton or xenon) mixed with traces of a halogen element (usually bromine or iodine). The halogen allows the filament to be run at a higher temperature for longer than an equivalent tungsten filament lamp.

Typical use

Applicable to all AtoN from buoys to large optics.

Figure 3 Tungsten halogen lamps

##### Discharge lamps

Discharge lamps differ from filament lamps in that their operation depends not on the incandescence of a hot piece of wire but on electrical discharge through an ionised gas or vapour causing an arc of hot gaseous particles. All discharge lamps require some form of igniter to start the ionisation

Fluorescent

Fluorescent tubes consist of two electrodes at either end of a glass tube filled with an inert gas and some mercury vapour under low-pressure. The inside of the tube is coated with a phosphor or phosphors. When a high voltage is placed briefly across the electrodes the mercury vapour ionises and conducts current forming an arc between the electrodes. The light emitted by the arc has a few spectral lines in the visible spectrum and in the ultra-violet region that provide a light of poor quality. However, the phosphor coating absorbs energy from these spectral lines and re-emits a broader spectrum of white light. Different phosphors can be used to give warm or cold white effects and even some individual colours, although the colours tend to appear somewhat pale.

Typical use

Direction arrows, signs and light tubes or bars used for leading lights or where large areas of illumination are an advantage.

Low-pressure sodium vapour lamps

- In a low-pressure sodium lamp a current is passed between electrodes in an arc tube filled with sodium vapour at low pressure. The colour of light emitted is yellow.

Typical use:

- Used extensively for street lighting, but also for flood lighting and external illumination of structures, towers, locks etc.

Figure 4 Low-pressure sodium lamps

High Pressure Sodium Vapour Lamps

- In a high-pressure sodium lamp a current is passed between electrodes in an arc tube filled with sodium vapour at high-pressure. The colour of light emitted is whiter than that of low-pressure sodium but still with a yellow/orange hue. White versions are available but they are still less white than a typical filament lamp.

Typical Use

- White lamps may be used as AtoN light source.

Metal halide

The metal halide lamp is one of a family of HID (High Intensity Discharge) lamps, and its arc tube is made of silica glass. The principle of emission is as follows: 1) High voltage from the ballast initiates current flow in the mercury vapour between the electrodes; 2) As the lamp temperature rises, metals in the lamp evaporate, and light emission occurs across a broad spectrum.

Typical use

- Used as a fixed light source in rotating optics, fixed lenses with rotating screens and general floodlighting.

Figure 5 Metal halide lamps

Xenon Lamps (short arc)

Xenon lamps are continuous discharge lamps, with the xenon gas enclosed in a silica tube at high pressure. An electrical discharge across two closely spaced electrodes, through the xenon gas, generates a high intensity white light.

Typical use

A specialised light source used where high intensity is of paramount importance. Can be used in fixed or rotating optics.

Figure 6 A diagram of a short-arc Xenon lamp

Xenon Flash Tubes

A high voltage charge is placed across electrodes within a tube filled with Xenon gas and a third electrode is used to trigger a discharge of electrical energy through the gas producing a brief but bright flash of light.

Typical use

Xenon discharge is commonly used in camera flashguns but larger versions can be used as a flashing marine AtoN light.

##### Light Emitting Diode (LED)

An LED is not a lamp but a solid-state light source (photo-luminescent semiconductor junction). It emits light in a narrow spectral band when a current is passed through the junction. Different types of junction material emit different colours of light, typically red, orange, yellow, green, blue and violet. An LED can be made to emit white light by using a blue LED in conjunction with a yellow phosphor. The blue light is absorbed by the phosphor and re-emitted as yellow. The resultant mixture of blue and yellow light appears white to the eye.

Typical use

- Lighted beacons on buoys and other short range AtoN, range lights consisting of flat arrays of LEDs or single high power LEDs, and signs or signals formed by arrays of LEDs in the shape of letters, numerals, signs etc.

Figure 7 LED lantern (courtesy Sabik Oy)

##### Lasers

A laser is a device that produces a very narrow but intense beam of coloured light.

Typical use

Their use has not been established in AtoN light systems, despite the work over some decades. However there are two areas of development:

- High power lasers can be used to provide a line of light in the sky where particles of dust, water etc. are illuminated by the laser beam to provide a leading line. These devices require considerable electrical power (several kW). Detailed safety procedures are required for operation and servicing;

- Low power lasers are being trialled by the Canadian Coast Guard where the laser is aimed directly at the mariner. Different coloured lasers are used to identify areas of a river.

Figure 8 Canadian Coast Guard Hay River laser trial

- –

Refer to IALA publication:

IALA Guideline 1043 on Light Sources used in Visual Aids to Navigation.

Table 1  - Light Source Summary and Comparison

|  | Usual AtoN use | Cost of light Source (USD) | Lifetime (hours) | Robustness | Maximum Input Power (Watt) | Lumen per Watt | Emission Geometry | Colour Spectrum | Flashable | Safety and Other issues |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Filament Lamp | All Round | 5-20 | 300-2,000 | Low | 3,000 | up to 16 | Spherical | Broadband | YES | Hot envelope Glass may break. |
| Tungsten Halogen Lamp | All Round | 10-20 | Up to 3,000 | Medium | 3,000 | Up to 25 | Spherical | Broadband | YES | .Very hot envelope Glass may break Fingerprints on envelope may cause failure. |
| Metal Halide Lamp | Rotating Optics and medium range Range Lights ? | 30-100 | 6000-20,000 | Low (vibration) | 10,000 | Up to 120 | Spherical | Strong spectral lines and UV | NO | Hot envelope. Glass may break Arc tube can explode. UV Hazard |
| Lasers | Range Lights | 1,000 | 10,000 | Medium | 20 | ?? | Narrow Beam | Monochromatic | YES | May cause damage to the eyes. |
| Low Power LED | All Round except long range | 0,25-1 (Need many 4-600) | >100,000 White <10,000 | High | 150mW | 25 - 50 | Medium to wide Beam | Narrow Bandwidth (50nm) but pcLED broadband | YES | May cause damage to the eyes. |
| High Power LED | All Round except long range | 8 – 15 | >100,000 White >50000 | High | 5 | 25 - 50 | Medium to wide Beam | Narrow Bandwidth (50nm) but pcLED broadband | YES | May cause damage to the eyes. |
| Low Pressure Sodium | Inland waterways & marking structures | 10-20 | >10,000 | Medium | 135 | 150 | Spherical | Monochromatic (yellow) | No | Glass may break |
| Xenon | Sector lights & high range lights | 300 | Up to 3,000 | Medium | 35 – 15K | 40 | Spherical | Broadband | No | Hazardous: UV, explosion risk, very hot envelope, glass hazard. |

#### =hotometry of Marine Aids to Navigation Signal Lights

##### Measurement of Light

Refer to IALA publication:

IALA Recommendation E-122 on the Photometry of Marine Adis to Navigation Signal Lights.

The behaviour of light, in physics, is normally seen in the context of either a form of electromagnetic radiation or particle motion.  The latter includes the concept of “rays” of light that are used in analyzing the interactions of light and lenses.

The units of interest for electromagnetic applications of light are generally metres (wavelength) and watts (power).

The study of photometry and the use of lights for signal application has necessitated a parallel set of units to be developed to account for the physiological aspects of how the human eye evaluates a light source, as shown in Table 1.

The spectral sensitivity of the human eye (or the response of the eye to different coloured light) has been evaluated in tests of large numbers of people.  The results have been presented as a standard spectral sensitivity distribution or V( curve for photopic observers and V’(  for scotopic observers.

Figure 9 - Spectral sensitivity distributions or V(  and V’(  curves for the human observer also showing the difference between day and night vision.

##### Units of Measurement

Table 1 – Photometric units of measurement.

| Term | Description | Unit | Abbreviation |
| --- | --- | --- | --- |
| Luminous flux | This is the total light emitted from the source (ie. lamp) The peak sensitivity of the human eye occurs at about 555 nanometres, a wavelength that corresponds to green. At this wavelength, the photometric equivalent of one watt is defined as 680 lumens. | lumens | lm |
| Luminous intensity | This is the part of the luminous flux in a particular direction. Also expressed as the luminous flux per solid angle (or steradian) | candela | cd |
| Luminance | This is the portion of the luminous flux emitted in a specific direction by the surface area of a luminous body. This variable is an important term for rating the brightness impression of light sources and illuminated objects. | candelas per square meter and also as candelas per square centimetre | cd/m2 cd/cm2 |
| Illuminance | This is the density of the luminous flux incident on a surface. It is the quotient of the luminous flux by the area of the surface when the surface is uniformly illuminated | lux (lumens/square metre) | lx |
| Luminous efficacy | This is the ratio of luminous output to radiometric output of a light source. It can also be applied to the efficiency with which electrical power is converted to visible radiation. | lumens per watt of electrical power consumed |  |
| Colour temperature | This related to the temperature of a black body. As a body heats up, it goes through a series of different colours from red through yellow and white, to blue white. The colour appearance of a tungsten filament lamp is similar to a black body at the same temperature. | Kelvin | ºK |
| Colour rendering index | Characterises the colour rendering quality of the light from a lamp. It is the same for all incandescent lamps by definition and equal to the maximum value of 100. |  | CRI |

##### Threshold of Illuminance

In physical terms, the threshold of illuminance is the lowest level of illuminance from a point source of light, against a given background level of luminance, that causes a visual response at the eye.

For visual signalling applications, the threshold of illuminance (E) is taken to be 0.2 micro lux at the eye of the observer.

In the case of leading lights of limited range and with a high level of shore illumination, the above figures may be found too low. It is recommended that to observe the relative position of the lights easily and to derive the maximum possible accuracy from leading and sector lights, it is generally necessary to have a minimum illuminance of 1 micro lux  at the eye of the observer. This condition is to be met at the outer limits of the useful segment for the minimum meteorological visibility under which the leading lights are to be used.

IALA Recommendation on the Definition of the Nominal Daytime Range of Maritime Signal Lights, Intended for the Guidance of Shipping by Day (1974) provides the method of designing AtoN lights for use in daylight.

For lights on floating aids, care must be taken to provide adequate vertical divergence so that the minimum illuminance at the observer is maintained as the floating aid rolls and pitches.

##### Luminous intensity

Luminous intensity is the quantity of visible light emitted by a point source in any one direction.  The Candela (cd) is the measurement unit used to quantify the luminous intensity of a lighted aid to navigation.

Within an optical system for producing beams of light in a marine AtoN, the luminous intensity of the beam is directly proportional to the luminance of the light source.

The size of the light source is inversely proportional to its luminance and directly proportional to the divergence of the optic system.

##### Inverse Square Law

Light emitted from a source radiates out in all directions. For a point source, the wave fronts of light can be imagined to generate a series of spherical surfaces.  As shown in Figure 10, the further the light travels from the source, the greater is the surface area of the sphere and consequently, the lower the illuminance.  Since illuminance is measured in lumens per square metre, and the surface area of a sphere increases in proportion to the square of the radius, the illuminance decreases in proportion to the square of the distance from the source.  The decline in illuminance with distance is described as an inverse-square law.

.

Figure 10 – Illustration of the Inverse-Square-Law concept.

##### Allard’s Law

The illuminance of a light source reaching an observer’s eye determines whether the light is seen.  The relationship between the illuminance produced at the observer’s eye, the luminous intensity of the light source, the distance to the observer and the atmospheric transmissivity is given by Allard’s Law:

E = I×Tdd2

where:

| E = | illuminance at the observer’s eye (lm/m2) |
| --- | --- |
| I = | effective intensity of the light source (cd) |
| T = | atmospheric transmissivity |
| d = | distance between the light source and the observer. (Because T is measured per nautical mile, d in the numerator must also be in nautical miles. In the denominator, d is in metres) |

Allard’s law applies only when the luminance of the background is small compared to the average illuminance of the light.

Where average background luminance is large, as typically happens during the daytime, the equation becomes:

E = [I − (L − L')A] × Tdd2

where:

| L = | luminance (cd/m2) of the background light, measured in the direction of the line of sight from a position near the AtoN light (e.g. a patch of sky next to the lighthouse) |
| --- | --- |
| L' = | average luminance (cd/m2) of the unlighted AtoN measured in the direction of the line of sight from a position near the lighthouse (e.g. a luminance measurement of a lighthouse optic lens with the lamp switched off) |
| A = | area (m2) of the AtoN light beam projected onto a plane normal to the line of sight (e.g. a measurement of the lit area of a lighthouse optic) |
| (L - L' )A= | Since I = LA, this is the light intensity required to make the average luminance of the projector equal to that of the background (cd) |

Note 1: 	Luminance is equal to Intensity divided by Area (L = I / A). Luminance measurements can be carried out with a luminance meter; typically these are devices that measure a quantity of light through a fixed aperture of known area.

Note 2: 	When the AtoN light exhibited is of a significantly different colour to the background light, Allard’s Law may not apply.

#### Colorimetry of Marine Aids to Navigation Signal Lights (Colour measurement)

The measurement of the colour of lights is described in CIE Publication No 15.2 (1986) Colorimetry. There are two main types of instrument for measuring the colour of a light: one is a colorimeter; the other is a spectroradiometer.

Colorimeters usually comprise three photoreceptors, each with a coloured filter. Each filter is matched to the response of one of the three eye receptors, red green and blue and such devices are called ‘tristimulus’ colorimeters. The colorimeter gives three outputs, one for each filtered receptor, and these correspond to the X, Y and Z functions of the human observer.

Spectroradiometers consist of a monochromator and photoreceptor. The monochromator splits the light into individual wavelengths (much like a prism makes a rainbow) and is usually rotated in steps past an exit slit. The photoreceptor, behind the exit slit, measures the radiant power in different sections of the spectrum as the monochromator is rotated. The output is a series of readings enabling a graph of power against wavelength to be displayed. Results may then be weighted with the X, Y and Z functions of the human observer to produce colour information.

Stepping monochromators of the type described previously are fairly slow in operation and are not suitable for measuring flashing lights. Tristimulus colorimeters, on the other hand, enable much faster measurements of colour. New types of spectroradiometer, known as ‘array-based’ spectroradiometers, are now available. Instead of a single photoreceptor and a rotating monochromator, a fixed monochromator has its output directed at an array of charge-coupled devices (CCDs). Such devices are capable of much faster measurement speeds than stepping monochromators.

Recent developments in colour measurement have resulted from the technology of digital cameras. ‘Imaging meters’, as they are known, are little more than calibrated digital cameras, some with tristimulus filtering. They are capable of fast measurement of a whole scene, making them useful for work outside the laboratory. However, the accuracy of some cheaper devices leaves much to be desired.

In summary:

- Tristimulus colorimeters are fast, however cheaper models suffer errors when measuring narrowband light sources such as LEDs.

- Stepping monochromators are expensive and slow but very accurate.

- Array-based spectroradiometers are fast relatively inexpensive but can suffer stray light errors.

- Imaging photometers are expensive and not very accurate but can record a whole scene and not just one light.

Figure 11 – CIE 1931 Colour Functions

Resultant data from colour measurements is usually displayed on a chromaticity chart, developed by the CIE in 1931. The three X, Y, Z values are reduced to two x, y values as shown in Figure 12.

Figure 12 – CIE 1931 x,y Chromaticity Chart

#### Rhythms / Character

IALA has produced a recommendation on the characters for light on aids to navigation

Refer to IALA publication:

Recommendation E-110 for the rhythmic characters of lights on aids to navigation.

The tables of classifications and specifications of aid to navigation characters are provided in Table 2 – Classification Of The Rhythmic Characters Of Lights

The Rhythmic Characters of Lights are provided in Table 3 – Rhythmic Characters of the Lights in the IALA Maritime Buoyage System

Table 2 – Classification Of The Rhythmic Characters Of Lights

|  |  |  | Class | Abbreviation | General description | IALA Specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Particular use in the IALA Maritime Buoyage System |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | FIXED LIGHT | F | A light showing continuously and steadily. | A single fixed light should be used with care because it may not be recognized as an aid to navigation light. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A single fixed light shall not be used. |
| 2 |  |  | OCCULTING LIGHT |  | A light in which the total duration of light in a period is longer than the total duration of darkness and the intervals of darkness (eclipses) are usually of equal duration. | A light in which the total duration of light in a period is clearly longer than the total duration of darkness and all the eclipses are of equal duration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 |  |  | Single-occulting light | Oc | An occulting light in which an eclipse is regularly repeated | The duration of an appearance of light should not be less than three times the duration of an eclipse. The period should not be less than 2 s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A single-occulting White light indicates a safe-water mark. |
|  |  |  |  |  |  |  |  |  |  |  | Example: l = 3 s; d = 1 s; p = 4 s |  |  |  |  |  |  |  |  |  | 1 ≥ 3 d p ≥ 2 s |  |  |  |  |
| 2.2 |  |  | Group-occulting light | Oc(#) eg. Oc(2) | An occulting light in which a group of eclipses, specified in number, is regularly repeated. | The appearances of light between the eclipses in a group are of equal duration, and this duration is clearly shorter than the duration of the appearance of light between successive groups. The number of eclipses in a group should not be greater than four in general, and should be five only as an exception. The duration of an appearance of light within a group should not be less than the duration of an eclipse. The duration of an appearance of light between groups should not be less than three times the duration of an appearance of light within a group. In a group of two eclipses, the duration of an eclipse together with the duration of the appearance of light within a, group should not be less than 1 s. In a group of three or more eclipses, the duration of an eclipse together with the duration of an appearance of light within the group should not be less than 2 s. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A group-occulting Yellow light indicates a special mark. |
|  |  |  |  |  |  | Oc(2) |  |  |  |  | Example: l’ = 6 s; l = 2 s; d = 1 s; c = 3 s; p = 10 s |  |  |  |  |  | l' ≥ 3 l 1 ≥ d c ≥ 1 s |  |  |  |  |  |  |  |  |
| 2.3 |  |  | Composite group-occulting light | Oc(# + #) eg.Oc(2 + 1) | A light similar to a group-occulting light except that successive groups in a period have different numbers of eclipses. | This class of light character is not recommended because it is difficult to recognize. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Oc(2+1) |  |  |  |  |  | Example: l’’ = 9 s; l’ = 3 s; l = 1 s; d = 1 s; c = 2 s; p = 16 s |  |  |  |  |  |  | 1’’ ≥ l’ l’ ≥ 3 l l ≥ d c ≥ 1 s |  |  |  |  |  |  |
| 3 |  |  | ISOPHASE LIGHT | Iso | A light in which all the durations of light and darkness are clearly equal. | The period should never be less than 2 s, but preferably it should not be less than 4 s in order to reduce the risk of confusion with occulting or flashing lights of similar periods. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | An isophase White light indicates a safe-water mark. |
|  |  |  |  |  |  |  |  |  |  |  | Example: l = d = 2 s; p = 4 s |  |  |  |  |  |  |  |  |  |  | l = d p ≥ 2 s |  |  |  |
| 4 |  |  | FLASHING LIGHT |  | A light in which the total duration of light in a period is shorter than the total duration of darkness and the appearances of light (flashes) are usually of equal duration. | A light in which the total duration of light in a period is clearly shorter than the total duration of darkness and all the flashes are of equal duration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 |  |  | Single-flashing light | Fl | A flashing light in which a flash is regularly repeated (at a rate of less than 50 flashes per minute). | The duration of the interval of darkness (eclipse) between two successive flashes should not be less than three times the duration of a flash. The period should not be less than 2 s (or not less than 2.5 s in those countries where a quick rate of 50 flashes per minute is used). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A single-flashing Yellow light indicates a special mark. |
|  |  |  |  |  |  |  |  |  |  |  | Example: d = 3 s; l = 1 s; p = 4 s |  |  |  |  |  |  |  | d ≥ 3 l p ≥ 2 s |  |  |  |  |  |  |
| 4.2 |  |  | Long-flashing light | LFI | A single-flashing light in which an appearance of light of not less than 2 s duration (long flash) is regularly repeated. |  | Example: d = 8 s; l = 2 s; p = 10 s |  |  |  |  |  |  |  |  |  |  |  |  | d ≥ 3 l l ≥ 2 s |  |  |  |  | A long-flashing White light with a period of 10 s indicates a safe-water mark. |
| 4.3 |  |  | Group-flashing light | Fl(#) eg. Fl(2) | A flashing light in which a group of flashes, specified in number, is regularly repeated. | The eclipses between the flashes in a group are of equal duration, and this duration is clearly shorter than the duration of the eclipse between successive groups. The number of flashes in a group should not be greater than five in general, and should be six only as an exception. The duration of an eclipse within a group should not be less than the duration of a flash. The duration of an eclipse between groups should not be less than three times the duration of an eclipse within a group. In a group of two flashes, the duration of a flash together with the duration of the eclipse within the group should not be less than 1 s. In a group of three or more flashes, the duration of a flash together with the duration of an eclipse within a group should not be less than 2 s (or not less than 2.5 s in those countries where a quick rate of 50 flashes per minute is used). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A group-flashing White light with a group of two flashes, in a period of 5 s or 10 s, indicates an isolated-danger mark. A group-flashing Yellow light with a group of four, five or (exceptionally) six flashes indicates a special mark |
|  |  |  |  |  |  | Fl(2) |  |  |  | Example: d’ = 6 s; d = 2 s; l = 1 s; c = 3 s; p = 10 s |  |  |  |  | d’ ≥ 3 d d ≥ l c ≥ 1 s |  |  |  |  |  |  |  |  |  |  |
| 4.4 |  |  | Composite group-flashing light | Fl(# + #) eg Fl(2 + 1) | A light similar to a group-flashing light except that successive groups in a period have different numbers of flashes. | Light characters should be restricted to (2 + 1) flashes in general, and should be (3 + 1) flashes only as an exception. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A composite group-flashing Red or Green light with a group of (2 + 1) flashes indicates a modified lateral (preferred-channel) mark. A composite group-flashing Yellow light indicates a special mark. |
|  |  |  |  |  |  | Fl(2+1) |  |  | Example: d’’ = 9 s; d’ = 3 s; d = 1 s; l = 1 s; c = 2 s; p = 16 s |  |  |  |  |  | d’’ ≥ d’ d’ ≥ 3 d d ≥ l c ≥ 1 s |  |  |  |  |  |  |  |  |  |  |
| 5 |  | QUICK LIGHT |  |  | A light in which flashes are repeated at a rate of not less than 50 flashes per minute but less than 80 flashes per minute. | A light in which identical flashes are repeated at the rate of 60 (or 50) flashes per minute. The higher rate of flashing is preferred. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 |  | Continuous quick light |  | Q | A quick light in which a flash is regularly repeated. |  |  |  | Example: l = d = 0.5 s; p = 1 s |  |  |  |  |  |  |  |  | d ≥ l l s ≤ p ≤ 1.2 s |  |  |  |  |  |  | A continuous quick White light indicates a north cardinal mark. |
| 5.2 |  | Group quick light |  | Q(#) eg Q(3) eg Q(9) eg Q(6) +LFl | A quick light in which a specified group of flashes is regularly repeated. | The number of flashes in a group should be three or nine. An exceptional light character is reserved for use in the IALA Maritime Buoyage System to indicate a south cardinal mark. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Q(3) |  |  | Example: d’ = 7.5 s; l = d = 0.5 s; c = 1 s ; p = 10 s |  |  |  |  |  |  |  |  | d ≥ l d’>d 1 s ≤ c ≤ 1.2 s |  |  |  |  |  |  | A group quick White light with a group of three flashes, in a period of 10 s, indicates an east cardinal mark. |
|  |  |  |  |  |  | Q(9) |  |  | Example: d’ = 6.5 s; l = d = 0.5 s; c = 1 s; p = 15 s |  |  |  |  |  |  |  |  | d ≥ l d’>d 1 s ≤ c ≤ 1.2 s |  |  |  |  |  |  | A group quick White light with a group of nine flashes, in a period of 15 s, indicates a west cardinal mark. |
|  |  |  |  |  |  | Q(6)+LF1 |  |  | Example: d’ = 7 s; l’ = 2 s; l = d = 0.5 s; c = 1 s.; p = 15 s |  |  |  |  |  |  |  |  | d' ≥ 3 l’ l’ ≥ 2 s d ≥ l 1 s ≤ c ≤ 1.2 s |  |  |  |  |  |  | A group quick White light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 15 s, indicates a south cardinal mark. |
| 6 |  | VERY QUICK LIGHT |  |  | A light in which flashes are repeated at a rate of not less than 80 flashes per minute but less than 160 flashes per minute. | A light in which identical flashes are repeated at the rate of 120 (or 100) flashes per minute. The higher rate of flashing is preferred. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 |  | Continuous very quick light |  | VQ | A very quick light in which a flash is regularly repeated. |  | Example: l = d = 0.25 s; p = 0.5 s |  |  |  |  |  |  |  |  | d ≥ l 0.5 s ≤ p ≤ 1.6 s |  |  |  |  |  |  |  |  | A continuous very quick White light indicates a north cardinal mark. |
| c |  | Group very quick light |  | VQ(#) eg VQ(3) eg VQ(9) eg VQ(6)+LFl | A very quick light in which a specified group of flashes is regularly repeated. | The number of flashes in a group should be three or nine. An exceptional light character is reserved for use in the IALA Maritime Buoyage System to indicate a south cardinal mark. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | VQ(3) |  | Example: d’ = 3.75 s; l = d = 0.25 s; c = 0.5 s; p = 5 s |  |  |  |  |  |  |  | d' ≥ 1,5 s d ≥ l 0.5 s ≤ c ≤ 0.6 s |  |  |  |  |  |  |  |  | A group very quick White light with a group of three flashes, in a period of 5 s, indicates an east cardinal mark. |
|  |  |  |  |  |  | VQ(9) |  | Example: d’ = 5.75 s; l = d = 0.25 s; c = 0.5 s; p = 10 s |  |  |  |  |  |  |  | d' ≥ 1.5 s d ≥ l 0.5 s ≤ c ≤ 0.6 s |  |  |  |  |  |  |  |  | A group very quick White light with a group of nine flashes, in a period of 10 s, indicates a west cardinal mark. |
|  |  |  |  |  |  | VQ(6)+LF1 |  | Example: d’ = 5 s; l’ = 2 s; l = d = 0.25 s; c = 0.5 s; p = 10 s |  |  |  |  |  |  |  | d' ≥ 1.5 l’ l’ ≥ 2 s d ≥ l 0.5 s ≤ c ≤ 0.6 s |  |  |  |  |  |  |  |  | A group very quick White light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 10 s, indicates a south cardinal mark. |
| 7 | ULTRA QUICK LIGHT |  |  |  | A light in which flashes are repeated at a rate of not less than 160 flashes per minute. | A light in which flashes are repeated at a rate of not less than 240 flashes per minute and not more than 300 flashes per minute. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Continuous ultra quick light |  |  | UQ | An ultra quick light in which a flash is regularly repeated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | MORSE CODE LIGHT |  |  | Mo(#) eg. Mo(A) | A light in which appearances of light of two clearly different durations are grouped to represent a character or characters in the Morse Code. | Light characters should be restricted to a single letter in the Morse Code in general, and should be two letters only as an exception. The duration of a "dot" should be about 0.5 s, and the duration of a "dash" should not be less than three times the duration of a "dot". |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A Morse Code White light with the single character "A" indicates a safe-water mark. A Morse Code Yellow light, but not with either of the single characters "A" or "U"*, indicates a special mark. |
|  |  |  |  |  |  | Mo(A) |  |  |  |  |  |  | Example: l’ = 1.5 s; l = 0.5 s; d = 0.5 s; d’ = 4.5 s; p = 7 s |  |  |  |  |  |  |  |  |  |  | l' ≥ 3 l d ≥ l l = 0.5 s |  |
| 9 | FIXED AND FLASHING LIGHT |  |  | FFl | A light in which a fixed light is combined with a flashing light of' higher luminous intensity. | This class of light character should be used with care because the fixed component of the light may not be visible at all times over the same distance as the rhythmic component. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Example: d = 3 s; l = 1 s; p = 4 s |  |  |  |  |  |  |  |  |  | d ≥ 3 l l ≤ 1 s |  |
| 10 | ALTERNATING LIGHT |  |  | Al## eg A1WR | A light showing different colours alternately. | This class of light character should be used with care, and efforts should be made to ensure that the different colours appear equally visible to an observer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | AlWR |  |  |  |  |  |  | Example: l = d = 2 s; p = 4 s |  |  |  |  |  |  |  |  |  |  | l  d |  |

Table 3 – Rhythmic Characters of the Lights in the IALA Maritime Buoyage System

| Mark | Rhythmic character of the light | Remarks and further recommendations. |
| --- | --- | --- |
| LATERAL | All recommended classes of rhythmic character , but a composite group flashing light with a group of (2 + 1) flashes is solely assigned to modified lateral marks that indicate preferred channels. | Only the colours Red and Green are used. |
| Modified lateral (preferred channel) | Composite group flashing light with a group of (2 + 1) flashes, in a period of not more than 16 s | The duration of the eclipse after the single flash should not be less than three times the duration of the eclipse after the group of two flashes. |
| CARDINAL |  | Only the colour White is used. |
| North cardinal | (a) Continuous very quick light. (b) Continuous quick light. |  |
| East cardinal | (a) Group very quick light with a group of three flashes, in a period of 5 s. (b) Group quick light with a group of three flashes, in a period of 10 s. |  |
| South cardinal | (a) Group very quick light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 10 s. (b) Group quick light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 15 s. | The duration of the eclipse immediately preceding a long flash should be equal to the duration of the eclipses between the flashes at the very quick rate. The duration of a long flash should not be greater than the duration of the eclipse immediately following the long flash. The duration of the eclipse immediately preceding a long flash should be equal to the duration of the eclipses between the flashes at the quick rate. The duration of a long flash should not be greater than the duration of the eclipse immediately following the long flash. |
| West cardinal | (a) Group very quick light with a group of nine flashes, in a period of 10 s. (b) Group quick light with a group of nine flashes, in a period of 15 s. |  |
| ISOLATED DANGER | (a) Group-flashing light with a group of two flashes, in a period of 5 s. (b) Group-flashing light with a group of two flashes, in a period of 10 s. | Only the colour White is used. The duration of a flash together with the duration of the eclipse within the group should be not less than 1 s and not more than 1.5 s. The duration of a flash together with the duration of the eclipse within the group should be not less than 2 s and not more than 3 s. |
| SAFE-WATER | (a) Long-flashing light with a period of 10 s. (b) Isophase light. (c) Single-occulting light. (d) Morse Code light with the single character "A". | Only the colour White is used. |
| SPECIAL | (a) Group-occulting light. (b) Single-flashing light, but not a long-flashing light with a period of 10 s. (c) Group-flashing light with a group of four, five or (exceptionally) six flashes. (d) Composite group-flashing light. (e) Morse Code light, but not with either of the single characters "A" or “U”`. | Only the colour Yellow is used. A group-flashing light with a group of five flashes at a rate of 30 flashes per minute, in a period of 20 s, is assigned to Ocean Data Acquisition Systems (ODAS) buoys. |

#### Maximum Periods for Light Characters

Refer to IALA publications:

Recommendation E-110 for the rhythmic characters of lights on aids to navigation;

Recommendation for the calculation of the effective intensity of a rhythmic light (November, 1980)

Table 4 is an extract of the recommended maximum periods for rhythmic characters of lights.

Table 4 – Maximum period for rhythmic characters of aids to navigation lights.

| Character Class | Maximum period (seconds) |
| --- | --- |
| Isophase light | 12 |
| Single-occulting light | 15 |
| Single-flashing light | 15 |
| Group very quick light | 15 |
| Interrupted very quick light | 15 |
| Interrupted ultra quick light | 15 |
| Group-occulting light | 20 |
| Long-flashing light | 20 |
| Group-flashing lights of two flashes | 20 |
| Group-quick light | 20 |
| Interrupted quick light | 20 |
| Group-occulting light of three or more eclipses | 30 |
| Group-flashing light of three or more flashes | 30 |
| Composite group-flashing light | 30 |
| Morse code light | 30 |

#### Timing of Astronomical Events

The description of a lighted Aid to Navigation emphasises the night time operations but daytime role is often as important.  The astronomical events that define the transitions from day to night are shown in Table 5..

Table 5 – Timing of Astronomical Events.

| Event | Condition | Typical Illumination Lux | Comment (Assuming the absence of moonlight, artificial lighting or adverse atmospheric conditions) |
| --- | --- | --- | --- |
| Sunset/Sunrise | Upper edge of the sun’s disc is coincident with the horizon. | 600 |  |
| Civil Twilight (beginning / ending) | Centre of the sun is at a depression angle of six (6) degrees below the horizon. | 6 | Illumination is sufficient for large objects to be seen but no detail is discernible. The brightest stars and planets can be seen. For navigation at sea, the sea horizon is clearly defined. |
| Nautical Twilight (beginning / ending) | Centre of the sun is at a depression angle of twelve (12) degrees below the horizon. | 0.06 | It is dark for normal practical purposes. For navigation at sea, the sea horizon is not normally visible. |
| Astronomical Twilight (beginning / ending) | Centre of the sun is at a depression angle of eighteen (18) degrees below the horizon. | 0.0006 | Illumination due to scattered light from the sun is less than that from starlight and other natural light sources in the sky. |

#### Switch-on / switch off Light levels

For lighted aids to navigation that only operate at night, the ambient light levels at which AtoN lights switches should be chosen so that the AtoN light switch-on while the ambient light level is sufficiently high to allow safe navigation, while not switching on during overcast conditions when the AtoN is not necessary for safe navigation.

Refer to IALA publication:

IALA Guideline 1038 on Ambient Light Levels at which Aids to Navigation should switch on and off

#### Night Operations

Refer to IALA publication:

Recommendation for the notation of luminous intensity and range of lights (November 1966)

##### Nominal Range and Luminous Intensity

Table 6 is an extract of the IALA recommendation for the notation of luminous intensity and range of lights and provides a conversion between nominal range and luminous intensity.

Table 6 – IALA conversion table for Luminous Intensity and Nominal Range for night observations.

Note - This assumes an atmospheric transmissivity of T=0.74 and a threshold of illumination of 0.2 microlux.

| Nominal Range (nautical miles) | Luminous Intensity (candela) | Nominal Range (nautical miles) | Luminous Intensity (candela) |
| --- | --- | --- | --- |
| 1 | 0.9 | 12 | 3600 |
| 1.5 | 2.4 | 13 | 5700 |
| 2 | 5 | 14 | 8900 |
| 2.5 | 9 | 15 | 14000 |
| 3 | 15 | 16 | 21000 |
| 3.5 | 24 | 17 | 32000 |
| 4 | 36 | 18 | 49000 |
| 4.5 | 53 | 19 | 73000 |
| 5 | 77 | 20 | 110000 |
| 6 | 150 | 21 | 160000 |
| 7 | 270 | 22 | 240000 |
| 8 | 480 | 23 | 360000 |
| 9 | 820 | 24 | 520000 |
| 10 | 1400 | 25 | 770000 |
| 11 | 2200 | 26 | 1100000 |

##### Background Lighting

Nominal range at night is calculated with no allowance for glare from background lighting.  Excessive background lighting, from street lights, neon signs etc., frequently makes an aid to navigation light less effective and, in some cases, it becomes completely lost in the general background clutter.

The light can be made more conspicuous by increasing its intensity, changing its colour or by varying its rhythm.

##### Glare

Glare can be caused by bright lights emitted from the shore, such as car headlights, or from another vessel indiscreetly using a search-light.  An aid to navigation light can also cause glare if it is too bright for the shortest viewing distance, especially when the focal plane of the light and the observer's eye are at the same height.  This situation can arise with two station leading lines.

For aids to navigation lights it is generally accepted that the illuminance at the eye of the navigator from the light:

- should not exceed 0.1 lux, and;

- should be reduced to 0.01 lux if the background is very dark .

Refer to IALA publications:

Recommendation E-112 for leading lights (including excel program);

Guideline 1023 for the design of leading lines.

In situations where glare is a problem, one or more of the following alterations may lead to a satisfactory result:

- raise the focal plane of the light so that the mariner uses the loom of the light or a less intense part of the vertical distribution of the light;

- reduce the intensity of the light by:

- reducing the illuminance of the light source,

- reducing the size of the optic,

- masking the optic with, for example, perforated metal sheet;

- screen unnecessary arcs of the light;

- use two or more lower intensity lights instead of one higher intensity light.

Whatever methods are used, it will be necessary to measure or calculate the intensity and distribution of the modified light or lighting system.

##### Intensity Losses

Some lighting equipment has to be installed inside a protective lantern housing.  Unless it is practicable to measure the luminous intensity of the complete installation, it is normal practice to apply a de-rating factor to the intensity of the lighting equipment to allow for the reflection and transmission losses at the lantern glazing.  Generally referred to as the glazing loss factor.  Glazing bars or astragals may reduce the intensity of the light at certain bearings.  The installation of non-vertical astragals will overcome this reduction to a certain extent.  The focal plane of the light should be positioned away from any horizontal glazing bars or intersection.

IALA recommends that, in the absence of more definitive information, the glazing loss factor be taken as 0.85 for a system in clean condition.

Refer to IALA publication:

Recommendation on the determination of the luminous intensity of a marine aid to navigation light (December 1977)

##### Service Conditions Factor

Under normal operating conditions the luminous intensity of a light is likely to degrade between service (maintenance) intervals.  There are several components to this degradation:

- meteorological conditions (which may only be temporary);

- dirt and salt deposition (which can be minimised by an efficient regular programme of cleaning of the optical system and housing), and;

- progressive deterioration of the light source over the service interval.

It is clearly impossible to represent such a complex array of factors in any simple way, and a proper assessment of the various effects could only be made by measurements on site at regular intervals.  However, in order to give a more realistic figure for the performance of the light under normal operating conditions than when the luminous intensity is measured in a laboratory or on a photometric range, it may be appropriate to apply a service conditions factor to the measured intensity.

IALA recommends that, in the absence of more definitive information, the service condition factor be taken as 0.75.

#### Day Operations

A number of authorities have established daytime lighted leading lines in major ports and waterways to achieve a more consistent performance than is possible with dayboards.

##### Nominal Daytime Range and Luminous Intensity

Refer to IALA publication:

Recommendation for a definition of the nominal daytime range of maritime signal lights intended for the guidance of shipping by day (April 1974)

Recommendation E-111 on Port Traffic Signals

Table 7 is an extract of this Recommendation and provides a conversion between nominal daytime range and luminous intensity.

Table 7 – IALA conversion table for Luminous Intensity and Nominal Daytime Range.

| Nominal Daytime Range (nautical miles) | Luminous Intensity (candela) | Nominal Daytime Range (nautical miles) | Luminous Intensity (candela) |
| --- | --- | --- | --- |
| 1 | 4600 | 11 | 11000000 |
| 2 | 25000 | 12 | 18000000 |
| 3 | 75000 | 13 | 28000000 |
| 4 | 182000 | 14 | 45000000 |
| 5 | 383000 | 15 | 69000000 |
| 6 | 745000 | 16 | 105000000 |
| 7 | 1400000 | 17 | 161000000 |
| 8 | 2400000 | 18 | 244000000 |
| 9 | 4100000 | 19 | 367000000 |
| 10 | 6900000 | 20 | 549000000 |

##### Daymarks (Dayboards)

The size of a dayboard should be determined for the maximum useful viewing distance and minimum visibility conditions.  Daymarks used on leading lines are typically rectangular with the long side vertical.  The aspect ratio for the rectangle is commonly 2:1(height = 2 x width).

The typical operational range of daymarks under different visibility conditions is shown in Table 8.

Table 8 – Typical operational range of daymarks.

| Operational Range of Daymarks (Nautical Miles) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Daymark height (metres). Aspect ratio h=2w |  |  |  |  |
| Minimum visibility (Nautical Miles) | 1.8 | 2.4 | 3.7 | 4.9 | 7.3 |
| 1 | 0.5 | 0.7 | 0.9 | 1.0 | 1.1 |
| 2 | 0.6 | 0.9 | 1.2 | 1.4 | 1.5 |
| 3 | 0.6 | 1.1 | 1.5 | 1.9 | 2.1 |
| 4 | 0.7 | 1.3 | 1.8 | 2.3 | 2.7 |
| 5 | 0.8 | 1.5 | 2.1 | 2.7 | 3.3 |
| 6 | 0.8 | 1.6 | 2.3 | 2.9 | 3.6 |
| 7 | 0.9 | 1.7 | 2.4 | 3.3 | 4.0 |
| 8 | 0.9 | 1.7 | 2.6 | 3.5 | 4.2 |
| 9 | 0.9 | 1.9 | 2.8 | 3.8 | 4.5 |
| 10 | 1.0 | 2.0 | 3.0 | 4.0 | 5.0 |

#### Luminous Range Diagram

The Luminous Range Diagram, shown in Figure 13, enables the mariner to determine the approximate range at which a light may be sighted, by night or by day, in the meteorological conditions prevailing at the time, and for various levels of background lighting or sky luminance, respectively.

Refer to IALA publications:

IALA Recommendation for the notation of luminous intensity and range of lights (November 1966)

IALA Recommendation for a definition of nominal daytime range of maritime signal lights Intended for the guidance of shipping by day (April 1974)

IALA Recommendation E-111 on Port Traffic Signals

#### Using the Luminous Range Diagram

Example 1

There are some countries that publish the range of lights to a different meteorological visibility than the 10 nautical miles meteorological visibility used in the definition of nominal range.  If the meteorological visibility value is known, the quoted range of a light can be converted to a nominal range using the graph as follows:

- Locate the quoted range on the left hand axis.

- Run a horizontal line across to the intercept with the curve for the stated meteorological visibility.

- Then move vertically down to the bottom scale to read the equivalent nominal range.

- For example:  A light with a published range of 24 nautical miles with a meteorological visibility of 20 nautical miles has a nominal range of 15 nautical miles.

Example 2

To determine the luminous range of a light after making an estimate of the prevailing meteorological conditions:

- Locate the nominal range on the bottom axis.

- Then move vertically to the intercept with the curve for the estimated meteorological visibility.

- Read off the luminous range on the left hand scale.

- For example:  A light with a nominal range of 9 nautical miles will have a range of 6 nautical miles when the visibility is 5 nautical miles.

Figure 13 – Luminous Range Diagram

The top scale on the graph gives the equivalent intensity values in candelas for both day and night operation, and should be used when only the intensity of a light is known.   It must be remembered that:

- The atmosphere transmissivity is not necessarily consistent between the observer and the light.

- No allowance is made for range limitations imposed by the height-of-eye of the observer or the elevation of the light.  A separate check should be made using the geographic range table to confirm that the light will be visible at the estimated luminous range.

- Glare from background lighting will reduce considerably the range at which lights are sighted at night.

Example 3

To determine the luminous range at night when the light is affected by glare from background lighting.

- Estimate the level of background lighting between none and substantial.

- Select the adjustment interval from the right hand auxiliary scale on the graph.

- Subtract this interval from the nominal range of the light.

- Then move vertically to the intercept with the curve for the estimated meteorological visibility.

- Read off the luminous range on the left hand scale.

- For example: If there is substantial background lighting from a port facility or large urban development, an 18 mile light must first be corrected to 8 miles before the luminous range can be determined for the prevailing meteorological conditions.

Example 4

To determine the luminous range of a light intended for day-time use under prevailing day-time luminance conditions:

- Estimate the day sky luminance between glaring cloud and dark overcast. The nominal condition for day-time use is bright cloud or a clear sky (looking away from the sun).

- Select the adjustment interval from the nominal condition from the left hand auxiliary scale on the graph.

- Add or subtract this interval from the nominal range of the light.

- Then move vertically to the intercept with the curve for the estimated meteorological visibility.

- Read off the luminous range on the left hand scale.

- For example:

- When a light with a 5 nautical mile nominal range by day is viewed against an overcast sky, the nominal range can be increased to around 8.8 nautical miles before determining the luminous range for the prevailing meteorological conditions;

- If a light with a 5 nautical mile nominal range by day is viewed against very bright glaring cloud the nominal range would be reduced to 3 nautical miles before determining the luminous range for the prevailing meteorological conditions.

Example 5

The graph can also be used to show:

- Using the top and bottom scales that a light with an intensity of 100,000 candelas has a nominal range of a little over 20 nautical miles.

- If the same light was sighted at 12 nautical miles it would imply that the meteorological visibility was about 5 nautical miles.

- The same light would have a nominal day-time range of a little over 3 nautical miles.

The steradian is the equivalent in solid geometry to the definition of radian in plane geometry.  The steradian is defined as the unit of measure of a solid angle with its vertex at the centre of a sphere and enclosing an area of the spherical surface equal to that of a square with sides equal in length to the radius.  There are 4π steradians in a sphere. For further explanation see IALA Recommendation E122.

This condition is to be met at the outer limits of the useful segment for the minimum meteorological visibility under which the leading lights are to be used.

The term “long flash”, which is used in the descriptions of the long-flashing light and of the light characters reserved for south cardinal marks, means an appearance of light of not less than 2 seconds duration. The term “short flash” is not commonly used and does not appear in the Classification. If an Authority requires discrimination between two flashing lights that only differ in having flashes of different durations, then the longer flash should be described as “long flash” and be of not less than 2 seconds duration, and the shorter flash may be described as “short flash” and should be of not more rhythmic character of such a light is than one third of the duration of the longer flash.

A single fixed light shall not be used on a mark within the scope of the IALA Maritime Buoyage System because it may not be recognized as an aid to navigation light.

A Morse Code white light with the single character "U" is assigned to offshore structures.

The timing of astronomical events can also be applied to calculations (computer programs) for sizing solar power supplies.

The most common of the alternatives is a meteorological visibility of 20 nautical miles.