AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Guideline No. XXXX

On

Illumination

Of

Structures

Draft

Spring 2008

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      	Internet -  http://iala-aism.org

Table of Contents

Table of Contents	2

1	Introduction	3

2	MAIN Applications	3

2.1	Lighthouse and minor lights	3

2.2	Beacons and marks	3

2.3	Obstacles, breakwater, rocks, etc	4

3	Design	4

3.1	Type of light sources	4

3.1.1	Lamps and housing	4

3.2	Intensity ( illumination level)	4

3.2.1	Light absorption and reflectivity of the surfaces	4

3.3	Location of light sources	5

3.4	Examples on Conventional constructions	5

3.4.1	Light houses and beacons	5

3.4.2	Marks:	5

4	Further consideration	6

4.1	Alternate solutions	6

4.1.1	Pipe light LED-floodlighting	6

4.1.2	Extended light sources	7

4.2	Flood lighting on demand	8

5	References	8

# Introduction

This guideline does not address architectural flood-lighting, instead it focus on navigational aspects. Flood-lighting can be an important AtoN function. It can help the user to improve his location visually, also allow him to have access to information noted on the AtoN, eg marking.

# MAIN Applications

Floodlighted structures are mainly used in locations where mariners navigate close by.

## Lighthouse and minor lights

Floodlighted lighthouses are today an appreciated feature, some times even regarded as more important than the traditional flashing light of the lighthouse. The floodlight has to be considered as a complement to the main AtoN light.

(it helps the mariner to keep track of the bearing to the lighthouse in the characters dark period.)

## Beacons and marks

For this application fixed or flashing lights are not applicable. By floodlight a beacon or a mark the mariner will be able to have a better appreciation of the distance and the direction to the AtoN.

Example of a floodlighted mark

## Obstacles, breakwater, rocks, etc

For this application, traditional way of marking e.g. lantern or beacon is not practically or economically feasible.

picture

# Design

Flood-lighted objects are normally used within a distance up to 1 NM.

## Type of light sources

### Lamps and housing

Characteristics from the lamp guide; colour, lifetime, etc

To be added later

If the colour of the surface has to be distinguished, one has to take in to account the chromaticity of the light source.

## Intensity ( illumination level)

Sufficient number of light sources should be used to ensure an adequate intensity, a uniform light distribution and a satisfactory redundancy.

Light levels from 5 lx to 50 lx are commonly used in architectural floodlight applications.

### Light absorption and reflectivity of the surfaces

To be added later

(IESNA Lighting Handbook?)

#### Type of reflectivity. (Lambertian or Retroreflection)

When light strikes a rough or granular surface, it bounces off in all directions due to the microscopic irregularities of the interface. The exact form of the reflection depends on the structure of the surface. One common model for diffuse reflection is Lambertian reflectance, in which the light is reflected with equal luminance in all directions, as defined by Lambert's cosine law.

Some surfaces exhibit retro reflection. The structure of these surfaces is such that light is returned in the direction from which it came. If illumination is used in combination with a retro reflection surface the position of the light is very important.

## Location of light sources

Considerations for the remote location

## Examples on Conventional constructions

### Light houses and beacons

The light source in these constructions is Seald Beam lamp PAR 36 SB4589 28V 50W. To prolong the lifetime of the lamps they can be runned on a lower voltage, 22V. Normal use is 4 lamps on a beacon (2 m in diameter) and abt. 8–12 lamps on our bigger caisson light houses, i.e.total illumination power 200– 600W.

### Marks:

Marks consist in the most cases of a plastic cone, which is illuminated from the top downwards. When yellow light is needed Low Pressure Sodium lamp is used alt. a Mercury Vapour lamp when white light is needed.

# Further consideration

A large number of lighthouses are externally floodlighted with projected

indirect reflection illumination. This important feature rely on mains supply due to the power required, typical 250W continuous 14h/day, the economical implications for new installations are limiting. Also the service and repair of the necessary underwater cables takes its increasing toll of budget.

Floodlighted lighthouses are today a appreciated feature by the mariners, some times regarded as more important than the traditional flashing light.

Installations connected to mains by sea cables mean high investments and running cost and therefore alternative installations which can run on PV is under developtning.

## Alternate solutions

### Pipe light LED-floodlighting

An LED pipe light is designed to solve problems connected to traditional AtoN floodlights with need of network electricity, high power consumption and maintenance.

Fig 4, 5, 6, 7 shows the first main installation of LED-floodlighting on an

inaccessible lighthouse in the busy approach to Gothenburg, Sweden.

Fig 8, 9, 10, 11 illustrate the individual components.

Fig 12 is a draft drawing of the type of beacons erected, in the new

Gothenburg approach channel. All fitted with this type of PV-LED-floodlighting

and PV-GPS-synchronised LED-lanterns.

### Extended light sources

The conventional flood lights are normally fitted with ordinary lights for mercury or sodium vapour lamps to illuminate the mark. The drawback of this method is the confusion of disturbing background lighting from roads and bridges, especially at port entrances or other heavily populated areas

To increase the conspicuity in areas with disturbing background lighting as for example at break water lights, an additional feature is needed.

The outcome is a lighting pole with an illuminated area of 200 x 6000mm

= 1.2 m2. The pole is an 8m steel framework hot-dip galvanised lighting mast. The

light consists of 1800 LED`s. The power consumption is 150W at 220V AC.

The distinguishing factor for these light poles is the use of lateral colours red/green

and the distinct vertical length. In a fairway many red/green fixed or flashing lights

can be observed, but few that stick out like an illuminated sore thumb.

For daytime use the pole is fitted vertical narrow day-mark boards at each side of the pole (see fig 5). The boards are lined with fluorescent sheeting red or green.

The calculated service life is at least 8 years and no maintenance is needed.

Due to the LED-technique it’s possible to flash the whole lighted area, which

further increase the conspicuity with respect to background light. This feature is

used particular in leading line applications with character: Rear Oc and Front Q in

order to avoid dangerous mirror effects. This application is suitable as leading lights in port area with its special appearance. The range of visibility is at least 2M

## Flood lighting on demand

Flood lighting is very energy demanding and has to be runned by the mains supply. Some installations, which by remote control are switched on when needed can be runned by PV.

# References