Editing of Sections 1 to 3, MDC

IALA Guideline  XXXX

DRAFT

On

Vertical Divergence and Focal Height of Lights on Floating Aids to Navigation

Edition 1

3 April 2008

AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      		Internet -  www.iala-aism.org

IALA Guideline on Vertical Divergence and 
Focal Height of Lights 
on Floating Aids to Navigation

# SUMMARY

# This guideline provides a general informative overview of factors affecting the recognition of the signal from a light on a floating aid to navigation and discusses how to take them into consideration when selecting a lantern and floating aid combination. It particularly discusses the effect of the motion of the floating aid on the recognition of the character of the light signal.

# OVERVIEW

The Technical Note following the IALA Recommendations on the Determination of the Luminous Intensity of a Marine Aids to Navigation Light (1977) states that “the usefulness of a marine aid-to-navigation light depends greatly on the proportion of time for which it may be seen at any required distance”.

Moreover, the same text points out that “in enclosed waters the maximum distance at which it (the light) is required to be seen may be limited, but it is of interest to guarantee this range in as poor a visibility as it is economical to provide for”.

This Guideline will discuss the different factors affecting floating AtoN Light signal recognition, and the consequent selection of lanterns, and of the buoy, float, or lightvessel on which it is to mounted, assuming that the range of the light is previously defined by the competent authority.

# RECOGNITION OF THE LIGHT SIGNAL

For identification of an AtoN the mariner can observe two properties of the signal from the lantern during nighttime:

- color of the light

- character of the light

If the mariner observes a red or green light, he will be aware that if it is a floating aid, then it is a lateral mark. This means that he already has information useful for navigation. On cardinal and danger marks is it essential  for the mariner to recognize the character of light also, in order to have any navigation information.

## Definition of character recognition

The process of recognizing the character of the light can be divided into three steps :

- Detection : Mariner observes the signal

- Identification : Mariner identifies the character of the signal

- Confirmation : Mariner identifies the character of the signal once again, to verify

## Time required for recognition of the character of the light

There are two important time factors which define the time of recognition of the character . These are:

- Character duration (Tc)

- Observation duration (To)

The mariner therefore needs at least two character duration periods ( 2 x  Tc) to recognize the character of the light.

# factors affecting RECOGNITION OF A signal LIGHT  ON A FLOATING aTon light

A light intended for use on a floating AtoN is generally a compact, high quality lantern for which there is sufficient photometric data available. For this kind of lantern, the published luminous intensity and the spatial distribution of the emitted light are the useful parameters for establishing its range. Generally, it is not necessary do de-rate its luminous intensity by factors related to the installation itself, such as lantern housing, and, so, the main factors affecting its range are external to the installation and may be grouped as follows:

- Focal Height of lantern, and height and distance of observer: geographical range

- Light transmission and human perception

  - Meteorological visibility

  - Background lighting

- Dynamic effects

- Obscuration by wave crests

- Buoy motion and lantern light distribution

- Variation in geographical range due to waves and swell

## Focal Height of lantern and height and distance of observer

The focal height of a light defines the geographical range of that light for a given observer.

According to the NAVGUIDE, the geographical range is “the greatest distance at which an object or a light source could be seen under conditions of perfect visibility, as limited only by the curvature of the earth, by refraction of the atmosphere, and by the elevation of the observer and the object or light”.

This maximum distance is determined by the equation:

Rg=2.03×(ho+ Hm)

where:

| Rg = | geographical range (nautical miles) |
| --- | --- |
| ho = | elevation of observer’s eye (meters) |
| Hm = | elevation of the mark (meters) |

The factor 2.03 accounts for refraction in the atmosphere and also for the conversion of units between the heights in meters and the range in nautical miles. Climatic variations around the world may lead to different factors being recommended. Typical range of factors is 2.03 to 2.12.

The practical focal height varies according to the design criteria adopted for a particular kind of buoy. Generally these criteria consider service conditions and allow buoys to be grouped in four general categories: .  An example of a classification from one authority is as follows.

- Open sea buoys – focal heights varying from 4 to 6 meters.

- Moderate sea buoys – focal heights varying from 2.5 to 3.5 meters

- Coastal and Channel buoys – focal heights varying from 1.5 to 2.5 meters

- Restricted water buoys – focal heights at or below 1.5 meters.

Obviously, the focal heights indicated are not exclusive of each category and one can find different heights. These are only heights found in common models available at the market. Authorities also have their own designs,, with great variety of focal heights.

The formula above was used to calculate the ranges of the table below, limited to the heights defined for the different buoy categories. It shows that all categories of buoys present geographical ranges at or above the available limits for luminous ranges, except for the lowest possible combination of heights. This means that focal height will not be the main limiting factor for establishing the desired range for a floating aid.

| Geographical Range in Nautical Miles |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Observer eye height (meters) | Elevation of light (meters) |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | 2.0 | 4.1 | 4.9 | 5.5 | 6.1 | 6.6 | 7.0 |
| 2 | 2.9 | 4.9 | 5.7 | 6.4 | 6.9 | 7.4 | 7.8 |
| 5 | 4.5 | 6.6 | 7.4 | 8.1 | 8.6 | 9.1 | 9.5 |
| 10 | 6.4 | 8.5 | 9.3 | 9.9 | 10.5 | 11.0 | 11.4 |
| 20 | 9.1 | 11.1 | 12.0 | 12.6 | 13.1 | 13.6 | 14.1 |
| 30 | 11.1 | 13.2 | 14.0 | 14.6 | 15.2 | 15.7 | 16.1 |

Other concern is the ability of the mariner to see the light at close range, when viewing angles are well above the focal plane. The problem can be solved to some extent by choosing lanterns of adequate vertical divergence.

## Light transmission and human perception

### Meteorological visibility and light transmission

The luminous range of a light is highly dependent of the prevailing meteorological visibility (V). It can be observed that once the desired range is established, the authority generally takes this range as the nominal range under V = 10 nautical miles and selects a lantern with suitable intensity for that visibility factor. In some countries, the value of V taken as ‘standard’ for that purpose is even higher.

It can be demonstrated that this approach does not consider the recommended “proportion of time for which (the light) may be seen at any required distance”. The following table presents the meteorological visibility and its equivalent in atmospheric transmission factor versus percentage of time for the Guanabara Bay in Rio de Janeiro – Brazil. The data correspond to more than 40 years of regular meteorological observations in that area and can be taken as a good long term representation of visibility trends.

| WMO Visibility Ranges (V) | Number of observed occurrences | Mean Values of Meteorological Visibility V (km) | Mean Values of V (nautical miles) | Mean Atmospheric Transmission Factor (T) T = 0.051/ V | Percentage of observed occurrences | Proportion of time for which a light may be seen at any required distance |
| --- | --- | --- | --- | --- | --- | --- |
| < 50 m | 50 | 0.025 | 0 | 0.00 | 0.08% |  |
| 50 to 200 m | 34 | 0.125 | 0 | 0.00 | 0.05% |  |
| 200 to 500m | 110 | 0.35 | 0 | 0.00 | 0.18% |  |
| 500m to 1 km | 1686 | 0.75 | 0 | 0.00 | 2.71% |  |
| Sum of occurrences of T = 0.00 |  |  |  |  | 3.02% |  |
| 1 to 2 km | 3132 | 1.5 | 1 | 0.02 | 5.03% | 96.98% |
| 2 to 4 km | 10994 | 3 | 2 | 0.16 | 17.64% | 91.96% |
| 4 to 10 km | 15327 | 7 | 4 | 0.45 | 24.59% | 74.32% |
| 10 to 20 km | 23152 | 15 | 8 | 0.69 | 37.15% | 49.72% |
| 20 to 50 km | 7248 | 35 | 19 | 0.85 | 11.63% | 12.57% |
| > 50 km | 587 | 50 | 27 | 0.89 | 0.94% | 0.94% |
| Sum of observations | 62320 |  |  |  | 100.00% |  |

The observational data is contained in the two columns to the left and were collected at a single meteorological station using the SYNOP observation method established by the World Meteorological Organization (WMO).

The remaining columns are extracted from the number of observations and mean values of V. Those values were further converted to nautical miles and to transmission factors (T). The near zero values of T were grouped as a general percentage of occurrences.

The last column to the right contains the useful data. The proportions assume that a light designed to provide the required range under the correspondent value of “T” is, of course, able to reach that range under better conditions. So, the conclusion is that, to reach the maximum usefulness (nearly 97% in this case), the authority should select a light of sufficient intensity to operate under T = 0.02 (or V = 1 nautical mile) for the desired range.

The visibility of 10 nautical miles (or T = 0.74) required for the nominal range is not directly represented in the table, but can be estimated and is clearly less than 50%. That percentage would be the expected proportion of time a light could be seen in the region if designed to operate under nominal range conditions for the desired range.

Small lanterns for use on buoys typically have peak intensities ranging from 20 to 200 candelas. Discarding background lighting effects, those values would offer a range of 2.2 to 2.7 nautical miles under T = 0.02. For comparison, their nominal ranges vary from 3.3 to 6.5 nautical miles, and those would be the ranges declared in nautical charts and other documents.

Authorities can use meteorological station data assuming that they indicate a general trend over a limited region and their reliability is directly proportional to the number of observations. A perhaps better alternative is to install visibility measuring equipment and use the recorded data. Available commercial devices are capable of measuring visibility within a 20 km radius from their installation point, rendering them useful for range evaluation of aids to navigation lights.

Would this discussion above be better included in another IALA publication?

### Background Lighting

The background lighting is an ever growing problem, caused by the increasing human activity surrounding ports and harbours, where the aids to navigation are deployed. There are basically two ways to address the problem and they are likely to be adopted together:

- Limiting background lighting (e.g.: by means of restrictive measures); and

- Increasing conspicuity through increase of  intensity of aids, character change, colour change, synchronisation and sequencing of lights.

For both, it is necessary a numerical determination of the background lighting, by means of measurement or estimation and, for increasing luminous intensity of aids, it is necessary do include that parameter in Allard’s Law. Please refer to The IALA NAVGUIDE and to IALA Guideline 1051 for more detailed information.,

Returning to the small lanterns for use on buoys and their typical intensities of 20 to 200 candelas, let us now consider background lighting effects and T = 0.02. The luminous ranges would be:

- 0.5 to 0.9 nautical miles for minor background lighting (L = 12 cd/m²) ; and

- 0.3 to 0.5 nautical miles for considerable background lighting (L = 203 cd/m2

## Buoy Motion

Add something about buoy being obscured from view in wave troughs?

All the aspects addressed so far are also applicable to lights on fixed aids. From now on we will focus on phenomena exclusively affecting floating aids.

- The buoy movements can be described using the terms below, Buoy Pitch – the buoy tilts towards and away from the observer

- Buoy Roll – the buoy tilts from side to side, relative to the observer

- Buoy Yaw – the buoy spins about its vertical axis.

- Buoy Heave – the buoy moves up and down vertically.

- Buoy List – the average, permanent buoy angle of roll.

- Buoy Trim – the average permanent buoy angle of pitch.

- Angle of Heel – a general term used to refer to  either pitch,  roll, list or trim.

- Squat – the increased submergence of  the buoy due to dynamic forces arising from  water movement.

These movements, isolated or combined, contribute to affect the desired range, by lowering the focal height and/or by tilting the focal plane of the lantern away from the observer’s eye. The illustration below is of a typical 155 mm lantern and the points where the intensity pattern crosses the focal plane represent the luminous intensity reduction towards the observer’s eye.

Note that the vertical divergence of a marine lantern is now commonly stated as equal to Full Width Half Maximum, (FWHM), or 2θ in the above diagram where θ is the angle at which the intensity is 50% of the intensity at zero degrees.

The random nature of the movements is a challenge for designers. The solution may come from two complimentary approaches:

- Buoy design – with the help of modern computation techniques and the use of massive statistic data, in order to improve buoy behaviour under prevalent sea conditions, which is beyond the scope of this Guideline; and

- Lantern design and selection – which will be discussed in the rest of this Guideline.

## Buoy Motion Effects on Light Observations

Two studies presented during IALA Conferences dealt with the problem. Their outcomes will be summarized below to help understand the magnitude of the effects of buoy motion in light perception. They will also be helpful in discussing the possible solutions. Let us examine the first of them:

- Detecting Buoy Lights: Effects of Motion and Lantern Divergence – XIIth IALA Conference – Holland – 1990.

This paper was presented by the U.S. Coast Guard and describes a practical method of measuring buoy movement by digitalizing video images and measuring angles. The aim was to determine the range at which a mariner had an 80% probability of detecting the signal (by observation of its light). In this document the use of the term “list” should be read as “trim” in accordance with the above definitions.

The 80% POD (probability of detection) is part of the USCG design criteria for floating aids. The other assumptions were a 10-mile standard atmospheric visibility and no background lighting. The researchers recorded twelve data sets gathered from different buoys, all of them located in open sea environments, except one river buoy. The samples were considered representative of the different buoy models in use by USCG at that time.

The graph to the right shows how the combined effect of flashing light and buoy roll sum up to reduce the POD of a light on a floating aid. It is an actual 30-second roll recording of a buoy equipped with a red 155 mm lantern operating with 12V/1.15 Amp lamp. Steady light (the middle graph) would be seen flashing or flickering and the observer would detect only the first two flashes of the actual flashing light (the bottom graph). The remaining three flashes become invisible by the buoy motion.

Other important result was the unexpected list varying between 1º and 7.4º. The researchers could not find a direct correlation between those lasting tilts and tidal currents or other likely effects and concluded that the list could not be attributed to any single dominant force (e.g. wind/buoy sail area, moorings, balancing, etc.). The causes are yet to be determined.

The overall effect on POD of lights is that a permanent list angle is to be added to the roll, decreasing further the POD. The graph below illustrates this effect.

The study demonstrates that increasing the vertical divergence of the lights has a significant impact on improving the POD, but at the expense of some loss in lantern peak intensity, resulting in an overall reduction in the maximum range.

The paper presents six graphs similar to the one below, depicting the relationship between range and vertical divergence. There are three curves. The solid line represents de loss in peak intensity versus divergence, for a fixed aid in line with the observer’s eye. The composite dot/dash line is the effect of divergence versus 5.1º list and the dotted line represents the 2.6º list. It is clear that the three curves tend to align around 4 nautical miles nominal range for vertical divergence angles above 10 degrees. The study concluded that for the majority of samples, this alignment occurred for divergences of 7º to 10º.

In some graphs, the improvement of range in the presence of list continues well above 10 degrees, but the paper pointed out that it would not be practical to have lanterns with divergences tailored to each application.

The paper presented the following conclusions:

The designed buoy nominal ranges are never achieved with the slightest amount of buoy movement or list. Buoy movement and list are present in all but the calmest atmospheric and sea conditions.

The following general observations and trends are supported by the results:

* - Nominal range is not a realistic measure of the detection range of a lighted buoy.

* - Buoy list, more than buoy motion, is a severe problem, which merits further study.

* -Increasing the lens divergence can significantly increase the 80% POD range of a buoy.

Further studies have been carried out by Trinity House concentrating particularly on the interruption of a light characteristic by the dynamic motion of the buoy.  In a simplified analysis of the buoy motion, and in agreement with the other studies, the major effects are caused by pitch and trim.  The motion of a 3m diameter steel buoy was recorded by video and subsequently analysed to give the angle of heel shown as the lower curve in the graph below.

The apparent range of the lantern is plotted as the upper curve in the graph below.  This was obtained by equating the angle of heel of the buoy with the intensity as measured in the indoor light range, and then a calculation applied to convert intensity to nominal luminous range.

The results of the analysis above were then used as an input into a study to determine the apparent character of a lantern, as observed from a static distance of four miles, in the worst possible location with respect to the buoy motion.  A south cardinal character is represented as a series of dots and dashes at the top of the graph shown below.  The apparent character, as modified by the buoy behaviour, is shown as a second series of dots and dashes.  In this example, it is apparent that much of the character is lost.  It is also apparent that, over the observation period of one minute, the character of a south cardinal could be detected, and possibly identified, but not confirmed.

It is important to note some assumptions that have been made in order to simplify the study.

  - The observer is stationary, not closing in on the buoy

  - No allowance is made for the intelligence or experience of the observer.

The final part of this study is a comparison of the modified characters of north and south cardinal lanterns, set in the same location.  It is clear that the two flash patterns are very different, but it is also clear that neither character could be confirmed in the methodology described in section 3 of this document.  It is also indicative of the problems that may be associated with identification and confirmation of east and west cardinal characters in that they are less salient.

# CONSIDERATIONS IN THE SELECTION OF THE light AND BUOY COMBINATION

Experience has shown that while the character of a light can be seriously affected by buoy movement, the colour of a light is unaffected. The use of cardinal marks (with their white lights) should therefore be minimised to take advantage of the unambiguous lateral colour.

This section sums up the various topics discussed earlier, from the standpoint of the Authority responsible for the establishment of floating aids.

The following steps are a general conceptual approach for a good floating AtoN design, considering the various factors affecting luminous range:

- Decide the visible range required of the light to meet the navigational needs of the AtoN station

- Consider the Meteorological Visibility profile in the region of interest.

- Measure or estimate the worst case background luminance in the intended directions of observation, .

- Calculate the luminous intensity needed to reach the desired range above all the limiting conditions mentioned in steps 2 and 3 above.

- Decide a suitable light character and calculate its effective intensity.

- Choose the appropriate buoy for the location, taking into consideration the typical observers’ heights and expected wave amplitude when selecting the focal height.

- Consider the expected buoy motion, for the chosen buoy and local conditions, and choose a lantern with a suitable vertical divergence curve.

- Now check whether, with the chosen lantern, and its character and intensity, together with the expected motion of the buoy selected, the light signal will be recognized by the mariner and does not conflict with other nearby aids. .

- If the equipment selected does not correspond to your expectations of performance, retrace the steps above by considering:

  - other equipment, lantern and buoy;

  - a new light character with a better effective intensity and/or colour,; or

  - means for lowering the background lighting.

Sometimes, practical limitations can leave the designer with only the option to reduce the desired range and, consequently, to increase the number of aids with adverse consequences in overall cost.

# LANTERN DESIGN

This final section considers the recommendations of the TTC study mentioned above and another paper presented to IALA by Trinity House of London in 1982. The purpose of both documents was to establish an ‘optimum’ vertical profile for AtoN lanterns for buoys.

Lantern manufacturers can benefit from the following discussion for the conception and manufacture of suitable lanterns for buoys. It is also strongly recommended that they consult the original works for a complete discussion of the topics below.

The 1982 document author declared, based on his expertise that “the beam for a buoy principally in calm weather with tidal flow should be as the curve 2B and that for rough water should be as 3B”. The total spread for the two beams are 35º and 55º respectively.

The curve 1B is the result of a practical test and the author recommends the corresponding lantern for use on buoys “in still water giving a tilt not more than  10º”.

The aim of the document is “to prepare appropriate optical designs of lenses… to yield the 2B and 3B distributions of intensity…”

This proposal is amazingly similar to the conclusions taken from the other two studies and point out a kind of solution that cannot be ignored.

Please refer to IALA Guideline 1051 for the Provision and Identification of Aids to Navigation in Built-up Areas - Edition 1 - December 2005.

Brown, D. M. “Probabilities of Detection and Recognition of Flashing Lights on Rolling Buoys”, U.S. Coast Guard Research and Development Center, Report no. CG-D-10-88, August 1987.

Please refer to IALA Recommendation E. 122 on the Photometry of Marine Aids-to-Navigation Signal Lights.