Proposal for a revision of the IALA standard for cardinal buoy lights.

Tests have clearly indicated that character recognition for Cardinal Buoys can become very ambiguous due to the buoy motion. However recognition of the colour of a light is unaffected by buoy motion.

It is therefore suggested that advantage be taken of modern technology with LED-lanterns on cardinal buoys to use colour to as the means of character recognition.

Lateral buoys are idenfied by red and green for port and starboard marks respectively. A consistency of approach could be introduced by using a combination of white/red and white/green to mark the West and East cardinals respectively. South cardinal could be identied with a white/yellow/ green flash with no interval between the individual colours within the flash. The North cardinal remains unchanged. The proposal is summarised as follows:

LP			Red, as current standard

LS			Green, as current standard

NC			White, QFl

SC			White / Yellow/ Green in one long flash, repeating 10 sec

EC			White / Green with short eclipse, repeating 5 sec

WC			White / Red with short eclipse, repeating 5 sec

Wreck 			Blue / Yellow, as current standard

Special mark		Yellow, as current standard

Safe water		White with special character as current

This proposal has the advantage that a mariner can quickly identify cardinal marks by colour combinations, without the need for counting individual flashers and period time. It avoids long and complex identification characters that may be compromised by buoy motion.

This proposal would imply review of the IALA Buoyage System A/B standard.

3 April 2008.