IALA Discussion Paper on the Colour of Marine Signal Lights

Executive Summary

In this report, comparisons are made between several different colour systems for marine signal lights. Documents from the International Maritime Organisation (IMO), European Commission (EC), International Illumination Commission (CIE) and International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) are compared with a view to identifying the strengths and weaknesses of each.

Criteria for the design of any colour system for light signals should include: the number of different colours needed; the possibility of confusion between colours; light source technology; and any external affects or influences. Individual colour regions are discussed along with the impact of colour deficient observers.

Contents

1	Introduction	7

2	Objective	7

3	Scope	7

4	Current Colour Systems	8

5	Discussion	12

6	Colour Confusion	17

7	System Design Considerations	18

List of Figures

Figure 1 CIE 1931 Chromaticity Chart	8

Figure 2 IMO COLREGs Colour Boundaries	9

Figure 3 EN 14744 Colour Boundaries	9

Figure 4 CIE S 004/E:2001 - Class A Colour Boundaries	10

Figure 5 IALA 1977 Colour Boundaries (general & preferred)	10

Figure 6 IALA E-200 (draft) Colour Boundaries	11

Figure 7 White Region Close-up showing White LED and Filament Lamp Coordinates	12

Figure 8 Red Region Close-up	13

Figure 9 Green Region Close-up	14

Figure 10 Yellow Region Close-up	15

Figure 11 Blue Region Close-up	16

Figure 12 Chromaticity  Diagrams showing Lines of Colour Confusion	17

List of Tables

Table 1 Percentages of Colour Deficient Observers within a Given Population	17

Table 2 Explanation of Different Types of Colour Deficiency	18

Reference Documents

- IMO, COLREGs Annex 1, 1972

- EN 14744, “Inland navigation vessels and sea-going vessels - Navigation light”, 2005

- IALA, “Colours of Light Signals on Marine Aids-to-Navigation”, 1977

- Draft IALA Recommendation E-200 part 1 “Colours of Light Signals on Marine Aids-to-Navigation”, 2008

- ICAO Annex 14

- CIE S 004/E:2001, “Colours of Light Signals”

- CIE Technical Report 107, 1994

- CIE S 014-1/E:2006, “CIE Standard Colorimetric Observers”

- CIE Publication No. 15, “Colorimetry 3rd Edition”, 2004.

- Soon & Cole, “Critical Test of the CIE Domains for Signal Colours”, 1999

- Chapman and Hall, London “Light, Colour and Vision”, 1968

Glossary of Terms

| CIE | The Commission Internationale de l’Eclairage (International Commission on Illumination) |
| --- | --- |
| Chromaticity | The quality of a colour, as determined by its dominant wavelength, irrespective of its luminance |
| COLREGs | Convention on the International Regulations for Preventing Collisions at Sea |
| EN | European Normative |
| IALA | International Association of Marine Aids to Navigation and Lighthouse Authorities |
| ICAO | International Civil Aviation Organisation |
| Illuminance | The quantity of visible light falling onto a surface, expressed in lumens per metre squared (lux) |
| IMO | International Maritime Organization |
| LED | Light Emitting Diode |
| Standard Colorimetric Observer | CIE standard set of colour-matching functions based on colour-matching properties of a standard human observer |
| Subtense Angle | The angular size which an object subtends the eye of an observer, calculated as 2 x arctan r/d where r is the radius of the object and d is the distance from the object to the observer |

# Introduction

There are currently three signal light colour systems used by the marine sector:

- IMO, COLREGs Annex 1;

- EN 14744, “Inland navigation vessels and sea-going vessels - Navigation light”;

- IALA, “Colours of Light Signals on Marine Aids-to-Navigation”.

The mariner may also be exposed to signal lights from other transport sectors, such as aeronautical obstruction beacons (ICAO) and rail or road traffic signals when close to shore.

The Commission Internationale de l’Eclairage (CIE) produced S 004/E:2001, a standard for colours of signal lights. This standard gives colour regions for five different colours and, within those regions, different classes of colour. Although the colour boundaries given in CIE S 004 are generally adequate for most signal applications, care should be taken when designing a colour system using this standard:

- a maximum of four colours in any one system is recommended;

- restricted boundaries should be used for some colours when used in conditions of low illuminance and narrow subtense angle.

CIE Technical Report 107 of 1994 should be consulted in conjunction with CIE S 004 when designing a colour system for signal lights. This technical report contains the background to the standard.

The design of a signal light colour system should be based on at least four criteria:

- the number of different colours needed to convey information;

- the possibility of the observer confusing one colour with another;

- available light source technology to produce the required light colour;

- external effects, such as background lighting.

When making changes to an existing colour system, there is the further consideration of light source technology already in use (legacy lights).

When designing a colour system for maritime signal lights, IALA also considered the following:

- the long distances over which lights are viewed (sometimes with poor visibility):

- colour shift due to atmospheric extinction; and

- the fact that many signal lights are flashing.

# Objective

The objective of this report is to discuss the different signal light colour systems used in the maritime sector and to provide guidance on suitable colour regions.

# Scope

The scope of this report is signal lights used in the maritime environment, both as navigational lights on vessels and aid-to-navigation (AtoN) lights.

# Current Colour Systems

Five different colour systems will be considered and discussed, IMO COLREGs, EN 14744, CIE S 004/E Class A, IALA 1977 and IALA E-200 draft. The IALA E-200 draft follows a review of the IALA 1977 colour boundaries and is to include a blue region. All colour systems are shown with reference to the CIE 1931 Standard Colorimetric Observer and x, y chromaticity diagram.

Figure 1 CIE 1931 Chromaticity Chart

note: colours in this chart are for demonstration only and may vary with printer/display

Figure 2 IMO COLREGs Colour Boundaries

Figure 3 EN 14744 Colour Boundaries

Figure 4 CIE S 004/E:2001 - Class A Colour Boundaries

Figure 5 IALA 1977 Colour Boundaries (General & Preferred)

Figure 6 IALA E-200 (draft) Colour Boundaries (Optimum & Temporary)

# Discussion

## Comparisons between Different colour Systems

### White

Figure 7 White Region Close-up showing White LED and Filament Lamp Coordinates

The white regions of COLREGs and EN 14744 have similar cut off towards blue but EN 14744 is cut off earlier than COLREGs towards the yellow region (at x = 0.5). Whilst both regions will include most incandescent filament lamps, EN 14744 has a reduced risk of confusion between white and yellow. The effects of atmospheric extinction should also be considered for lights at sea. Over large distances, the atmosphere scatters shorter wavelengths of light preferentially. This can result in a white light appearing yellow and even red when viewed over distance in certain atmospheric conditions.

Compared to COLREGs and EN 14744, CIE S 004/E Class A has a shortened white region towards yellow but is extended slightly towards blue. This will further reduce colour confusion between white and yellow but will exclude many incandescent lamps currently in use as white navigation signal lamps.

Compared to all other systems, the IALA 1977 and IALA E-200 draft system has an extended white region towards blue. This is to capture white LED light sources with high colour temperature, typically used in marine signalling applications, that have significant advantages over incandescent lamps for marine use.. The phosphor conversion LEDs (pcLEDs) currently used in many short range AtoN signal lights are robust, long-lived and efficient. Furthermore, their blue-white colour is much preferred by the mariner as it is more noticeable in peripheral vision and stands out well against a background of yellow sodium lighting proliferating on our shorelines.

The white region shown in IALA E-200 draft recommendation was designed to include incandescent filament lamps but improve differentiation between white and yellow regions. The cut-off towards yellow is placed accordingly.

### Red

Figure 8 Red Region Close-up

The red regions of CIE S 004/E Class A, EN 14744 and draft IALA E-200 Optimum systems agree. The IALA 1977 system had a preferred region smaller than the IALA E-200 Optimum region. The COLREGs system has a larger red region continuing to the purple boundary. The reason why the long wavelength red region is cut off in the CIE standard, and other colour systems, is that for protanopes (see 6.2) lacking a red-sensitive retinal pigment, long-wavelength reds will not be seen (CIE Technical Report 107).

### Green

Figure 9 Green Region Close-up

The green regions of EN 14744 and draft IALA E-200 Optimum systems are the same. CIE S 004/E is similar but has the border with blue extended. This extension provides a useful area of signal green for lights with high illuminance but low illuminance green lights can be confused with white and blue, especially when they are flashing.

The IALA 1977 green preferred region extended further towards the yellow than the draft IALA E-200 Optimum. It was reduced to counteract the extension of the IALA red region (this was done in order to agree with CIE and EN 14744), because of concerns about red/green confusion for deuteranopes (see 6.2 and figure 12). The draft IALA E-200 green Temporary region was designed to include a legacy of incandescent lamps with green glass filters still in use as AtoNs.

The COLREGs green region is larger than EN 14744, CIE Class A and IALA 1977 preferred or draft IALA E-200 Optimum. It will however include incandescent lamps with green glass filters, where others will not. It should also be remembered that the existing COLREGs system does not include a region for blue.

### Yellow

Figure 10 Yellow Region Close-up

The yellow regions of the systems under consideration differ considerably. The largest region is that of CIE S 004/E. It encroaches on the Planckian locus. EN 14744 and COLREGs have the same boundaries for yellow whilst IALA has the smallest region, furthest away from the Planckian locus.  The IALA E-200 region is so placed to reduce confusion between yellow and red that can occur at distance when illuminance at the eye of the observer is low. It is also further away from the Planckian locus to avoid confusion with unfiltered incandescent lamps operating below their design voltage and close to the Planckian locus.

The draft IALA E-200 Temporary region is designed to capture yellow LEDs, currently in service as ‘special mark’ AtoNs.  The yellow region falls between two LED technologies, phosphide and nitride types. Phosphide LEDs operate at wavelengths no shorter than 590nm and their wavelength increases significantly when the junction temperature is raised.

### Blue

Figure 11 Blue Region Close-up

The current COLREGs shows no blue region. EN 14744 included a blue region for signal lights used by emergency services (fire, police, coastguard, lifeboats etc.). IALA 1977 did not consider the use of blue lights as AtoN signals but, due to the recent introduction of emergency wreck marking buoys that exhibit an alternating blue and yellow flashing light, a blue region has been specified in the IALA E-200 draft recommendation.

EN 14744 and CIE Class A differ slightly in their boundaries for blue but both extend well towards the white in order to capture filtered incandescent lights commonly used for emergency service beacons.

CIE Technical Report 107 recommends truncating the blue region if blue lights are to be used for conditions of low illuminance. Since this is often the case at sea, the IALA E-200 draft recommendation shows an Optimum blue region that is a truncated version of the CIE Class A region. Further reasons for using a truncated region are:

- to reduce colour confusion with white and green;

- to discourage the use of filtered incandescent lamps (these can have significant red output which can dominate over distance due to atmospheric scattering and make a blue light appear red).

Emergency wreck marker buoys utilise LED technology and the saturated colour of blue LEDs falls within the draft IALA E-200 blue Optimum region.

# Colour Confusion

Shown below are three chromaticity diagrams with lines of colour confusion for various types of colour deficient observers. The colour regions shown on each diagram are those of the IALA E-200 Optimum.

Figure 12 Chromaticity  Diagrams showing Lines of Colour Confusion

Left: Protanope; Centre: Deuteranope; Right: Tritanope

## Percentages of Colour Vision Deficient Observers

| Type of Deficiency |  | % males | % females |
| --- | --- | --- | --- |
| Overall |  | ~8% | ~0.5% |
| Anomalous trichromasy | Protanomaly | 1% | 0.01% |
|  | Deutanomaly | 5% | 0.4% |
|  | Tritanomaly | rare | rare |
| Dichromasy | Protanopia | 1% | 0.01% |
|  | Deuteranopia | 1.5% | 0.01% |
|  | Tritanopia | 0.008% | 0.008% |

Table 1 Percentages of Colour Deficient Observers within a Given Population

## Description of Different Types of Colour Vision Deficiency

| Protan | Protanope | Dichromat | missing longer wavelength (red) cone pigment |
| --- | --- | --- | --- |
|  | Protanomal | Anomalous Trichomat | anomalous (misaligned) longer wavelength cone pigment |
| Deutan | Deuteranope | Dichromat | missing middle wavelength (green) cone pigment |
|  | Deuteranomal | Anomalous Trichomat | anomalous (misaligned) middle wavelength cone pigment |
| Tritan | Tritanope | Dichromat | missing shorter wavelength (blue) cone pigment |
|  | Tritanomal | Anomalous Trichomat | anomalous (misaligned) shorter wavelength cone pigment |

Table 2 Explanation of Different Types of Colour Deficiency

# System Design Considerations

From the bullet points in the introduction, it would be worth considering the following when designing a system of colours for signal lights:-

- How many different colours are needed? Is IMO to recommend blue as a region for signal lights? If so, CIE S 004/E recommends a maximum of four colours in any system. The inclusion of blue would make five colours.

- If the intended users of this system comprise all mariners, a significant percentage of them will not have been tested for colour deficiency. Of those, a percentage will be colour deficient (see table 1). Of those that are colour deficient, a percentage will not know that they are colour deficient.

- What is currently being used to provide signal lights and what emerging technologies are being proposed or trialled? It is important not to unnecessarily proscribe existing equipment or potentially useful or popular technology.

- If the effects of background lighting are becoming more evident with the proliferation of lighting and signage on our coastlines, ports and harbours. The effects of this on the ability of a mariner to recognise a signal should be considered.