IALA Aids to Navigation Manual

IALA NAVGUIDE

AISM-IALA : 20 ter rue Schnapper  - 78100 Saint-Germain en Laye  -  France

Telephone: + 33 1 34 51 70 01      -         fax: + 33 1 34 51 82 05

E-mail :  iala-aism@wanadoo.fr       -      Internet: //www.iala-aism.org

© IALA-AISM 2006

Reproduction for training / educational purposes permitted.

- AIDS TO NAVIGATION

## Visual Aids to Navigation

### Fixed Aids to Navigation

The IALA International Dictionary of Aids to Marine Navigation defines a beacon as “a fixed artificial navigation mark” that can be recognised by its shape, colour, pattern, topmark or light character, or a combination of these. While this functional definition includes lighthouses and other fixed aids to navigation, the terms lighthouse and beacon are used more specifically to indicate importance and size.

#### Definitions

Photo courtesy of Australian Maritime Safety Authority

Lighthouse:  A lighthouse is generally considered to be:

A large conspicuous structure (visual mark) on land, close to the shoreline or in the water that:

- acts as a daymark, and;

- provides a platform generally for higher range marine AtoN signal lights.

Other aids to navigation, such as audible signals and RACONs, may be located on or near the lighthouse;

A Lighthouse may be a staffed or automated facility, although the staffing of Lighthouses is becoming less common.

An automated lighthouse will often be remotely monitored and in some cases remotely controlled.

Beacon: Visual characteristics are often defined by daymarks, topmarks, and by numbers.  A marine signalling light, if fitted, would generally be of a lower range than lighthouses.

In navigable channels a pile beacon may be used as an alternative to a buoy.

#### Purpose of Lighthouses and Beacons

A lighthouse or beacon may perform one or more of the following navigational functions:

- mark a landfall position;

- mark an obstruction or a danger;

- indicate the lateral limits of a channel or navigable waterway;

- indicate a turning point or a junction in a waterway;

- mark the entrance of a Traffic Separation Scheme (TSS);

- form part of a leading (range) line;

- mark an area;

- provide a reference for mariners to take a bearing or line of position (LOP).

Oother purposes include:

- base for AIS equipment;

- base for RACON;

- platform for radar;

- base for radionavigation systems;

- reference station for dGNSS;

- coastwatch or coastguard functions;

- VTS functions;

- base for audible (fog) signals;

- collection of meteorological and oceanographic data;

- radio and telecommunication facilities;

- tourist facilities.

In these situations the beacon will generally show a colour scheme and topmarks in accordance with the IALA Maritime Buoyage System.