Probability of Detection

Larry Jaeger of USCG has kindly provided a paper by Montonye, submitted to the IALA 1970 conference in Stockholm, that attempts to assign probabilities to several visual experiments. The conclusion is that they were all carried out by different methods and therefore have different probabilities but Montonye provides a table for several different experiments for a ‘dark adapted 99% probability of seeing foveal threshold’ in microlux for 11 experiments.

After much discussion in a meeting of the Ad Hoc WG at Koblenz, we looked at the illuminance of 0.2microlux, used as the international standard for marine observation. It is stated that, for the conditions of observation, the probability of detection is 99%. It seems that Montonye’s efforts were directed towards complying with this standard.

In view of the requirement for ‘safe’ navigation, it is assumed that the probability of detection should be as high as practically possible. Montonye suggests that this should be 99%.

It seems logical therefore, for a conspicuity model based on Bayesian networks, to base probabilities on a ‘99% probability of detection’. Where normal distributions are used within the model, three sigma should be used when calculating probability.

Then there is the question of measurement uncertainty. In most measurement systems, and recommended by ISO GUM, measurement uncertainty is based on a standard deviation to two sigma. This typically gives around 95.4% probability. If three sigma is used, the probability rises to 99.7%.

It may be found that a level of probability of 99% is impractical for some parts of the model. If this is the case, a 95% probability should be considered for parts of the model and comparisons made to 99% where possible.

Ian Tutt 12.03.2008