- AIDS TO NAVIGATION

A marine Aid to Navigation (AtoN) is a device or system external to vessels that is designed and operated to enhance the safe and efficient navigation of vessels and/or vessel traffic.

A marine aid to navigation should not be confused with a navigational aid.  A navigational aid is an instrument, device, chart, etc., carried on board a vessel for the purpose of assisting navigation.

This chapter describes the major types of aids to navigation in current use and provides comments on the application and performance of the technology.  Automatic Identification System (AIS) and Vessel Traffic Services (VTS) are covered in the chapter, since IALA considers these services as satisfying the definition of an aid to navigation.

The concept of e-navigation is gaining momentum.  At MSC81, IMO was asked to consider the development of an e-navigation strategy.  IALA has identified e-navigation in its strategic planning, and has formed an e-navigation committee for the 2006-2010 work programme.  IALA has also developed a working definition for e-navigation:

“e-Navigation is the collection, integration and display of maritime information onboard and ashore by electronic means to enhance berth-to-berth navigation and related services, safety and security at sea and protection of the marine environment.”

## Visual Aids to Navigation

Visual marks for navigation can be either natural or man-made objects. They include structures specifically designed as short range aids to navigation, as well as conspicuous features such as headlands, mountain-tops, rocks, trees, church-towers, minarets, monuments, chimneys, etc.

Short range aids to navigation can be fitted with a light if navigation at night is required, or left unlit if daytime navigation is sufficient.

Navigation at night is possible, to a limited extent, if the unlit aids are provided with:

- retro-reflecting material, and the vessel has a searchlight.  This approach would generally only be acceptable for small boats operating in safe waterways and with the advantage of local knowledge.

### Description

Visual aids to navigation are purpose-built facilities that communicate information to a trained observer on a vessel for the purpose of assisting the task of navigation. The communication process is referred to as Marine Signalling.

Common examples of visual aids to navigation include lighthouses, beacons, leading (range) lines, lightvessels, buoys (lit or unlit), daymarks (dayboards) and traffic signals.

The effectiveness of a visual aid to navigation is determined by factors such as:

- type and characteristics of the aid provided;

- location of the aid relative to typical routes taken by vessels;

- distance (range) of the aid from the observer;

- atmospheric conditions;

- contrast relative to background conditions (conspicuity);

- the reliability and availability of the aid.

Refer to IALA publications:

IALA Recommendation O-130 on Categorisation and Availability Objectives for Short Range Aids to Navigation;

IALA Guideline 1035 on Availability and Reliability of Aids to Navigation.

### Distinguishing Features

Visual aids to navigation are distinguished by:

- type;

- fixed structure;

- floating platform;

- location;

- inclusion of subsidiary aids;

- relationship to other aids to navigation and observable features;

- shape;

- size;

- elevation;

- colour;

- lit/unlit;

- signal character;

- light intensity;

- sectors;

- construction materials;

retro-reflective features;

Photo courtesy of IALA

- names, letters and numbers.

### Signal Colours

IALA has made recommendations on colours for lighted aids to navigation and for surface colours for visual signals on aids to navigation.

Marine aid-to-navigation signal lights use a five-colour system comprising white, red green, yellow and blue, as will be defined in IALA recommendation E-XXX Part 1, to be issued in the 2006-2010 time-frame.  Although the colour regions defined in this IALA recommendation agree with those given in the International Commission on Illumination (CIE) Standard S 004/E 2001 “Colours of Light Signals”, the boundaries of each colour region differ in some cases. Furthermore, in their standard, CIE recommend that signal systems should normally comprise no more than four colours.

Recommended surface colours for visual signals on aids to navigation are as follows:

- Ordinary colours should be limited to white, black, red, green, yellow or blue.

- Orange and fluorescent red, yellow, green or orange may be used for special purposes requiring high conspicuity.

Refer to IALA publications:

IALA Recommendation E-106 for the use of retro-reflecting material on aids to navigation marks within the IALA Maritime Buoyage System;

Recommendation E-108 for the surface colours used as visual signals on aids to navigation.

IALA Guideline 1015 for painting aids to navigation buoys (including reference to the practical guide on surface colours)

The CIE standard on the measurement of colours (colorimetry) is based on three reference colours (ie. a tri-stimulus system) that in varying combination can generate the spectrum of colours.  A particular colour function is described by the symbols; X, Y and Z that represent the proportions of the reference colours.

Using ratios of the tri-stimulus values, such that: x + y + z = 1, colours can be defined in terms of chromaticity using just the x [= X / (X+Y+Z)] and y [= Y/ (X+Y+Z)] values.  The advantage of this arrangement is that colours can be mapped on a two-dimensional chromaticity diagram.

CIE colour standards for marine signalling can be depicted as areas on the chromaticity diagram.  These areas are defined by boundaries expressed as functions of x and y (equations).

If the chromaticity co-ordinates of a coloured light, filter material or a paint product are known, its acceptability for marine signalling applications can easily be determined.

Figure 1 – Illustration of the colour zones on the 1931 CIE chromaticity diagram.  (Note that the colour rendering is only indicative and should not be taken as fully accurate)

The CIE standard for signalling colours has recently been revised, with some adjustments to the boundaries of signal colours.  Further information on surface colours can be found in IALA Recommendation E108 on the surface colours used as visual signals on aids to navigation.  Information for light signal colours is shown in IALA Recommendation for the colours for light signals on aids to navigation, December 1977. For further details on this issue please refer to CIE S 004/E-2001 Colours of Light Signals.

Figure 2 – IALA allowed chromaticity areas for red, orange, yellow, green, blue, white and black of Ordinary Surface Colours plotted on the 1931 CIE chromaticity diagram (Courtesy of CIE)

### Visibility of a Mark

Photo courtesy of Trinity House, UK

The visibility of a mark is affected by one or more of the following factors:

- observing distance (range);

- curvature of the Earth;

- atmospheric refraction;

- atmospheric transmissivity (meteorological visibility);

- height of the aid above sea level;

- observer's personal visual perception;

- observer's height of eye;

- observing conditions (day or night);

- conspicuity of the mark (shape, size, colour, ability to reflect light in the direction of the observer);

- contrast (Type of background – lighting, vegetation, snow, etc…);

- mark lighted or unlit;

- intensity and character.

### Meteorological Visibility

Meteorological visibility (V) is defined as the greatest distance at which a black object of suitable dimensions can be seen and recognised by day against the horizon sky, or, in the case of night observations, could be seen if the general illumination were raised to the normal daytime level.  It is usually expressed in kilometres or nautical miles.

### Atmospheric Transmissivity

The atmospheric transmissivity (T) is defined as the transmittance or proportion of light from a source that remains after passing through a specified distance through the atmosphere, at sea level.

Since the atmosphere is not uniform over the observing distances of most visual aids, a representative value is used:

- typically, the atmospheric transmissivity is taken as T = 0.74 over one nautical mile;

- a figure of T = 0.85 is occasionally used in regions where the atmosphere is very clear.

In practice the value can vary a lot and a number of countries collect data on atmospheric transmissivity for different parts of their coastline.  This enables the luminous range of lights to be:

- calculated more precisely, and;

- better matched to local conditions and user requirements.

### Atmospheric Refraction

This phenomenon results from the normal decrease in atmospheric density from the Earths’ surface to the stratosphere.  This causes light rays that are directed obliquely through the atmosphere to be refracted (or bent) towards the Earth.

### Contrast

The ability to detect differences in luminance between an object and an otherwise uniform background is a basic visual requirement and is used to define the term contrast.  It is represented by the equation:

C = (Lo −LB)LB

where:

C   =       contrast

| LB = | luminance of background (cd/m2) |
| --- | --- |
| Lo = | luminance of object (cd/m2) |

The contrast at which an object can be detected against a given background 50% of the time is called the threshold contrast.  For meteorological observations, a higher threshold must be used to ensure that the object is recognised.

A contrast value of 0.05 has been adopted as the basis for the measurement of meteorological optical range.

### Use of Binoculars

While it is generally assumed that observations will be made with the naked eye, mariners will quite often use binoculars.  This can allow:

- a light being observed, or the characteristics resolved, at a greater luminous range than with the naked eye;

- a limited improvement in the sensitivity of leading lights;

- about a 30% improvement in the detectable difference from a given bearing;

- the identification of a light operating against background lighting conditions.

Generally, the most suitable binoculars for use at sea are considered to be:

- 7 x 50 type at night, and;

- 10 x 50 binoculars by day.

### Range of a Visual Mark

The range of an aid to navigation can broadly be defined as the distance at which the observer’s receiver can detect and resolve the signal.  In the case of visual marks the observer’s receivers are his/her eyes.  This broad definition of range leads to a number of more specific definitions that are described below.

#### Geographical Range

This is the greatest distance at which an object or a light source could be seen under conditions of perfect visibility, as limited only by the curvature of the earth, by refraction of the atmosphere, and by the elevation of the observer and the object or light. (IALA International Dictionary of Aids to Marine Navigation).

A Geographical Range table is shown in Table 1.

Table 1 – IALA Geographical Range Table

| Geographical Range in Nautical Miles |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Observer eye height metres | Elevation of Mark / metres |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 10 | 50 | 100 | 200 | 300 |
| 1 | 2.0 | 4.1 | 4.9 | 5.5 | 6.1 | 6.6 | 8.5 | 16.4 | 22.3 | 30.8 | 37.2 |
| 2 | 2.9 | 4.9 | 5.7 | 6.4 | 6.9 | 7.4 | 9.3 | 17.2 | 23.2 | 31.6 | 38.1 |
| 5 | 4.5 | 6.6 | 7.4 | 8.1 | 8.6 | 9.1 | 11.0 | 18.9 | 26.9 | 33.3 | 39.7 |
| 10 | 6.4 | 8.5 | 9.3 | 9.9 | 10.5 | 11.0 | 12.8 | 20.8 | 26.7 | 35.1 | 41.6 |
| 20 | 9.1 | 11.1 | 12.0 | 12.6 | 13.1 | 13.6 | 15.5 | 23.4 | 29.4 | 37.8 | 44.2 |
| 30 | 11.1 | 13.2 | 14.0 | 14.6 | 15.2 | 15.7 | 17.5 | 25.5 | 31.4 | 39.8 | 46.3 |

The values in Table 1 are derived from the formula:

Rg=2.03×(ho+ Hm)

where:

| Rg = | geographical range (nautical miles) |
| --- | --- |
| ho = | elevation of observer’s eye (metres) |
| Hm = | elevation of the mark (metres) |

Note - The factor 2.03 accounts for refraction in the atmosphere.

Climatic variations around the world may lead to different factors being recommended.   The typical range of factors is 2.03 to 2.12.

#### Meteorological Optical Range

This is the distance through the atmosphere that is required for 95% attenuation in the luminous flux of a collimated beam of light using a source colour temperature of 2700ºK.

The meteorological optical range is related to the atmospheric transmissivity by the formula:

V=dlog0.05logT	or	T=0.05dV

Where:

V  =	meteorological optical range (nautical miles)

d  =	distance (nautical miles)

T  =	atmospheric transmissivity

It is often convenient to simplify the above expression by giving the distance term a  value of one, such that:

T=0.051V	or	TV=0.05

#### Visual Range

This is the maximum distance in daytime at which the contrast of the object against its background is reduced by the atmosphere to the contrast threshold of the observer.  The visual range can be enhanced if the observer uses binoculars, although the effectiveness depends on the stability of the observer’s platform. Visual Range can be interpreted as the maximum distance that a given object is seen by an observer in daytime.

#### Luminous Range

This is the maximum distance at which a light signal of a specific intensity can be seen by the eye of the observer at a given time, as determined by the meteorological visibility prevailing at that time. The geographical range limits the luminous range for practical purpose.

It does take into account the:

- height of the light,

- observer's height of eye,

- curvature of the Earth

#### Nominal Range

Nominal range is the luminous range when the meteorological visibility is 10 nautical miles, which is equivalent to a transmission factor of T = 0.74.  Nominal Range is generally the figure used in official documentation such as nautical charts, Lists of Lights, etc.

Nominal range assumes that the light is observed against a dark background, free of background lighting.

New IALA Recommendation, due for completion in the 2006-2010 work term, will replace IALA “Colours of Light Signals”, December 1977.

CIE S 004/E 2001 replaces CIE 2.2 - 1975, “Colours of Light Signals”.

Blue surface colours may be used in inland waterways, estuaries and harbours where the colours may be seen at close range.  See IALA Recommendation E108.  In addition, blue lights are being tested for use on emergency wreck marking buoys – IALA Recommendation O-133 refers.

CIE web site address: http://www.cie.co.at/cie

That is, a magnifying power of 7 and an objective lens of 50 mm diameter.