# Navguide revision

## Section 3

### Sector Lights and Leading (Range) Lines

Note – Bearings, directions of leading (range) lines and limits of sectors should always be stated in terms of the bearings that would be seen by the mariner.  Bearings may carry a suffix ‘TBS” or True Bearing from Seaward as confirmation.

#### Sector Lights [Update in light of new technologies eg LED]

A sector light is an aid to navigation that displays different colours and/or rhythms over designated arcs.

A common means of creating a sector is to fit a coloured filter in front of the main light. However, sector lights with LED light sources are being introduced to the market.  A sector can be produced by filtering or by using a secondary light (or several lights) on the same structure.  The secondary light can take any of the following forms:

- Range (directional) light;

- Beacon with a coloured lens, masked to achieve the sector angle;

- Beacon fitted with internal or external filter panels;

- Beacon or beacons with different coloured light sources, masked to achieve the sector angle;

- Precision Direction Light.

The limits or boundaries of a sector are not always precisely cut off due to the characteristics of the light source, fading of colours or changing rhythms between adjacent sectors.

For a beacon fitted with coloured filter panels, the reason for the lack of a precise transition at the sector boundary is readily apparent from Figure 1 which shows the light source, lens and filter geometry.  The transition zone is defined by an "angle of uncertainty".

A similar geometry exists with multiple coloured beacons and masking.

Figure 1 – Angle of Uncertainty

It can also be noted that:

- the observed angle of uncertainty is generally less than the geometric angle due to the relative intensities of sector colours (ie. colour mixing) as the observer passes through the transition zone;

- if space on the aid to navigation structure is not a limiting factor, it is usually possible to achieve an angle of uncertainty of around 0.25 with this type of sector arrangement;

- the angle of uncertainty can be reduced by decreasing the physical width of the light source or by increasing the radial distance to the coloured filter;

- in situations where the main light has a large projected area, such as a rotating lens or reflector array, it is generally preferable to use a separate sector light rather than installing a coloured filter in front of the main light.

From time to time specialised sector lights have been developed to exhibit different rhythms over different sector bearings.  This capability is found in some Precision Direction Lights (PDL).

A PDL is a specialised form of sector light that can generate sharply defined sector boundaries.  This feature is particularly useful for applications that require one or several narrow sectors. The PDL may use a white light source with coloured filter, but newer designs are utilising LED or laser lights.

PDL Sector Lights are very precise, providing a complete colour change at a sector boundary occurs over an angle of less than 1 minute of arc in most models.

##### Applications

The design of sector lights can be a complex task.  The process should be carried out with reference to an accurate chart of the area.  In some cases good local knowledge is also required.

A sector light may indicate one or more of the following:

- boundaries of a navigable waterway;

- change of course position;

- shoals, banks, etc.;

- an area or position (eg. an anchorage);

- the deepest part of a waterway;

- position checks for floating aids.

A Precision Direction Light (PDL) allows for further applications that include the ability to:

- produce narrow sectors with an angles of uncertainty down to approximately one minute of arc;

- define the central zone of a channel;

- accurately mark one side of a straight channel.  (A pair of PDLs can cover the permutations of converging, diverging and parallel channels);

- define different rhythms over adjacent sectors.

##### Examples

Some examples of sector lights applications are illustrated in Figure 2 and Figure 3.

Figure 2 – Sector Light Application

This illustration follows the IALA Maritime Buoyage System colour convention for Region A (‘red to port when approaching the aid from seaward’).  The white sector should, if possible, be wide enough to provide a margin of safety for a vessel that inadvertently leaves the white sector. Curves C and D indicate depth contours or limiting dangers that dictate the boundaries of sectors.

Figure 3 – Shows several applications for sector lights.

The function of each light is described below:

- Light 1 is a coastal white light with a red sector indicating a danger.

- Light 2 is a sector light obscured over the shore, with two white sectors indicating a safe channel.  When sailing towards the sector light it shows red to port and green to starboard following the IALA Maritime Buoyage System colour convention for Region A.  The boundary between the red and the green sector also indicates the position of a buoy.

- Light 3 is a sector light with a red light and 4 white sectors indicating four anchorage positions.  It is obscured over the shore.

- Light 4 is a sector light with a white sector indicating a safe channel.

##### Design Considerations for Sector Lights

Where a single sector light defines a navigable channel the following points should be considered:

- Lateral position:  There is no reference of the vessel’s lateral position within the channel until a sector boundary is reached.  This may cause a problem in channels subject to a strong cross current.  For vessels with local knowledge, the zones defined by the angle of uncertainty can sometimes provide a useful guide to the vessel’s proximity to a sector boundary;

- Safety margin:  Where practicable, there should be a margin of safety between the sector boundary and adjacent hazards.  If an appropriate safety margin cannot be achieved within the sector boundary, the hazards could be marked separately.

- Angle of uncertainty:  Zones defined by the angle of uncertainty should be considered an additional margin of safety over the actual sector boundary;

- Vessel size:  The design process for a sector light needs to consider the draught and manoeuvrability of the largest vessels likely to utilise the sector, how quickly they can respond once they cross a sector boundary and the situations that may develop when other vessels are in the vicinity;

- Lights and filters:  A sector design should take account of the spectral distribution of the light source and the proportion of this light transmitted through the filter material as this will affect the resultant colour and intensity of the light exhibited.  The process should also check for potential glare problems;

- Flash characteristic:  The period of the light flash should be selected to provide ample time for a mariner to recognise the transitional phases that occur at the sector boundary;

- Sector colours:  A white light is normally the first preference for a lighthouse or beacon.  If a single coloured sector is added, red is often used.  If a white sector light is used to mark a navigation channel, coloured sectors may be used either side of the white to indicate the lateral limits.  In such cases it is common practice to use red and green sectors that follow the convention of the IALA Maritime Buoyage System;

- Multiple sectors:  Multiple sectors can be used to provide a better indication of a vessel’s lateral position within the channel but at the expense of complexity for both the system designer and navigators.

- Lamp position and type:  The position of the light source within the optical system is critical for the correct alignment of the sectors. When replacing lamps or using lampchangers, it is important to ensure that the light source (e.g. filament) position is repeatable. If a lampchanger is incorporated, the sector system should be designed for the widest light source used in the lampchanger.

Refer to IALA publication:

IALA Guideline 1041 on sector lights

#### Transits / Leading Lines (Range Lines)

A transit is defined as the alignment of two or more marks.

A Leading (or Range) light is a specialised application of a transit.

A simple transit can be used to:

- Provide a turning reference;

- Define a clearing line for the limits of safe navigation;

- Provide a distance mark along a waterway.

##### Leading Lines (Range Lines)

A leading line is an aid to navigation system that comprises two separated structures with marks or lights that, when viewed from the centreline or deepest route along a straight section of channel, are aligned.

In a two station leading line, the structures lie along an extension of the centreline of the nominated channel.  The rear structure must have a greater elevation than the front structure to enable both marks or lights to be viewed simultaneously.

A leading line provides a vessel with a heading reference and a visual indication of the size and direction of any cross track error.

Figure 4 – Leading Line (photo courtesy of Canadian Coast Guard)

##### Purposes of Leading Lines

A leading line may be used to:

- indicate the centreline of a straight section of a navigable channel;

- indicate to deep draught vessels the deepest part of the waterway;

- indicate the navigable channel where fixed and floating aids to navigation are not available or do not satisfy the accuracy requirements for safe navigation;

- define a safe approach bearing to a harbour or river entrance, particularly where there are cross currents;

- separate two-way traffic (i.e. when passing a bridge).

##### Design Considerations for Leading Lines

A well-designed leading line will enable vessels of the type and size that commonly use the channel to:

- Identify the marks or lights when the ship is at the inner and outer sections of the channel and readily detect cross track position errors from the centreline of the channel;

- Detect cross track position errors with sufficient sensitivity that the channel can be utilised without abrupt changes to the vessels heading and speed;

- Observe both lights together, by selection of leading light character rhythms that appropriately overlap in their free running condition. In some situations it may be preferable to provide additional equipment to synchronise the light characters.

- Observe the lights in all ambient conditions for which they are designed to be used without glare, if lights are to be used for both day and night operations light intensities will need to be varied.

The characters of rhythmic leading lights should be selected so that the front and rear lights, in their free running states, can generally be observed together.  In some situations it may be preferable to provide additional equipment to synchronise the light characters;

If lights are to be used both day and night, the light intensities should be adapted for each situation to avoid glare at night.

Radar transponders (RACONs) may be used as leading line markers.

Refer to IALA publications:

IALA Recommendation E-112 for leading lights (including excel program)

IALA Guideline 1023 for the design of leading lines

IALA Recommendation for a definition of nominal daytime range of maritime signal lights intended for guidance of shipping by day, (April 1974)

Also known by the trade name of PEL Light

See also IALA Recommendation E 110, The Rhythmic Character of Lights on Aids to Navigation.

For example, in waterways where the aid may be drifting or destroyed due to ice conditions.