- EEP 12/WG1/WP1

- IALA Guideline No. ???

- On

- FLOATAID MOORINGS

- Edition ??

- October 2008

20ter, rue Schnapper, 78100
Saint Germain en Laye, France
Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05
E-mail - iala-aism@wanadoo.fr      Internet - http:lliala-aism.org

Table of contents

INTRODUCTION	5

1	GENERAL CONSIDERATIONS	5

1.1	Design Process for a new Mooring	6

1.1.1	Data Required	6

1.1.2	Calculation Process	6

2	VARIOUS SECTIONS OF THE MOORING	7

2.1	Tail Chain	7

1	HIGH WATER	7

2.2	Bridle	8

2.3	The Riding Chain	8

2.4	The Thrash Chain	8

2.5	The Ground Chain	8

2.6	Swinging Radius	8

2.7	Sinkers	9

3	MOORING DESIGN	9

3.1	Mooring Types	9

3.2	Transitional Mooring	10

3.2.1	Transitional moorings Loads	10

3.2.2	Chain Size for transitional mooring	11

3.2.3	Chain Length  for transitional moorings	12

3.2.4	Reserve Buoyancy for transitional mooring	12

3.2.5	Swinging Radius for transitional mooring	13

3.3	Slack Moorings	14

3.3.1	Ground Chain Resistance for slack mooring	14

1.2	Taut Mooring	15

3.3.2	Taut Mooring Loads	16

3.3.3	Chain Size for taut moorings	16

3.3.4	Reserve Buoyancy for taut moorings	16

3.3.5	Swinging Radius for taut mooring	17

3.4	Sinker Weight	17

3.4.1	Sinker Weight for Transitional and Slack Moorings	18

3.5	Buried Sinkers	18

4	CHAIN MOORING COMPONENTS	19

4.1	Chain	19

4.1.1	Material Used In The Manufacture Of Chains	20

4.1.2	Heat Treatment	20

4.1.3	Chain finish	20

4.1.4	Specifications And Standardisation	21

4.2	Shackles	21

4.2.1	Forelock Shackles	21

4.2.2	Clenching Shackles (heat and beat - American)	22

4.2.3	Bolt Shackles	22

4.2.4	Screw Pin Shackles	23

4.2.5	Kenter Shackles	23

4.2.6	Quick release links	24

4.3	Swivels	24

4.4	Sinkers	25

4.4.1	Concrete sinkers	25

4.4.2	Rock sinkers	26

4.4.3	Cast iron sinkers	26

5	MOORING WEAR AND CORROSION	27

5.1	Wear	27

6	SERVICE LIFE	28

7	ROPE MOORINGS	28

7.1	Rope Buoy Moorings	28

7.2	Mooring Design	29

7.3	Rope size	29

7.4	Rope Construction	30

7.5	Types Of Fibre	31

7.5.1	Nylon	31

7.5.2	Polyester	31

7.5.3	Polypropylene	31

7.5.4	Advanced Fibres	31

7.6	Mixed Construction	32

7.7	Rope Terminations	32

7.7.1	Thimbles	32

7.7.2	Splices	32

7.8	Hybrid Rope/Chain Moorings	32

7.9	Tension Leg Moorings	32

7.10	Handling Rope Moorings	33

7.10.1	Deployment	33

7.10.2	Recovery	33

7.11	Safety	33

8	ELASTIC MOORINGS	34

8.1	Introduction	34

8.2	Elastic Mooring Construction	34

8.3	Elastic Mooring Design	34

8.4	Handling	35

8.4.1	Advantages	35

9	RIVER MOORINGS	35

9.1	GERMANY	35

9.1.1	Rock mooring of buoys on the River Rhine	35

9.2	HOLLAND	37

9.2.1	Mooring with anchors on the Rhine	37

APPENDIX 1	CHAIN MOORING DESIGN	38

APPENDIX 2	example of a mooring design	39

APPENDIX 3	concrete sinkers characteristics	45

APPENDIX 4	Illustrated details of Canadian Coast Guard’s rope mooring system	47

# INTRODUCTION

This guideline combines three existing IALA papers:‘Recommendation E107 On the Design of Normal Moorings, 1998’, ‘Guidelines on Synthetic Moorings 1024, 2001’,and ‘Practical Notes On The Use Of Mooring Chains For Floating Aids To Navigation, 1989’ and provides information based on current practices used by IALA members’ Authorities in 2008.

# GENERAL CONSIDERATIONS

In order to ensure the safety of the mariner, lighthouse authorities maintain buoys and lightvessels as aids to navigation (AtoN). These floating Atons are maintained in position by their moorings

The mooring system must maintain the floating aid in a sufficiently accurate position for it to perform its function as an AtoN.

The mooring consists of a flexible cable connecting the floating AtoN to an anchoring device.

In most cases we will be considering a buoy connected by a length of mooring chain to a sinker on the seabed. However the concepts described can equally be applied to large floating aids such as lightvessels moored with anchors.

The design of the mooring will depend on:

- The depth of water at the mooring site

- The buoyancy of the floating aid

- The seabed conditions at the site

- The loads imposed on the mooring by the floating aid due to wind, wave action, water flow and ice.

- Loads imposed on the mooring cable by water flow

- Local conditions that cause wear and corrosion to the mooring

- Available servicing facilities

- Required life of the mooring

- Cost

The most common form of mooring cable is steel chain. The chain will form a catenary between the buoy and the seabed and will be able to absorb considerable amounts of energy. Chain has good wear resistance. With the correct equipment it is easy and safe to handle. Chain can easily be joined with shackles with little reduction in its tensile strength.

Buoy moorings usually consist of open link chain whose size is specified by the bar diameter of the material from which the chain is manufactured. Buoy mooring chain might range from 12.5 to 50mm. The chain material may be mild steel alloy or a medium carbon alloy steel may be used where its extra cost can be justified by improved wear resistance.

Stud link chain is not normally used for buoy moorings due to its initial high cost but may be considered where its increased weight, in comparison with open link chain may be an advantage.

However there are two particular situations where the use of chain is problematic:

- Where very deep moorings are planned, perhaps more than 60m water depth, a chain mooring may be too heavy for a normal buoy to support the weight of the mooring chain. In this case synthetic rope may provide a suitable lightweight material for part of the mooring cable. Section (7) provides details of rope moorings.

- Shallow moorings, particularly where breaking waves are encountered may also be problematic for conventional chain moorings. If breaking waves regularly occur at the buoy site, this would typically be when the wave height was ¼ (or more) of the water depth, then each breaking wave will impart considerable energy to the buoy. The length of the mooring chain which could typically be 7x water depth,  may not be able to absorb the very high loads generated by a wave swept buoy, resulting in the chain being broken or the sinker dragging from position. One solution to this problem is the use of elastic mooring cable. Section (8) provides details of rubber cord moorings.

# Design Process for a new Mooring

To undertake the mooring calculations it will be necessary to know the physical details of the buoy to be moored and the environmental conditions at the mooring site. To refine the calculations it will also be necessary to know the size and strength of mooring chain that is available, details of sinkers that are available and the capacity of the lifting equipment on the servicing vessels.

### Data Required

The AtoN requirements for the buoy, that is the intensity of the light, the focal plane height, the daymark size, other AtoN systems and their associated power systems will define the type or class of buoy that will be deployed. The required AtoN positional accuracy may influence the design of the mooring that will be used and results for the mooring loads obtained from the calculations may require a larger buoy body than was originally envisaged.

To calculate the loads imposed on the mooring by the wind dimensions of the buoy superstructure and daymark will be needed.

The calculation of the loads imposed on the mooring by tidal flow or current will require dimensional details of the buoy body and tailtube or skirt.

The calculation of freeboard under maximum mooring load conditions will require the physical dimensions and the displacement of the chosen buoy.

Environmental information for the mooring site will provide details of the worst anticipated wind forces, tidal flow or current and wave heights. Information on the type of bed on which the sinker sits is also needed.

### Calculation Process

The relevant buoy data and environmental forces can then be used in the ‘Transitional Mooring Formulae’ to calculate mooring loads, chain size, chain length, sinker size, swinging circle radius and the reserve buoyancy of the buoy.

If the calculated swinging radius does not meet navigational requirements then the mooring design can be re-calculated with a heavier chain or with the ‘ Taught Mooring  Formulae’ so that an acceptable mooring can be achieved. The suitability of the chosen buoy can then be re-assessed by calculating the new reserve buoyancy.

If the swinging radius obtained from the ‘Transitional Formulae’ is not a problem then the ‘Slack Mooring Formulae’ may be investigated to see the effects of increasing the chain length to reduce loads on the sinker or possibly reduce the size of the sinker.

# VARIOUS SECTIONS OF THE MOORING

HIGH WATER FINE WEATHER

STORM CONDITIONS

LOW WATER

SWINGING CIRCLE RADIUS

SINKER

GROUND CHAIN

RIDING CHAIN

EXTENT OF THRASH

– ALL CONDITIONS

- Buoy Mooring

A mooring is normally made up of the following parts suitably assembled:

- Either a Tail Chain or a Bridle (depending upon the method of securing to the buoy)

- Riding Chain

- Thrash Chain

- Ground Chain

## Tail Chain

For buoys with the mooring attached to a single a single mooring eye, this is the length of chain connected to the buoy. It suffers from interlink wear as it absorbs a significant portion of the energy generated by the buoy as it rises and falls following the surface movement of the sea. It also has to absorb the rotational energy of the mooring induced by the buoy rotating around its axis. To limit this effect a stabilising fin can be fitted to the buoy on the opposite side to the mooring eye. This fin also has the advantage of reducing the oscillation of the buoy in running water.

In order to allow buoys with a single mooring attachment point to ride vertically in a variety of water depths, and currents, and with differing weights of chain it is necessary to provide a several mooring eyes at different distances from the buoy axis.

In the case of a tail tube buoy the tail chain should be increased in size to allow for extra wear where the chain rubs against the tube. To lessen this chafe some Services attach a wooden rubbing band around the bottom of the tube.

## Bridle

A bridle comprises two equal lengths of chain shackled to the buoy on opposite sides. The two chains are joined below the buoy by means of either a triangular or circular link, or a shackle.

Below this point there can be a swivel or a direct connection to the next section of chain.

The use of a bridle enables buoys to ride vertically without having several mooring eyes to cope with different tidal conditions or weights of moorings. It also allows a chain size some 10 % smaller, and smaller mooring eyes.

## The Riding Chain

The Riding Chain or Riser connects the bridle or tail chain to the thrash chain. It can vary in chain size (bar diameter) and length depending on the buoyancy of the aid, the depth of water and expected forces on the chain. The chain must be of sufficient strength to recover the sinker. Usually chains are delivered in standard lengths and joined together with shackles.

This section of the mooring remains suspended beneath the buoy and is not subjected to wear on the seabed. Hence this section of the mooring may utilise synthetic rope where there are special requirements for a lightweight mooring, usually in deep water situations, or a rubber cord in very shallow moorings where large amounts of wave energy must be absorbed.

## The Thrash Chain

The Thrash Chain is the part of the mooring lying on or near the seabed, connecting the riding chain to the ground chain. Due to tidal influences (rise, fall and current) and the sea and wind conditions the thrash chain continually moves on the seabed. In the case of a sandy bottom severe wear can be expected in this area of the mooring.

It is usual to renew this part of the mooring first, either with new chain or by exchanging with a length of chain from elsewhere in the mooring.

In some cases the chain size (diameter) is increased in this section to offset the wear.

## The Ground Chain

The Ground Chain lies on the seabed between the thrash chain and the sinker, and sometimes becomes embedded.

This section of the mooring may be increased in length or a larger chain size used to reduce loads transmitted to the sinker.

The ground chain, the sinker and the resilience of the other parts of the mooring keep the buoy in position.

Note: For small to moderate sized moorings the riding chain, ground chain and thrash chain may be formed by one continuous length of chain. However these terms are still used to identify the various sections of the mooring.

## Swinging Radius

The swinging radius is the radius of  the circular movement of the buoy about the position of the sinker at low water. Ice may increase the swinging radius as the increased load imposed by the ice may pull the chain into a straight line.

In tidal conditions, the excursion of the buoy will be in the direction of water flow.

## Sinkers

The sinker usually provides the attachement of the mooring to the seabed. They have the advantage that they can resist loads from all directions. Anchors may be used when the mooring load is always in one direction. An anchor can provide considerably more resistance than a sinker of the same weight, when correctly burried in the sae bed.

# MOORING DESIGN

The following section provides a methodology to calculate the dimensions of a chain mooring and the weight of sinker required. The process requires knowledge of the environmental conditions at the mooring site, details of the local wave regime and the dimensions of the buoy that will be moored. Details of the type of seabed will be needed to establish the required weight of the sinker.

Navigational requirements regarding an acceptable swinging radius for the buoy may require further refinement of the mooring design.

At sites where damage to flora and fauna on the seabed is of concern then the mooring may have to be designed so that there is no ground chain sweeping the seabed.

In areas such as the Baltic, where there is very little tidal range, tensioned moorings are often employed. These connect the buoy directly to the sinker so that the tension in the mooring chain maintains the buoy in an upright position.

Information gained from practical experience with other buoys in the proposed mooring area will prove valuable in verifying the results of the calculations.

The calculations make the assumptions that:

- The buoy axis is vertical under the most common conditions of current and wind.

- The reserve buoyancy of the fully equipped buoy is sufficient under the worst conditions of wind and tide.

- The drag due to water flow on the mooring chain is not significant in currents less than 5 knots and water depths less than 40m. Additional calculations will be required at deeper sites with fast currents.

## Mooring Types

Details are provided for the three main types of mooring.

- The Transitional Mooring.

- The Slack Mooring

- The Taut Mooring

The following text describes these three types of moorings and how the moorings loads may be calculated. The detailed calculations are presented as annex 1.

## Transitional Mooring

Wind IND

TIDE

- Transitional Mooring

The transitional mooring (see Fig.2) is the ideal mooring state where the mooring chain catenary meets the seabed tangentially exactly at the sinker when there are the maximum wind and tide loads on the buoy. The mooring loads will be transferred horizontally to the sinker, which will be working as effectively as possible.

### Transitional moorings Loads

The loads caused by wind and tide on the buoy are calculated from the following formulae:

THO = FW + FD

Where

THO	stands for the horizontal load caused by wind and tide in Newtons

Fw 	stands for the maximum wind load on the buoy in Newtons

FD	stands for the maximum tide (or current) load on the buoy in Newtons

FW = ½ρaV2WACW

Where

ρa	stands for the unit mass of air in Kg/m3

VW	stands for the maximum wind velocity in m/s

A		stands for the cross sectional area of the parts of the buoy exposed to the wind in m2

CW 	stands for the aerodynamic drag coefficient of the relevant parts of the buoy exposed to wind loads.

Typical values for CW are as follows:

Cylinder	0.3 to 0.4

Flat Plate	1.0

Lattice construction (angles)   1.7 to 2.0

Lattice construction (tubes)      0.3 to 0.4

Give a specific value for users who wonder the coefficient they should choose

FD = 1/2ρoSV2Cd

Where

ρo	stands for the unit mass of sea water in Kg/m3

S		stands for the cross sectional area of the immersed areas of the buoy in  m2

V	stands for the maximum velocity of the current or tide in m/s

Cd	stands for the hydrodynamic drag coefficient of the various immersed sections of the buoy

Typical values for Cd are

For buoys with a draught ≥ 1/5 of their diameter    0.8 to 1.2

For buoys with a draught ≤ 1/5 of their diameter   0.3 to 0.5

Give a specific value for users who wonder the coefficient they should choose

Add an example of a shape from Holland

### Chain Size for transitional mooring

The practical size of the chain can then be ascertained by trying the strength and immersed weight of various sizes of commercially available chain in the following formulae

RC ≥ 5 ( p g H  +  THO )

Where

Rc	stands for the proof load of the chain in Newtons.

p	stands for the linear immersed mass of the chain (mass minus the buoyancy of the chain) in Kg/m.

H	stands for the maximum water depth at the station in metres.  This should include wave height.

THO	stands for the horizontal load imposed by the buoy which is equal to the horizontal mooring load at the sinker in Newtons.

g	Stands for the acceleration due to gravity in m/s2

The safety factor of 5 takes account of the continual cyclic load and wave effects that the chain is subjected to by the motion of the buoy.

This calculation will provide a guide to the size of chain required and the following formulae can then be used to establish the required length of the mooring chain.

### Chain Length  for transitional moorings

L=H(H+2THOpg)

### Reserve Buoyancy for transitional mooring

- ?????

The load imposed on the buoy can then be calculated to see if the proposed buoy will have sufficient reserve buoyancy.

Rb=U−Mb+mcLρw

Where

Rb	stands for the reserve buoyancy in m3.

U	stands for the total volume of the float in m3.

Mb	stands for the mass of the buoy in Kg.

mc	stands for the linear immersed mass of the chain in Kg/m.

L	stands for the suspended length of the mooring in m.

ρw	stands for the density of water in Kg/m3

### Swinging Radius for transitional mooring

The swinging radius (see Fig.1) of the mooring can then be calculated from the formulae.

Rm=L−Hm(H+2THOpg)+THOpgcosh−1(HmpgTh0+1)

Where

Rm	stands for the maximum swinging radius in meters,

L	stands for the total length of the mooring in meters,

Hm	stands for minimum depth of the station in meters,

Th0	stands for the horizontal mooring tension at the connection with the sinker in Newtons,

p	stands for  the linear immersed mass of the chain (mass minus the buoyancy in mass of the chain) in Kg/m.

It will then be possible to see if the designed mooring meets navigational and operational requirements.

Will the swinging radius be acceptable? If the buoy is marking a navigable channel then the swinging radius may need to be reduced. This can be achieved by using a larger size of mooring chain. The effect of this can be investigated by re-calculating the mooring design with increasingly larger sizes of chain.

If further reduction of the swinging circle radius is required then the ‘Taught Mooring’ may be considered. See 3.4

The reserve buoyancy of the buoy must also be monitored as larger sizes of chain are considered.

Consideration may be given to the use of a resilient beacon, a piled bacon or a two legged mooring if precision marking is required.

## Slack Moorings

WIND

TIDE

- Transitional Mooring

In practice, this is the most common form of mooring.

Slack moorings have chain permanently on the seabed. When the loading on the buoy is at its maximum the mooring chain catenary will meet the seabed some way from the sinker. The chain on the seabed increases the security of the mooring or may be used to reduce the size of the sinker (See Fig.4)

### Ground Chain Resistance for slack mooring

The following formulae evaluates the increase in mass that the ground chain provides to the sinker.

Mgained=mCLgtanφK

Where

Mgained	stands for the effective increase of sinker mass provided by the ground chain.

mc	tands for the linear immersed mass of the chain (mass minus the buoyancy of the chain) in Kg/m.

K	stands for a safety coefficient (generally taken equal to 1,5),

Φ	stands for the internal friction angle of the sea bed (this depends on the type of soil at the station), 45° ( 0,7855 in radian) is a practical approximation that can be used in most cases. Chalk and some gravel bottoms provide  a lower friction angle.

Lg	stands for the length of the ground mooring that lies on the seabed in m.

This mooring arrangement is often used in open waters where the consequently large swinging radius of the buoy is acceptable. The length of ground chain provides energy absorbing capabilities in extreme weather conditions.

Another consideration may be that the weight of the sinker may be reduced by using a long ground chain if the required sinker weight for the transitional mooring is greater than the servicing vessel’s lifting capacity.

## Taut Mooring

WIND

TIDE

- Taut Mooring

If the swinging radius derived from the mooring design is not acceptable, for example the edges of dredged channels often require very precise marking, then a Taut Mooring may have to be considered.

Similar design constraints will apply if the local seabed is an important marine ecosystem. It may then be necessary to design a taut mooring such that most of the mooring chain is suspended from the buoy. Any damage that the mooring chain causes to the seabed will then be minimised.

In areas with rough rocky seabeds there is a chance of the mooring chain catching on outcrops of rock. If the mooring is designed as a transitional or a slack mooring then it is possible that part of the chain on the seabed will catch on a rock and effectively shorten the mooring. The loads on the chain and the buoy will then be increased, possibly to the point of failure. These problems can be minimised by designing the mooring as a taut mooring.

The taut mooring (see Fig.4) has the mooring chain meeting the sinker at an angle when the maximum wind and tide loads are acting on the buoy. There will then be a vertical component of the mooring load applied to the sinker. This will require a larger sinker to maintain the buoy on station.

The chain size and length and sinker weight may now be calculated to meet the swinging radius requirements for the particular buoy station. A check must again be made to ensure that the buoy has sufficient buoyancy to support this mooring in the worst environmental conditions.

The following formulae are provided for this mooring calculation;

### Taut Mooring Loads

For the vertical mooring load on the sinker   TVO=pH4THO2+(pL)2−(pH)22(pL)2−(pH)2−pL2

Where

Tv0	stands for the vertical mooring tension at the connection with the sinker in Newtons,

Th0	stands for the horizontal mooring tension at the connection with the sinker in Newtons (equal to the horizontal mooring tension at the connection with the buoy as the horizontal tension is integrally transmitted along the mooring),

p	stands for the linear immersed mass of the chain  (mass minus the buoyancy of the chain) in Kg/m.  check the formula

H	stands for the maximum depth of water in m, including wave height

L	tands for the length of the mooring in m.

### Chain Size for taut moorings

For the chain strength Rr≥5(pH+TVO2+THO2)

Where

Rc	stands for the proof load of the chain in Newtons.

p	stands for the linear immersed mass of the chain (mass minus the buoyancy in mass of the chain) in Kg/m.

H	tands for the maximum water depth at the station in metres. Including wave height

THO	stands for the horizontal load imposed by the buoy which is equal to the horizontal mooring load at the sinker in Neutons.

Tvo	stands for the vertical load imposed by the weight (in water) of the chain plus the vertical load on the sinker in Neutons

g	stands for the acceleration due to gravity in m/s2

The safety factor of 5 takes account of the continual cyclic load that the chain is subjected to.

### Reserve Buoyancy for taut moorings

For the reserve buoyancy Rb=U−Mb+mcL+TVOgρw

Where

Rb	stands for the reserve buoyancy in m³,

U	stands for the total volume of the float in m³,

Mb	stands for the mass of the buoy in Kg,

mc 	stands for the linear immersed mass of the chain (mass minus the buoyancy of the chain) in Kg/m.

L	stands for the suspended length of the mooring in m,

Tv0	stands for the vertical mooring tension at the connection with the sinker in Newtons,

g	stands for the acceleration due to gravity in m/s²,

ρw	stands for the density of water Kg/m3 (taken equal to 1024 Kg/m3 for salt water),

### Swinging Radius for taut mooring

For the taut mooring swinging radius:

[add a condition to see if the mooring is still taut with the low level]

Rm=ThOpcosh−1[pHmThO+ThO2+TVO2Th0]−ThOpsinh−1(TVOThO)

Where

Rm	stands for the swinging radius in meters,

L	stands for the length of the mooring in meters, [does not appear in formula]

Hm	stands for minimum depth of the station in meters,

Th0	stands for the horizontal mooring tension at the connection with the sinker buoy in Newtons,

T'v0	stands for the vertical mooring tension at the connection with the sinkerbuoy in Newtons, calculated with the formulation (5) at the minimum depth,

Mass of the chain

p	stands for  the immersed linear mass of the chain  (mass minus the buoyancy of the chain) in N/m.

## Sinker Weight

A very simple way to design the sinker weight consist in taking only into account the effect of the friction with the sea bottom (the ‘burying’ and the ‘rock stopping’ effects are not taken into account ).

Under these hypothesis, the minimum weight of the sinker is provided by the formula

M≥KTHOδg(δ−ρW)tanφ+TVOg

Where

M	stands for the mass of the sinker in Kg.

K	stands for a safety coefficient (generally taken equal to 1,5)

Th0	stands for the horizontal mooring tension at the connection with the sinker in Newtons,

Tv0	stands for the vertical mooring tension at the connection with the sinker in Newtons,

δ	stands for the mean density of the sinker in Kg/m3 (generally taken equal to 2400 Kg/m3 for a reinforced concrete sinker and 7800kg/m3+ for cast iron),

g	stands for the acceleration due to gravity in m/s²,

ρw	stands for the density of water Kg/m3 (taken equal to 1024 Kg/m3 for salt water),

Φ	stands for the internal friction angle of the sea bed  (it depends  on the type of soil at the station), 45° ( 0,7855 in radian) is a practical approximation that can be used in most cases.  Chalk and some gravel bottoms may provide  a lower friction angle. Relevant information can be found in civil engineering textbooks.

### Sinker Weight for Transitional and Slack Moorings

In the case of a Transitional or a Slack mooring, the vertical tension of the mooring at the connection with the sinker is equal to zero, and consequently, the relationship above can be simplified:

M≥KTOδg(δ−ρW)tanφ

## Buried Sinkers

It must be noted that in service sinkers often become buried in the sand or mud on the seabed. In order to recover the sinker the servicing vessel will then have to haul up the mooring chain and break the sinker out of the seabed.

The following empirical formulae provides an estimate of the maximum load that may be encountered:

HC≥2Mδ−ρWδ+HmC

Where

Hc	stands for the hauling capacity of the servicing boat (likely a buoy tender) in Kg,

M	stands for the mass of the sinker in Kg,

δ	stands for the mean density of the sinker in Kg/m3 (generally taken  as 2400 Kg/m3 for a reinforced concrete sinker and 7800 Kg/m3 for cast iron),

ρw	stands for the density of water Kg/m3 (taken equal to 1024 Kg/m3 for salt water),

H	stands for the depth of water when the mooring is lifted (generally, it is the maximum depth of the site that is taken into account) in m.

mc	stands for the immersed linear mass of the chain  (mass  minus the buoyancy of the chain) in Kg/m.

See appendix2: Example of mooring calculation for French service’s standard buoy

# CHAIN MOORING COMPONENTS

The components used to assemble a mooring system are:

- Chain

- Shackles

- Swivels

- Sinkers or Anchors

It is important that all components (including sinker and anchor eyes). are manufactured from the same quality material to minimize electrolytic corrosion and wear.

## Chain

Chain size is defined by the bar diameter of the chain links. However various lengths of chain links have been established depending on local manufacturing processes, national standards and operational requirements.

- 4D chain

The proportions of the chain link are described as the ratio between the bar diameter and the inside length of the chain link.

Each Service should define a dimensional ratio in consultation with the supplier and manufacturer that will allow the use of their standard shackles and other chain connections.

Examples of ratios currently in use are:

France 3,4 and 5

UK 4

Netherlands 5

Germany 9

The shorter link chain requires more links per length. It is heavier and therefore can be more expensive, but it is easier to handle with a winch. However the links may not be of sufficient size to accept a joining shackle and enlarged end links may be required on each length of chain.

The longer link chain may not require an enlarged end link in order to insert a joining shackle. Long lengths of chain can then be cut to the required length for a particular mooring. It is easier to handle with a hook and permits one size of hook on the hoisting equipment to be used for several sizes of chain. There is also less risk of ‘knotting’.

Consideration must be given to the chain handling method, and particularly the dimensions of the servicing vessels’ chain handling capstan if different link lengths are to be used.

### Material Used In The Manufacture Of Chains

Chain is manufactured from grades of steel that are suitable for forging into the shape of the link and also capable of being reliably welded. A variety of steels fall into these categories and provide different combinations of strength, hardness, wear resistance and cost. A particular steel may be slow to wear but not be resistant to corrosion or vice versa. Generally a compromise must be found in the quality of material that will minimise both wear and corrosion at an acceptable cost.

Steels with a high carbon content (0.2%) and high manganese content (1.5%) have proved to be very effective for buoy mooring chain. It should be noted that Lloyd’s U grades and German DIN grades are based on the chain strength and do not specify a particular grade of steel. The wearing performance of the chain will therefore not be defined. It is recommended that national authorities should specify the steel used in the manufacture of their chain.

This is an example of the steel used by the Netherlands Authority. This is an acceptable high performance chain mooring steel.  The chain is heat treated by quenching and tempering.

| Material | Min % | Max % |
| --- | --- | --- |
| Aluminium | 0.015 | 0.025 |
| Carbon | 0.25 | 0.26 |
| Chromium | 0.20 | 0.30 |
| Copper | - | - |
| Manganese | 1.40 | 1.60 |
| Molybdenum | 0.10 | 1.20 |
| Nickel | 0.20 | 0.30 |
| Phosphorus | - | 0.02 |
| Silicon | 0.20 | 0.35 |
| Sulphur | - | 0.02 |
| Vanadium | 0.08 | 0.12 |

- ????

### Heat Treatment

Chains may be heat treated to reduce stress concentrations around the area of the welds and to generally improve the strength of the chain.

Some Authorities have found non-heat treated chain to be acceptable in use and hence more cost effective.

### Chain finish

Some authorities specify a protective coating to limit corrosion while the chain is stored awaiting deployment.

### Specifications And Standardisation

A specification should include the material from which the chain is to be made, the heat treatment, the finished dimensions of the chain and the mechanical properties of the material .  It should also include means of identification of individual chain lengths.

Tests must be included which will verify the requirements of the specification and will be practical for the purchaser or his agent to carry out.

## Shackles

The shackle is the most widely used device for attaching the mooring to the buoy, and joining the other components of the mooring together.

The strength of the shackle must equal that of the chain which it is joining. Thus, the diameter of the shackle bow will be greater than the chain size.

Confusingly the size of a shackle may refer to the size of the chain which it fits or it may refer to the diameter of the shackle pin. Local standards must be consulted to ensure the correct definition of sizes.

The types of shackles in use are:

- Forelock shackles

- Clenching shackles

- Bolt shackles

### Forelock Shackles

- Forelock shackle with oval pin

The forelock shackle is a very reliable type and it is easy to use. It can be manufactured in size and shape to suit the components that it is connecting and it has a forelock to secure the pin. This has the advantage that in use the forelock is the least stressed component.

The pin can be round or oval in cross section. The round pin is cheaper to manufacture but it has the disadvantage that in service it can rotate, thus wearing the forelock and eyes of the shackle. The more expensive oval pin overcomes this disadvantage.

### Clenching Shackles (heat and beat - American)

Hammer forged

- Clenching shackle with oval pin

The clenching shackle is the most reliable method of joining lengths of chain. The pin end is enlarged by hammering (forging) to prevent the pin withdrawing whilst in use.

The disadvantages are that considerable heat is required to bring the pin up to forging temperature, and the pin is not reusable. An alternative to forging is to weld the pin end to the shackle eye but in this case neither the pin nor the shackle is reusable. It is essential to use the correct grade welding rod in order to ensure that electrolytic corrosion does not take place. It is very difficult to obtain a good quality of weld on the deck of a boat.

### Bolt Shackles

- ?????

The bolt shackle has a round pin secured by one or two nuts. If only one nut is used then a cotter pin or a split pin must secure the nut.

The round pin can rotate in the eyes of the shackle resulting in rapid wear.

Authorities who choose to use these shackles may find it prudent to weld the nuts and pins in place.

The protruding parts of shackle pins may be subject to severe abrasion on the sea bed and may be worn away with the subsequent loss of the shackle pin unless it is otherwise secured.

### Screw Pin Shackles

- ???????

These should not be used in permanent moorings, as any rotation of the shackle pin will result in failure of the shackle.

### Kenter Shackles

- ???????

These are commonly used in ship moorings but are not usually used for permanent moorings where long term corrosion and wear will result in the shackle literally falling apart.

### Quick release links

Many types are available One of which is illustrated below. These may be used close to the buoy where the mooring is always in tension if there is a requirement to regularly or quickly disconnect the mooring.

- G hook and connecting link

## Swivels

- ???????

A swivel permits two lengths of chain to be joined without transmitting a twisting motion (torque) from one length to another.

Due to the rotation of a buoy around its axis, a twisting action is applied to the chain. This may result in the chain knotting, ultimately affecting the riding quality of the buoy and the effectiveness of the mooring. In such a case one or more swivels can be inserted into the mooring.  Usually a swivel is placed between the Tail Chain or Bridle and the Riding Chain.

Experience shows that the quality of swivels requires careful monitoring to ensure that their strength and working life matches the other components of the mooring.  They should be made of the same material as the chain.

Swivels may seize due to corrosion, wear and/or marine growth.  If severe marine growth occurs, regular inspection and cleaning is recommended.

## Sinkers

**

[add a paragraph about anchors in general and especially river moorings (Sipke Hoekstra and the German authority has valuable information)]**

Experience has shown that sinkers are adequate to keep buoys in their assigned positions. They do not provide the resistance or holding power of anchors of the same weight but have the great advantage that they will provide the same resistance irrespective of the direction that the mooring load is applied.

Sinkers may be made of concrete, cast iron, rock or bundles of used chain. Whilst most Authorities use sinkers, in some cases anchors are necessary to resist high mooring loads. for example LANBYS (LNBs) on exposed stations use a combination of sinkers and anchors, and floating aids in rivers may use anchors  where the current direction is constant.

The effective weight will be equal to the weight of the sinker in air minus the weight of the water displacement.

### Concrete sinkers

- ???????

These are of moderate cost and widely used. They are manufactured by casting concrete into a suitably shaped mould with a metal eye for attachment of the mooring chain cast in place. Concrete sinkers can be made with local labour using local materials. However, they are considerably larger than cast iron sinkers of the same submerged weight and thus take up valuable storage space in the depot or on the buoy tender’s deck. Care must be taken in all aspects of their manufacture so that they do not disintegrate or allow the mooring eye to pull out. The quality of the concrete must be carefully  monitored to ensure it is of the specified density.

The density is often improved by including steel reinforcement or scrap chain in the concrete.

Hematite may be used as an aggregate to increase the density of the concrete.

See annex 3 for manufacturing details of sinkers used by American and French services.

### Rock sinkers

An alternative sinker can be made by fixing a mooring eye in a piece of rock, or large stone. This may be effective if there is a local supply of dense stone in suitable sizes. However, it is very difficult to assess the condition of an individual rock and it can break in service.

### Cast iron sinkers

- Trinity House 3 tonne cast iron sinker

The most expensive but the most durable and effective sinkers are made of cast iron.  Their density compared with concrete is approximately in the ratio of 3 : 1.

Cast iron sinkers are robust and the repair or replacement of the mooring eye is possible.

If a mooring is prone to sanding in, a cast iron sinker will be easier to extricate than a concrete sinker because its physical size is smaller.  However if lost its replacement will be more expensive than a concrete sinker.

There are many designs and shapes of cast iron sinkers. It may be cost effective to use scrap cast iron if suitably dimensioned pieces are available.

# MOORING WEAR AND CORROSION

The two main factors which influence the service life of a mooring system are wear and corrosion These two factors are interactive since the products of corrosion rapidly accelerate wear.

The performance of the mooring chain depends upon the choice of the correct steel specification and must take into account the acceptable amount of wear within the working life of the mooring.

Rust, electrolytic action and pollution may reduce the normal life of a mooring line.

Rust, which is due to oxidation, occurs both ashore and at sea.

Electrolytic Action results from a potential difference between components of a mooring. This is caused by using dissimilar metals in the same mooring immersed in a common electrolyte, in this case water.

It shows as pitting near and in the welds of chain links and in the chain each side of connecting shackles, over a length, of about 1-2 metres.

Electrolytic Action can be significantly reduced by making the buoy body and all the metallic components of the mooring from the same or very similar grades of steel.

Polluted waters also have an influence on the extent of pitting corrosion. It is mainly observed in the riding part of the mooring chain and has occasionally been observed in the skirts of steel buoys.

## Wear

The extent to which wear occurs mainly depends on the quality of the materials from which the mooring is made. Wear is also affected by local environmental conditions, in particular:

- Type of seabed

- Amount of sand transport in the water

- Tidal conditions

- Depth of water

- Sheltered or open waters

- Weather conditions

The mooring cannot change these conditions but can only react to them.

A reduction in bar diameter will be observed at the contact point of the links.

This may be due to wear resulting from the movement between links or to local deformation of the metal caused by impact loads due to the chain ‘snatching’.

Snatching can also cause elongation of the links.

In an area where the seabed is sandy and in particular during bad weather or in strong tidal currents, a heavy sand transport can be expected. This will cause severe abrasion of the mooring components.

The thrash section of the mooring has the most pronounced wear as it is continuously moving. Even in waters with small tidal range, where only a small part of the mooring chain moves over the sea bed, very significant wear may be found.

It is common practice to establish a suitable chain size for the thrash area of the mooring and then to use this size of chain for the entire mooring. For deep water moorings, a smaller chain size may be used for the riding chain due to limitations of the reserve buoyancy of the buoy.

Larger size of chain may be used for the ground mooring chain where the increased weight would be an advantage.

Chain inspections are carried out at intervals that may range from 6 to 24 months. This will depend on the safety factors built into the mooring and the environmental conditions at the AtoN location. Experience may show that on some stations a more frequent check is necessary.

# SERVICE LIFE

The service life of chain normally varies from 1 to 5 years, but in sheltered, muddy conditions it may be as long as 20 years.

Chains are scrapped or down graded for use in other locations when the diameter in any point of a link is reduced to a predetermined percentage  of its original size. 60% is used by some authorities for buoys in inland waterways, whereas 85% may be used in exposed situations.  It may be practical for  a worn part of the chain to be  moved to another part of the mooring that is experiencing little wear.

A detailed inspection regime including elements of predictive maintenance may allow greater or lesser wear down allowances. This regime must take account of wear rate and predicted remaining cross section.

Accurate record keeping is a vital part of this inspection process.

Detailed dimensional and material specifications must be maintained to ensure reliable, predictable wear  performance of all mooring components.

These specifications are of particular importance regarding the planning of future maintenance. It will only be possible to utilise experience gained with the working life and necessary maintenance to a group of moorings to plan the future repair and replacement of similar moorings if the same quality of chains, shackles and other components can be purchased in the future.

There are benefits in terms of stockholding and cost reduction to be gained by standardisation of the various components of mooring systems. The quality of the components should be related to the required service life.

It is important to note that the normally quoted regulations for the manufacturers of chain for merchant ships, as laid down by classification societies, only specify the breaking and impact strength of the chain and give no guide to wear and corrosion resistance. A ship normally spends little time at anchor, and so the chain does not suffer significant wear and corrosion. Often the chain will last the life of the ship or only be replaced if it has been overstrained.

See IALA Guideline No. 1040 on the maintenance of buoys and small Aids to Navigation

structures.

# ROPE MOORINGS

## Rope Buoy Moorings

The primary advantage of rope moorings is their light weight and elasticity when compared with normal chain moorings. Modern ropes can easily match the strength of steel chain and experience has shown that a similar or better working life than chain can be achieved if chafe is carefully avoided.

The conventional chain mooring utilises energy absorption of the chain catenary to absorb much of the wind and wave energy acting on the buoy and prevent this being transferred to the sinker or anchor. The elasticity of the rope performs a similar function and choosing a suitable combination of fibre type and rope construction can optimize this energy absorption.

Chafe and cutting are the greatest dangers to a rope mooring. It is easily demonstrated that a sharp knife will rapidly cut through a piece of rope and any sharp edges presented by rocks, sea shells or the servicing ships own capstan can rapidly cause permanent damage to the surface of the rope. Allowing the rope to slip on the drum of a capstan or pulling it through an unsuitable fairlead may not only result in abrasive damage but also in localised heating such that the surface fibres of the rope may melt, resulting in significant weakening.

Rope may suffer abrasive damage from sand particles in suspension in the water resulting in reduction in breaking load.

The US Coastguard have used rope moorings in fast flowing rivers. Here the particular advantage is the light weight of the mooring. This reduces the tendency of the buoy to be submerged as the current drag on the buoy lifts the mooring off the river bed.

In areas of severe marine growth ropes may attract considerable weed and shell fouling. This can make the resistance of the rope mooring unacceptably high in fast flowing streams.

## Mooring Design

The mooring must be designed so that the rope is never in contact with the buoy body or tail tube and is never in contact with the sea bed (although this may not be a problem in areas with soft, muddy bottoms).

These criteria can be achieved in a normal buoy mooring by utilising a ground chain that absorbs the wear on the sea bed to whic h a rope “riser” is attached. Floats may be incorporated to keep the rope off the seabed. Floats may also be used in this way in environementally sensitive areas. The rope “riser” component of the mooring is of such a length that even at the lowest tides the rope is never chaffing on the sea bed.

The rope may be attached directly to the buoy if the mooring eye is in a suitable position such that the rope will always be clear of the buoy.

In other cases a short length of chain (or bridle in the case of two mooring eyes) may be used to absorb any chafe.

Cutting by trawl wires may also be a hazard in some areas where commercial fishing takes place. It may be possible to utilise chain in the part of the mooring that may be subject to abrasion from trawl wires.

## Rope size

The decision on the size of rope to be used will depend on the load imparted by the buoy, due to wind/wave action and water velocity and the strength necessary to lift the sinker (or anchor).

A rope with strength equal to twice the lifting capacity of the servicing vessel may be a safe guide for the selection of a rope to account for the case when the sinker is buried.

Detailed information will be required from the rope manufacturer regarding the energy absorbing properties and practical working load for the chosen rope.

The method to be used to handle the rope may also influence the size of rope chosen.

The user must be aware of the inherant danger resulting from the high levels of energy stored within an ellastic rope when under load.

## Rope Construction

A great variety of ropes are now available with many different fibre types as well as types of construction. The use of natural fibre ropes for load carrying applications has almost disappeared. Natural fibre ropes have low strength, will have a short life when compared with synthetic fibres and are no longer cheaper than their synthetic counterparts.

Traditional 3-strand rope construction has also largely been superseded by plaited or braided constructions where high strength and long life are the primary requirements.

- 3-strand Constructions

This is the oldest and simplest rope construction, consisting of 3 twisted strands laid together. 3 strand ropes are hard wearing and easily spliced.

- Multiplait Constructions

Multiplait rope consists of 8 strands plaited in pairs, each pair passing over one pair and under the next. It is easily spliced and the twisted strands offer good resistance to abrasion.

- Braided constructions

This illustration shows a rope with a braided core encased in a braided jacket, 3 strand cores are also used. Variations in jacket and core construction allow ropes to be designed for specific working situations. The highest strengths are possible in this construction but splicing is complex.

Each internal strand of the rope may be constructed in several different ways. Manufacturers informations should be studied in detail.

## Types Of Fibre

Modern rope constructions utilise the following fibre groups:

### Nylon

This provides high strength, elastic rope with good shock absorbing qualities.

Some ultimate strength is lost due to water absorption if the rope is permanently immersed in water.

### Polyester

This is widely used to construct high strength, low stretch ropes with good wear resistance and long life.

### Polypropylene

This has been used for cheap, general purpose rope which floats, however recent developments in fibre manufacture and rope construction have resulted in moderate performance ropes, which are considerably cheaper than nylon or polyester.

### Advanced Fibres

These include Aramid fibres (trade name Kevlar) and high modulus polyethylene (HMPE such as

Spectra, Dyneema and Vectran) which have very high strengths associated with very low stretch. HMPE ropes are also buoyant. However, these are very expensive products being approximately three times the cost of nylon or polyester.

Rope identification can be difficult as different manufacturers may use trade names for fibre type rather than generic names.

Some HMPE rope constructions are very easy to splice.

- Behaviour of diferent types of fiber under traction

## Mixed Construction

Large ropes, such as those used for ship mooring may be constructed from a mix of fibres to achieve particular performance parameters.

Annex 4 illustrates details of a standard mooring system developped by the Canadian Coastguards to moor small (less than half meter diameter) buoys in depths up to 30 meters.

## Rope Terminations

### Thimbles

The use of fibre rope rather than wire rope for towing and mooring ships and oil rigs has lead to the development of thimbles which allow ropes carrying very high loads to be shackled to chain or mooring eyes without damaging the rope fibres.

These thimbles completely enclose the rope leaving no unprotected rope surface to chafe against the joining shackle. They may be fabricated from steel tube, made in cast iron, or moulded from high strength plastic. Movement of the rope within the thimble can be further prevented by filling the thimble with a flexible resin system (usually polyurethane), however opinions differ as to the need for this process.

### Splices

The modern rope constructions, i.e. braided and plaited constructions, both allow high strength splices to be made when the rope has been installed around the thimble. It should be noted that detailed splicing information must be obtained from the rope manufacturer and that these instructions have to be followed precisely in order to retain the majority of the rope strength at the splice. Special tools will be needed for splicing braided rope and training of those making the splices in any modern rope construction will be necessary.

The mooring calculation must take into account the reduction in strength resulting from the splices, usually on the order of 10% for correctly made splices.

## Hybrid Rope/Chain Moorings

One of the most successful applications of rope has been for the “riser” ( see’ Riding Chain’ Fig.1) component of deep-water buoy moorings. The lighter weight of the rope component will allow a service standard buoy to be used at stations where the weight of an all chain mooring would sink the standard buoy or the lighter weight of the rope mooring may allow a smaller buoy to be used when compared with the size of buoy that would be required to support the chain mooring (providing daymark size and focal plane height are adequate).

The design of the buoy being used must be carefully examined to ensure that the riding performance of the buoy is adequate if the rope mooring is used. Some buoy designs rely on chain weight to achieve positive stability.

This type of mooring does not work well for shallow water applications.

## Tension Leg Moorings

Rope moorings are particularly suitable for tensioned mooring configurations such as spar buoys and resilient beacons, where the mooring goes directly from the buoy to the sinker and tension in the mooring line holds the buoy upright. The rope being in tension is not in danger of chafing on the seabed or on the buoy. This configuration has the advantage of maintaining the buoy precisely on station (i.e. there is no “swinging circle” as there is with a conventional mooring) but is only practical in areas with little tidal range or current. However, the mooring sinker or anchor will need to be considerably larger than that associated with a conventional catenary mooring.

## Handling Rope Moorings

### Deployment

When compared with chain, rope is light and easy to manually handle. The components for quite large moorings can be moved about onshore or on deck by hand. Moorings can be deployed by flaking (faking) the rope on deck (or in a flakingbox, large version of line throwing gun rope box). The buoy is placed in the water, the sinker and ground chain simply pushed overboard (or released by cutting lashings) and the rope will follow into the water.

The deck must be clean  and free from sand and the deck edge must be sufficiently smooth so as not to damage the rope.

### Recovery

If the mooring is to be lifted for removal or inspection then two areas need special attention:

- Any fairlead that the rope runs over must be of sufficient diameter for the rope used, be of the roller type and present no sharp edges;

- The winch or capstan must be designed for handling rope and must not allow the rope to slip on the winch drum when under load.

Conventional capstans as used for tensioning mooring warps etc., may be capable of recovering a rope mooring however their tendency to allow the rope to slip on the capstan drum will result in considerable heat being generated at the rope/drum interface which will result in serious damage to the rope. Successful techniques have been developed using large spooling winches where the rope is wound onto a large rotating drum. This technique is limited by the length of rope and hence the number of moorings that can be carried on the drum at any one time.

The preferred method, where a large number of rope moorings are to be handled, is to use a specialised rope hauling winch. These can be installed at the vessel’s deck edge so that the rope can lead directly to the winch without a fairlead being required. The winch consists of an arrangement of large rubber wheels, which grip the rope without causing damage to the surface fibres. The rope usually only passes over a segment of hauling wheel rather than being wrapped around a drum and can thus be placed in, or removed from, the hauling winch as may be necessary. This type of winch placed on the deck edge also has the advantage that there is no rope under load passing across the vessel’s deck, which may present a serious hazard, should the rope break.

An alternative concept is to incorporate handling loops or shackles   in the rope allowing the mooring to be hoisted in sections by the deck crane.

The deep water mooring design used by the French authority ensures that the ground chain is sufficiently long so that as the rope part of the mooring is retrieved the tension in the rope will only be the weight of the ground chain being lifted. The weight of the sinker will not be felt until all the rope has been recovered and the vessel is lifting the chain part of the mooring.

## Safety

It must be noted that the energy stored in the more elastic types of rope when under tension may be considerable and suitable precautions must be taken to ensure that no personnel will be in any area that may be swept by the end of a broken rope.

# ELASTIC MOORINGS

## Introduction

As the ‘on station’ life of navigation buoys has increased the wear of the mooring chain has become the controlling factor in planning service intervals. The continuous movement of the chain links and abrasive particles suspended in the water will cause the chain to wear. This wear can be reduced if the mooring line can be kept under constant tension. A solution to this problem is the fully elastic mooring line. This can be designed to always be under tension to minimise wear.

The elastic mooring can be compared to a rope mooring (see section *** )  with the difference that the upper part of the mooring line consists of a length of rubber cord which will absorb the energy caused by the motion of the buoy and compensate for the difference in water levels, tide, waves etc.

## Elastic Mooring Construction

The elastic cord used is made of solid natural rubber. This is capable of considerable elongation and has high tear strength. The rubber cord is installed in parallel with a rope safety line and is connected to the rope by a simple knot.  The different rope lengths are connected by special H-shackles.

## Elastic Mooring Design

The dimensions of an elastic mooring are determinated by the local sea conditions, the water depth and the size/shape of the buoy. For chain moorings a certain length, chain size and quality of steel are needed to keep the buoy on station for a certain time. For an elastic mooring the variables to consider are the correct length of rubber cord and the diameter and hardness of rubber to provide the necessary elongation to compensate for different water levels and to absorb the buoy’s energy.

- ??????

## Handling

An elastic mooring can be deployed in a similar way to a rope mooring. For recovery the rope sections are lifted, length by length by the crane. A special stopper is required on the deck edge to safely hold the rope during the recovery process.

### Advantages

An elastic mooring is light in weight, easy to assemble and flame cutting is not required to cut the mooring to length.

As an elastic mooring is always under tension and so does not damage the seabed.

The dimensions (diameter) of an elastic mooring is much smaller than a comparable chain mooring and hence the marine growth is much less. Thus the elastic mooring has less resistance in water than chain.

A particular advantage is that the elastic mooring can be used in shallow water and breaking waves because the rubber itself absorbs the energy in the system. A chain mooring needs a certain length and weight of chain and also a certain minimum water depth to function properly. Hence a chain mooring is not a good solution in shallow water and breaking waves.

An elastic mooring is very light in weight and for this reasons it has almost no influence on the reserve buoyancy of the buoy.

As the mooring line is always under tension, the buoy has a very high positional accuracy.

The very smooth absorbsion of energy by the rubber cord results in the forces in the mooring line being approximately half of those in a chain mooring.

As the mooring line is always under tension, there is little wear and therefore almost no maintenance. For this reason the lifetime of an elastic mooring is approximately double that of chain.

# RIVER MOORINGS

The following examples illustrate special moorings that have been developed for use in fast flowing rivers with a current flow in one direction only.

## GERMANY

### Rock mooring of buoys on the River Rhine

In the area of St. Goar the waterway is 120m wide and winding. The current is about 7kn and ground is mainly rock. This section of the river is used by ships and pushing units up to a length of 140m and a width of 15m. About 70.000 ships carry 67 Million ton per year. For safety reasons only one-way traffic is allowed and a VTS-Centre is installed to control traffic. In this area about 1/3 of the buoys are moored to mooring rings set in the rock bed of the river. Sinkers would be too large and could be a danger for the passing ships.

Data of the waterway

- Current of 3,6m/s (7kn)

- Minimum width 120m

- Minimum depth (not more than 20 days per year ) 1,9m

- Average depth 3,4m

- Seasonal variation of depth 1,39m to 9,29m

Data of mooring system

- Depth of mooring fixing 60cm

- Mooring ring bar diameter 50mm

- Ground chain length 12m, bar diameter18-20mm

- Riding chain length 15m, bar diameter 10mm

- If the mooring ring is lost then temporary chain length (used until a new mooring eye is installed) 40m, bar diameter 20-22mm (with a weight of 600-700kg)

Data of the buoy

- Lightweight Steel buoy with 1m diameter

- Volume 400 litre

- Filled with polystyrene, in case of leakage 40litre of water could ingress

- Weight 62 kg + tail tube 13 kg + counterbalance 8 kg = 73 kg

## HOLLAND

### Mooring with anchors on the Rhine

Instead of a sinker a 125kg anchor is used .

The chain length is 25 m including the bridle of 4 m.

The diameter of the chain is 22 mm.

The water depth varies from 3 m to 12 m.

The current is usually around 4 kn, but may increase to 6 kn.

The second line on the buoy is called the “neuringline”. The bottom part is chain of 10 mm and the upper part is rope. This neuringline is used by a small vessel to tow the buoy in to a new position, required by changes in water levels. The small vessel can easily lift the anchor in this way and move the buoy and mooring to a new position. Buoy lifting is not necessary and the small vessel would not be able to lift a sinker that would have the same holding power as the anchor.

The buoy used is made of steel, has a diameter of 2,00 m, a weight of 2500 kg and the draught is 1,90 m.

- CHAIN MOORING DESIGN

- example of a mooring design

Chosen situation

This numerical application has been carried out for a Bouée Nouvelle generation A8L. The aerodynamic and hydrodynamic drags have been calculated as per section 3.2.1. ‘Mooring Loads’.

Environmental conditions for the example

Buoy: New Generation Buoy (BNG) A8L

Depth: 20 m

Tide: 6,9 m

Maximum wave height: 5 m

Maximum wind speed:180 km/h

Maximum current speed: 2 knots

Chain diameter: 35 mm

Chain type: 3D

sinker density: 2100 Kg/m³

Steel density: 7850 Kg/m³

Steel tensile strength: 500 N/mm²

Air density: 1,29 Kg/m³

Water density: 1024 Kg/m³

Internal Friction angle of the seabed: 45°

A8L new generation buoy parameters

Tension on the sinker--- See 3.3.1. Taut mooring Loads

The figure bellow shows the vertical tension on the sinker depending on the length of chain.

Vertical tension on the sinker

Maximum « depth »

mooring

tangential

We can note that the vertical tension is equal to zero for slack and transitional moorings

If the mooring is shorter, the vertical tension on the sinker will be increasing.

If the length of the mooring is equal to the depth, the tension on the sinker will be very high, in reality the buoy will be submerged before this can occur.

Tension on the buoy	See 3.3.3. Reserve Buoyancy formulae

mooring tangential

In the case of a tangential mooring (slack or transitional) the tension on the buoy is only due to the weight of supported chain, whereas if the length of chain is less than the recommended length for the transitional mooring, the gain in chain weight is not enough to compensate the extra effort due to the tension on the sinker.

Tension on the mooring line----see 3.3.2.  Chain Size

The graph above shows that the "new" chain strength may not be a problem for the mooring as long as the chain length is not too close to the "depth". It follows that if you have a taut mooring, you may not have to choose a higher strength chain. However, the taut mooring can have an impact on chain wear. As the chain is continuously in tension, the rate of wear will be faster with this type of mooring.

Recommended mass on the sinker------See 3.4.  Sinker

The graph above illustrates that taut moorings require heavier sinkers.

Reserve buoyancy of the buoy

Similarly, reducing the length of chain cause a reduction of the reserve buoyancy that has to be taken account at the design stage. In some cases, the choice of a larger buoy may be necessary.

- concrete sinkers characteristics

Sinker with a square base (easier to produce, but not recommended for fast currents sites)

| M | Mass of sinker | 500 kg | 1000 kg | 2000 kg | 3000kg | 4000kg | 6000kg |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | Side of the large base | 100 cm | 125 cm | 155 cm | 175 cm | 190 cm | 215 cm |
| B | Side of the narrow base (=2A/3) | 66.7 cm | 73.3 cm | 86.7 cm | 106.7 cm | 133.3 cm | 153.3 cm |
| C | Height of sinker (=A/3) | 33.3 cm | 41.7 cm | 51.7 cm | 58.3 cm | 63.3 cm | 71.7 cm |
| D | Base of the suction hole (=22A/30) | 73.3 cm | 91.7 cm | 113.7 cm | 128.3 cm | 139.3 cm | 157.7 cm |
| E | Summit of the suction hole (=21A/30) | 70 cm | 87.5 cm | 108.5 cm | 122.5 cm | 133 cm | 150.5 cm |
| F | Height of the suction hole (=A/30) | 3.3 cm | 4.2 cm | 5.2 cm | 5.8 cm | 6.3 cm | 7.2 cm |
| G | Length of reentrant for shackle (=9ф) | 36 cm | 45 cm | 54 cm | 54 cm | 54 cm | 54 cm |
| H | Width of reentrant for shackle (=3 ф) | 12 cm | 15 cm | 18 cm | 18 cm | 18 cm | 18 cm |
| I | Depth of of reentrant for shackle (=2 ф) | 8 cm | 10 cm | 12 cm | 12 cm | 12 cm | 12 cm |
| J | Height of reentrant for storage (=2.5 ф) | 10 cm | 12.5 cm | 15 cm | 15 cm | 15 cm | 15 cm |
| K | Length of reentrant for storage (=8 ф) | 32 cm | 40 cm | 48 cm | 48 cm | 48 cm | 48 cm |
| L | Width of reentrant for storage (=2 ф) | 8 cm | 10 cm | 12 cm | 12 cm | 12 cm | 12 cm |
| ф | Diameter of mooring eye | 40 mm | 50 mm | 60 mm | 60 mm | 60 mm | 60 mm |
|  | Mass of incorporated steel (incl. Scrap Chain and mooring eye) | 50 kg | 100 kg | 300 kg | 500 kg | 800 kg | 1400 kg |
| P | weight of sinker in water | 2500 N | 4000 N | 6500 N | 13000 N | 26500 N | 39000 N |

Sinker with a circular base (more dificult to produce, but suitable for fast currents sites)

| M | Mass of sinker | 400 kg | 600 kg | 1000 kg | 2000kg | 4000kg | 6000kg |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | Base diamter | 100 cm | 110 cm | 130 cm | 160 cm | 200 cm | 230 cm |
| B | Upper diameter (=2A/3) | 66.7 cm | 73.3 cm | 86.7 cm | 106.7 cm | 133.3 cm | 153.3 cm |
| C | Height of sinker (=A/3) | 33.3 cm | 36.7 cm | 43.3 cm | 53.3 cm | 66.7 cm | 76.7 cm |
| D | Large diameter of the suction hole (=22A/30) | 73.3 cm | 80.7 cm | 100.3 cm | 122.3 cm | 151.7 cm | 173.7 cm |
| E | Narrow diameter of the suction hole (=21A/30) | 70 cm | 77 cm | 91.7 cm | 111.7 cm | 138.3 cm | 158.3 cm |
| F | Height of the suction hole (=A/30) | 3.3 cm | 3.7 cm | 4.3 cm | 5.3 cm | 6.7 cm | 7.7 cm |
| G | Length of reentrant for shackle (=9ф) | 36 cm | 36 cm | 45 cm | 54 cm | 54 cm | 54 cm |
| H | Width of reentrant for shackle (=3 ф) | 12 cm | 12 cm | 15 cm | 18 cm | 18 cm | 18 cm |
| I | Depth of of reentrant for shackle (=2 ф) | 8 cm | 8 cm | 10 cm | 12 cm | 12 cm | 12 cm |
| J | Height of reentrant for storage (=2.5 ф) | 10 cm | 10 cm | 12.5 cm | 15 cm | 15 cm | 15 cm |
| K | Upper diameter of reentrant for storage | 20 cm | 20 cm | 20 cm | 20 cm | 20 cm | 20 cm |
| L | Lower dia. of reentrant for storage(=K+2J) | 40 cm | 40 cm | 45 cm | 50 cm | 50 cm | 50 cm |
|  | Mooring eye diameter | 40 mm | 40 mm | 50 mm | 60 mm | 60 mm | 60 mm |
|  | Mass of incorporated steel (incl. Scrap Chain and mooring eye) | 50 Kg | 150 Kg | 250 Kg | 600 Kg | 1200 Kg | 1600 Kg |
| P | weight of sinker in water | 2500 N | 4000 N | 6500 N | 13000 N | 26500 N | 39000 N |

- Illustrated details of Canadian Coast Guard’s rope mooring system