IALA EEP COMMITTEE

Comments on Draft Guideline No. 1006 on Plastic Buoys, Edition 2 May 2008

- Purpose of submission

Following the submission of the abovementioned document for approval to the IALA Council in May 2008, the following comments are submitted for consideration by the EEP Committee.

It should be noted that South Africa did not have a representative at the IALA Workshop on Developments in Floating Aids and Associated Moorings Workshop that took place in Brest, France in April 2008, or at the EEP 11 Committee meeting where this document was reviewed and updated.  So these comments below are offered for consideration, taking into account that these issues may have been debated by the experts in the two forums and therefore do not appear in the Draft Guideline.

- Comments

  - Clause 2.1, following on point no. 7 - The mooring and lifting eyes should be cross-braced and secured in such a manner that no undue strain is placed on the body when the buoy is being lifted by the lifting eyes or by the forces exerted by the mooring.

  - Clause 2.1, point 10. – The sentence starting with “More guidance is available in ……..” is incomplete in the document that was submitted to the IALA Council.

  - Clause 3.1.2 – Question: Should the minimum wall thickness of the buoy body be indicated, or be mentioned that the wall thickness of plastic buoys should be proportional to the size of the buoy to be sufficiently robust?

  - Clause 3.2.2 -  The two sentences under the heading Quality Control should form one sentences, i.e. “As many commonly available types of glass fibre and polyester resin have limited resistance to extended immersion in water, the manufacturing materials must be carefully specified and certification obtained from the manufacturer to ensure that correct quality materials have been used.”

  - Clause 3.2.3 - When oily water (which are usually in and around Port/harbour areas) is allowed to penetrate foam-filled plastic buoys (being damaged), the foam may gets soaked with this oily mixture.  This may result in the foam having to be replaced.  Furthermore the oil on the surfaces of the buoy makes it very difficult for GRP to be repaired.

  - Appendix 1, 3rd bullet under the heading ADVANTAGES - Please note that not every administration have suitable craft (BTV) to carry out maintenance at sea, so this bullet is only relevant should such craft be available.

  - Appendix 1, 3rd bullet under the heading DISADVANTAGES - Where appropriate, the following could be mentioned:

- Plastic buoys, in particular GRP are prone to impact damage, particularly in extremes of hot and cold weather and with aging after prolonged UV exposure.

- Weathering and ultraviolet light (UV) can degrade the surface finish of GRP buoys, which can cause fibreglass splinters to develop and present a hazard for servicing personnel.

  - General

  - Metal parts of plastic buoys

As we have a very high corrosive atmosphere in South Africa, we have found it to be useful have all metal parts, including the lifting eyes, mooring eye, radar reflector, mounting hardware, the topmark mounting facility, lantern guard, etc. on plastic buoys to be made of stainless steel grade 316L, which had been passivated, in order to increase the lifespan.

All slag and impurities of the cutting edges and welding should be removed and the sharp edges smoothed.

  - Sinkers & Mooring

I have noticed that there is a work item on the EEP Work Programme to update the existing Guideline on Synthetic Moorings, however I cannot recall if there is a guideline addressing moorings and sinkers in general, e.g. addressing the various types of mooring, their advantages and disadvantages, how to calculate the length of mooring lines required for different types of buoys and depths of water, etc.

The same applies to sinkers, e.g. addressing the various types of sinkers, their advantages and disadvantages, what type of sinkers to be used for different types of seabed, what the weight of the various types of sinkers should be for different types of buoys, mooring lines in various depths of water, etc.

If guidelines on moorings & sinkers are not available/envisaged, I would suggest that it should be considered.

I trust this may be of some value.

Keep up the good work!

____________________________

James Collocott

Lighthouse Services

Transnet National Ports Authority

South Africa

6 September 2008

HQTECH/IALA Committees/EEP/ IALA EEP Committee - Comments on Draft Guideline No. 1006 on Plastic Buoys, Edition 2 May 2008_1.doc