German Federal Waterways Administration

Traffic Technologies Centre

Frank Hermann

EEP12/9/1

Input to IALA EEP 12

EEP 11, Action Item 4.7

Document EEP11/WG4/WP7

‘Draft IALA Guideline on synchronisation of Lights’

The title of the guide does not restrict synchronisation to the application of buoys or lateral marks only. So it might be worth considering whether the synchronisation of the following lights could be included in this guideline:

- leading (range) lights,

- aeronautical and marine lights on offshore windfarms.

If not the Guideline could be named ‘Synchronisation of lateral lights’.

Information about Synchronisation of leading lights in Germany

The front and rear lights are always synchronised. The recommended flash characters are occulting lights or iso-phase in order to have a long period, when front and rear light are visible at the same time.

For some applications there has been the proposal to have a delay of the rear light for 0.1 to 0.4 seconds, so the front light appears first.

Some of the lights will be driven with solar power and the synchronisation has to be realised with radio signals. As a lot of e-navigation depends on the GPS-system, it is recommended not to use GPS-time signal to synchronize a leading light. This is because the leading light should act as a backup system. If GPS is not available the synchronisation of the leading lights gets lost just in the same moment when e-navigation is not possible. For the German waterways administration an unsynchronized leading light is not acceptable.

However the US Coast Guard has skipped synchronisation of front and rear light in these cases and introduced a tricky alternative. US Coast Guard could provide more information about this in EEP 12.

Lights on offshore wind-turbines

In the recommendation on the marking of offshore-windfarms it is stated that aeronautical lights should not be visible below the horizontal plane. Unfortunately the relevant ICAO-lights must be visible below the horizontal plane and will be visible for the mariner as well.

So in practice there are flashing yellow (IALA) and red (ICAO) lights visible. Because the aeronautical lights are very bright (2000 cd) some administrations have already made modifications to the aeronautical lights to minimize the disturbance of marine aids to navigation.

In addition the aeronautical and nautical lights should have some kind of synchronisation or harmonisation. (We use the word ‘synchronisation’ for lights which are switch on and off at the same time, we use ‘harmonisation’ when lights with different flash characters have the same period and are switched on and off simultaneously for as many instances as possible).

In the following figure the ‘harmonisation’ of lights for German offshore-windfarms is shown. It is recommended that all flashes are synchronized to UTC, so every windfarm appears with harmonized flashes automatically.

The aeronautical light is a non-ICAO-light used in Germany and is called ‘Feuer W, rot’. It shows the characters 1 + (0.5) + 1 + (1.5). This was done to avoid conflict with marine lights: The flash character does not fit into the scheme of IALA Recommendation E-110.