German Federal Waterways Administration

Traffic Technologies Centre

Frank Hermann

EEP12/9/2

Input to IALA EEP 12

EEP 11, Action Item 4.1

Document EEP11/WG4/WP8

‘Draft IALA Guideline on Integrated Power System Lanterns’

The draft includes a lot of technical information for the mechanical, electronic and optical design of an Integrated Power System Lantern (IPS-lantern). The guideline should include information for the authorities and customers as well.

Geographical latitude / solar irradiation information

The German Waterways Administration has some experience with IPS- lanterns. Though offered by manufacturers for the use at the geographical latitude range of the German waterways, it turned out that the lanterns were not suitable: All Integrated Power System Lanterns failed in the first or second winter period.

So German Authority expects that the IALA guideline gives very precise information for the customer, whether this type of lantern can be used at all.

The required information is mentioned in chapter 5, 16.6 and 17 of the draft. These chapters should be worked out in detail to give the authorities the guarantee that the expected lifetime can actually be achieved.

A look at the technical information of IPS-lanterns provided by some manufacturers, shows that there is currently no information about the latitude range (July 2008). There is only weak information as ‘operates up to 5 years maintenance -> footnote: dependent on location’.

The guideline ‘IPS-lantern’ should be linked to IALA-guideline 1039.

Perhaps the guideline can propose that the manufacturers make calculation according to 1039 when offering lanterns.

Lantern in stock

To be able to replace a defect lantern quickly many authorities have their own stock for spare parts (e.g. lanterns for buoys). When a lantern is in stock, it gets no solar power and the battery is discharged.

The guideline should give information to the authorities how to handle the IPS-Lanterns as a spare part.

From the technical point of view it could be recommend that the battery can be disconnected from the battery by a mechanical switch. So the battery will be discharged by internal effects only. When an electronic switch is used the current of the switch will accelerate the discharge (quiescent current).

When the lantern stays in stock for a long time and is set up for operation, it would be useful that the battery can be charged to full capacity by an external device, before mounting on a buoy.

For all IPS-lanterns the customer should be able to get information about the present battery voltage or charge (measurement by external or internal equipment).

Disposal of defect lanterns

Some manufacturers offer IPS-lanterns with a lifetime of less than 5 years (see example above) which are totally maintenance-free (battery cannot be changed). After some years the authority has a lot of defect lanterns. The guideline should give information how to dispose these lights.

It could be proposed to make a contract with the manufacturer, so that he becomes responsible for an environmentally sound disposal.

Otherwise the guideline could recommend that the manufacturers describe how to dismantle the lantern and give information about the environmental hazard of each part. So the customer will be able to manage the disposal by himself.

Technical information about components

The guideline includes a lot of important information about the components of the lanterns (LED, Solar Panel, battery, GPS). However this information is not specific for an IPS-lantern. So it could be considered to put this information to existing IALA documents to give them an update.

E.g. the information about LED could be moved to IALA guideline ‘Modern Light Sources’, the information about batteries to IALA guideline ‘On secondary batteries for aids to navigation’.

Otherwise it should be investigated, whether the guideline ‘IPS-lantern’ conforms to existing IALA-documents.