IALA EEP Committee

Guideline on Flash Synchronisation and Sequencing

Work item 4.7

Introduction

IALA EEP Committee is preparing a guideline on flash synchronisation and sequencing.

Flash synchronisation and sequencing is commonly achieved using GPS timing signals. Recent GPS jamming trials by the GLA R&RNav Directorate in the UK have demonstrated that synchronised lights will continue to free run in synchronisation for a significant period of time in the presence of GPS jamming so long as the lights are synchronised at the commencement of the jamming. However if jamming occurs before lighting up time so that the lights come on in the absence of valid GPS timing signals, synchronisation will not occur. Flashers using GPS timing for synchronisation should therefore be designed so that the flasher timers remain synchronised even when the light is off during daytime.

Proposal

A note to this effect should be added to the Guideline on Flash Synchronisation and Sequencing.

Seamus Doyle

Commissioners of Irish Lights

3 September 2008.