AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Guideline No. ????

On

Light Applications

Floodlights

Part 5, Ed 1

Octber 2008

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      	Internet -  http://iala-aism.org

Table of Contents

1	Introduction	3

2	The PRACTICAL Application of ILLUMINATION OF FIXED STRUCTURES	3

3	Basic Principles	3

4	Types of FLOOD ILLUMINATION	4

5	Further considerations	Error! Bookmark not defined.

# Introduction

# The PRACTICAL Application of ILLUMINATION OF FIXED STRUCTURES

Illumination of fixed structures means, that the supporting structure of a navigational light or a navigational obstacle is illuminated or flood lighted with a fixed light of non-glaring properties.

The purpose of this illumination is to enable the mariner to positive identify the object and to allow the human brain to estimate the distance to the object. (The brain can not estimate the distance to a flashing light)

The main application is for close range navigation such as in ports and constricted waters.

- Off shore structures, beacons

- Moles, piers, jetties, breakwaters, dolphins

- Locks, bridges, signs

## Technical Considerations for Illumination of fixed structures

The illumination must not interfere with any navigational light. Good vertical separation between the two types of lights is necessary, if mounted on the same structure.

The risk of glaring the observer must be considered. This is a common mistake that must be avoided. Screening of the light sources is often required.

Normally the illumination system does not have a back-up system, due to heavy power requirements by the fixed light. But with the introduction of LED-technology (LED-tubes) this can be achieved.

White light is commonly used, but lateral red and green can be used particularly by means of LED.

## illumination system terminology

Illumination of fixed structures is frequently called Flood-lighting or Facade-light.

# Basic Principles

There are two ways to achieve illumination of structures, the traditional in-direct illumination or by direct emitted light towards the observer.

## In-direct illumination

In-direct illumination means that a lamp illuminates an object and the observer sees the reflected light from the object. This method has low efficiency, as only a fraction of the emitted light is radiated towards the observer. This method is simple to apply by use of standard light-fittings and normally requires mains-grid connection. An advantage is that the whole structure can be illuminated and also the surroundings. The indirect Light can help the observer to identify the structure (e.g. fixed Lighthouse) by illuminating identification text (or identification number etc.).

Using the same colour of indirect Light as the illuminated area the visibility may be enhanced.

## Direct illumination

Direct emitted light towards the observer is a fairly newly adopted method making the use of diffused LED to create the impressing of a weakly illuminated area resembling flood-light. The light directed to the observer is usually emitted from vertical tubes arranged around the structure. From a distance this lighted tubes form a stripped band of light. The light efficiency is high as mainly all light generated is directed towards the observer.

Figure 1 – Lighthouse Almgrund in Sweden equipped with LED pipes

# Types of FLOOD ILLUMINATION

## Sealed Beam lanterns

The most robust illuminator is the Sealed-Beam lamps which also have long life if run at lower voltage than marked.

## Sodium (Na) and Mercuri (Hg) lamps

Na and Hg lamp are frequently used and has the advantage of long life. Na emits yellow light and Hg emits white light, this is often used to mark opposite sides of channels with yellow and white side marks.

## LED lights

Also LED can be used for indirect illumination, but than with low light efficiency however supplied from low voltage systems.

## LED pipes

LED pipe light is designed to solve problems connected to traditional AtoN floodlights without need of mains electricity, high power consumption and maintenance. These direct light emitters rely on low voltage supply which means that PV-systems can be utilised.