Draft Guideline on Synchronisation of Lights (Work Item 4.7)

The ANM Committee work on synchronisation of lights should be taken into account when completing the Guideline on Synchronisation of Lights. The following is taken from the Report of ANM11.

# Presentation on the Trial of Synchronised Lights on Buoys in the River Swale

The following note on trials in Trinity House could be added to the Guideline.

Duncan Glass presented information on the trials conducted by Trinity House, using synchronised lights on buoys in the River Swale (southern part of the River Thames estuary).

A field trial revealed the merits of synchronised and sequential buoy lights when used as lateral marks in a channel. In particular, the use of these innovations when lighting straight channels (and those with bends), offered the mariner ease of identification of the lateral boundaries and ends of the channel.

Synchronised and sequential lights also overcome many of the detrimental effects that background lighting has on conspicuity.

During the trial, the characters of lights were altered using a remote control.  Varying the character of the channel entrance lights helped greatly in differentiating the beginning and end of a buoyed channel from the rest of the synchronised lights.

It was observed that if a channel had a bend in it, synchronising the lights of one leg at a time assisted the mariner greatly.  That is, if the straight legs were not delineated, the mariner could easily be misled.

It is aimed to include all lessons learnt and information gathered on synchronisation and sequential lights in the relevant IALA Guideline being developed.

Trinity House is expected to carry out more trials in May 2008.

A copy of the presentation made by Duncan Glass is provided as part of the output of the meeting.

# Maritime Buoyage System (MBS)

ANM are revising the MBS. The revised MBS booklet should also make reference to innovations and integration of new technologies (such as sequential and synchronised lights, blue lights, radio aids and AIS as AtoN).

# IALA Guideline on Synchronisation of Lights

The committee reviewed the draft Guideline on Synchronisation of lights prepared by the EEP Committee.  In light of recent trials reported by ANM members, especially in the UK, on sequential and synchronised lighted buoys, the committee is of the opinion that the ANM Committee should contribute to the development of this document by adding sections from the mariner’s and service provider’s point of view.  A liaison note to the EEP committee, on Draft Guidance on Synchronised Lights (ANM11-output-09), was prepared to inform them that the ANM committee will work intersessionally on proposed additions to the draft Guideline, in time for EEP12.

Action Items

The IALA Secretariat is requested to inform the IALA Council that the ANM Committee will work intersessionally on the Draft Guideline on Synchronisation of lights prepared by the EEP in order to add information and details from a mariner’s and service provider’s point of view.

The IALA Secretariat is requested to forward the liaison note on Draft Guidance on Synchronised Lights (ANM11-output-09) to the EEP committee.

Gerry Brine, Bjorn Erik Krosness, Duncan Glass, Juan Rebollo and Andre Chateauvert are requested to form an intersessional drafting group to develop revised Draft Guidance on Synchronised Lights (ANM11-output-09) for submission to EEP12.

When received, the IALA Secretariat is requested to provide the revised Draft Guidance on Synchronised Lights (ANM11-output-09) as an input paper for EEP12.

Seamus Doyle.