#### EEP 12    PROPOSED REVISION OF NAVGUIDE SECTION

### ENVIRONMENT/HAZARDOUS MATERIALS/MERCURY

#### Adrian Wilkins

# Mercury

A number of historic lighthouses still utilise rotating glass lenses and mercury float pedestals.  This was a clever method for providing a heavy lens with an almost frictionless bearing so that it could be turned by a clockwork mechanism.  However, given the toxic and corrosive properties of mercury, the following information may assist Authorities to implement appropriate safety procedures.

The mercury float pedestal for a first-order rotating lens contains about 13 litres of mercury.  Quantities of mercury may also be found in the electrical slip-ring units in rotating lamp array lighting equipment, some tilt-action switches, high current contact breakers, manometers and thermometers.

Physical Properties

Mercury is a heavy metal that has the unusual property of remaining liquid at normal temperatures ( above – 38 degC ).

Spill Risk

The mercury in a lighthouse optic system does not present a significant hazard, unless personnel come into contact with "uncontained" mercury as a result of accidental spills.  Such events are usually the result of a mishap during maintenance work, or as a result of a natural disaster such as an earth tremor that displaces mercury from its containment bath.

If spilt, the mercury can enter cracks in floors, and is readily absorbed into porous surfaces such as concrete, masonry and timber.  When broken into small globules or droplets, the surface area and vaporisation rate rises rapidly.  Minute droplets will adhere readily to dust and can form particles that can be inhaled.

Mercury is a corrosive substance if it comes into contact with metals such as zinc and aluminium.

Occupational Hazard

The occupational hazard associated with mercury relates to:

- Vapour inhalation: Some vaporization from a free mercury surface will occur at normal room temperature and this is the most likely first contact that lighthouse personnel will have with mercury.  Unless the mercury vapour levels have been measured, personnel are unlikely to be aware of the hazard.  If the work-space around lighthouse equipment containing mercury is not well ventilated, the concentration levels can rise above recommended limits and there is potential for mercury poisoning:

- mercury vapour is heavier than air and in still air will tend to concentrate in low parts of the work-space. Well designed ventilation will allow such concentrations to disperse.

- Ingestion:- is less common than vapour inhalation but can lead to acute mercurial poisoning;

- Absorption through the skin:- mercury is not easily absorbed through the skin.

Precautions

It is essential for the Authority to have detailed and strictly managed working procedures for all personnel working with, or in close proximity to mercury.

Staff must be trained in these work procedures and regularly medically monitored to ensure that they do not become contaminated with mercury.

The working procedures must follow national health and safety regulations and should be written by an expert in this field.

For work on optics the procedure will cover emptying, cleaning and re-filling the optic bath. Clean-up procedures will detail methods to recover all visible particles of mercury and the use of chemicals to neutralise smaller spills.

Personal protective equipment must be supplied that is specifically designed for use with mercury. This will include overalls, gloves, overshoes, respirator and eye protection. Procedures for the safe storage and disposal of this equipment must be in place.

A mercury vapour meter must be available to monitor the working environment and procedures in place for regular testing and calibration.

Consignment

Mercury is a hazardous substance and the relevant national and international regulations must be followed with regard to the type of container to be used, the packaging of this container for transport and the marking of this packaging.

Note:

Both IMO and the International Air Transport Association (IATA) have regulations covering the transportation of mercury.

The quantity of mercury used in higher order optics is shown in Section Error: Reference source not found.