EEP12-output-19

Formerly EEP10 / WG3 / WP2

PRODUCT CERTIFICATION TEMPLATE 4b

Buoy Mooring Chain and Bridles

Purpose

Product templates identify parameters that should be tested and measured and the standards against which each parameter should be tested and measured to enable a comprehensive evaluation of products.

Scope

This template is applicable for open link, welded steel chain and bridles used for mooring aids to navigation buoys.

Applicable Documents, Recommendations, and Guidelines

IALA Documents

- 1. IALA Guideline 1059, On Float Aid Moorings

- 2. IALA Guideline 1036, On Environmental Considerations in Aids to Navigation Engineering

Test Standards

The Templates quote national and/or international standards. If the manufacturer wishes to use an alternative standard, it is the responsibility of the manufacturer to confirm that the standard used conforms to the IALA quoted standard. Where no test standard is mentioned, the manufacturer shall state the test standard used, preferably with reference to international or national standards.

EEP12-output-19

Formerly EEP10 / WG3 / WP2

Product Certification Template 4b – Buoy Mooring Chain and Bridles

Product Type / Manufacturer Name and Reference Number:

| Para No. | Parameter Category | Parameter | Measured Value | Test Method∗ | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Physical |  |  |  |  |
| 1.1 |  | Material | Chemical composition | The manufacturer shall provide material certifications from the steel mill for each heat (batch) of steel used. The certifications shall indicate the heat (batch) number, steel designation, chemical composition, and quantity of steel represented by the certification. | Each length of chain and each bridle offered by the manufacturer shall be traceable to the heat (batch) of steel from which it was made. Serial numbers traceable to the heat (batch) of steel shall be stamped or engraved on the chain and bridles (see 1.9 below). |

If the chain has been heat treated after manufacture then details of the heat treatment process must be provided along with records of the treatment oven temperature and the times that the lengths of chain are maintained at temperature. Hardness and impact tests may be carried out on material samples taken from chain links to ensure that the heat treatment process has been effective and uniform.

∗ Where no test standard is mentioned, the manufacturer shall state the test standard used, preferably with reference to international or national standards.

| Para No. | Parameter Category | Parameter | Measured Value | Test Method∗ | Comments |
| --- | --- | --- | --- | --- | --- |
| 1.2 |  | Proof strength | lb and/or kg or kN | This test may be performed in one of two ways: 1) on a horizontal test bed, in which the entire length of chain or bridle is tested at one time, or 2) on a calibrator, in which the chain is tested in increments. In either case, the proof load shall be applied, and the chain or bridle subsequently inspected to ensure the product successfully passed the test. | The manufacturer shall indicate the load that the chain or bridle can withstand without rupture, permanent deformation, cold welding, or stretching beyond the dimensional tolerances of the links. Each length of chain and each bridle offered by the manufacturer shall have been tested to this load, and the manufacturer shall certify that the products successfully passed the test. The actual test data shall be provided upon request. The test equipment shall have been calibrated to International Standards within 12 months prior to the testing, and certification to this effect shall be made available upon request. |

| Para No. | Parameter Category | Parameter | Measured Value | Test Method∗ | Comments |
| --- | --- | --- | --- | --- | --- |
| 1.3 |  | Breaking strength | lb and/or kg or kN | Samples for this test shall consist of from 3 to 7 links, as required by the testing machine being used. Samples shall be free from twist, and all stresses bearing on the terminal links shall be the same as those applied to every link in the sample. The samples shall be brought to the breaking load and held for 15 seconds. The highest load applied to each sample shall be recorded. | The manufacturer shall indicate the load that the chain or bridle can withstand for a minimum of 15 seconds without rupture. Samples of the chain and bridles from each heat (batch) of steel shall be subjected to this load, and the manufacturer shall certify that the samples successfully passed the test. The actual test data shall be provided upon request. The test equipment shall have been calibrated to International Standards within 12 months prior to the testing, and certification to this effect shall be made available upon request. |

| Para No. | Parameter Category | Parameter | Measured Value | Test Method∗ | Comments |
| --- | --- | --- | --- | --- | --- |
| 1.4 |  | Elongation | Percent | Samples for this test shall consist of from 3 to 7 links, as required by the testing machine being used. Samples shall be free from twist, and all stresses bearing on the terminal links shall be the same as those applied to every link in the sample. Load shall be applied until the samples reach the required elongation. Elongation shall be determined using an autographic recorder, or by measuring the sample manually before and after the test. When performing manual measurements, ten percent of the proof load shall be applied for the initial measurement to ensure that the links are properly set. Subsequent measurements shall be taken with the load reduced to ten percent of the proof load. The elongation for each sample shall be recorded. | The chain and bridles shall be able to elongate at least 15% before rupture. Samples of the chain and bridles from each heat (batch) of steel shall be subjected to the elongation test, and the manufacturer shall certify that the samples successfully passed the test. The actual test data shall be provided upon request. The test equipment shall have been calibrated to International Standards within 12 months prior to the testing, and certification to this effect shall be made available upon request. |

| Para No. | Parameter Category | Parameter | Measured Value | Test Method∗ | Comments |
| --- | --- | --- | --- | --- | --- |
| 1.5 |  | Overall length | ft and/or m | Visual inspection | The manufacturer shall indicate the length of chains and bridles offered, and the respective tolerances, and certify that the products have been measured and are in conformance with these values. |
| 1.6 |  | Link dimensions (length, width, bar diameter) | in and/or mm | Visual inspection | The manufacturer shall indicate the dimensions of the products offered, and the respective tolerances, and certify that the products have been measured and are in conformance with these values. |
| 1.7 |  | Surface quality |  | Visual inspection | Links shall be free from mill defects, burrs, slag, weld spatter, or rough surfaces which might present a safety hazard or cause kinking of the chain or bridle in service. The manufacturer shall inspect the products offered and certify compliance with this requirement. Chain and bridles are normally offered in their natural color and finish. If a coating or other finish is used, the manufacturer shall describe the purpose and function of the material. |
| 1.8 |  | Weld quality |  | Visual inspection | Welds shall not contain craters, undercutting, overlap, porosity, or cracks. The diameter of the weld shall not be less than the bar diameter. Bar end misalignment shall not exceed 5 percent of the bar diameter. The manufacturer shall inspect the products offered and certify compliance with this requirement. |

| Para No. | Parameter Category | Parameter | Measured Value | Test Method∗ | Comments |
| --- | --- | --- | --- | --- | --- |
| 1.9 |  | Marking |  | Visual inspection | The last link on both ends of each length of chain and one end link on each bridle shall be legibly and permanently stamped or engraved with the manufacturer's name or trademark, the year of manufacture, and a unique serial number by which the chain or bridle can be traced back to the heat (batch) of steel from which it was made (see 1.1 above). The manufacturer shall inspect the products offered and certify compliance with this requirement. |
| 2 | Disposal and recycling |  |  |  |  |
|  |  | Recycling |  |  | The manufacturer shall indicate whether the products are recyclable. The manufacturer shall state if it will undertake the recycling of the products. Reference IALA Guideline 1036. |
|  |  | Disposal |  |  | The manufacturer shall state any material content that would harm the environment if the products are disposed of as normal waste. Reference IALA Guideline 1036. |
| 3 | Safety |  |  |  |  |
|  |  | Worker safety |  |  | The manufacturer shall state any special requirements to ensure workers’ safety in the use of the products. |