- IALA Guideline No. ???

- On

- FLOATAID MOORINGS

- Edition ??

- April 2008

20ter, rue Schnapper, 78100
Saint Germain en Laye, France
Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05
E-mail - iala-aism@wanadoo.fr      Internet - http:lliala-aism.org

Table of contents

INTRODUCTION	4

1.	GENERAL CONSIDERATIONS	4

1.1.	Design process for a new mooring	5

2.	VARIOUS SECTIONS OF THE MOORING	5

2.1.	Tail Chain	6

2.2.	Bridle	6

2.3.	The Riding Chain	6

2.4.	The Thrash Chain	6

2.5.	The Ground Chain	7

2.6.	Swinging Radius	7

3.	MOORING DESIGN	7

3.1.	Mooring Types	8

3.2.	Transitional Mooring	8

3.2.1.	Transitional moorings Loads	8

3.2.2.	Chain Size for transitional mooring	9

3.2.3.	Chain Length  for transitional moorings	10

3.2.4.	Reserve Buoyancy for transitional mooring	10

3.2.5.	Swinging Radius for transitional mooring	11

3.3.	Slack Moorings	12

3.3.1.	Ground Chain Resistance for slack mooring	12

3.4.	Taut Mooring	13

3.4.1.	Taut Mooring Loads	14

3.4.2.	Chain Size for taut moorings	14

3.4.3.	Reserve Buoyancy for taut moorings	15

3.4.4.	Swinging Radius for taut mooring	15

3.5.	Sinker Weight	16

3.5.1.	Sinker Weight for Transitional and Slack Moorings	17

3.6.	Buried Sinkers	17

4.	CHAIN MOORING COMPONENTS	18

4.1.	Chain	18

4.1.1.	Material Used In The Manufacture Of Chains	19

4.1.2.	Heat Treatment	19

4.1.3.	chain finish	19

4.1.4.	Specifications And Standardisation	20

4.2.	Shackles	20

4.2.1.	Forelock Shackles	20

4.2.2.	Clenching Shackles (heat and beat - American)	21

4.2.3.	Bolt Shackles	21

4.2.4.	Screw Pin Shackles	22

4.2.5.	Kenter Shackles	22

4.2.6.	Quick release links	23

4.3.	Swivels	23

4.4.	Sinkers	24

4.4.1.	Concrete sinkers	24

4.4.2.	Rock sinkers	25

4.4.3.	Cast iron sinkers	25

5.	MOORING WEAR AND CORROSION	26

5.1.	Wear	26

6.	SERVICE LIFE	27

7.	ROPE MOORINGS	27

7.1.	Rope Buoy Moorings	28

7.2.	Mooring Design	28

7.3.	Rope size	28

7.4.	Rope Construction	29

7.5.	Types Of Fibre	30

7.5.1.	Nylon	30

7.5.2.	Polyester	30

7.5.3.	Polypropylene	30

7.5.4.	Advanced Fibres	30

7.6.	Mixed Construction	31

7.7.	Rope Terminations	31

7.7.1.	Thimbles	31

7.7.2.	Splices	31

7.8.	Hybrid Rope/Chain Moorings	31

7.9.	Tension Leg Moorings	32

7.10.	Handling Rope Moorings	32

7.10.1.	Deployment	32

7.10.2.	Recovery	32

7.11.	Safety	33

8.	ELASTIC MOORINGS	33

Appendix 1: Chain mooring design	45

Appendix 2: example of a mooring design	46

Appendix 3: concrete sinkers characteristics	51

Appendix 4: Illustrated details of Canadian Coast Guards rope mooring system	53

# INTRODUCTION

This guideline combines three IALA papers:

Recommendation E107 On the Design of Normal Moorings, 1998,

Guidelines on Synthetic Moorings 1024, 2001,

Practical Notes On The Use Of Mooring Chains For Floating Aids To Navigation, 1989;

and provides information based on current practices used by IALA members Authorities in 2008.

# GENERAL CONSIDERATIONS

In order to ensure the safety of the mariner, lighthouse authorities maintain buoys and lightvessels as aids to navigation (AtoN). These floating Atons are maintained in position by their moorings

The mooring system must maintain the floating aid in a sufficiently accurate position for it to perform its function as an AtoN.

The mooring consists of a flexible cable connecting the floating AtoN to an anchoring device.

In most cases we will be considering a buoy connected by a length of mooring chain to a sinker on the seabed. However the concepts described can equally be applied to large floating aids such as lightvessels moored with anchors.

The design of the mooring will depend on:

- The depth of water at the mooring site

- The buoyancy of the floating aid

- The seabed conditions at the site

- The loads imposed on the mooring by the floating aid due to wind, wave action, water flow and ice.

- Loads imposed on the mooring cable by water flow

- Local conditions that cause wear and corrosion to the mooring

- Available servicing facilities

- Required life of the mooring

- Cost

The most common form of mooring cable is steel chain. The chain will form a catenary between the buoy and the seabed and will be able to absorb considerable amounts of energy. Chain has good wear resistance. With the correct equipment it is easy and safe to handle. Chain can easily be joined with shackles with little reduction in its tensile strength.

Buoy moorings usually consist of open link chain whose size is specified by the bar diameter of the material from which the chain is manufactured. Buoy mooring chain might range from 12.5 to 50mm. The chain material may be mild steel alloy or a medium carbon alloy steel may be used where its extra cost can be justified by improved wear resistance.

Stud link chain is not normally used for buoy moorings due to its initial high cost but may be considered where its increased weight, in comparison with open link chain may be an advantage.

However there are two particular situations where the use of chain is problematic:

- Where very deep moorings are planned, perhaps more than 60m water depth, a chain mooring may be too heavy for a normal buoy to support the weight of the mooring chain. In this case synthetic rope may provide a suitable lightweight material for part of the mooring cable. Section (7) provides details of rope moorings.

- Shallow moorings, particularly where breaking waves are encountered may also be problematic for conventional chain moorings. If breaking waves regularly occur at the buoy site, this would typically be when the wave height was ¼ (or more) of the water depth, then each breaking wave will impart considerable energy to the buoy. The length of the mooring chain may not be able to absorb the very high loads generated by a wave swept buoy, resulting in the chain being broken or the sinker dragging from position.  This problem can be solved by the use of elastic mooring cable. Section (8) provides details of rubber cord moorings.

## Design process for a new mooring

[Adrian to write]

# VARIOUS SECTIONS OF THE MOORING

HIGH WATER FINE WEATHER

STORM CONDITIONS

# HIGH WATER

LOW WATER

SWINGING CIRCLE RADIUS

SINKER

GROUND CHAIN

RIDING CHAIN

EXTENT OF THRASH

– ALL CONDITIONS

Fig.1 Buoy Mooring

A mooring is normally made up of the following parts suitably assembled:

- Either a Tail Chain or a Bridle (depending upon the method of securing to the buoy)

- Riding Chain

- Thrash Chain

- Ground Chain

## Tail Chain

For buoys with the mooring attached to a single a single mooring eye, this is the length of chain connected to the buoy. It suffers from interlink wear as it absorbs a significant portion of the energy generated by the buoy as it rises and falls following the surface movement of the sea. It also has to absorb the rotational energy of the mooring induced by the buoy rotating around its axis. To limit this effect a stabilising fin can be fitted to the buoy on the opposite side to the mooring eye. This fin also has the advantage of reducing the oscillation of the buoy in running water.

In order to allow buoys with a single mooring attachment point to ride vertically in a variety of water depths, and currents, and with differing weights of chain it is necessary to provide a several mooring eyes at different distances from the buoy axis.

In the case of a tail tube buoy the tail chain should be increased in size to allow for extra wear where the chain rubs against the tube. To lessen this chafe some Services attach a wooden rubbing band around the bottom of the tube.

## Bridle

A bridle comprises two equal lengths of chain shackled to the buoy on opposite sides. The two chains are joined below the buoy by means of either a triangular or circular link, or a shackle.

Below this point there can be a swivel or a direct connection to the next section of chain.

The use of a bridle enables buoys to ride vertically without having several mooring eyes to cope with different tidal conditions or weights of moorings. It also allows a chain size some 10 % smaller, and smaller mooring eyes.

## The Riding Chain

The Riding Chain or Riser connects the bridle or tail chain to the thrash chain. It can vary in chain size (bar diameter) and length depending on the buoyancy of the aid, the depth of water and expected forces on the chain. The chain must be of sufficient strength to recover the sinker. Usually chains are delivered in standard lengths and joined together with shackles.

This section of the mooring remains suspended beneath the buoy and is not subjected to wear on the seabed. Hence this section of the mooring may utilise synthetic rope where there are special requirements for a lightweight mooring, usually in deep water situations, or a rubber cord in very shallow moorings where large amounts of wave energy must be absorbed.

## The Thrash Chain

The Thrash Chain is the part of the mooring lying on or near the seabed, connecting the riding chain to the ground chain. Due to tidal influences (rise, fall and current) and the sea and wind conditions the thrash chain continually moves on the seabed. In the case of a sandy bottom severe wear can be expected in this area of the mooring.

It is usual to renew this part of the mooring first, either with new chain or by exchanging with a length of chain from elsewhere in the mooring.

In some cases the chain size (diameter) is increased in this section to offset the wear.

## The Ground Chain

The Ground Chain lies on the seabed between the thrash chain and the sinker, and sometimes becomes embedded.

This section of the mooring may be increased in length or a larger chain size used to reduce loads transmitted to the sinker.

The ground chain, the sinker and the resilience of the other parts of the mooring keep the buoy in position.

Note: For small to moderate sized moorings the riding chain, ground chain and thrash chain may be formed by one continuous length of chain. However these terms are still used to identify the various sections of the mooring.

## Swinging Radius

The swinging radius is the radius of  the circular movement of the buoy about the position of the sinker at low water. Ice may increase the swinging radius.

In tidal conditions, the excursion of the buoy will be in the direction of water flow.

2.7 Sinkers

The sinker usually provides the attachement of the mooring to the seabed. Anchor may be used when the mooring load is always in one direction.

# MOORING DESIGN

The following section provides a methodology to calculate the dimensions of a chain mooring and the weight of sinker required. The process requires knowledge of the environmental conditions at the mooring site, details of the local wave regime and the dimensions of the buoy that will be moored. Details of the type of seabed will be needed to establish the required weight of the sinker.

Navigational requirements regarding an acceptable swinging radius for the buoy may require further refinement of the mooring design.

At sites where damage to flora and fauna on the seabed is of concern then the mooring may have to be designed so that there is no ground chain sweeping the seabed.

In areas such as the Baltic, where there is very little tidal range, tensioned moorings are often employed. These connect the buoy directly to the sinker so that the tension in the mooring chain maintains the buoy in an upright position.

Information gained from practical experience with other buoys in the proposed mooring area will prove valuable in verifying the results of the calculations.

The calculations make the assumptions that:

- The buoy axis is vertical under the most common conditions of current and wind.

- The reserve buoyancy of the fully equipped buoy is sufficient under the worst conditions of wind and tide.

- The drag due to water flow on the mooring chain is not significant in currents less than 5 knots and water depths less than 40m. Additional calculations will be required at deeper sites with fast currents.

## Mooring Types

Details are provided for the three main types of mooring.

The Transitional Mooring.

The Slack Mooring

The Taut Mooring

The following text describes these three types of moorings and how the moorings loads may be calculated. The detailed calculations are presented as annex 1.

## Transitional Mooring

Fig.3  Transitional Mooring

WIND

TIDE

The transitional mooring (see Fig.2) is the ideal mooring state where the mooring chain catenary meets the seabed tangentially exactly at the sinker when there are the maximum wind and tide loads on the buoy. The mooring loads will be transferred horizontally to the sinker, which will be working as effectively as possible.

### Transitional moorings Loads

The loads caused by wind and tide on the buoy are calculated from the following formulae:

THO = FW + FD

Where

THO     stands for the horizontal load caused by wind and tide in Newtons

Fw       stands for the maximum wind load on the buoy in Newtons

FD       stands for the maximum tide (or current) load on the buoy in Newtons

FW = ½ρaV2WACW

Where

ρa      stands for the unit mass of air in Kg/m3

VW         stands for the maximum wind velocity in m/s

A         stands for the cross sectional area of the parts of the buoy exposed to the wind in m2

CW       stands for the aerodynamic drag coefficient of the relevant parts of the buoy exposed to wind loads.

Typical values for CW are as follows:

Cylinder   0.3  to  0.4

Flat Plate    1.0

Lattice construction (angles)   1.7 to 2.0

Lattice construction (tubes)      0.3 to 0.4

Give a specific value for users who wonder the coefficient they should choose

FD = 1/2ρoSV2Cd

Where

ρo    stands for the unit mass of sea water in Kg/m3

S     stands for the cross sectional area of the immersed areas of the buoy in  m2

V     stands for the maximum velocity of the current or tide in m/s

Cd    stands for the hydrodynamic drag coefficient of the various immersed sections of the buoy

Typical values for Cd are

For buoys with a draught ≥ 1/5 of their diameter    0.8 to 1.2

For buoys with a draught ≤ 1/5 of their diameter   0.3 to 0.5

Give a specific value for users who wonder the coefficient they should choose

Add an example of a shape from Holland

### Chain Size for transitional mooring

The practical size of the chain can then be ascertained by trying the strength and immersed weight of various sizes of commercially available chain in the following formulae

RC ≥ 5 ( p g H  +  THO )

where

Rc stands for the proof load of the chain in Newtons.

p  stands for the linear immersed mass of the chain (mass minus the buoyancy of the chain) in Kg/m.

H stands for the maximum water depth at the station in metres.

THO  stands for the horizontal load imposed by the buoy which is equal to the horizontal mooring load at the sinker in Newtons.

g  Stands for the acceleration due to gravity in m/s2

The safety factor of 5 takes account of the continual cyclic load and wave effects that the chain is subjected to by the motion of the buoy.

This calculation will provide a guide to the size of chain required and the following formulae can then be used to establish the required length of the mooring chain.

### Chain Length  for transitional moorings

L=H(H+2THOpg)

### Reserve Buoyancy for transitional mooring

The load imposed on the buoy can then be calculated to see if the proposed buoy will have sufficient reserve buoyancy.

Rb=U−Mb+mcLρw

where

Rb  stands for the reserve buoyancy in m3.

U  stands for the total volume of the float in m3.

Mb stands for the mass of the buoy in Kg.

mc stands for the linear immersed mass of the chain in Kg/m.

L stands for the suspended length of the mooring in m.

ρw stands for the density of water in Kg/m3

### Swinging Radius for transitional mooring

The swinging radius (see Fig.1) of the mooring can then be calculated from the formulae.

Rm=L−Hm(H+2THOpg)+THOpgcosh−1(HmpgTh0+1)

where

Rm stands for the maximum swinging radius in meters,

L stands for the total length of the mooring in meters,

Hm stands for minimum depth of the station in meters,

Th0 stands for the horizontal mooring tension at the connection with the sinker in Newtons,

p stands for  the linear immersed mass of the chain (mass minus the buoyancy in mass of the chain) in Kg/m.

It will then be possible to see if the designed mooring meets navigational and operational requirements.

Will the swinging radius be acceptable? If the buoy is marking a navigable channel then the swinging radius may need to be reduced. This can be achieved by using a larger size of mooring chain. The effect of this can be investigated by re-calculating the mooring design with increasingly larger sizes of chain.

The reserve buoyancy of the buoy must also be monitored as larger sizes of chain are considered.

Consideration may be given to the use of a resilient beacon, a piled bacon or a two legged mooring if precision marking is required.

## Slack Moorings

WIND

TIDE

Fig.2 Slack Mooring

In practice, this is the most common form of mooring.

Slack moorings have chain permanently on the seabed. When the loading on the buoy is at its maximum the mooring chain catenary will meet the seabed some way from the sinker. The chain on the seabed increases the security of the mooring or may be used to reduce the size of the sinker(See Fig.3)

### Ground Chain Resistance for slack mooring

The following formulae evaluates the increase in mass that the ground chain provides to the sinker.

Mgained=mCLgtanφK

where

Mgained stands for the effective increase of sinker mass provided by the ground chain.

mc stands for the linear immersed mass of the chain (weight minus the buoyancy of the chain) in Kg/m.

K stands for a safety coefficient (generally taken equal to 1,5),

Φ stands for the internal friction angle of the sea bed  (it relies on the type of soil at the station), 45° ( 0,7855 in radian) is a practical approximation that can be used in most cases. The chalk and some gravel bottom may lead to choose a lower friction angle.

Lg stands for the length of the ground mooring that lies on the seabed in m.

This mooring arrangement is often used in open waters where the consequently large swinging radius of the buoy is acceptable. The length of ground chain provides energy absorbing capabilities in extreme weather conditions.

Another consideration may be that the weight of the sinker may be reduced by using a long ground chain if the required sinker weight for the transitional mooring is greater than the servicing vessel’s lifting capacity.

## Taut Mooring

Fig.4 Taut Mooring

WIND

TIDE

If the swinging radius is still not acceptable, for example the edges of dredged channels often require very precise marking, then a Taut Mooring may have to be considered.

Similar design constraints will apply if the local seabed is an important marine ecosystem. It may then be necessary to design a taut mooring such that most of the mooring chain is suspended from the buoy. Any damage that the mooring chain causes to the seabed will then be minimised.

In areas with rough rocky seabeds there is a chance of the mooring chain catching on outcrops of rock. If the mooring is designed as a transitional or a slack mooring then it is possible that part of the chain on the seabed will catch on a rock and effectively shorten the mooring. The loads on the chain and the buoy will then be increased, possibly to the point of failure. These problems can be minimised by designing the mooring as a taut mooring.

The taut mooring (see Fig.4) has the mooring chain meeting the sinker at an angle when the maximum wind and tide loads are acting on the buoy. There will then be a vertical component of the mooring load applied to the sinker, which will require a larger sinker to maintain the buoy on station.

The chain size and length and sinker weight may now be calculated to meet the swinging radius requirements for the particular buoy station. A check must again be made to ensure that the buoy has sufficient buoyancy to support this mooring in the worst environmental conditions.

The following formulae are provided for this mooring calculation;

### Taut Mooring Loads

For the vertical mooring load on the sinker   TVO=pH4THO2+(pL)2−(pH)22(pL)2−(pH)2−pL2

where

Tv0 stands for the vertical mooring tension at the connection with the sinker in Newtons,

Th0 stands for the horizontal mooring tension at the connection with the sinker in Newtons (equal to the horizontal mooring tension at the connection with the buoy as the horizontal tension is integrally transmitted along the mooring),

p stands for the linear immersed mass of the chain  (mass minus the buoyancy of the chain) in Kg/m.  check the formula

H stands for the maximum depth of water in m,

L stands for the length of the mooring in m.

### Chain Size for taut moorings

For the chain strength                             Rr≥5(pH+TVO2+THO2)

where

Rc stands for the proof load of the chain in Neutons.

p  stands for the linear immersed mass of the chain (mass minus the buoyancy in mass of the chain) in Kg/m.

H stands for the maximum water depth at the station in metres.

THO  stands for the horizontal load imposed by the buoy which is equal to the horizontal mooring load at the sinker in Neutons.

Tv Stands for the vertical load imposed by the weight (in water) of the chain plus the vertical load on the sinker in Neutons

g  Stands for the acceleration due to gravity in m/s2

The safety factor of 5 takes account of the continual cyclic load that the chain is subjected to.

### Reserve Buoyancy for taut moorings

For the reserve buoyancy                    Rb=U−Mb+mcL+TVOgρw

where

Rbstands for the reserve buoyancy in m³,

U stands for the total volume of the float in m³,

Mbstands for the mass of the buoy in Kg,

mc   stands for the linear immersed mass of the chain (weight minus the buoyancy of the chain) in Kg/m.

L    stands for the suspended length of the mooring in m,

Tv0  stands for the vertical mooring tension at the connection with the sinker in Newtons,

g    stands for the acceleration due to gravity in m/s²,

ρw  stands for the density of water Kg/m3 (taken equal to 1024 Kg/m3 for salt water),

### Swinging Radius for taut mooring

For the taut mooring swinging radius:

[add a condition to see if the mooring is still taut with the low level]

Rm=ThOpcosh−1[pHmThO+ThO2+TVO2Th0]−ThOpsinh−1(TVOThO)

where

Rm stands for the swinging radius in meters,

L stands for the length of the mooring in meters,

Hm stands for minimum depth of the station in meters,

Th0 stands for the horizontal mooring tension at the connection with the sinker buoy in Newtons,

T'v0 stands for the vertical mooring tension at the connection with the sinkerbuoy in Newtons, calculated with the formulation (5) at the minimum depth,

Mass of the chain

p stands for  the immersed linear weight of the chain  (weight minus the buoyancy of the chain) in N/m.

## Sinker Weight

A very simple way to design the sinker weight consist in taking only into account the effect of the friction with the sea bottom (the ‘burying’ and the ‘rock stopping’ effects are not taken into account ).

Under these hypothesis, the minimum weight of the sinker is provided by the formula

M≥KTHOδg(δ−ρW)tanφ+TVOg

where

M stands for the mass of the sinker in Kg.

K stands for a safety coefficient (generally taken equal to 1,5)

Th0 stands for the horizontal mooring tension at the connection with the sinker in Newtons,

Tv0 stands for the vertical mooring tension at the connection with the sinker in Newtons,

δ stands for the mean density of the sinker in Kg/m3 (generally taken equal to 2400 Kg/m3 for a reinforced concrete sinker),

g stands for the acceleration due to gravity in m/s²,

ρw stands for the density of water Kg/m3 (taken equal to 1024 Kg/m3 for salt water),

Φ stands for the internal friction angle of the sea bed  (it relies on the type of soil at the station), 45° ( 0,7855 in radian) is a practical approximation that can be used in most cases. The chalk and some gravel bottom may require a lower friction angle. Relevant information can be found in civil engineering textbooks.

### Sinker Weight for Transitional and Slack Moorings

In the case of a Transitional or a Slack mooring, the vertical tension of the mooring at the connection with the sinker is equal to zero, and consequently, the relationship above can be simplified:

M≥KTOδg(δ−ρW)tanφ

## Buried Sinkers

It must be noted that in service sinkers often become buried in the sand or mud on the seabed. In order to recover the sinker the servicing vessel will then have to haul up the mooring chain and break the sinker out of the seabed.

The following empirical formulae provides an estimate of the maximum load that may be encountered:

HC≥2Mδ−ρWδ+HmC

where

Hc stands for the hauling capacity of the servicing boat (likely a buoy tender) in Kg,

M stands for the mass of the sinker in Kg,

δ stands for the mean density of the sinker in Kg/m3 (generally taken  as 2400 Kg/m3 for a reinforced concrete sinker and 7800 Kg/m3 for cast iron),

ρw stands for the density of water Kg/m3 (taken equal to 1024 Kg/m3 for salt water),

H stands for the depth of water when the mooring is lifted (generally, it is the maximum depth of the site that is taken into account) in m.

mc stands for the immersed linear mass of the chain  (weight minus the buoyancy of the chain) in Kg/m.

See appendix2: Example of mooring calculation for French service’s standard buoy

# CHAIN MOORING COMPONENTS

The components used to assemble a mooring system are:

- Chain

- Shackles

- Swivels

- Sinkers or Anchors

It is important that all components (including sinker and anchor eyes). are manufactured from the same quality material to minimize corrosion and wear.

## Chain

Chain size is defined by the bar diameter of the chain links. However various lengths of chain links have been established depending on local manufacturing processes, national standards and operational requirements.

Fig.: 4D chain

The proportions of the chain link are described as the ratio between the bar diameter and the inside length of the chain link.

Each Service should define a dimensional ratio in consultation with the supplier and manufacturer that will allow the use of their standard shackles and other chain connections.

Examples of ratios currently in use are:

France 3,4 and 5

UK 4

Netherlands 5

Germany 9

The shorter link chain requires more links per length. It is heavier and therefore can be more expensive, but it is easier to handle with a winch. However the links may not be of sufficient size to accept a joining shackle and enlarged end links may be required on each length of chain.

The longer link chain may not require an enlarged end link in order to insert a joining shackle. It is easier to handle with a hook and permits one size of hook on the hoisting equipment to be used for several sizes of chain. There is also less risk of ‘knotting’. Consideration must be given to the chain handling method, and particularly the dimensions of any chain handling capstain if different link lengths are to be used.

### Material Used In The Manufacture Of Chains

Chain is manufactured from grades of steel that are suitable for forging into the shape of the link and also capable of being reliably welded. A variety of steels fall into these categories and provide different combinations of strength, hardness, wear resistance and cost. A particular steel may be slow to wear but not be resistant to corrosion or vice versa. Generally a compromise must be found in the quality of material that will minimise both wear and corrosion at an acceptable cost.

Steels with a high carbon content (0.2%) and high manganese content (1.5%) have proved to be very effective for buoy mooring chain. It should be noted that Lloyd’s U grades and German DIN grades are based on the chain strength and do not specify a particular grade of steel. The wearing performance of the chain will therefore not be defined. It is recommended that national authorities should specify the steel used in the manufacture of their chain [see 4.3.3].

This is an example of the steel used by the Netherlands Authority. This is an acceptable high performance chain mooring steel. This product would be heat treated by quenching and tempering.

| Country | Netherlands |  |
| --- | --- | --- |
| Material | Min % | Max % |
| Aluminium | 0.015 | 0.025 |
| Carbon | 0.25 | 0.26 |
| Chromium | 0.20 | 0.30 |
| Copper | - | - |
| Manganese | 1.40 | 1.60 |
| Molybdenum | 0.10 | 1.20 |
| Nickel | 0.20 | 0.30 |
| Phosphorus | - | 0.02 |
| Silicon | 0.20 | 0.35 |
| Sulphur | - | 0.02 |
| Vanadium | 0.08 | 0.12 |

### Heat Treatment

Chains may be heat treated to reduce stress concentrations around the area of the welds and to generally improve the strength of the chain.

Some Authorities have found non-heat treated chain to be acceptable in use and hence more cost effective.

### chain finish

Some authorities specify a protective coating to limit corrosion while the chain is stored.

### Specifications And Standardisation

A specification should include the material from which the chain is to be made, the heat treatment, the finished dimensions of the chain and the mechanical properties of the material .  It should also include means of identification of individual chain lengths.

Tests must be included which will verify the requirements of the specification and will be practical for the purchaser or his agent to carry out.

## Shackles

The shackle is the most widely used device for attaching the mooring to the buoy, and joining the other components of the mooring together.

The strength of the shackle must equal that of the chain which it is joining. Thus, the diameter of the shackle bow will be greater than the chain size.

Confusingly the size of a shackle may refer to the size of the chain which it fits or it may refer to the diameter of the shackle pin. Local standards must be consulted to ensure the correct definition of sizes.

The types of shackles in use are:

- Forelock shackles

- Clenching shackles

- Bolt shackles

### Forelock Shackles

Forelock shackle with oval pin

The forelock shackle is a very reliable type and it is easy to use. It can be manufactured in size and shape to suit the components that it is connecting and it has a forelock to secure the pin. This has the advantage that in use the forelock is the least stressed component.

The pin can be round or oval in cross section. The round pin is cheaper to manufacture but it has the disadvantage that in service it can rotate, thus wearing the forelock and eyes of the shackle. The more expensive oval pin overcomes this disadvantage.

### Clenching Shackles (heat and beat - American)

Hammer forged

Clenching shackle with oval pin

The clenching shackle is the most reliable method of joining lengths of chain. The pin end is enlarged by hammering (forging) to prevent the pin withdrawing whilst in use.

The disadvantages are that considerable heat is required to bring the pin up to forging temperature, and the pin is not reusable. An alternative to forging is to weld the pin end to the shackle eye but in this case neither the pin nor the shackle is reusable. It is essential to use the right grade welding rod in order to ensure that electrolytic corrosion does not take place. It is very difficult to obtain a good quality of weld on the deck of a boat.

In certain circumstances the protruding parts of pins are subject to abrasion on the sea bed and may be worn away with the subsequent loss of the shackle pin unless it is otherwise secured.

### Bolt Shackles

Fig.6

The bolt shackle has a round pin secured by one or two nuts. If only one nut is used then a cotter pin or a split pin must secure the nut.

The round pin can rotate in the eyes of the shackle resulting in rapid wear.

Authorities who choose to use these shackle may find it prudent to weld the nuts and pins in place.

### Screw Pin Shackles

Fig. 7

These should not be used in permanent moorings, as any rotation of the shackle pin will result in failure of the shackle.

### Kenter Shackles

These are commonly used in ship moorings but are not usually used for permanent moorings where long term corrosion and wear will result in the shackle literally falling apart.

### Quick release links

Many types are available One of which is illustrated below. These may be used close to the buoy where the mooring is always in tension if there is a requirement to regularly disconnect the mooring.

G hook

## Swivels

Fig. 8

A swivel permits two lengths of chain to be joined without transmitting a twisting motion (torque) from one length to another.

Due to the rotation of a buoy around its axis, a twisting action is applied to the chain. This may result in the chain knotting, ultimately affecting the riding quality of the buoy and the effectiveness of the mooring. In such a case one or more swivels can be inserted into the mooring. Usually a swivel is placed between the Tail Chain or Bridle and the Riding Chain.

Experience shows that the quality of swivels requires careful monitoring to ensure that their strength and working life matches the other components of the mooring. They should be made of the same material as the chain.

Swivels may seize due to corrosion, wear and/or marine growth. If severe marine growth occurs, regular cleaning is recommended.

## Sinkers

[add a paragraph about anchors in general and especially river moorings (Sipke Hoekstra and the German authority has valuable information)]

Experience has shown that sinkers are adequate to keep buoys in their assigned positions. They do not provide the resistance or holding power of anchors of the same weight but have the great advantage that they will provide the same resistance irrespective of the direction that the mooring load is applied.

Sinkers may be made of concrete, cast iron, rock or bundles of used chain. Whilst most Authorities use sinkers, in some cases anchors are necessary to resist high mooring loads. for example LANBYS (LNBs) on exposed stations use a combination of sinkers and anchors, and floating aids in rivers may use anchors  where the current direction is constant.

The effective weight will be equal to the weight of the sinker in air minus the weight of the water displacement.

### Concrete sinkers

Fig. 10

These are of moderate cost and widely used. They are manufactured by casting concrete into a suitably shaped mould with a metal eye for attachment of the mooring chain cast in place. Concrete sinkers can be made with local labour using local materials. However, they are considerably larger than cast iron sinkers of the same submerged weight and thus take up valuable storage space in the depot or on the buoy tender’s deck. Care must be taken in all aspects of their manufacture so that they do not disintegrate or allow the mooring eye to pull out. The quality of the concrete must be carefully  monitored to ensure it is of the specified density.

The density is often improved by including steel reinforcement or scrap chain in the concrete.

Hematite may be used as an aggregate to increase the density of the concrete.

See annex 3 for manufacturing details of sinkers used by American and French services.

### Rock sinkers

An alternative sinker can be made by fixing a mooring eye in a piece of rock, or large stone. This may be effective if there is a local supply of dense stone in suitable sizes. However, it is very difficult to assess the condition of an individual rock and it can break in service.

### Cast iron sinkers

Trinity House 3 tonne cast iron sinker

Fig. 11

The most expensive but the most durable and effective sinkers are made of cast iron. Their density compared with concrete is approximately in the ratio of 3: 1.

Cast iron sinkers are robust and the repair or replacement of the mooring eye is possible.

If a mooring is prone to sanding in, a cast iron sinker will be easier to extricate than a concrete sinker because its physical size is smaller. However if lost its replacement will be more expensive than a concrete sinker.

There are many designs and shapes of cast iron sinkers. It may be cost effective to use scrap cast iron if suitably dimensioned pieces are available.

# MOORING WEAR AND CORROSION

The two main factors which influence the service life of a mooring system are wear and corrosion These two factors are interactive since the products of corrosion rapidly accelerate wear.

The performance of the mooring chain depends upon the choice of the correct steel specification and must take into account the acceptable amount of wear within the working life of the mooring.

Rust, electrolytic action and pollution may reduce the normal life of a mooring line.

Rust, which is due to oxidation, occurs both ashore and at sea.

Electrolytic Action results from a potential difference between components of a mooring. This is caused by using dissimilar metals in the same mooring immersed in a common electrolyte, in this case water.

It shows as pitting near and in the welds of chain links and in the chain each side of connecting shackles, over a length, of about 1-2 metres.

Electrolytic Action can be significantly reduced by making the buoy body and all the metallic components of the mooring from the same or very similar grades of steel.

Polluted waters also have an influence on the extent of pitting corrosion. It is mainly observed in the riding part of the mooring chain and has occasionally been observed in the skirts of buoys.

## Wear

The extent to which wear occurs mainly depends on the quality of the materials from which the mooring is made. Wear is also affected by local environmental conditions, in particular:

-     Type of seabed

-     Amount of sand transport in the water

- Tidal conditions

- Depth of water

-     Sheltered or open waters

-     Weather conditions

The mooring cannot change these conditions but can only react to them.

A reduction in bar diameter will be observed at the contact point of the links.

This may be due to wear resulting from the movement between links or to local deformation of the metal caused by impact loads due to the chain ‘snatching’.

Snatching can also cause elongation of the links.

In an area where the seabed is sandy and in particular during bad weather or in strong tidal currents, a heavy sand transport can be expected. This will cause severe abrasion of the mooring components.

The thrash section of the mooring has the most pronounced wear as it is continuously moving. Even in waters with small tidal range, where only a small part of the mooring chain moves over the sea bed, very significant wear may be found.

It is common practice to establish a suitable chain size for the thrash area of the mooring and then to use this size of chain for the entire mooring. For deep water moorings, a smaller chain size may be used for the riding chain due to limitations of the reserve buoyancy of the buoy.

Larger size of chain may be used for the ground mooring chain where the increased weight would be an advantage.

Chain inspections are carried out at intervals that may range from 6 to 24 months. This will depend on the safety factors built into the mooring and the environmental conditions at the AtoN location. Experience may show that on some stations a more frequent check is necessary.

# SERVICE LIFE

The service life of chain normally varies from 1 to 5 years, but in sheltered, muddy conditions it may be as long as 20 years.

Chains are scrapped or down graded for use in other locations when the diameter in any point of a link is reduced to a predetermined percentage  of its original size. 60% is used by some authorities for buoys in inland waterways, whereas 85% may be used in exposed situations. Quite, often a worn part of the chain is moved to another part of the mooring that is experiencing little wear.

A detailed inspection regime including elements of predictive maintenance may allow greater or lesser wear down allowances. This regime must take account of wear rate and predicted remaining cross section.

Accurate record keeping is a vital part of this inspection process.

Detailed dimensional and material specifications must be maintained to ensure reliable, predictable wear  performance of all mooring components.

These specifications are of particular importance regarding the planning of future maintenance. It will only be possible to utilise experience gained with the working life and necessary maintenance to a group of moorings to plan the future repair and replacement of similar moorings if the same quality of chains, shackles and other components can be purchased in the future.

There are benefits in terms of stockholding and cost reduction to be gained by standardisation of the various components of mooring systems. The quality of the components should be related to the required service life.

It is important to note that the normally quoted regulations for the manufacturers of chain for merchant ships, as laid down by classification societies, only specify the breaking and impact strength of the chain and give no guide to wear and corrosion resistance. A ship normally spends little time at anchor, and so the chain does not suffer significant wear and corrosion. Often the chain will last the life of the ship or only be replaced if it has been overstrained.

See IALA Guideline No. 1040 on the maintenance of buoys and small Aids to Navigation

structures.

# ROPE MOORINGS

## Rope Buoy Moorings

The primary advantage of rope moorings is their light weight and elasticity when compared with normal chain moorings. Modern ropes can easily match the strength of steel chain and experience has shown that a similar or better working life than chain can be achieved if chafe is carefully avoided.

The conventional chain mooring utilises energy absorption of the chain catenary to absorb much of the wind and wave energy acting on the buoy and prevent this being transferred to the sinker or anchor. The elasticity of the rope performs a similar function and choosing a suitable combination of fibre type and rope construction can optimize this energy absorption.

Chafe and cutting are the greatest dangers to a rope mooring. It is easily demonstrated that a sharp knife will rapidly cut through a piece of rope and any sharp edges presented by rocks, sea shells or the servicing ships own capstan can rapidly cause permanent damage to the surface of the rope. Allowing the rope to slip on the drum of a capstan or pulling it through an unsuitable fairlead may not only result in abrasive damage but also in localised heating such that the surface fibres of the rope may melt, resulting in significant weakening.

Rope may suffer abrasive damage from sand particles in suspension in the water resulting in reduction in breaking load.

## Mooring Design

The mooring must be designed so that the rope is never in contact with the buoy body or tail tube and is never in contact with the sea bed (although this may not be a problem in areas with soft, muddy bottoms).

These criteria can be achieved in a normal buoy mooring by utilising a ground chain that absorbs the wear on the sea bed to whic h a rope “riser” is attached. Floats may be incorporated to keep the rope off the seabed. Floats may also be used in this way in environementally sensitive areas. The rope “riser” component of the mooring is of such a length that even at the lowest tides the rope is never chaffing on the sea bed. The rope may be attached directly to the buoy if the mooring eye is in a suitable position such that the rope will always be clear of the buoy.

In other cases a short length of chain (or bridle in the case of two mooring eyes) may be used to absorb any chafe.

Cutting by trawl wires may also be a hazard in some areas where commercial fishing takes place. It may be possible to utilise chain in the part of the mooring that may be subject to abrasion from trawl wires.

## Rope size

The decision on the size of rope to be used will depend on the load imparted by the buoy, due to wind/wave action and water velocity and the strength necessary to lift the sinker (or anchor).

A rope with strength equal to twice the lifting capacity of the servicing vessel may be a safe guide for the selection of a rope to account for the case when the sinker is buried.

Detailed information will be required from the rope manufacturer regarding the energy absorbing properties and practical working load for the chosen rope.

The method to be used to handle the rope may also influence the size of rope chosen.

## Rope Construction

A great variety of ropes are now available with many different fibre types as well as types of construction. The use of natural fibre ropes for load carrying applications has almost disappeared. Natural fibre ropes have low strength, will have a short life when compared with synthetic fibres and are no longer cheaper than their synthetic counterparts.

Traditional 3-strand rope construction has also largely been superseded by plaited or braided constructions where high strength and long life are the primary requirements.

Fig.12a    3-strand Constructions

This is the oldest and simplest rope construction, consisting of 3 twisted strands laid together. 3 strand ropes are hard wearing and easily spliced.

Fig.12b  Multiplait Constructions

Multiplait rope consists of 8 strands plaited in pairs, each pair passing over one pair and under the next. It is easily spliced and the twisted strands offer good resistance to abrasion.

Fig.12c Braided constructions

This illustration shows a rope with a braided core encased in a braided jacket, 3 strand cores are also used. Variations in jacket and core construction allow ropes to be designed for specific working situations. The highest strengths are possible in this construction but splicing is complex.

Each internal strand of the rope may be constructed in several different ways. Manufacturers informations should be studied in detail.

## Types Of Fibre

Modern rope materials utilise the following fibre groups:

### Nylon

This provides high strength, elastic rope with good shock absorbing qualities.

Some ultimate strength is lost due to water absorption if the rope is permanently immersed in water.

### Polyester

This is widely used to construct high strength, low stretch ropes with good wear resistance and long life.

### Polypropylene

This has been used for cheap, general purpose rope which floats, however recent developments in fibre manufacture and rope construction have resulted in moderate performance ropes, which are considerably cheaper than nylon or polyester.

### Advanced Fibres

These include Aramid fibres (trade name Kevlar) and high modulus polyethylene (HMPE such as

Spectra, Dyneema and Vectran) which have very high strengths associated with very low stretch. HMPE ropes are also buoyant. However, these are very expensive products being approximately three times the cost of nylon or polyester.

Rope identification can be difficult as different manufacturers may use trade names for fibre type rather than generic names.

Some HMPE rope constructions are very easy to splice.

Behaviour of diferent types of fiber under traction

## Mixed Construction

Large ropes, such as those used for ship mooring may be constructed from a mix of fibres to achieve particular performance parameters.

Annex 4 illustrates details of a standard mooring system developped by the canadian coastguards to moore small (less than half meter diameter) buoys in depths up to 30 meters.

## Rope Terminations

### Thimbles

The use of fibre rope rather than wire rope for towing and mooring ships and oil rigs has lead to the development of thimbles which allow ropes carrying very high loads to be shackled to chain or mooring eyes without damaging the rope fibres.

These thimbles completely enclose the rope leaving no unprotected rope surface to chafe against the joining shackle. They may be fabricated from steel tube, made in cast iron, or moulded from high strength plastic. Movement of the rope within the thimble can be further prevented by filling the thimble with a flexible resin system (usually polyurethane), however opinions differ as to the need for this process.

### Splices

The modern rope constructions, i.e. braided and plaited constructions, both allow high strength splices to be made when the rope has been installed around the thimble. It should be noted that detailed splicing information must be obtained from the rope manufacturer and that these instructions have to be followed precisely in order to retain the majority of the rope strength at the splice. Special tools will be needed for splicing braided rope and training of those making the splices in any modern rope construction will be necessary.

The mooring calculation must take into account the reduction in strength resulting from the splices, usually on the order of 10% for correctly made splices.

## Hybrid Rope/Chain Moorings

One of the most successful applications of rope has been for the “riser” ( see’ Riding Chain’ Fig.1) component of deep-water buoy moorings. The lighter weight of the rope component will allow a service standard buoy to be used at stations where the weight of an all chain mooring would sink the standard buoy or the lighter weight of the rope mooring may allow a smaller buoy to be used when compared with the size of buoy that would be required to support the chain mooring (providing daymark size and focal plane height are adequate).

The design of the buoy being used must be carefully examined to ensure that the riding performance of the buoy is adequate if the rope mooring is used. Some buoy designs rely on chain weight to achieve positive stability.

This type of mooring does not work well for shallow water applications.

## Tension Leg Moorings

Rope moorings are particularly suitable for tensioned mooring configurations such as spar buoys and resilient beacons, where the mooring goes directly from the buoy to the sinker and tension in the mooring line holds the buoy upright. The rope being in tension is not in danger of chafing on the seabed or on the buoy. This configuration has the advantage of maintaining the buoy precisely on station (i.e. there is no “swinging circle” as there is with a conventional mooring) but is only practical in areas with little tidal range or current. However, the mooring sinker or anchor will need to be considerably larger than that associated with a conventional catenary mooring.

## Handling Rope Moorings

### Deployment

When compared with chain, rope is light and easy to manually handle. The components for quite large moorings can be moved about onshore or on deck by hand. Moorings can be deployed by flaking (faking) the rope on deck (or in a flakingbox, large version of line throwing gun rope box). The buoy is placed in the water, the sinker and ground chain simply pushed overboard (or released by cutting lashings) and the rope will follow into the water.

The deck must be celan and free from sand and the deck edge must be sufficiently smooth so as not to dammage the rope.

### Recovery

If the mooring is to be lifted for removal or inspection then two areas need special attention:

(i). Any fairlead that the rope runs over must be of sufficient diameter for the rope used, be of the roller type and present no sharp edges

(ii). The winch or capstan must be designed for handling rope and must not allow the rope to slip on the winch drum when under load.

Conventional capstans as used for tensioning mooring warps etc., may be capable of recovering a rope mooring however their tendency to allow the rope to slip on the capstan drum will result in considerable heat being generated at the rope/drum interface which will result in serious damage to the rope. Successful techniques have been developed using large spooling winches where the rope is wound onto a large rotating drum. This technique is limited by the length of rope and hence the number of moorings that can be carried on the drum at any one time.

The preferred method, where a large number of rope moorings are to be handled, is to use a specialised rope hauling winch. These can be installed at the vessel’s deck edge so that the rope can lead directly to the winch without a fairlead being required. The winch consists of an arrangement of large rubber wheels, which grip the rope without causing damage to the surface fibres. The rope usually only passes over a segment of hauling wheel rather than being wrapped around a drum and can thus be placed in, or removed from, the hauling winch as may be necessary. This type of winch placed on the deck edge also has the advantage that there is no rope under load passing across the vessel’s deck, which may present a serious hazard, should the rope break.

Handling loops or shackles can be incorporated in the rope allowing the mooring to be hoisted in sections by the deck crane.

The deep water mooring design used by the French authority ensures that the ground chain is sufficiently long so that as the rope part of the mooring is retrieved the tension in the rope will only be the weight of the ground chain being lifted. The weight of the sinker will not be felt until all the rope has been recovered and the vessel is lifting the chain part of the mooring.

## Safety

It must be noted that the energy stored in the more elastic types of rope when under tension may be considerable and suitable precautions must be taken to ensure that no personnel will be in any area that may be swept by the end of a broken rope.

# ELASTIC MOORINGS

Rope moorings will absorb some of the buoy’s energy due to the elasticity of the rope and prevent this energy being transferred to the sinker. Fully elastic moorings have been developed where all of the buoys’ motion is absorbed by an elastic cord, which forms part of the mooring cable. The Netherlands authority is using elastic cord cables to moor relatively small buoys in areas subject to wave action. The elastic moorings allow the use of a short cable and hence a limited swinging circle.

Elastic mooring of navigation buoys in shallow water

H.P. Joosten, Datawell B.V., Zomerluststraat 4, 2012 LM Haarlem, The Netherlands

S. Hoekstra, Directie Noordzee, Koopmanstraat 1, 2288 BC Rijswijk, The Netherlands

Recent improvements of navigation buoys have made wear of the mooring chain one of the weakest point of the complete system. The use of an elastic mooring for navigation buoys - common practice for over 40 years in the case of wave measuring buoys - constitutes a substantial improvement in life expectation. In the case of shallow water, the severe requirement on the maximum elongation of the elastic mooring is met by the combination of natural rubber and bollard terminals.

Introduction

With increasing life time of navigation buoys, it is the wear of the mooring chain that mainly determines the service interval for these systems. Grinding by sand contributes substantially to this chain wear. The sand grains get between the chain links when the mooring line is slack, and do their damage when it is tightened again. The ongoing wave movement causes the mooring line to be alternately under tension and without tension. Although tides and currents distribute this wear somewhat over the chain, it is better to prevent this wear altogether. A way to reduce the grinding is by keeping the mooring line continuously tight, thus keeping the grains out of the link spacings. This is achieved by putting in an elastic cord in the mooring line.

Problem of shallow water

Designing an elastic cord mooring is a challenge especially in shallow water with relatively large tidal amplitude. In these circumstances, the ratio of the maximum cord length - needed to keep the buoy visible at high tide - and the minimum cord length - needed to keep the line under tension at low tide - is very large. This large ratio constitutes a requirement on the maximum elongation of the elastic cord.

In order to discuss this problem in quantitative terms, we introduce the symbol λ, denoting the ratio of the actual cord length at any moment and the initial cord length when being unstressed.

An elongation of 100% is thus indicated by λ = 2, a 200% elongation by λ = 3, etc.

The maximum elongation of a elastic cord is denoted by λmax. When we indicate the length ratio required by the circumstances as λreq, the problem is reformulated as finding a cord mooring for which λmax > λreq.

Let us consider a typical shallow water situation having a low tide depth of 2.5 m, a tidal cycle of 3.5 m and a maximum current of 1 m/s. In addition, a maximum wave height in the range of a few metres is assumed. In these circumstances, λreq is about 4. Traditional overbraided rubber cords cannot meet this requirement, since their λmax is only 2 [1]. A rubber cord with a loosely woven overbraid, sometimes used in open sea [2], does have a λmax of 4, however, in this case the wear problems are not solved but transferred to the rubber/braid strangles.

Figure 1.  A bare cord of natural rubber terminated by special bollard terminals

Solution

The proper choice in this situation is a bare cord of natural rubber; terminated by special bollard terminals, see Fig. 1. When terminated by the standard Datawell rubber cord terminals, the maximum elongation of the rubber itself is circa 4. However, since the bollard of the terminal holds a certain length of cord - that, when stressed, slips from the bollard and adds up to the cord length -, the effective λmax of the combination of cord and terminals is larger than this 4, and can in fact be as large as 5. The shorter the rubber cord is, the stronger is this positive bollard effect.

The elastic mooring line described above is basically a shortened version of the standard

Datawell rubber cord used for over 40 years to moor (Directional) Waveriders and Wavecs [3]. Basically these buoys are motion sensors. They become wave sensors when enabled to follow the orbital waves movement. For this reason, an elastic cord is incorporated in the mooring line of these buoys. The size of the wave buoys ranges from 0.7 m diameter (spherical) to 2.5 m diameter (discuss shape). Standard lengths of the rubbercord are 15 and 30 m of rubber with a ardness of either 45 or 60 Shore A, depending on size of the buoy and required specification of the wave buoy.

The elegance of the Datawell rubber cord design is its simplicity. It consists of an arbitrary length of rubber and two stainless steel terminals, which are easily mounted and removed from the rubber cord. Since the rubber cord is bare and solid, wear due to sand is principally impossible. Also, no fouling is encountered, since it cannot stick to the surface of the continuously oscillating cord. Finally, the fouling of the stainless steel bollard is prevented by the piece of the rubber cord at the terminal moving on and off the bollard.

The flexible mooring for navigation buoys serves basically the same purpose as the mooring for wave buoys: allowing the buoy to follow as much as possible the wave motion without getting off location. Often the elastic cord has been promoted by suppliers with the argument of shock absorption. However, also in properly chain-moored buoys no shocks show up. The craftsmanship and experience of the local lighthouse authorities help them in determining the minimum chain length long enough to avoid the chain being tightly stretched between buoy and mooring stone.

This minimum chain length will always exceed the length of an optimally designed elastic mooring, making the latter preferable in terms of precise localization. It also suggests that the flexible mooring, though perhaps more expensive per unit length, could well be the most profitable when comparing complete designs. Apart from this reduction of initial expense, it is expected that inspection, maintenance and retrieval will occur less frequently, thus making the elastic mooring the more economical one.

Measurements and Results

The rubber cord mooring has proved itself for the Wave-buoy over the past decades. In order to check the suitability for navigation buoys, the Dutch lighthouse authorities have performed extensive experiments over the past years. With increasing confidence in the performance of the rubber cord mooring, a chain/rubber comparison experiment was performed.

3 graphs

For figures 2AB and 3: mooring force of a 1.8 diameter light buoy. Water depth: minimal3.7m; tidal cycle: 4m. Minimum/maximum is determined over ½ hour periods (1 Hz sampling)

Two identical buoys (1.8 m diameter) are moored close to each other, (minimum water depth 3.7 m, maximum tidal cycle: 4m, maximum current: 2 kn). One buoy is moored using chain (length 20 m, weight: 8.8 kg/m), the other buoy is moored using rubber (length 1.5 m 45 Shore A, standard Datawell terminals). Both buoys had a load cell plus data logger in the mooring line. During one year, the forces in both systems were monitored: every second the force was measured, and the minimum and maximum value of each half hour was stored.

Some typical autumn results are shown in Figures 2ab and 3. The weather in the shown time span is a combination of mild autumn weather, and a severe storm.

All curves show tidal oscillations. In the rubber cord mooring, the force increases with high tide due to extra elongation. In the chain mooring, the force increases since more chain is lifted from the seabed. One can see that the force of the rubber cord is typically much larger than the force of the chain mooring. As expected, the rubber cord is kept under tension, avoiding wear on the shackle, mooring eyes, and terminal connections.

In the first halve of the figure, smooth oscillations in the chain mooring can be observed, whereas in the storm period some peaks loads are observed. These peaks however do not exceed the forces on the rubber cord.

Handling

A typical weight reduction by a factor 10 or 20 compared to a chain mooring makes the handling of a rubber cord mooring system a delight on deck or in store. The rubber cord itself is solid, and made of natural rubber known for its excellent properties. Although life on the sea can be rough, no rubber cord has ever been damaged on a ship.

For safety reasons, Datawell’s policy has always been to have no force on the elastic part of the mooring when retrieving a buoy. The Dutch lighthouse authorities have made the same decision. Handling procedures varying with circumstances have been developed. In the shallow water situation, two methods have been tested: 1) using a ground chain that is picked up by a drag, 2) with barbed hooks welded onto the terminals. The latter method, still under development, is only suited for single point mooring systems.

Often a kind of safety line is used to limit the elongation of the elastic mooring, and suggesting the retrieval of the mooring stone via the safety line. This has the obvious risk that the unknown state of the mooring gets the full load. Although serious accidents are only expected once in a lifetime, the risk should not be run.

Future

In view of the excellent results of the shallow water situation with relatively small navigation buoys, larger navigation buoys on new locations will be moored with Datawell rubber cord moorings. The experience of thousands of Waverider moorings and hundreds of Wavec buoy moorings, in both near shore and open sea, and with breaking waves and large tidal currents, will be used to moor navigation buoys. Various designs with or without safetyline, with 45 or 60 shore rubber and varying diameter of rubber are considered to make the most successful mooring for navigation buoys.

Conclusion

A proven system for Waveriders has shown to be a reliable mooring for navigation buoys in shallow water as well. Inspection of moored buoy show indeed the expected reduced wear of the metal/metal connections. Since the length of the rubber mooring system is much shorter than the length of the chain mooring, the buoy stays closer to its anchoring. An expected advantage is the increase of the inspection intervals, resulting in a reduction of annual inspection costs.

load on the chain. The same model is used to evaluate the mooring forces on a rubber moored

navigation buoy. Comparison of the results explains why in shallow water rubber cords perform better

than chain moorings.

Introduction

Initially mooring a navigation buoy with rubber was faced with scepticism. Experience has shown that chain moorings sometimes break. Replacing it by a stronger, longer or heavier chain makes the mooring survive. Based on this experience it is contra-intuitive that a rubber cord, having only a fraction of the maximum load, can handle the mooring forces equally well or sometimes even better. Now, after some 5 years of applying rubber cords on navigation buoys [1,2,3], this initial scepticism has changed to confidence and willingness to accept a rubber cord as a suitable mooring material. This paper deals with some background considerations concerning the mooring forces in both the chain and the rubber cord moored navigation buoy, and shows that a proper rubber cord mooring limits the mooring load well below the maximum force a rubber cord can handle.

The initial conflict

Basically the mooring of a buoy serves two conflicting purposes. On the one hand it has to

keep the buoy on position, while on the other hand the mooring has to allow the buoy to

follow the dynamics of the waves to a certain degree in order to reduce the mooring forces.

This conflict most clearly shows up when the buoy is picked up by a wave at the moment the

end of the mooring line length is reached. Especially in shallow water, the chain mooring

suddenly comes to its end resulting in enormous peak forces. A rubber cord can elongate, to a

couple of times its original length, smoothing the stop of the buoy and thus avoiding peak

forces. In the next sections of this presentation the results of a numerical simulation of a chain

and rubber moored navigation buoy are presented. Finally, in the last section this topic is

presented on the basis of the concept of conservation of energy.

Description of the moored navigation buoy at sea

In order to make the results as transparent as possible, the sea, the buoy and the mooring are

reduced to their most essential features: The buoy is modelled as a single-pointed moored

buoy which keeps perfectly upright at all times. A train of breaking waves is modelled as a

Figure 1:A train of breaking waves

Figure 2: The horizontal force on a buoy as a function the displacement _X.

wall of water having a width equal to its height. The breaking waves are separated by still

water of three times the waves width (figure 1).

Although the chain mooring is at least two dimensional, it is cast into a one-dimensional

model. This is done by calculating the horizontal-force/displacement relation of a chain

moored buoy. The horizontal force of the chain on the buoy is calculated as a function of the

displacement (_X) from the zero-velocity position (see figure 2). In equilibrium this force

Figure 3: The horizontal force on a buoy versus the displacement of the buoy from its zero-current position .

equals the drag force on the buoy. In a dynamic situation this force also accelerates or decelerates the buoy. This chain-force/displacement relation depends on the specific weight (weight per meter), the length of the chain and last but not least the water depth. In figure 3 the horizontal-force versus displacement is presented for various values of the specific chain

weight. As expected, the lighter the chain is, the sharper the edge in the force-displacement

relation.

The impact of the varying specific chain weight can be shown by modeling various sizes of chain in a

mooring. The buoy, the sea and the mooring are defined as follows:

The specifications of the buoy are:

Underwater surface area: 1 m2

Total surface area: 2 m2

Mass incl. added mass: 300 Kg

Mass incl. added mass when flushed over: 400 Kg

Drag coefficient: 1

The specifications of the sea are:

Water depth: 7 m

Water velocity of the breaking wave: 8.3 m/s

Water velocity in between the breaking wave: 0 m/s

Height of the breaking wave: 7 m

Distance between two breaking waves: 21 m

The specifications of the chain mooring are:

Length 50 m

Specific mass (in water) 14, 17, 20, 23, 26, 29 Kg/m

When the breaking wave hits the buoy, the buoy is completely flushed over. Tilting of the

buoy is not taken into account. The inertia of the chain itself is neglected, as is the drag of the

chain. The velocity of the water in a (breaking) wave is determined by physics and depends,

in shallow water, on the water depth. The standard water depth in this numerical study is 7 m.

For reasons of compatibility, the velocity of the water in the breaking wave is considered

constant throughout this study independent of the actual water depth.

The mooring force as a function of time when the breaking waves pass the buoy is presented

in figure 4. Figure 5 zooms in on the observed peak forces. For all chains, the mooring force

comes to a steady level, equal to the drag force on a steady buoy. The way this steady state

force is reached depends very much on the specific weight of the chain. As can be seen in

figure 5, with the most heavy chain the force smoothly increases until the equilibrium force is

reached. The light-weight chains on the other hand do not slow down the buoy sufficiently

when the end of the chain length is reached, and the mooring force peaks to approximately 10

times the steady state value. Here one can see what the problem is with light-weight chain:

Light-weight chain is not heavy enough to keep the mooring forces small. In that case the

mooring forces can reach very high peak values, with all the undesired possible consequences.

Figure 4:                                                                                       Figure 5:

The horizontal force on a buoy                                                     The horizontal force on a buoy

while a breaking wave passes                                                     while a breaking wave passes.

Shallow water

In the previous section the mooring design was completely in our hand. With a proper choice

of the chain, both in specific weight and in length, the mooring forces could be kept low. In

the case of shallow water the key parameter is the precise water depth and this is not

controlled by man. From the configurations described in the previous section, the chain with a

specific weight of 20 Kg/m is chosen, and the water level is taken 3, 4, 5, 6 and 7 m

respectively. The mooring forces due to a passing breaking wave are presented in figure 6. At

t = 0 the front of the breaking wave is at the buoy’s equilibrium position in case of zero water

velocity. With decreasing water level from 7 to 4 m, the maximum force increases dramatically. At 3 m the peak force increases to over 10 times the

static force of the braking wave. The impact of this observation is that in an occasional case of

lower water level than expected the mooring design is not adequate anymore and can fail.

Figure 6:The horizontal force on a buoy while a breaking wave passes.

Rubber moored navigation buoy

The sharp peaks in the mooring force in shallow water under breaking wave conditions can be

avoided by applying a rubber cord in the mooring. When the buoy is picked up by the wave,

and the mooring line reaches its end, the buoy has to be slowed down. The rubber cord will do

this in a gradual manner, thus avoiding high peaks in the mooring force. In order to demonstrate this, the numerical simulation presented in the previous section has been

performed with rubber cord, instead of chain. The used rubber has a diameter of 50 mm, and a

hardness of 60 Shore A.

Figure 7: The force on a rubber cord as a function of the relative length.

Figure 8:The mooring force on a buoy moored with a rubber cord while breaking waves pass.

Rubber cord length: 2 cm 8 cm 32 cm 1.3 m 5.1 m.

The force elongation relation of the used rubber is presented in figure 7. Elongation is

presented as a percentage, 100 % corresponds to an un-stretched condition, at 200 % the

rubber cord is doubled in length. For varying length of the rubber cord the simulation has

been performed, see figure 8. In figure 9 is zoomed in on the second breaking wave pass. The

peaked mooring forces are due to the simulation with an unrealistically short rubber cord of

only 2 cm! With increasing length of the rubber cord the peak immediately disappears.

Further increase of the rubber cord length results in a reduction of the maximum force to a

level below the steady state level. In those cases the buoy can move along with the wave

during the whole passage and is pulled back during the time in-between two breaking waves.

Figure 9: The mooring force on a buoy moored with a rubber cord while breaking waves pass.

Rubber cord length: 2 cm 8 cm 32 cm 1.3 m 5.1 m.

With the first breaking wave passing the buoy, the maximum forces are not as high as during

the sequential waves. At t = 0 the rubber cord is in the un-stretched condition, and the front of

the breaking wave just tips at the buoy. When the breaking wave passes the buoy, it is

displaced in the positive direction. After this passage the buoy is pulled back by the mooring.

At the time the front of the second wave hits the buoy, it may be on the left or right side of the

original position point. With this new starting position the second pass will be slightly

different.

Energy considerations

The applicability of the chain and rubber mooring can be considered from an energy point of

view. When the buoy is picked up by a (breaking) wave it will gain kinetic energy. This

energy has to be absorbed by the mooring. A chain mooring absorbs this energy by lifting up

chain. The kinetic energy is transferred to potential energy. The rubber mooring transfers the

kinetic energy into internal (elastic) energy. Here the difference shows up, the energy a chain

mooring can absorb depends on, among others, the water depth. The energy a rubber cord can

absorb only depends on the cord itself. Thus, in shallow water, elastic mooring has the

greatest asset.

Conclusion

By numerical analysis it is shown that a buoy which is not adequately moored may suffer

from high force peaks in the mooring line. The quality of the chain mooring depends on the

length and specific weight of the chain and the water depth. If the chain is too light in weight,

or not sufficiently long, peak forces can increase dramatically. The quality of the rubber

moored buoy only depends on the rubber cord itself. Peak forces can be avoided

independently of the water depth, making the rubber cord mooring superior in shallow water

situations.

References

[1] Elastic mooring of navigation buoys in shallow water; by H.P. Joosten and S.

Hoekstra; in “Guidelines on Synthetic Mooring Lines”, December 2001 annex A; Also

available at www.datawell.nl

[2] Mooring buoys with natural rubber; by H.P. Joosten and S. Hoekstra; in “Newsletter

of the Rubber Foundation Information Center for Natural Rubber”/No. 32, 4th quarter 2003.

Also available at www.datawell.nl

[3] Evaluation “Project Onderhoudsvriendelijke Markering” Rijkswaterstaat, Directie

Noordzee, by Sipke Hoekstra, 28 Mei 2004, Translated into English, March 2005.

# Appendix 1: Chain mooring design

# Appendix 2: example of a mooring design

Chosen situation

This numerical application has been carried out for a Bouée Nouvelle generation A8L. The aerodynamic and hydrodynamic drags have been calculated as per section 3.2.1. ‘Mooring Loads’.

Environmental conditions for the example

Buoy: New Generation Buoy (BNG) A8L

Depth: 20 m

Tide: 6,9 m

Maximum wave height: 5 m

Maximum wind speed:180 km/h

Maximum current speed: 2 knots

Chain diameter: 35 mm

Chain type: 3D

sinker density: 2100 Kg/m³

Steel density: 7850 Kg/m³

Steel tensile strength: 500 N/mm²

Air density: 1,29 Kg/m³

Water density: 1024 Kg/m³

Internal Friction angle of the seabed: 45°

A8L new generation buoy parameters

Tension on the sinker--- See 3.3.1. Taut mooring Loads

The figure bellow shows the vertical tension on the sinker depending on the length of chain.

Vertical tension on the sinker

Maximum « depth »

mooring

tangential

We can note that the vertical tension is equal to zero for slack and transitional moorings

If the mooring is shorter, the vertical tension on the sinker will be increasing.

If the length of the mooring is equal to the depth, the tension on the sinker will be very high, in reality the buoy will be submerged before this can occur.

#### Tension on the buoy-----See 3.3.3. Reserve Buoyancy formulae

mooring tangential

In the case of a tangential mooring (slack or transitional) the tension on the buoy is only due to the weight of supported chain, whereas if the length of chain is less than the recommended length for the transitional mooring, the gain in chain weight is not enough to compensate the extra effort due to the tension on the sinker.

#### Tension on the mooring line----see 3.3.2.  Chain Size

The graph above shows that the "new" chain strength may not be a problem for the mooring as long as the chain length is not too close to the "depth". It follows that if you have a taut mooring, you may not have to choose a higher strength chain. However, the taut mooring can have an impact on chain wear. As the chain is continuously in tension, the rate of wear will be faster with this type of mooring.

#### Recommended mass on the sinker------See 3.4.  Sinker

The graph above illustrates that taut moorings require heavier sinkers.

#### Reserve buoyancy of the buoy

Similarly, reducing the length of chain cause a reduction of the reserve buoyancy that has to be taken account at the design stage. In some cases, the choice of a larger buoy may be necessary.

# Appendix 3: concrete sinkers characteristics

Sinker with a square base (easier to produce, but not recommended for fast currents sites)

| M | Mass of sinker | 500 kg | 1000 kg | 2000 kg | 3000kg | 4000kg | 6000kg |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | Side of the large base | 100 cm | 125 cm | 155 cm | 175 cm | 190 cm | 215 cm |
| B | Side of the narrow base (=2A/3) | 66.7 cm | 73.3 cm | 86.7 cm | 106.7 cm | 133.3 cm | 153.3 cm |
| C | Height of sinker (=A/3) | 33.3 cm | 41.7 cm | 51.7 cm | 58.3 cm | 63.3 cm | 71.7 cm |
| D | Base of the suction hole (=22A/30) | 73.3 cm | 91.7 cm | 113.7 cm | 128.3 cm | 139.3 cm | 157.7 cm |
| E | Summit of the suction hole (=21A/30) | 70 cm | 87.5 cm | 108.5 cm | 122.5 cm | 133 cm | 150.5 cm |
| F | Height of the suction hole (=A/30) | 3.3 cm | 4.2 cm | 5.2 cm | 5.8 cm | 6.3 cm | 7.2 cm |
| G | Length of reentrant for shackle (=9ф) | 36 cm | 45 cm | 54 cm | 54 cm | 54 cm | 54 cm |
| H | Width of reentrant for shackle (=3 ф) | 12 cm | 15 cm | 18 cm | 18 cm | 18 cm | 18 cm |
| I | Depth of of reentrant for shackle (=2 ф) | 8 cm | 10 cm | 12 cm | 12 cm | 12 cm | 12 cm |
| J | Height of reentrant for storage (=2.5 ф) | 10 cm | 12.5 cm | 15 cm | 15 cm | 15 cm | 15 cm |
| K | Length of reentrant for storage (=8 ф) | 32 cm | 40 cm | 48 cm | 48 cm | 48 cm | 48 cm |
| L | Width of reentrant for storage (=2 ф) | 8 cm | 10 cm | 12 cm | 12 cm | 12 cm | 12 cm |
| ф | Diameter of mooring eye | 40 mm | 50 mm | 60 mm | 60 mm | 60 mm | 60 mm |
|  | Mass of incorporated steel (incl. Scrap Chain and mooring eye) | 50 kg | 100 kg | 300 kg | 500 kg | 800 kg | 1400 kg |
| P | weight of sinker in water | 2500 N | 4000 N | 6500 N | 13000 N | 26500 N | 39000 N |

Sinker with a circular base (more dificult to produce, but suitable for fast currents sites)

| M | Mass of sinker | 400 kg | 600 kg | 1000 kg | 2000kg | 4000kg | 6000kg |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | Base diamter | 100 cm | 110 cm | 130 cm | 160 cm | 200 cm | 230 cm |
| B | Upper diameter (=2A/3) | 66.7 cm | 73.3 cm | 86.7 cm | 106.7 cm | 133.3 cm | 153.3 cm |
| C | Height of sinker (=A/3) | 33.3 cm | 36.7 cm | 43.3 cm | 53.3 cm | 66.7 cm | 76.7 cm |
| D | Large diameter of the suction hole (=22A/30) | 73.3 cm | 80.7 cm | 100.3 cm | 122.3 cm | 151.7 cm | 173.7 cm |
| E | Narrow diameter of the suction hole (=21A/30) | 70 cm | 77 cm | 91.7 cm | 111.7 cm | 138.3 cm | 158.3 cm |
| F | Height of the suction hole (=A/30) | 3.3 cm | 3.7 cm | 4.3 cm | 5.3 cm | 6.7 cm | 7.7 cm |
| G | Length of reentrant for shackle (=9ф) | 36 cm | 36 cm | 45 cm | 54 cm | 54 cm | 54 cm |
| H | Width of reentrant for shackle (=3 ф) | 12 cm | 12 cm | 15 cm | 18 cm | 18 cm | 18 cm |
| I | Depth of of reentrant for shackle (=2 ф) | 8 cm | 8 cm | 10 cm | 12 cm | 12 cm | 12 cm |
| J | Height of reentrant for storage (=2.5 ф) | 10 cm | 10 cm | 12.5 cm | 15 cm | 15 cm | 15 cm |
| K | Upper diameter of reentrant for storage | 20 cm | 20 cm | 20 cm | 20 cm | 20 cm | 20 cm |
| L | Lower dia. of reentrant for storage(=K+2J) | 40 cm | 40 cm | 45 cm | 50 cm | 50 cm | 50 cm |
|  | Mooring eye diameter | 40 mm | 40 mm | 50 mm | 60 mm | 60 mm | 60 mm |
|  | Mass of incorporated steel (incl. Scrap Chain and mooring eye) | 50 Kg | 150 Kg | 250 Kg | 600 Kg | 1200 Kg | 1600 Kg |
| P | weight of sinker in water | 2500 N | 4000 N | 6500 N | 13000 N | 26500 N | 39000 N |

# Appendix 4: Illustrated details of Canadian Coast Guards rope mooring system

Diagram provided by Georges Cossette via e-mail in May 08

(this is in visio – and is parked at

\\Newfilesrv\partage\COMMITTEE\EEP\EEP12

And titled

Recommeded Syntehtic Mooring small V0.2 Vsd