AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Guideline No. XXXX

On

Illumination

Of

Structures

Draft

Spring 2008

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      	Internet -  http://iala-aism.org

Table of Contents

Table of Contents	2

1	Introduction	3

2	MAIN Applications	3

2.1	Lighthouse and minor lights	3

2.2	Beacons and marks	3

2.3	Obstacles, breakwater, rocks, etc	4

3	Design	4

3.1	Type of light sources	4

3.1.1	Lamps and housing	4

3.2	Intensity ( illumination level)	4

3.2.1	Light absorption and reflectivity of the surfaces	4

3.3	Location of light sources	5

3.4	Examples on Conventional constructions	5

3.4.1	Light houses and beacons	5

3.4.2	Marks:	5

4	Further consideration	6

4.1	Alternate solutions	6

4.1.1	Pipe light LED-floodlighting	6

4.1.2	Extended light sources	7

4.2	Flood lighting on demand	8

5	References	8

# Introduction

This guideline does not address architectural flood-lighting, instead it focus on navigational aspects. Flood-lighting can be an important AtoN function. It can help the user to improve his location visually, also allow him to have access to information noted on the AtoN, eg marking.

What

Applicable to fix or floating aids, generally less than 1nm

Why

To transfer the daytime information visibility to the night time condition.

To estimate the distance between the object and an observer

To improve the conspicuity between two atons with similar characters

Where

Narrow and restricted channels or high lighted background

How

Floodlighting, backlighting or contour lighting

Definition:

Floodlighting: Technique based on projectors used to illuminate from a variable distances, structures or objects….

Backlighting: where lights source is behind a screen or transparent panel to improve the conspicuity of a structure …

Contour lighting: Strips of lights are used to profile the shape of a structure or an object to be seen….

Limitations. Where and why would you use floodlighting. Managerial decision making aid. Benefits – spatial awareness, perception of distance and bearing. Confined waters application.

Japan contour lighted floating aid

fig. 1

Sweden floodlighted lighthouse

fig. 2

titles to pictures.

# MAIN Applications

## Lighthouse,minor lights and piles of bridges

Floodlighted lighthouses are today an appreciated feature, some times even regarded as more important than the traditional flashing light of the lighthouse. The floodlight has to be considered as a complement to the main AtoN light.

It helps the mariner to keep track of the bearing to the lighthouse in the characters dark period by have an permanent illuminated object to look at.

It makes the aton more recognisable  (It helps the mariner to confirm he is looking at the right aton)

Additional information to the existing signage to approximate the distance to the object eg bridges’s piles

## Beacons and marks

For this application fixed or flashing lights are not applicable. By floodlight a beacon or a mark the mariner will be able to have a better appreciation of the distance and the direction to the AtoN.

Picture to be added from Germany

Example of a floodlighted mark

## Obstacles: breakwater, rocks, etc

For this application, traditional way of marking e.g. lantern or beacon on the location is not practically or economically feasible.

Picture coming from Japan

picture

# Design

Flood-lighted, back-lighted or contour lighted objects are normally used within generally a distance up to 1 NM.  Beyond this range, the flood-lighted or back-lighted structure tends to become more like a point source and the luminance becomes too low to be detected.. It has to be kept as a lighted area giving a minimum of 3 minutes (0.05 degree) as stated by  ????……see  physiological behaviour of the eyes)

For the contoured lighted structure it is a little bit different because of the two dimensional effect which is difficult to address because there is no reference book  ????  Road signs reference?

## Type of light sources

### Lamps and housing

General considerations:

- No direct light to the eyes of the mariner

- Avoid or minimise any direct light on the surface of the water

- Any glass used on the housing must not allow to deviate the light directly in the direction of the eyes of the mariner. A flat glass inside the lamp housing is recommended;

Lamp:

- efficiency (Lum/w), Power consumption, Life, Colour rendition, spectral distribution, intensity

Housing:

-    divergence of the light, non corrosive material, weight, bird repellent, water immersion protection, easy maintainability,   shock resistant.

If the colour of the surface has to be distinguished, one has to take in to account the chromaticity of the light source.

## Luminance ( illumination level)

Question for optic specialists : What is at the eyes of the mariner the minimum intensity to detect and recognise an illuminated object given the highly illuminated background ….20 Microlux (IALA E200-2)????: Uniform intensity with a ratio of 1 to 5 is  applicable ?

Possibly make 2 sections – one for illuminating surfaces and one for contour lighting.

Sufficient number of light sources should be used to ensure an adequate luminance, a uniform light distribution and a satisfactory redundancy. Luminance levels from 5 to 100 Cd/sq m are commonly used in architectural floodlight applications. 5lux to 50 lux for architectural applications.

### Light absorption and reflectivity of the surfaces

Help ………

(Lighting Handbook?)

#### Type of reflectivity. (Lambertian or Retroreflection)

When light strikes a rough or granular surface, it bounces off in all directions due to the microscopic irregularities of the interface. The exact form of the reflection depends on the structure of the surface. One common model for diffuse reflection is Lambertian reflectance, in which the light is reflected with equal luminance in all directions, as defined by Lambert's cosine law.

Some surfaces exhibit retro reflection. The structure of these surfaces is such that light is returned in the direction from which it came. If illumination is used in combination with a retro reflection surface the position of the light is very important but there will be no significant gain from the retro reflective material.

## Location of light sources

Position ideally all light sources perpendicular to the water surface to avoid reflection or direct light in the direction of mariners. The lower the angle of light is, the higher the probability of direct light or reflection that could blind mariners.

Avoid any interference with the main signal light

## Examples on Conventional constructions

### Light houses and beacons

The light source in these constructions is Seald Beam lamp PAR 36 SB4589 28V 50W. To prolong the lifetime of the lamps they can be runned on a lower voltage, 22V. Normal use is 4 lamps on a beacon (2 m in diameter) and abt. 8–12 lamps on our bigger caisson light houses, i.e.total illumination power 200– 600W.

Picture ?

### Marks:

Marks consist in the most cases of a plastic cone, which is illuminated from the top downwards. When yellow light is needed Low Pressure Sodium lamp is used alt. a Mercury Vapour lamp when white light is needed.

### Back-lighted structure

Backlighted structure is typically made of transparent material such as fiberglass or glass blocks.

Picture of lighthouse from Japan

3.4.4. Contour lighted structure

In this particular case, strips are positioned on the edges of the structure

Picture of a building from ……

# Further consideration

A large number of lighthouses are externally floodlighted with projected indirect reflection illumination. This important feature rely on mains supply due to the power required, typical 250W continuous 14h/day, the economical implications for new installations are limiting. Also the service and repair of the necessary underwater cables takes its increasing toll of budget.

Floodlighted lighthouses are today a appreciated feature by the mariners, some times regarded as more important than the traditional flashing light. Repeat of text.

Installations connected to main power sources by sea cables mean high investments and maintenance cost and therefore alternative installations which can run on solar panel would be preferable.

## Alternate solutions

### Pipe light LED-floodlighting

An LED pipe light is designed to solve problems connected to traditional AtoN floodlights with need of network electricity, high power consumption and maintenance.

Fig 4, 5, 6, 7 shows the first main installation of LED-floodlighting on an

inaccessible lighthouse in the busy approach to Gothenburg, Sweden.

Fig 8, 9, 10, 11 illustrate the individual components.

Fig 12 is a draft drawing of the type of beacons erected, in the new

Gothenburg approach channel. All fitted with this type of PV-LED-floodlighting

and PV-GPS-synchronised LED-lanterns.

check if this is in Guide on light sources.

### Extended light sources

The conventional flood lights are normally fitted with ordinary lights for mercury or sodium vapour lamps to illuminate the mark. The drawback of this method is the confusion of disturbing background lighting from roads and bridges, especially at port entrances or other heavily populated areas

To increase the conspicuity in areas with disturbing background lighting as for example at break water lights, an additional feature is needed.

The outcome is a lighting pole with an illuminated area of 200 x 6000mm

= 1.2 m2. The pole is an 8m steel framework hot-dip galvanised lighting mast. The

light consists of 1800 LED`s. The power consumption is 150W at 220V AC.

The distinguishing factor for these light poles is the use of lateral colours red/green

and the distinct vertical length. In a fairway many red/green fixed or flashing lights

can be observed, but few that stick out like an illuminated sore thumb.

For daytime use the pole is fitted vertical narrow day-mark boards at each side of the pole (see fig 5). The boards are lined with fluorescent sheeting red or green.

The calculated service life is at least 8 years and no maintenance is needed.

Due to the LED-technique it’s possible to flash the whole lighted area, which

further increase the conspicuity with respect to background light. This feature is

used particular in leading line applications with character: Rear Oc and Front Q in

order to avoid dangerous mirror effects. This application is suitable as leading lights in port area with its special appearance. The range of visibility is at least 2M

## Flood lighting on demand

Flood lighting is very energy demanding and has to be runned by the mains supply. Some installations, which by remote control are switched on when needed can be runned by PV.

# References

Any ?